3GPP TSG RAN WG1 Meeting #97	R1-1907219

Reno, USA, May 13th – 17th, 2019

Source:	Sharp

Title:	Synchronization mechanism for NR sidelink

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In RAN1#96bis, S-SSB structure and SLSS sequence design were mainly discussed for NR sidelink synchronization mechanism, with the following agreements [1]:

Agreements: - RAN1 will consider the design of NR SLSS where - S-PSS of LTE and NR candidates have similar PAPR. - S-SSS of LTE and NR candidates have similar PAPR. - SLSS design should take into consideration PAPR difference between S-PSS and S-SSS. Agreements: - In NR V2X, S-SSB bandwidth is 11RBs. - PSBCH spans 11RBs. - The S-SSB is designed following combination 1. - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS - Two symbols are used for each of S-PSS and S-SSS, respectively. Agreements: - For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC.

In this document, we mainly discuss our views on several issues for NR sidelink synchronization mechanism.

This document is a revision of R1-1905394 [2].

Discussion

S-SSB transmission

In LTE V2X, UE transmits SLSS/PSBCH either when E-UTRAN configures it to do so by dedicated signalling (i.e. network based), or when not configured by dedicated signalling (i.e. UE based) and E-UTRAN broadcasts for the in-coverage case or pre-configures a threshold for the out-of-coverage case. When not configured for SLSS/PSBCH transmission, UE transmits SLSS/PSBCH if measured RSRP is lower than (pre-) configured threshold. From our perspective, we do not see the need to change condition of SLSS/PSBCH transmission for NR sidelink. Hence, it’s natural to maintain the same condition as in LTE V2X.

Proposal 1: Reuse the condition of SLSS/PSBCH transmission in LTE V2X for S-SSB transmission in NR sidelink, which includes:

- Network configuration.

- Measured RSRP compared with (pre-) configured threshold.

PSBCH contents

Considering in LTE V2X, if UE is in-coverage on the frequency for sidelink operation and TDD configuration is included in SIB1, the UE shall set TDD configuration information in LTE V2X MIB the same meaning as included in the received SIB1. As discussed above, e.g. when the network configures UE to transmit sidelink synchronization information in LTE V2X, the main purpose is to extend the coverage of the cell. UE which receives the transmitted SLSS/PSBCH shall obtain the same timing information and TDD configuration. In our point of view, it is natural to adopt the same mechanism in NR sidelink, or at least for the case of in coverage, TDD configuration information conveyed in sidelink system information does not violate the cell-specific TDD configuration. For the case of out-of-coverage, UE may either set TDD configuration as the SyncRef UE (if UE has selected sync source UE), or set as the pre-configured TDD configuration (if GNSS as sync source or UE has selected no sync source).

Proposal 2: For the case when UE is in coverage, at least TDD configuration information in PSBCH does not violate cell-specific TDD-UL-DL-ConfigCommon in SIB1.

In NR Rel-15, cell-specific IE: TDD-UL-DL-ConfigCommon in SIB1 includes at most 2 configuration patterns concatenated to ensure flexibility which takes more than 50 bits payload in total. It was already agreed that PSBCH payload is 56bits including CRC for evaluation purpose. Naturally, methods to reduce overhead on TDD configuration information should be studied, i.e. indicating only UL slots or sidelink available slots in sidelink MIB.

Proposal 3: FFS methods to reduce overhead of TDD configuration information in NR sidelink MIB.

In the frequency domain, it may be necessary to indicate an initial resource pool via PSBCH such that UEs that may communicate with each other can exchange physical layer broadcast messages in that resource pool e.g. in order for the higher layers to initiate unicast or groupcast sessions. Other mechanisms/triggering conditions should allow UEs to switch to other resource pools when the unicast or groupcast sessions are established.

Proposal 4: NR PSBCH indicates an initial resource pool within the (pre-) configured SL BWP in the NR SL carrier.

Synchronization references

The following working assumptions made during the SI phase are related to synchronization references [3],

RAN1#96 agreements:  Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured. o The following table is a working assumption GNSS-based synchronization; gNB/eNB-based synchronization. • P0: GNSS • P1: the following UE has the same priority: • UE directly synchronized to GNSS • P2: the following UE has the same priority: • UE indirectly synchronized to GNSS • P3: the remaining UEs have the lowest priority.; • P0: gNB/eNB • P1’: UE directly synchronized to gNB/eNB • P2’: UE indirectly synchronized to gNB/eNB • P3’: GNSS • P4’: UE directly synchronized to GNSS • P5’: UE indirectly synchronized to GNSS • P6’: the remaining UEs have the lowest priority..

We are in principle fine with confirming the working assumption, with only one comment on synchronization references for GNSS-based synchronization. In our view, in case a gNB/eNB is also synchronized to GNSS (e.g. a UE can derive the same DFN from the gNB/eNB and from GNSS) there is no reason why that gNB/eNB is excluded from being a synchronization reference. Hence we propose to confirm the working assumption with the addition of “gNB/eNB synchronized to GNSS” to priority level P1.

Proposal 5: Confirm the following working assumption with changes highlighted in yellow:

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| • P0: GNSS • P1: the following UE has the same priority: • gNB/eNB synchronized to GNSS • UE directly synchronized to GNSS • P2: the following UE has the same priority: • UE indirectly synchronized to GNSS • P3: the remaining UEs have the lowest priority. | • P0: gNB/eNB • P1’: UE directly synchronized to gNB/eNB • P2’: UE indirectly synchronized to gNB/eNB • P3’: GNSS • P4’: UE directly synchronized to GNSS • P5’: UE indirectly synchronized to GNSS • P6’: the remaining UEs have the lowest priority. |

Conclusion

In this contribution, we discuss several issues on synchronization mechanism in NR sidelink, and make the following proposals:

Proposal 1: Reuse the condition of SLSS/PSBCH transmission in LTE V2X for S-SSB transmission in NR sidelink, which includes:

- Network configuration.

- Measured RSRP compared with (Pre-) configured threshold.

Proposal 2: For the case when UE is in coverage, at least TDD configuration information in PSBCH does not violate cell-specific TDD-UL-DL-ConfigCommon in SIB1.

Proposal 3: FFS methods to reduce overhead of TDD configuration information in NR sidelink MIB.

Proposal 4: NR PSBCH indicates an initial resource pool within the (pre-) configured SL BWP in the NR SL carrier.

Proposal 5: Confirm the following working assumption with changes highlighted in yellow:

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| • P0: GNSS • P1: the following UE has the same priority: • gNB/eNB synchronized to GNSS • UE directly synchronized to GNSS • P2: the following UE has the same priority: • UE indirectly synchronized to GNSS • P3: the remaining UEs have the lowest priority. | • P0: gNB/eNB • P1’: UE directly synchronized to gNB/eNB • P2’: UE indirectly synchronized to gNB/eNB • P3’: GNSS • P4’: UE directly synchronized to GNSS • P5’: UE indirectly synchronized to GNSS • P6’: the remaining UEs have the lowest priority. |

References

Chairman’s notes, 3GPP RAN1#96bis.

R1-1905394, “Synchronization mechanism for NR sidelink”, Sharp, 3GPP RAN1#96bis.

Chairman’s notes, 3GPP RAN1#96.