3GPP TSG RAN WG1 Meeting #97	R1-1907226

Reno, USA, May 13th – May 17th, 2019

Source:	Sharp

Title:	Evaluation results of Uplink spectral efficiency in Indoor Hotspot – eMBB in FR1 for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion

Background

In this contribution, we share our evaluation results of uplink spectral efficiency in Indoor Hotspot – eMBB in FR1, i.e. InH-eMBB configuration A, for IMT-2020 self-evaluation.

Evaluation results

In [1], the minimum requirement for 5th percentile user spectral efficiency and average spectral efficiency are defined, and the requirements are shown in Table 1.

Table 1. Minimum requirement for 5th percentile user spectral efficiency and average spectral efficiency in UL

| Test environment | 5th percentile user spectral efficiency (bps/Hz) | Average spectral efficinency (bps/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.21 | 6.75 |

Table 2 shows evaluation results of UL 5th percentile user spectral efficiency and UL average spectral efficiency in InH-eMBB Configuration A. In this evaluation, we evaluate 12TxRPs case. Evaluation assumptions are shown in Table A1 in the Annex. Other assumptions are aligned with the reference [2].

Table 2. Evaluation results

|  |  | 5th percentile user spectral efficiency (bps/Hz) | Average spectral efficinency (bps/Hz) |
| --- | --- | --- | --- |
| TDD | InH_A | 0.355 | 8.89 |
|  | InH_B | 0.357 | 8.85 |
| FDD | InH_A | 0.406 | 10.14 |
|  | InH_B | 0.407 | 10.09 |

Observation:

NR meets UL 5th percentile user spectral efficiency and UL average spectral efficiency requirements in both TDD and FDD cases under Indoor Hotspot – eMBB test environment in FR1.

Conclusion

In this contribution, we have the following observation:

Observation:

NR meets UL 5th percentile user spectral efficiency and UL average spectral efficiency requirements in both TDD and FDD cases under Indoor Hotspot – eMBB test environment in FR1.

References

- ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, November 2017.

- ITU-R M2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Oct. 2017

- TR 36.873, “Study on 3D channel model for LTE”, v. 12.7.0, January, 2018.

Annex A: Evaluation assumptions

- Table A1. System level evaluation assumptions for UL

|  |  |  | - InH-eMBB Configuration A |
| --- | --- | --- | --- |
| - Channel model |  |  | - InH_A/InH_B |
| - Number of TRxP per site |  |  | - 1 |
| - Multiple access |  |  | - OFDMA |
| - Duplexing |  |  | - TDD/FDD |
| - Network synchronization |  |  | - Synchronized |
| - Frame structure for TDD |  |  | - DDDSU |
| - Modulation |  |  | - Up to 256QAM |
| - Channel coding on PUSCH |  |  | - LDPC with BP decoding - Max code-block size = 8448 bits |
| - Numerology |  |  | - TDD: 30 kHz, FDD: 15 kHz |
| - Simulation bandwidth |  |  | - TDD: 20 MHz, FDD: 10 MHz |
| - UE power class |  |  | - 23 dBm |
| - Antenna configuration at TRxP |  |  | - 32R (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 4, 4) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) |
| - Antenna configuration at UE |  |  | - 4T (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 0, 90 (degree) |
| - Transmission scheme |  |  | - UL SU/MU-MIMO with rank adaptation |
| - SU dimension |  |  | - Up to 2 layers |
| - Codeword to layer mapping |  |  | - For 1~2 layers, CW1 |
| - MU dimension |  |  | - Up to 4 layers |
| - Mechanical tilt |  |  | - 180° in GCS |
| - Electronical tilt |  |  | - 90° in LCS |
| - PRB bundling |  |  | - 4 PRBs |
| - UL codebook |  |  | - NR 4 Tx codebook |
| - Max CBG number |  |  | - 1 |
| - Re-transmission delay |  |  | - Next available UL slot after receiving retransmission indication |
| - Scheduling |  |  | - PF |
| - Outer-loop link adaptation |  |  | - ON |
| - UE speed |  |  | - 3 km/h |
| - Criteria for selection for serving TRxP |  |  | - Maximizing RSRP where the digital beamforming is not considered |
| - Wrapping around method |  |  | - No wrapping around |
| - Inter-site interference modeling |  |  | - Explicitly modeled |
| - Handover margin |  |  | - 1 dB |
| - Power control |  |  | - Open-loop power control: PO = -60 dBm,  = 0.6 |
| - Power backoff model |  |  | - Continuous RB allocation: follow TS 38.101 for FR1 |
| - Receiver |  |  | - Non-ideal MMSE-IRC |
| - Channel estimation |  |  | - Non-ideal |
| - Overhead assumption | - PUCCH | - TDD | - 2 PRBs and 14 OS for the slots without SRS transmission |
|  |  | - FDD | - 2 PRBs and 14 OS for the slots without SRS transmission; 2 PRBs and 12 OS for the slots with SRS |
|  | - DMRS |  | - Type II, 2 symbols (including one additional DMRS symbol), multiplexing with PUSCH |
|  | - SRS |  | - Non-precoded SRS, 4 SRS ports with 4 SRS resources, 2 symbols in every 5 slots |