3GPP TSG RAN WG1 Meeting #97	R1-1907227

Reno, USA, May 13th – 17th, 2019

Source:	Sharp

Title:	Updated evaluation results of mobility in Rural – eMBB for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Background

In this contribution, we share our evaluation results of mobility in Rural Macro – eMBB for IMT-2020 self-evaluation. This document is updated from R1-1902670.

Evaluation results

Mobility evaluation

According to [1], the following evaluation steps have been captured.

The evaluator shall perform the following steps in order to evaluate the mobility requirement. Step 1: Run uplink system-level simulations, identical to those for average spectral efficiency, and 5th percentile user spectral efficiency except for speeds taken from Table 4 of Report ITU-R M.2410-0, using link-level simulations and a link-to-system interface appropriate for these speed values, for the set of selected test environment(s) associated with the candidate RITs/SRITs and collect overall statistics for uplink SINR values, and construct CDF over these values for each test environment. Step 2: Use the CDF for the test environment(s) to save the respective 50th-percentile SINR value. Step 3: Run new uplink link-level simulations for the selected test environment(s) for either NLOS or LOS channel conditions using the associated speeds in Table 4 of Report ITU-R M.2410-0, as input parameters, to obtain link data rate and residual packet error ratio as a function of SINR. The link-level simulation shall use air interface configuration(s) supported by the proposal and take into account retransmission, channel estimation and phase noise impact. Step 4: Compare the uplink spectral efficiency values (link data rate normalized by channel bandwidth) obtained from Step 3 using the associated SINR value obtained from Step 2 for selected test environments, with the corresponding threshold values in the Table 4 of Report ITU-R M.2410-0. Step 5: The proposal fulfils the mobility requirement if the spectral efficiency value is larger than or equal to the corresponding threshold value and if also the residual decoded packet error ratio is less than 1%, for all selected test environments. For the selected test environment it is sufficient if one of the spectral efficiency values (using either NLOS or LOS channel conditions) fulfils the threshold.

We evaluated mobility performances following the defined steps.

Evaluation results

At the RAN1#92bis, the following agreement was made for ZoD spread of a UE because mean value of ZoD spread needs a distance (d2D) between a UE and a gNB and antenna height (hUT) of a UE.

- Use the 50%-tile point of CDF of the mean value of ZoD spread of all UEs based on UE dropping in system level simulation. The mean value of ZoD spread of a UE is calculated according to the formula given in Table 1 and Table 2 for Dense Urban and Rural test environment, respectively, where d2D and hUT of a UE is determined by the UE dropping.

Table 1 shows the mean value of ZoD spread by the system level simulation.

Table 1. 50%ile mean values of ZoD spread

| Mean ZoD | Configuration A |  | Configuration B |  |
| --- | --- | --- | --- | --- |
|  | LOS | NLOS | LOS | NLOS |
| RMa_A | 1.24 | 1.42 | 1.15 | 1.40 |
| RMa_B | 1.21 | 1.38 | 1.16 | 1.36 |

Observation 1:

- For Configuration A

  - Mean values of ZoD spread are 1.24 degrees for LOS and 1.42 degrees for NLOS in RMa_A channel.

  - Mean values of ZoD spread are 1.21 degrees for LOS and 1.38 degrees for NLOS in RMa_B channel.

- For Configuration B

  - Mean values of ZoD spread are 1.15 degrees for LOS and 1.40 degrees for NLOS in RMa_A channel.

  - Mean values of ZoD spread are 1.16 degrees for LOS and 1.36 degrees for NLOS in RMa_A channel.

Figures 1 shows the C.D.F. of UL SINR in RMa_A and RMa_B channel models. Evaluation assumptions are listed in Table A1 in Annex.

Figure 1. C.D.F. of UL SINR

Based on this figure, mean values of UL SINR are listed in Table 2

Table 2. mean values of UL SINR

|  | RMa_A | RMa_B |
| --- | --- | --- |
| Configuration A | 11.3 dB | 12.0 dB |
| Configuration B | 11.8 dB | 11.8 dB |

Observation 2:

- For Configuration A, 50%ile values of UL SINR are 11.3 dB and 12.0 dB in UMa_A and UMa_B respectively.

- For Configuration B, 50%ile values of UL SINR are 11.8 dB and 11.8 dB in UMa_A and UMa_B respectively.

In Table 3, the requirements of the normalized traffic channel link data rates are defined for mobility [3].

Table 3. Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Rural – eMBB | 0.8 | 120 |
|  | 0.45 | 500 |

Tables 4 and 5 shows the link data rate. Link level simulation assumptions are listed in Table A2 in Annex

Table 4. data rate with 120 km/h

|  | RMa_A | RMa_B |
| --- | --- | --- |
| Configuration A | 1.5523 | 1.5876 |
| Configuration B | 1.5700 | 1.5700 |

Table 5. data rate with 500 km/h

|  | RMa_A | RMa_B |
| --- | --- | --- |
| Configuration A | 1.0321 | 1.0447 |
| Configuration B | 1.0534 | 1.0534 |

Observation 3:

- NR meets mobility requirements in Rural Macro – eMBB test environment.

Conclusion

We have the following observations.

Observation 1:

- For Configuration A

  - Mean values of ZoD spread are 1.24 degrees for LOS and 1.42 degrees for NLOS in RMa_A channel.

  - Mean values of ZoD spread are 1.21 degrees for LOS and 1.38 degrees for NLOS in RMa_B channel.

- For Configuration B

  - Mean values of ZoD spread are 1.15 degrees for LOS and 1.40 degrees for NLOS in RMa_A channel.

  - Mean values of ZoD spread are 1.16 degrees for LOS and 1.36 degrees for NLOS in RMa_A channel.

Observation 2:

- For Configuration A, 50%ile values of UL SINR are 11.3 dB and 12.0 dB in UMa_A and UMa_B respectively.

- For Configuration B, 50%ile values of UL SINR are 11.8 dB and 11.8 dB in UMa_A and UMa_B respectively.

Observation 3:

- NR meets mobility requirements in Rural Macro – eMBB test environment.

References

- ITU-R M2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Oct. 2017

- ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, November 2017.

Annex A: Evaluation assumptions

- Table A1. System level evaluation assumptions for uplink

| - Parameters | - Assumptions/Values |
| --- | --- |
| - Deployment scenario | - Rural Macro – eMBB, Config. A, Config. B |
| - Cell layout | - Hexagonal grid layout, 19 sites with 3-sectored TRP |
| - Simulation bandwidth | - 10 MHz |
| - Number of antenna elements per TRP | - (M, N, P, Mg, Ng) = (8, 4, 2, 1, 1) for Config. A, (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) for Config. B, (dH, dV) = (0.5 λ, 0.8 λ), - Polarization: 45, −45 (degree) |
| - Number of TXRUs per TRP | - (Mp, Np, P, Mg, Ng) = (1, 4, 2, 1, 1) for Config. A, (Mp, Np, P, Mg, Ng) = (1, 8, 2, 1, 1) for Config. B |
| - BS antenna gain | - 8 dBi |
| - BS antenna radiation pattern | - Table 9 in [1], θ3dB = 65 (degree), SLAV = 30 (dB) - φ3dB = 65 (degree), Am = 30 (dB) |
| - BS mechanical downtilt | - 90° in GCS |
| - BS electrical downtilt | - 100° in LCS |
| - BS array boresight | - 30°, 150°, 270° - x. |
| - UE antenna height | - 1.5 m |
| - Number of antenna elements per UE | - (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) for Config. A, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) for Config. B, Polarization: 0, 90 (degree) |
| - Number of TXRUs per UE | - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 1) |
| - UE antenna gain | - 0 dBi |
| - UE antenna radiation pattern | - Omni-directional |
| - UE array orientation | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° |
| - Device deployment | - 20 % indoor, 80 % outdoor |
| - Percentage of high loss and low loss building type | - 80% low loss, 20% high loss |
| - UE mobility model | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| - UE speed of interest | - Indoor UE: 3 km/h, Outdoor UE: 30 km/h |
| - UE noise figure | - 7 dB |
| - Channel model | - RMa_A, RMa_B |
| - UE density | - 10 UEs per TRP |
| - Handover margin | - 0 dB |
| - Traffic model | - Full buffer |
| - Scheduler | - Random scheduling |
| - Power control | - Open-loop power control: PO = -80 dBm,  = 0.8 for Config. A, PO = -106 dBm,  = 1 for Config. B |

- Table A2. Link level evaluation assumptions for uplink

| - Parameters | - Assumptions/Values |
| --- | --- |
| - Simulation bandwidth | - 10 MHz |
| - Number of antenna ports at BS | - 4, (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 1) - 8, (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5 λ, N/A), Polarization: 45, −45 (degree) |
| - BS antenna radiation pattern | - Omni directional |
| - Number of antenna ports at UE | - 1, (Mp, Np, P, Mg, Ng) = (1, 1, 1, 1, 1) |
| - UE antenna radiation pattern | - Omni-directional |
| - UE speed | - 120, 500 km/h |
| - Channel model | - CDL-i (NLOS) |
| - Channel coding/decoding | - LDPC with sum-product decoding |
| - Channel estimation | - 2D MMSE channel estimation |
| - DMRS | - 1-symbol type 1 DMRS - 1 additional DMRS symbol(s) |
| - PUSCH length | - 2-symbol |
| - PUCCH RBs | - 4 RBs |