3GPP TSG-RAN WG1 Meeting #97	R1-1907255

May 13th  – May 17th,  2019

Reno, USA

Agenda item:	7.2.1.1

Source: 	Qualcomm Incorporated

Title: 	Channel Structure for Two-Step RACH

Document for:		Discussion and Decision

# 1. 	Introduction

At RAN #82 meeting, two-step RACH was approved as a new WI for NR Rel-16, which involves both RAN1 and RAN2 efforts to specify contention-based random access procedures applicable to both licensed and unlicensed spectrum [1].

In RAN1 #96 and RAN1#96B meetings, the following agreements were made on the channel structure for two-step RACH [2-3]:

Agreements:

- PUSCH occasion for 2-step RACH is defined as the time-frequency resource for PUSCH transmission.

Agreements:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period

Agreements:

PUSCH resource unit for 2-step RACH is defined as the PUSCH occasion and DMRS port/DMRS sequence used for an msgA payload transmission.

  - FFS support only one or both of DMRS port / DMRS sequence.

  - The DMRS sequence generation mechanism should follow Rel.15.

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follows the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following options:

    - Option 1: the numerology for msgA PUSCH follows that of msgA preamble.

    - Option 2: gNB configures whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP.

    - Option 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission.

In this contribution, we will discuss the remaining issues for message A (msgA) and message B (msgB) channel structure design. The procedures related to two-step RACH are discussed in a companion paper [4] and the references therein.

# 2. 	Discussion

## 2.1	Overview of Two-Step RACH

Table 1: Physical Signals and Channels Used by Two-Step RACH

Figure 1:  Example of Two-Step RACH

Table 1 lists the physical signals and channels used by msgA and msgB, as well as the contents of each message type. Figure 1 shows an example of contention-based two-step RACH procedure.

- Before starting two-step RACH, UE receives and processes SSB/SIB/RS from the serving gNB.

  - The configuration information for two-step RACH is carried by system information (SI).

  - For RRC_CONNECTED UE, the configuration information can also be carried by RRC signalling.

- Based on the configuration information for two-step RACH as well as the contents of message A (msgA), UE will select a RACH occasion (RO) [2] and a PUSCH resource unit (PRU) [3], and transmit the preamble and payload on the selected RO and PRU, respectively.

  - To record the number of two-step RACH re-attempts, a counter for msgA retransmissions can be configured on UE side. The maximum value of this counter is configured by the network.

- After receiving msgA, gNB will process the preamble first.

  - If the msgA preamble can be successfully detected, gNB will continue processing the payload of msgA. After that, a message B (msgB) will be transmitted to the two-step RACH UE within the RAR window. The contents of msgB [1, 4, 11] depend on the outcome of msgA processing:

    - If the payload of msgA can be successfully decoded, msgB will include at least the contention resolution information (e.g. a unique UE identifier).

    - If the payload of msgA cannot be decoded, msgB will include at least the RAPID and the UL grant for retransmitting the payload part of msgA.

      - If UE can successfully decode the RAPID and UL grant, it will fall-back to four-step RACH procedure, by re-transmitting the payload part of msgA on the granted resources.

  - If the msgA preamble fails to be detected, gNB will not transmit anything to the two-step RACH UE.

- After msgA is transmitted, a RAR window will be configured on UE side. msgB will be monitored within the RAR window [12]. The behaviours and procedures of UE depend on the outcome of msgB reception [4].

  - Before the RAR window expires, if UE can decode the msgB in response to its latest msgA transmission/re-transmission (or detect the contention resolution information associated with the identity of the UE), the two-step RACH procedure is considered complete with success.

  - When the RAR window expires, if UE does not receive a msgB and the counter of re-transmissions is less than the maximum value, UE will re-transmit its msgA preamble and payload, which is considered as a re-attempt of two-step RACH procedure.

  - When the RAR window expires, if no msgB is received and the counter of msgA re-transmissions has reached its maximum, the two-step RACH procedure is considered as ended with failure.

## 2.2 	Channel Structure for msgA

A general channel structure for msgA is shown in Figure 2, which is applicable to both licensed spectrum (FR1 and FR2) and unlicensed spectrum (FR1).

- The preamble and payload of msgA are TDMed [1], which are transmitted on pre-configured RO and PRU, respectively.

- A transmission gap (TxG) with configurable time duration, say Tg, can be inserted between the preamble and the payload. For low latency use cases, TxG can be waived by setting Tg=0.

- Depending on the payload size, MCS and slot format, the transmission bandwidth for msgA preamble and payload can be the same or different.

- Network has the flexibility to configure separate RO or shared RO for two-step RACH UE and four-step RACH UEs. When separate ROs are configured for two-step RACH and four-step RACH, they can be multiplexed in time/frequency. As a result, the configuration period for msgA PUSCH can be similar to or different from the configuration period of RO.

- To mitigate the ISI and ICI incurred by asynchronous transmission of msgA payload, the inclusion of a guard time (GT) and a guard band (GB) is helpful.

- To support contention-based msgA transmission without a valid TA, timing adjustment for msgA payload can be performed by gNB and/or UE. Examples of msgA transmission timeline, without or with UE assisted timing adjustment, are shown in Figures 3-5, respectively.

Figure 2: General Channel Structure of msgA

Figure 3: Transmission Timeline for msgA without UE Side Timing Adjustment

Figure 4: Transmission Timeline for msgA with UE Side Timing Adjustment for Preamble and Payload

Figure 5: Transmission Timeline for msgA with UE Side Timing Adjustment for Payload Only

Therefore, we have the following observation and proposals:

Observation 1: Two-step RACH can be supported for all cell sizes. Specifically,

- for small cell sizes, Tadj = 0 is sufficient;

- for large cell sizes:

- Tadj = 0 can be used as long as a sufficient gap is allowed for gNB to process the preamble for timing offset estimation and then use that information for PUSCH demodulation;

- Tadj ~= RTT can be used in the case of UE assistance (e.g., tracking timing related changes relative to last valid TA)

- UE-assisted timing adjustment can be applied to both preamble and payload of msgA, or to the payload part only.

Proposal 1:  To support two-step RACH operation in both licensed spectrum (FR1 and FR2) and unlicensed spectrum (FR1), with or without a valid timing advance, the channel structure of msgA should include a preamble, a payload, as well as a transmission gap with configurable duration.

Proposal 2:  Depending on the payload size, MCS and slot format, the transmission bandwidth of msgA preamble and payload can be same or different.

Proposal 3:  Network has the flexibility to configure separate or shared ROs for two-step RACH and four-step RACH. When separate ROs are configured, they can be multiplexed in time and/or frequency domain. The configuration period of msgA PUSCH can be similar to or different from the configuration period of RO.

### 2.2.1	Preamble Transmission of msgA

Table 2:  Parameters of Long PRACH Preambles Specified in NR Rel-15

Table 3:  Parameters of Short PRACH Preambles Specified in NR Rel-15, where μ∈{0,1, 2, 3}

According to the scope and objectives of the WI [1], two-step RACH will be applied to any cell size supported in NR Rel-15, regardless whether the UE has a valid timing advance (TA) or not. Tables 2-3 list the parameters of the long and short PRACH preambles specified in NR Rel-15, respectively [4].

For a RO specified for four-step RACH in NR Rel-15, up to 64 PRACH preambles are shared by contention-based random access (CBRA) and contention-free random access (CFRA). To support both two-step RACH and four-step RACH, separate or shared RO can be configured for two-step RACH. When a RO is shared between different types of CBRA procedures, i.e. two-step RACH and four-step RACH, the pool of preamble sequences can be partitioned into mutually exclusive subsets for different RACH procedures.

Depending on the deployment scenarios and use cases, the preamble and payload of msgA can be transmitted in the same slot or different slots. When msgA preamble and payload are transmitted in different slots, the numerology for msgA PUSCH follows the numerology configured for the UL BWP for msgA transmission. When msgA preamble and payload are transmitted in the same slot, gNB will configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP.

Therefore, we have the following proposal:

Proposal 4:  Two-step RACH should support preamble formats specified in NR Rel-15 for both large cells and small cells. The preamble and payload of msgA can be transmitted in the same slot or different slots. Specifically,

- when msgA preamble and payload are transmitted in different slots, the numerology for msgA PUSCH follows the numerology configured for the UL BWP of msgA transmission;

- when msgA preamble and payload are transmitted in the same slot, gNB will configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP.

### Payload Transmission of msgA

Figure 6 shows a block diagram for msgA transmission. The NR Rel-15 processing mechanism can be re-used for msgA preamble and payload generation, with extension in scrambling ID (sets) generation for PUSCH and DMRS. Such extensions aim to meet the new requirements inherent to two-step RACH design, and minimize the impacts on specifications and implementations.

- Different from msg3 transmission in four-step RACH, neither UL grant nor TC-RNTI will be assigned for a two-step RACH UE. Therefore, a modified RNTI can be used to initialize the bit level scrambling sequence of msgA PUSCH in any RRC state, which can be defined as a function of RA-RNTI and resource index of preamble/DMRS, i.e.

nRNTI≜RA-RNTI+K1×p_id+K2×r_id16 ,

where p_id and r_id denote the resource index of preamble and DMRS, respectively; K1 and K2 are integers common to all two-step RACH UEs, and 16 represents the operation of truncating the inner variable to 16 bits.

- The transport format and resource size of msgA payload can be indicated by the preamble sequence.  In some cases, the MCS and resource allocation for msgA PUSCH can be indicated by UCI. The PUCCH associated with UCI can be generated and multiplexed with PUSCH, based on NR Rel-15 specifications.

- To improve the reliability and reduce the complexity of msgA decoding, the mapping rule between the preamble and the DMRS should be specified. To this end, the DMRS signal generation can be made as a function of the preamble sequence index, as shown in Figure 6. Based on the NR Rel-15 mechanism for DMRS signal generation, the formulas of scrambling IDs (cinit or nIDRS) in DMRS sequence generation can be extended, by incorporating the index of preamble sequence used by the msgA preamble.

Figure 6:  Block Diagram for msgA Transmission

Proposal 5: The mapping rules between msgA preamble and msgA PUSCH resource unit should be specified for two-step RACH. To support the mapping rules in msgA channel structure, NR Rel-15 design mechanisms for DMRS and PUSCH can be re-used for msgA PUSCH resource unit, with extensions in the scrambling ID sets generation. The scrambling ID generation formulas for msgA DMRS and PUSCH should include at least the preamble sequence index of msgA as a parameter.

Based on the use cases of two-step RACH, the contents of msgA can include UL CCCH message, small UP data and MAC CE [1, 8-10]. As a result, for UE in different RRC states, the payload size of msgA can range from a few bytes to a few hundred bytes. To illustrate, Figure 7 shows the spectral efficiency (per UE) for different payload size and PO size. In this example, we assume the DMRS overhead is 1/7, and a PO spans 12 PUSCH symbols in time domain.

Due to the bursty traffic pattern and the nature of contention-based random access, it is inefficient to dedicate one PO to a single UE. To improve the spectral efficiency of two-step RACH, one PO can be shared by UEs with similar payload size, MCS and waveform. To accommodate different payload size, MCS and waveform, multiple POs can be configured within a PO configuration period, which can be configured with different TBS/MCS/waveform. When multiple ROs and POs are configured within a transmission period of msgA, the preamble and DMRS sequences from multiple ROs and POs can be aggregated into larger pools of size M and Q, respectively. The preamble and DMRS sequences within the aggregated pool can be ordered and indexed. Figure 8 shows the mapping relationship between indexed preamble sequence (Sm) and DMRS sequence (Dq), which can be one-to-one (M=Q), one-to-many (Q=ML, L>1) and many-to-one (M=QK, K>1). Moreover, Figure 9 shows the average collision probability of preamble/DMRS based on random selection, which indicates the probability of collision is determined by the smaller pool size of preamble and DMRS sequences.

Figure 7:  Resource Size Considerations for Different Payload Size

To summarize, we have the following observation and proposals on the configuration and indication of waveform/payload size/MCS for msgA payload transmission:

Observation 2:  Two-step RACH can operate in any RRC state. Different payload size, waveform and MCS can be specified for different RRC states and mapped to different POs.

Proposal 6: For msgA transmission, the following designs should be supported:

- a minimum payload size of 7 or 9 bytes can be supported by DFT-s-OFDM waveform and low MCS; payload sizes up to a few hundred bytes can be supported by higher MCS;

- multiple POs can be configured per PO configuration period, to accommodate different MCS, payload size and waveform;

- multiple two-step RACH UEs can share the same PO, if their msgA transmissions use similar payload size/MCS/waveform;

- resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain;

- the configurations and resource allocations for RO and PO need to be balanced to achieve a good trade-off in resource utilization efficiency and collision probability reduction.

- the mapping rules between msgA preamble sequence and msgA PUSCH resource unit can be indicated by SI or RRC signalling.

Figure 8: Preamble Pool (S) to DMRS Pool (D) Mapping

Figure 9:  Collision Probability of Preamble or DMRS Random Selection for msgA (CBRA)

### LLS Results for msgA

Figure 10 shows the BLER performance of msgA payload, for small payload size of 72 bits and TDL-C (300ns) channel model. 1 TX & 4 RX antennas configuration is considered, and realistic channel estimation is performed. Various number of PRBs and UEs are evaluated. The details of the LLS assumptions can be found in the Appendix.

#### 2.2.3.1	Resource Allocation and UE Multiplexing for Small Payload Size

Figure 10: msgA BLER performance for TBS=72 bits with 1, 2 UEs

As shown by Figure 10, for small payload size of 72 bits, both single UE and 2 UE cases show similar BLER performances. This example indicates that when the number of RX antennas are greater than the number of UEs, gNB can successfully decode multiple UEs sharing the same PO, even though their PUSCH resource units multiplexing are non-orthogonal in time/frequency/code domains.

Table 4: Parameters for MCL Computation

| (1) UE Tx Power (dBm) | 23 |
| --- | --- |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) gNB Receiver noise figure (dB) | 5 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) |  |
| (7) required SINR (dB) per RxAnt (dB) |  |
| (8) Receiver sensitivity = (6)+(7) (dBm) |  |
| (9) Receiver processing gain |  |
| (10) MCL = (1)-(8)+(9) (dB) |  |

Table 5: MCL for 72 bits msgA

| Number of PRB per PO | 1 | 2 | 3 | 4 | 6 | 12 |
| --- | --- | --- | --- | --- | --- | --- |
| MCL at 10% Target BLER (dB) | 142.35 | 142.54 | 142.58 | 142.53 | 142.47 | 142.16 |

Based on the formula shown in Table 4 and the BLER curves in Figure 10, MCL corresponding to different PO bandwidth are calculated and shown in Table 5. For a 72-bit payload, 3 RB allocation provides the best MCL performance.

#### 2.2.3.2	Waveform for Small Payload Size

In Figure 11, the msgA BLER performance with DFT-s-OFDM waveform is also simulated. For small payload transmission, the BLER performance of DFT-s-OFDM waveform is similar to that of CP-OFDM waveform, but the PAPR performance of DFT-s-OFDM waveform is much better. Therefore, DFT-s-OFDM waveform should be supported in two-step RACH transmission of small payload, especially for UE experiencing large pathloss or in RRC_IDLE/RRC_INACTIVE state.

Figure 11: msgA BLER performance for TBS=72 bits with CP-OFDM and DFT-s-OFDM

#### 2.2.3.3	Non-orthogonal DMRS Multiplexing

Figure 12 shows the LLS performance comparison between orthogonal DMRS and non-orthogonal DMRS for two UEs.

- For orthogonal DMRS configurations, two UEs use different DMRS ports and share the same DMRS scrambling sequence.

- For non-orthogonal DMRS configuration, two UEs are assigned to the same DMRS port but can be distinguished by different DMRS scrambling IDs.

As shown by Figure 12, the non-orthogonal DMRS configuration shows negligible performance degradation compared to orthogonal DMRS configuration, especially for larger PO size of 3 and 4 RBs.

#### 2.2.3.4	Resource Allocation, MCS and UE Multiplexing for Large Payload Size

Figures 13 and 14 show the BLER performance for msgA with large payload size (1000 bits). MCL at 10% BLER is 131.77 dB for 6 RB allocation and 133.26 for 12 RB allocation, respectively. Therefore, 12 RB allocation provides the better MCL for a 1000-bit payload size of msgA.

In Figure 13, the link level performance degradation due to two UE multiplexing is negligible. This indicates non-orthogonal UE multiplexing is also feasible for large payload size.

On the other hand, Figure 14 studies the impacts of different MCS for the same PO bandwidth and the same payload size. It shows that 16 QAM modulation and lower code rate does not provide gain in link level performance.

Figure 12: msgA BLER Performance for TBS 72 bits with Orthogonal and Non-Orthogonal DMRS

Figure 13: msgA BLER performance for TBS=1000 bits with 1, 2 UEs

Figure 14: msgA BLER performance for TBS=1000bits with 16 QAM modulation

Therefore, we have the following observation:

Observation 3   LLS results for msgA decoding with realistic channel estimation indicate that:

- MCL at a given BLER target can be used as the performance metric for PO size optimization;

- PO size optimization depends on the payload size and slot format;

  - PO size of 3 RB and 1 slot (14 symbols, DMRS overhead 1/7) is good choice for 72 bits;

  - PO size of 12 RB and 1 slot (14 symbols, DMRS overhead 1/7) is a good choice for 1000 bits.

- DFT-s-OFDM waveform should be supported for small payload transmission, especially for UE experiencing large pathloss or in RRC_IDLE/RRC_INACTIVE state;

- For both small and large payload size, non-orthogonal UE multiplexing in both DMRS and PUSCH does not cause performance degradation, if the number of RX antennas is greater than the number of UEs;

- Higher order modulation does not show performance gain for large payload size.

Based on the observation, we propose the following for msgA transmission.

Proposal 7: For msgA transmission, the following designs should be supported:

- Low-order linear modulations such as QPSK and pi/2 BPSK should be specified for msgA PUSCH transmission. In particular,

  - QPSK should be specified for CP-OFDM waveform;

  - pi/2 BPSK and QPSK should be specified for DFT-s-OFDM waveform.

- A finite set of PO size can be specified, to support different payload size (in the range of a few bytes to a few hundred bytes) and different coverage requirements;

  - PO bandwidth can be configured proportional to that of RO bandwidth;

  - In FR1, PO bandwidth of {3, 6, 12} PRBs can be considered for a 14-symbol slot;

  - Slot repetition should be supported.

- To enlarge the pool size of DMRS sequences without increasing its overhead, NR Rel-15 DMRS signal generation mechanism can be reused with extended scrambling ID sets configuration.

## 2.3	Channel Structure for msgB

After the transmission of msgA, the two-step RACH UE will monitor msgB within a pre-configured RAR window. The RAR window configuration information is broadcast in SI. A general channel structure for msgB is shown in Figure 15, wherein the PDCCH carries the resource assignment for PDSCH, and PDSCH carries the MAC PDU in response to the latest msgA transmissions/re-transmissions sharing the same RO.

Figure 15:  General Channel Structure of msgB

### 2.3.1	Contents of msgB

In msgA transmission, multiple UEs can be multiplexed on the same RO/PO. Due to the contention-based transmission and channel impairments, not all msgA can be successfully decoded by gNB. As a result, gNB will send a msgB in response to the two-step RACH UEs, if their msgA payloads have been decoded, or only their msgA preamble have been detected. In particular, the contents of msgB depend on the processing outcome of msgA as well as the RRC states of UE.

Therefore, we have the following proposal:

Proposal 8: Depending on the processing outcome of msgA, gNB will send different responses in msgB.

- If the msgA preamble is detected but the msgA payload is not decoded, gNB will send a RAR, which includes at least the RAPID, the TA command and the UL grant for msgA payload transmission.

- If the msgA preamble is detected and the msgA payload is also decoded, gNB will send a MAC CE, which includes at least the contention resolution information.

- For two-step RACH UEs sharing the same RO, their RARs and MAC CEs can be aggregated in constructing the MAC PDU of msgB.

  - For a UE in RRC_CONNECTED state, if its msgA payload can be decoded, gNB can choose to transmit the contention resolution information (C-RNTI) differently. For example, by unicasting a PDCCH and scrambling its CRC by C-RNTI.

### 2.3.2	Modified RA-RNRI for msgB

In four-step RACH, the CRC of msg2 PDCCH is scrambled by a 16-bit RA-RNTI. Based on NR Rel-15 design, the RA-RNTI has max decimal value of 17920, whose binary expansion occupies 15 out of the 16 bits allocation. On the other hand, when the same RO is shared by two-step RACH UE and four-step RACH UE, both msg2 and msgB will be transmitted by gNB. To avoid the ambiguity and reduce the complexity of PDCCH decoding, it is necessary to consider modified RA-RNTI design to differentiate msg2 of four-step RACH from msgB of two-step RACH. A low complexity solution is to modify the RA-RNTI calculation formula for two-step RACH msgB transmission, by considering the un-used space in the 16-bit allocation.  Therefore, we have the following proposal:

Proposal 9: Modified RA-RNTI calculation should be specified for msgB transmission. To reduce the impacts on specification, the RA-RNTI formula in NR Rel-15 can be extended to support modified RA-RNTI mapping into the un-used space of the 16-bit allocation.

### 2.3.3	RAR Window Configuration

After transmitting msgA, a two-step RACH UE will monitor msgB within a RAR window. Since network requires extra time to process the msgA payload, the RAR window length needs to be extended in two-step RACH, compared to the window configuration in four-step RACH.

In addition, to simplify the UE procedures for msgB monitoring, with different msgA payload size and in both licensed and unlicensed spectrum, it is preferred to configure the starting point of RAR window with respect to the completion of msgA PUSCH transmission.

Therefore, we have the following proposal:

Proposal 10:  In two-step RACH, RAR window configurations different from four-step RACH should be supported. The starting point of the RAR window should be specified with respect to the completion of a msgA payload transmission/retransmission, and the length of the RAR window should be extended appropriately.

# 3.	Conclusions

In this contribution, msgA and msgB channel structure for two-step RACH has been discussed. Based on the performance evaluation and analysis, we have the following observations and proposals:

Observation 1: Two-step RACH can be supported for all cell sizes. Specifically,

- for small cell sizes, Tadj = 0 is sufficient;

- for large cell sizes:

- Tadj = 0 can be used as long as a sufficient gap is allowed for gNB to process the preamble for timing offset estimation and then use that information for PUSCH demodulation;

- Tadj ~= RTT can be used in the case of UE assistance (e.g., tracking timing related changes relative to last valid TA)

- UE-assisted timing adjustment can be applied to both preamble and payload of msgA, or to the payload part only.

Observation 2:  Two-step RACH can operate in any RRC state. Different payload size, waveform and MCS can be specified for different RRC states and mapped to different POs.

Observation 3   LLS results for msgA decoding with realistic channel estimation indicate that:

- MCL at a given BLER target can be used as the performance metric for PO size optimization;

- PO size optimization depends on the payload size and slot format;

  - PO size of 3 RB and 1 slot (14 symbols, DMRS overhead 1/7) is good choice for 72 bits;

  - PO size of 12 RB and 1 slot (14 symbols, DMRS overhead 1/7) is a good choice for 1000 bits.

- DFT-s-OFDM waveform should be supported for small payload transmission, especially for UE experiencing large pathloss or in RRC_IDLE / RRC_INACTIVE state;

- For both small and large payload size, non-orthogonal UE multiplexing in both DMRS and PUSCH does not cause performance degradation, if the number of RX antennas is greater than the number of UEs;

- Higher order modulation does not bring performance gain for large payload size.

Proposal 1:  To support two-step RACH operation in both licensed spectrum (FR1 and FR2) and unlicensed spectrum (FR1), with or without a valid timing advance, the channel structure of msgA should include a preamble, a payload, as well as a transmission gap with configurable duration.

Proposal 2:  Depending on the payload size, MCS and slot format, the transmission bandwidth of msgA preamble and msgA payload can be same or different.

Proposal 3:  Network has the flexibility to configure separate or shared ROs for two-step RACH and four-step RACH. When separate ROs are configured, they can be multiplexed in time and/or frequency domain. The configuration period of msgA PUSCH can be similar to or different from the configuration period of RO.

Proposal 4:  Two-step RACH should support preamble formats specified in NR Rel-15 for both large cells and small cells. The preamble and payload of msgA can be transmitted in the same slot or different slots. Specifically,

- when msgA preamble and payload are transmitted in different slots, the numerology for msgA PUSCH follows the numerology configured for the UL BWP of msgA transmission;

- when msgA preamble and payload are transmitted in the same slot, gNB will configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP.

Proposal 5: The mapping rules between msgA preamble and msgA PUSCH resource unit should be specified for two-step RACH. To support the mapping rules specified for msgA channel structure, NR Rel-15 design mechanisms for DMRS and PUSCH can be re-used, with extensions in the scrambling ID sets generation. The scrambling ID generation formulas for msgA DMRS and PUSCH should include at least the preamble sequence index of msgA as a parameter.

Proposal 6: For msgA transmission, the following designs should be supported:

- a minimum payload size of 7 or 9 bytes can be supported by DFT-s-OFDM waveform and low MCS; payload sizes up to a few hundred bytes can be supported by higher MCS;

- multiple POs can be configured per PO configuration period, to accommodate different MCS, payload size and waveform;

- multiple two-step RACH UEs can share the same PO, if their msgA transmissions use similar payload size/MCS/waveform;

- resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain;

- the configurations and resource allocations for RO and PO need to be balanced to achieve a good trade-off in resource utilization efficiency and collision probability reduction.

- the mapping rules between msgA preamble sequence and msgA PUSCH resource unit can be indicated by SI or RRC signalling.

Proposal 7: For msgA transmission, the following designs should be supported:

- Low-order linear modulations such as QPSK and pi/2 BPSK should be specified for msgA PUSCH transmission. In particular,

  - QPSK should be specified for CP-OFDM waveform;

  - pi/2 BPSK and QPSK should be specified for DFT-s-OFDM waveform.

- A finite set of PO size can be specified, to support different payload size (in the range of a few bytes to a few hundred bytes) and different coverage requirements;

  - PO bandwidth can be configured proportional to that of RO bandwidth;

  - In FR1, PO bandwidth of {3, 6, 12} PRBs can be considered for a 14-symbol slot;

  - Slot repetition should be supported.

- To enlarge the pool size of DMRS sequences without increasing its overhead, NR Rel-15 DMRS signal generation mechanism can be reused with extended scrambling ID sets configuration.

Proposal 8: Depending on the processing outcome of msgA, gNB will send different responses in msgB.

- If the msgA preamble is detected but the msgA payload is not decoded, gNB will send a RAR, which includes at least the RAPID, the TA command and the UL grant for msgA payload transmission.

- If the msgA preamble is detected and the msgA payload is also decoded, gNB will send a MAC CE, which includes at least the contention resolution information.

- For two-step RACH UEs sharing the same RO, their RARs and MAC CEs can be aggregated in constructing the MAC PDU of msgB.

  - For a UE in RRC_CONNECTED state, if its msgA payload can be decoded, gNB can choose to transmit the contention resolution information (C-RNTI) differently. For example, by unicasting a PDCCH and scrambling its CRC by C-RNTI.

Proposal 9: Modified RA-RNTI calculation should be specified for msgB transmission. To reduce the impacts on specification, the RA-RNTI formula in NR Rel-15 can be extended to support modified RA-RNTI mapping into the un-used space of the 16-bit allocation.

Proposal 10:  In two-step RACH, RAR window configurations different from four-step RACH should be supported. The starting point of the RAR window should be specified with respect to the completion of a msgA payload transmission/retransmission, and the length of the RAR window should be extended appropriately.

# References

[1] RP-182894, “New Work Item: Two-Step RACH for NR,” ZTE, 3GPP TSG RAN #82, Sorrento, Italy.

[2] 3GPP TSG RAN WG1 Meeting #96, “RAN1 Chairman’s Notes,” Athens, Greece.

[3] 3GPP TSG RAN WG1 Meeting #96B, “RAN1 Chairman’s Notes,” Xi’an, China.

[4] R1-1907256, “Procedures Related to Two-step RACH,” Qualcomm, 3GPP TSG RAN1 #97, Reno, USA.

[5] 3GPP TS 38.211 V15.4.0, “NR; Physical Channels and Modulation (Release 15),” December 2018.

[6] R1-163939, “Summary of Offline Discussions on eMBMS Simulation Considerations,” 3GPP TSG RAN WG1 Meeting #84Bis, April 2016.

[7] 3GPP TR 38.901, “Study on Channel Model for Frequencies from 0.5 to 100 GHz,” June 2018.

[8] R2-1905472, “LS on MsgA Content Size,” 3GPP TSG RAN WG2 Meeting #105B, Xi’an, China.

[9] R1-1809023, “Support for transmission in preconfigured UL resources,” Qualcomm, 3GPP TSG RAN WG1 Meeting #94, Gothenburg, Sweden.

[10] 3GPP TR 36.822 V11.0.0, “LTE Radio Access Network (RAN) enhancements for diverse data applications (eDDA),” Release 11.

[11] R2-1907911, “MsgB Content in Two-Step RACH,” Qualcomm, 3GPP TSG RAN WG2 Meeting #106, Reno, USA.

[12] R2-1907909, “RAR Window for msgB Reception,” Qualcomm, 3GPP TSG RAN WG2 Meeting #106, Reno, USA.

# Appendix A: Link Level Simulation Assumptions

The following table summarizes the LLS assumptions used in this paper:

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM and DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz |
| Number of symbols in PUSCH slot | 14 symbols |
| DMRS overhead for PUSCH | 2 DMRS symbols |
| TBS | 72, 1000 bits |
| Number of PRBs | 1,2,3,4,6,12 |
| Number of UEs | 1, 2 UEs |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 4Rx |
| Propagation channel & UE velocity | TDL-C 300ns, 3km/h |
| Receiver | MMSE-IRC |
| Channel Estimation | Ideal and Realistic |
| Target BLER | [10%, 1%] |