3GPP TSG-RAN WG1 Meeting #97	Tdoc R1-1907287

Reno, USA, May 13th –17th, 2019

Agenda Item:	7.2.7.2

Source:	Qualcomm

Title:	Evaluation of NR FR2 for IMT-2020 eMBB Dense Urban

Document for:	Discussion & Decision

# 1	Introduction

Current 3GPP self-evaluation TR [1] and preliminary submission report [2] do not include results for NR spectral efficiency.in eMBB-Dense Urban configuration B (30GHz), left “TBD”.

This contribution presents some evaluation and results for that TE/configuration.

# 2	Evaluation Assumptions and Results

For the evaluation of spectral efficiency in eMBB-Dense Urban at 30GHz, given the challenges related to high frequency transmission in reaching large distances and/or high penetration-loss indoors, we investigated a certain evaluation methodology using “admission control”.

In particular, those few users with very high/bad pathloss (above a specific threshold) are not “admitted” by the system, allowing the served users to achieve better spectral efficiency.

Simulation assumptions and results are captured below.

2.1 Simulation Assumptions

| General assumptions | UL&DL |
| --- | --- |
| Duplexing | TDD |
| Numerology | 120kHz SCS, 14 OFDM symbol slot |
| Frame structure | DDDU |
| Modulation | Up to 256 QAM |
| Coding on PDSCH/PUSCH | LDPC; Max code-block size=8448bit |
| Codebook | Ideal |
| Codeword (CW)-to-layer mapping | For 1~4 layers, CW1; For 5 layers or more, two CWs |
| Network synchronization | Synchronized |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Non-ideal |
| Guard band ratio | 5% (for 80 MHz) |
| Mechanic tilt | 90 deg (gNB panel in each sector is placed vertically) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") |
| UT attachment | Based on RSRP. |
| UE antenna panel selection | The UE panel with the best receive SNR |
| Beam set at TRxP | DFT beam selection |
| Beam set at UE | DFT beam selection |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Analog beam selection for serving TRxP | Select the best beam pair among the set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Analog beam selection for interfering TRxP | Based on the analog beam selection according to scheduling results of non-serving TRxP |

| DL specific assumptions | DL |
| --- | --- |
| Transmission scheme | SU/MU-MIMO |
| MU dimension | Up to 2 layers (Up to 2 simultaneous analog beams are used) |
| SU dimension | Up to 2 layers |
| SRS transmission | For UE 2 Tx ports: Non-precoded SRS, 2 SRS ports, 1 symbol every 2 UL slots |
| PRB bundling | 4 PRBs |
| DL CSI measurement | Non-Precoded CSI-RS based |
| CSI feedback | CQI: every 8 slots; RI: every 8 slots, CRI: every 8 slots; Subband based |
| Interference measurement | SU-CQI;CSI-IM for inter-cell interference |
| CBG | 1 |
| ACK/NACK delay | The next available UL slot |
| Re-transmission delay | The next available DL slot after receiving NACK |
| Antenna configuration at TRxP | 8TXU: (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,2) (dH,dV) = (0.5, 0.5)λ |
| Antenna configuration at UE | 2RXU: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 1,2) (dH,dV) = (0.5, 0.5)λ; (dg,V,dg,H) = (0, 0)λ |
| Total (RE) overhead | 28.58 |
| Admission control criteria | 128 dB PL threshold |

| UL specific assumptions | UL |
| --- | --- |
| Transmission scheme | SU/MU-MIMO |
| MU dimension | Up to 2 layers (Up to 2 simultaneous analog beams are used) |
| SU dimension | Up to 2 layers |
| SRS transmission | For UE 2 Tx ports: Non-precoded SRS, 2 SRS ports, 1 symbol every 2 UL slots |
| Max CBG number | 1 |
| UL re-transmission delay | Next UL slot after receiving retransmission indication |
| Antenna configuration at TRxP | 2RXU: (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;1,1) (dH,dV) = (0.5, 0.5)λ |
| Antenna configuration at UE | 2TXU: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 1,2) (dH,dV) = (0.5, 0.5)λ; (dg,V,dg,H) = (0, 0)λ |
| Total (RE) overhead | 20.74 |
| Admission control criteria | 125 dB PL threshold |

Regarding the admission control criteria/thresholds, we think 125-1288dB range reflects a realistic deployment configuration/setting.

Other TE/configuration parameters align with Report ITU-R M.2412.

2.2 Simulation Results

Downlink

Admission control criteria: 128 dB PL threshold

| DL spectral efficiency | IMT-2020 target | Sim results |
| --- | --- | --- |
| Average [bit/s/Hz/TRxP] | 7.8 | 9.38 |
| 5th-tile [bit/s/Hz] | 0.225 | 0.28 |

Uplink

Admission control criteria:  125 dB PL threshold

| UL spectral efficiency | IMT-2020 target | Sim results |
| --- | --- | --- |
| Average [bit/s/Hz/TRxP] | 5.4 | 5.52 |
| 5th-tile [bit/s/Hz] | 0.15 | 0.16 |

It is observed that, with the above assumptions, NR can fulfill the spectral efficiency requirements (for the admitted users).

# 3	Conclusions and proposals

We think it would be valuable to show, in the 3GPP self-evaluation TR, some evaluation of NR spectral efficiency.in eMBB-Dense Urban configuration B (30GHz).

Proposal: consider including above evaluation and results in the 3GPP self-evaluation TR [1].

Some example text proposal is provided in Annex.

# 4	References

[1] TR 37.910: Self Evaluation towards IMT-2020 Submission

[2] RP-182097: Preliminary 3GPP self-evaluation report

# Annex: Pseudo text proposal for TR 37.910

##### 5.4.1.2.2	Evaluation configuration B (CF=30 GHz)

The evaluation results of DL and UL spectral efficiency for NR for evaluation configuration B are provided in Tables 5.4.1.2.2-1/2.

It is noted that the evaluation is based on the additional assumption of using a certain admission control, e.g. based on max. pathloss of 128dB. This implies that those few users with worse pathloss (e.g. indoor with bad penetration loss) are not admitted; the served users can then achieve better spectral efficiency, as shown in the results below.

Table 5.4.1.2.2-1 DL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration B, CF=30 GHz)

[to be filled…]

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 80MHz |
| … |  |  | Average [bit/s/Hz/TRxP] | 7.8 | 1 |  |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  |  |

Table 5.4.1.2.2-2 UL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration B, CF=30 GHz)

[to be filled…]

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 80MHz |
| … |  |  | Average [bit/s/Hz/TRxP] | 5.4 | 1 |  |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  |  |