3GPP TSG RAN WG1 Meeting #97 	R1-1907288

May 13th – 17th, 2019

Reno, U.S.A.

Agenda item:	7.2.8.1

Source: 	Qualcomm Incorporated

Title: 	CSI Enhancement for MU-MIMO Support

Document for:	Discussion/Decision

# Introduction

In RAN1#96, high rank codebook for Rel-16 Type II CSI with FD compression and UCI design were discussed. The agreements and observation of offline email discussion were captured in [1-6]. In this contribution, we discuss remaining issues on high rank design alternatives and UCI parameters. We also shed light on the high level concept for CSI omission.

# Discussion on RI={3,4} designs

## Discussion on settings of p for RI={3,4}

For high rank design aspect, the remaining issue lies in the setting of number of FD bases for each layer of RI={3,4}. After extensive discussion in the last meeting, there are three alternatives left [1-2], i.e., Alt2B, Alt3C and Alt6E.

Table 1. Parameter settings for high-rank design schemes Alt2B, Alt3C and Alt6E.

|  | Alt2B |  |  |  | Alt3C |  |  |  | Alt6E |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Layer 0 | Layer 1 | Layer 2 | Layer 3 | Layer 0 | Layer 1 | Layer 2 | Layer 3 | Layer 0 | Layer 1 | Layer 2 | Layer 3 |
| RI=1 | M |  |  |  | M |  |  |  | M |  |  |  |
| RI=2 | M | M |  |  | M | M |  |  | M | M |  |  |
| RI-3 | M | M | M2 |  | M2 | M2 | M2 |  | M3,0 | M3,1 | M3,2 |  |
| RI=4 | M | M | M2 | M2 | M2 | M2 | M2 | M2 | M4,0 | M4,1 | M4,2 | M4,3 |

From complexity wise, Alt2B is the simplest scheme as it provides a nested structure for the FD bases of a certain layer across all ranks. That is, once a FD bases of layer 0 is determined, there may be no need to update it anymore for any rank hypothesis RI={1,2,3,4}. Alt3C is the second simplest as it ensures a nested structure for layers of RI={3,4}. Alt6E does not provide nested structured and need to update 7 M values given a configuration of M for RI={3,4}. Besides, Alt6E requires more spec effort than Alt2B and Alt3C.

From the aspect of performance-overhead tradeoff, we compare the three alternatives with the following setup: Alt2B and Alt3C with M2=23M and M2=M2 and Alt6E with Mr,0=Mr,1=2MRI and  Mr,2=Mr,3=2M-Mr,0-Mr,1RI-2. Simulation results are shown in Figure 1, where the plots are based on p,β=14,14, 14,12, 14,34, 12,14, 12,12 and 12,34. We can see that Alt2B and Alt3C with M2=M/2 achieves similar performance-overhead tradeoff to Alt6E, which reveals that the extra effort in Alt6E does not provide substantial benefit. Hence, based on the discussion, we propose

Figure 1. Comparison between Alt2B, Alt3C and Alt6E for setting of M

Observation 1: Alt2B and Alt3C with M2=M/2 achieves similar performance with Alt6E.

Proposal 1: For the setting of number of FD basis selection, support Alt2B (first preference) or 3C (second preference), and FFS the value of M2.

## Discussion on max number of coefficients per layer

Another remaining issue of high rank codebook lies in whether there is constraint on max number of coefficients per layer. With Alt0, UE may freely assign the total max 2K0 across layers; with Alt1, the max number of coefficients per layer is K0.

In our view, with Alt0, UE may perform the coefficient assignment by ordering the coefficients across all layers, while Alt1 introduce an unnecessary constraint, which adds on UE implementation complexity and degrade the performance (1~2% at small value of K0 shown by Figure 2).

Figure 2. Comparison between no max NNZC per layer vs. max NNZC per layer

Besides, the motivation of Alt1 strives to save overhead, especially for per-layer NNZC indicator and SCI. For payer-layer NNZC indicator, we have shown a more effective way in section 3.3 which yields a lower overhead and does not reply on the max number of coefficients per layer. For SCI, if per-layer NNZC is adopted, then Alt1 is useless in saving overhead; if total NNZC is adopted, SCI Alt 3.2 with log2min{KNZtot,2LMi} at most requires 1 bit more than SCI Alt 3.3 with log2K0 because minKNZtot,2LMi≤2K0. This 1-bit difference is negligible. Hence, based on the discussion, we propose

Observation 2: Alt1 with K0 max NNZC per layer achieves 1~2% loss compared to Alt0 with no constraint on max NNZC per layer.

Proposal 2: In Rel-16 Type II codebook, support Alt0 where UE freely assign 2K0 coefficients across all layers with no restriction on per-layer NNZC.

## Exploiting orthogonality among layers

As it has been agreed that the payload for rank-4 should be comparable to the payload for rank-2, the compression ratio for rank-4 is half of the compression ratio of rank-1. For instance, for rank-2, UE reports 2K0 coefficients from totally 4LM coefficients; however for rank-4, UE reports 2K0 coefficients from totally 8LM coefficients. Schemes exploiting orthogonality among layers help enhance the compression ratio while not incurring more overhead.

In principle, for two orthogonal layers each with 2L coefficients, i.e., ci,l where i=0,1,⋯2L-1 and l=0,1, satisfying i=02L-1ci,0*ci,1=0, UE only needs to report 4L-1 coefficients, e.g., not reporting c2L-1,1, because the unreported coefficient, e.g., c2L-1,2, can be derived as c2L-1,2=i=02L-2ci,0*ci,1/c2L-1,0* and the solution is unique. Similarly, if there is a third and a fourth layer, UE only needs to report L-2 and L-3 coefficients for the third layer and fourth layer, respectively. Applying this logic to Rel-16 type II codebook,

- For the first layer, UE needs to perform compression for coefficients associated with 2L beams, and the NZ coefficients are selected/quantized from 2LM coefficients;

- For the second layer, UE performs compression for coefficients associated with  2L-1 beams, and the NZ coefficients are selected/quantized from 2L-1M coefficients;

- For the third layer, UE performs compression for coefficients associated with  2L-2 beams, and the NZ coefficients are selected/quantized from 2L-2M coefficients;

- For the fourth layer, UE performs compression for coefficients associated with  2L-3 beams, and the NZ coefficients are selected/quantized from 2L-3M coefficients;

Consequently, for rank-4, UE reports 2K0 coefficients from totally 8LM-6M coefficients. Moreover, if the coefficients of each layer is normalized to the first beam, all coefficients associated with this beam are not needed to be reported. Hence, for rank-4, UE reports 2K0 coefficients from totally 8LM-10M coefficients. As the compression ratio is increased without increasing more overhead, the performance can be improved. Based on the discussion, we propose.

Proposal 3: For RI={3,4} design of Rel-16 type II codebook with FD compression, further study schemes that exploit orthogonality among layers and unit norm of each layer.

# UCI design

## FD basis report

For FD basis report, the first open issue lies in whether adopting a single-stage FD basis report or a two-stage report. Since-stage scheme via a N3-bit bitmap for each layer or log2N3Ml-bit combinatorial indicator is the most straightforward and flexible way. However, due to layer-independent basis selection, the payload would be large. Evidently, since N3 may be as large as 38 (when R=2), this method will provide too much overhead for high rank, e.g., 76 more bits for rank-4 compared to rank-2 via bitmap.

In theory, the selected bases may be located relatively close, and the span of the M bases (e.g., from the first selected basis to the last selected basis) may occupy only a small portion of the entire N3 candidate basis. Moreover, the span of the M basis selected for different layers may partially overlapped. Hence, a two-step report of the FD basis is useful. In the two-stage FD basis report, a first stage uses an intermediate set to capture the commonality among the FD basis selection across layers, then the bitwidth in layer-specific FD bases report in the second stage can be reduced. Based on the discussion, we propose

Proposal 4: Support two-stage FD basis report for Rel-16 Type II codebook with FD compression, wherein

- The size of the intermediate set is RI-common and layer-common

- The intermediate set is RI-common and layer-common

### Discussion on the size of the intermediate set

Regarding the size of the intermediate set, i.e., N3', there are three options: 1) reported by UE, 2) configured by the NW, and 3) fixed in the spec. Our view is as follows.

From implementation wise, if N3' is reported by UE, the UE may first freely select the FD bases for each layer, and secondly determine the intermediate set and its size considering the union of the FD bases across layers. If N3' is configured by the NW or fixed in the spec, the UE needs to first perform a pruning for a best intermediate set for all layers of a certain rank, and the pruning has to be rank-specific. After that, the UE performs layer-specific FD bases reporting within the intermediate set. Hence, having N3' being configured by NW or fixed in the spec adds on implementation complexity.

From performance wise, as shown by Figure 3, UE reporting N3' leads to a better performance as the FD bases selection is from the entire N3 bases, while network configured or fixed N3' put on a restriction for FD bases selection across layers.

Moreover, from the simulation results, we also do see the performance loss is controllable by choosing a proper value of N3' based on the value of M, i.e., number of FD bases per layer. Since the max number of FD bases across all layers is around 2M, setting N3'=2M may achieve similar performance to UE reporting N3'. More generally, the proper N3' should be set by scaling the max number of FD bases across all layers and all ranks (i.e., M) by a ratio α.

Hence, based on the discussion, we propose

Proposal 5: For the size of the intermediate set used in the 1st stage of two-stage FD bases report, support

- UE reporting intermediate set N3' (1st preference); or

- The intermediate set size N3' being derived from the max number of FD bases across all layers of all ranks, e.g., N3'=α×M (2nd preference).

### Choice of intermediate set for the first stage

For the two-stage based FD bases selection, there are two candidates for the intermediate set: window-based or combinatorial based via log2N3N3'-bit.

For window-based approach, the FD basis in the intermediate set is given by Minitial+n,N3, where n=0,1,⋯N3'-1. In our view, the starting index Minitial should be fixed in the spec. The reason is as follows. Recall that in the discussion of O3, we explain that only a fixed rotation factor (i.e., oversampling O3=1 and rotation factor q3=0) is needed to be defined in the spec. This is because a cyclic-shift to the W2 of a certain layer (e.g., shifting from rotation factor q3=2 to q3=0) corresponds to a linear phase rotation to W2 due to the property of DFT bases and a phase rotation does not bring any performance difference. This theory can be applied to the FD bases selection for across layers. In general, assuming Ml bases, denoted by n3(0)+Minitial, n3(1)+Minitial,⋯,n3Ml-1+Minitial, are selected are selected from the window-based intermediate set for layer-l, then it is equivalent to report FD basis n30, n3(1),⋯,n3Ml-1. Mathematically, it can be written as

fn3m+MinitialH=1e-j2πn3(m)+MinitialN3⋯e-j2πn3(m)+Minitial(N3-1)N3=1e-j2πn3(m)N3⋯e-j2πn3(m)(N3-1)N3fn3mH×1e-j2π⋅Minitial⋅1N3⋱e-j2π⋅Minitial⋅N3-1N3.

In other words, UE may employ any starting index in calculating the W2 matrix and the window-based intermediate set for each layer, but always report FD basis comprised in the window starting from FD index 0 with size N3'.

Based on the above analysis, the comparison between window-based approach and combinatorial based approach is as follows.

- From throughout perspective, an argument against window-based approach is performance loss due to the fact that the window consists of consecutive FD bases while combinatorial approach has more flexibility. To justify how performance difference between the two approach, a comparison is illustrated in Figure 3. As shown, window-based approach and combinatorial based approach achieve similar performance.

- From overhead perspective, the window-based approach is only characterized by the window size N3', while combinatorial approach requires the intermediate set  N3' and the selection of N3' bases from total N3 FD bases. At worst case, combinatorial approach requires log2N3N3'=36 bits more than window-based approach, considering N3=38 and N3'=19. We also observe from Figure 2 that at some point the combinatorial-based approach may need more bits than the one stage approach due the large overhead resulted by log2N3N3'. An evident example is N3=16, N3'=6 and Ml=2 for each layer of rank-4. Combinatorial approach needs 1 bit more than one-stage approach.

- From implementation perspective, the encoding and decoding of log2N3N3' requires much more complexity than window-based approach. Moreover, if the intermediate set size is configured/fixed, window-based approach enables per-layer searching for the window. That is, UE may perform searching for window layer-by-layer independently, although the resultant window may be layer-specific, UE may perform a cyclic-shift to the W2 matrix to align the window across layers. However, combinatorial approach cannot perform a per-layer search. This is because the best N3' FD bases for layer 0 may be different from the best N3' bases for layer 1. The difference between the best N3' FD bases for layer 0 and the best N3' FD bases for layer 1 is more than a cyclic-shift. Hence, a joint-layer searching for the intermediate set is needed, which adds on the implementation complexity.

Figure 3. Comparison between window-based intermediate set and combinatorial-based intermediate set

Observation 3: UE may employ a layer-specific starting point in calculating the W2 matrix and the window-based intermediate set, but always report the intermediate set and FD basis associated with a window with FD basis 0,1,⋯,N3'-1.

Observation 4: Window-based approach achieves similar performance as the combinatorial-based approach.

Observation 5: Combinatorial-based intermediate set may need more overhead than one-stage FD basis report.

Proposal 6: Support window-based intermediate set for the two-stage basis report, wherein the starting index of the window is fixed.

### Discussion on the layer-specific FD basis report in the second stage

The second stage comprises reporting FD bases for each layer based on the intermediate set in the first stage. There are two open issues: 1) bitmap or combinatorial indicator, and 2) selecting Ml basis from the N3' basis or selecting Ml-1 basis from the N3'-1 basis with one additional fixed FD basis.

Regarding the first issue,

- from complexity wise, bitmap is easier while combinatorial indicator requires a mapping of n30,n3(1),⋯,n3(Ml-1) FD bases to a codepoint of the combination indicator. As Ml can be as large as 10, and N3' can be as large as N3 and equal to 38. Then, the mapping relies on large table in the spec and the gNB may need 10 rounds of iteration to recover the 10 FD bases indices.

- From overhead wise, combinatorial indicator yields less overhead than bitmap. The overhead difference is large if there is huge or very small gap between the value of Ml and N3'. However, based on the discussion in the previous section, a proper value of N3' would be 1.5x or 2x of M (the # of FD bases for rank-1 and 2) so as to cover the union of the FD bases across layers. With this setup, combinatorial indicator only saves 2~3 bits compared to bitmap.

Regarding the second issue, selecting Ml basis from the N3' basis specified by the intermediate set without any restriction is the easiest approach to implement, while the second option assumes there is one basis that must be selected by all layers, and that common basis is fixed and there is no need to report so that UE only needs to report Ml-1 bases from the N3'-1 bases.

The second option tries to adopt the theory of fixing the starting index of the window. However, in our view, the theory cannot be applied in the second stage. The reason is as follows. When the intermediate set size is configured or fixed, the window selection in the first stage is from the entire N3 DFT bases, where the cyclic-shift property holds. However, in the second stage, the FD bases report is within the intermediate set, which is a truncation of the whole DFT matrix. If there exists one layer does not select the reference FD basis (which is fixed), the UE cannot perform a cyclic shift within the window to shift some strong coefficients to the reference FD basis.

To address the aforementioned problem, one way is to align the strongest FD basis across different layers before the selecting the intermediate set, so that the strongest FD basis on each layer appear on the same position within the intermediate set. For instance, if the window-based intermediate set is adopted and the starting index is fixed to FD basis 0, then the UE needs to align the strongest FD bases across all layers on FD basis N3'2 to ensure a good intermediate selection by including some strong side lobes. Hence, the second option implies a restriction on intermediate set selection and would add on UE implementation complexity.

Proposal 7: For the layer-specific FD basis report in the second stage, the downselection between bitmap and combinatorial indicator should consider the value of N3'.

Proposal 8: For the layer-specific FD basis report in the second stage,

- Prefer bitmap over combinatorial indicator

- Support the indicator selecting Ml basis from the N3' basis specified by the intermediate set, e.g., N3'-bit bitmap or log2N3'Ml-bit combinatorial indicator.

## Basis sufficiency indicator

In current R16 Type II codebook with FD compression, the total feedback overhead is controlled by the number of basis (i.e., M or p) and the total number of non-zero coefficients (i.e., K0, K0tot or β). The configured value of M and K0 are based on a relatively long-term observation/measurement (e.g., based on SRS) by the network. However, when UE suffers from narrow-band interference (measured from CSI-IM or NZP IMR), the resultant channel after whitening the interference would become less correlated across different subbands. In this case, the configured values of p and/or β may be insufficient to capture the enhanced frequency selectivity, because the narrow band interference is relatively a short-term statistic and the network is not be able to track the interference observed by the UE. If the UE employs the configured value of p and β to calculate PMI, the reported CQI and the resulted throughout may be worse than R15 Type II or R15 Type I codebook.

To solve this issue, a basis sufficiency indication can be reported in UCI part 1 for the network to identify that the value of p and/or β are insufficient to provide a solid PMI. Then, based on this indication, the network may decide to trigger another CSI reporting configuration with larger values of p and/or β, or decide to fallback to R15 Type II or Type I codebook.

The basis sufficiency indication can be reported in different ways

- Implicit indication via reporting NNZC=0 or N3'=0 in UCI part 1. In this case, UE may only report W1 matrix in UCI part 1 or drop UCI part 2.

- Explicit indication via a dedicated 1-bit field in UCI part 1. In this case, UE may only report W1 matrix in UCI part 1, or drop UCI part 2, or report a full CSI with UCI part 1 and part 2 following the configured values of L, p and β (or K0tot).

Hence, based on the discussion, we propose

Proposal 9: For R16 Type II codebook with FD compression, support a basis sufficiency indication in UCI part 1 with one of the following alternatives:

- Implicit indication via reporting NNZC=0 or N3'=0 in UCI part 1. In this case, UE may only report W1 matrix in UCI part 1 or drop UCI part 2.

- Explicit indication via a dedicated 1-bit field in UCI part 1. In this case, UE may only report W1 matrix in UCI part 1, or drop UCI part 2, or report a full CSI with UCI part 1 and part 2 following the configured values of L, p and β.

## Other UCI parameters

In this part, we discuss the design for other UCI parameters, including RI+NNZC indication, bitmap to indicate coefficients indices and SCI.

For the indication of NNZC, our proposal in the last meeting was supporting an indication of total number of non-zero coefficients across all layers. However, after further consideration, we do see some benefit of reporting per-layer NNZC. One benefit is performing CSI omission. As will be discussed in more detail in Section 4, layer-based CSI omission is preferred as it has minimal impact on the PMIs and CQIs. Hence, per-layer NNZC provides information related to the payload of each layer, which is easier for the UE and gNB understand how much payload of UCI part II is omitted.

There are two schemes to facilitate joint indication of rank and per-layer NNZC:

- Scheme 1: independent per-layer NNZC indication for each layer. That is, each layer uses a log2(K0+1)-bit (NNZC=0,1,…,K0 per layer). When rank-2 is reported, then UE may set the last two NNZC indicator equal to zero.

- Scheme 2: joint encoding of per-layer NNZC across all layers. The bitwidth is log2K0+2K02+2K03+2K04 if UE supports max rank-4. The mapping of a codepoint to RI and per-layer NNZC is as Table 2. The philosophy is similar to SRI, where a first set of codepoints are used to indicate the NNZC for rank-1, while a second set of codepoints are used to indicate the per-layer NNZC for rank-2, etc. The encoding process is based on the accumulated NNZC (denoted by yi) upto layer-i, and the accumulated NNZC y0<⋯<yi can be represented by a combinatorial number in 0, ⋯2K0i-1.

Table 2: Encoding process of scheme 2 for per-layer NNZC and RI indication

| Codepoint y | Rank | Encoding of KNZ,i,i=0,⋯,RImax-1 to 7 |
| --- | --- | --- |
| 0≤y≤K0-1 | 1 | y0=KNZ,i-1 and y=y0 |
| K0≤y≤2K02+K0-1 | 2 | Step 1: obtain y0=KNZ,0-1 and y1=KNZ,0+KNZ,1-1; Step 2: map (y0,y1) to a combinatorial value z∈0, ⋯2K02-1; Step 3: obtain y=z+K0 |
| 2K02+K0≤y≤2K03+2K02+K0-1 | 3 | Step 1: obtain y0=KNZ,0-1, y1=KNZ,0+KNZ,1-1, y2=KNZ,0+KNZ,1+KNZ,2-1; Step 2: map (y0,y1,y2) to a combinatorial value z∈0, ⋯2K03-1; Step 3: obtain y=z+K0+2K02 |
| 2K03+2K02+K0≤y≤2K04+2K03+2K02+K0-1 | 4 | Step 1: obtain y0=KNZ,0-1, y1=KNZ,0+KNZ,1-1, y2=KNZ,0+KNZ,1+KNZ,2-1 and y3=KNZ,0+KNZ,1+KNZ,2+KNZ,3-1; Step 2: map (y0,y1,y2,y3) to a combinatorial value z∈0, ⋯2K04-1; Step 3: obtain y=z+K0+2K02+2K03 |

The payload comparison is shown in Figure 4, where the x-axis is the value of K0 (from 8 to 48). We can see that scheme 2 always achieves lower overhead than scheme 1.

Figure 4. Payload comparison between scheme 1 and 2 for per-layer NNZC and RI indication

For the indication SCI, since per-layer NNZC is provided in UCI part 1, then it is straightforward to use log2KNZ,i-bits to indicate the index of the strongest coefficient of layer-i.

Among the alternatives for the bitmap used to indicate the indices of the coefficients, the multi-bitmap approach is too complicated and is not easier to perform CSI omission. For the alternative of using size-LMi bitmap for the layer where weaker polarization is dropped is an over-optimization for a corner case. Thus, using 2LMi bitmap is our preference.

Hence, based on the discussion, we propose

Proposal 10: For Rel-16 Type II codebook, support a per-layer NNZC indication without RI (Alt1.2), wherein the NNZC of each layer and the rank are jointly encoded via combinatorial indication, and the bitwidth is log2K0+r=2RImax2K0r.

Proposal 11: For Rel-16 Type II codebook, support SCI indicator based on per-layer NNZC (Alt3.1) and the bitwidth for layer-i is log2KNZ,i.

Proposal 12: For Rel-16 Type II codebook, support using size-2LMi bitmap (Alt2.1) to indicate the coefficients indices for layer-i.

# CSI omission

## Discussion on CSI omission rule

According to the setting of L, p and β, the overhead for Rel-16 type II CSI can be as large as more than 500 bits for 10MHz bandwidth with 13 subbands. For larger bandwidth, the overhead can be as large as 1000 bits. However, considering UL throughput requirement, the max TB size for a cell-edge UEs is around 500 bits, while the max TB size for a cell-center UE is around 1000 bits. When multiple CSI report is configured to transmit on one PUSCH, a UE may process the CSI calculation in parallel to meet the PUSCH timing. However, after the CSI calculation, the UE may identify that the allocated UL resource is insufficient to carry the full CSI of each CSI report. Hence, CSI omission is necessary.

One may argue that the CSI omission for Rel-16 Type II can be done by UE implementation via adjusting the rank and/or NNZC. However, in our view, the CSI omission is performed after the CSI calculation, and there would be no time left for the UE to recalculate the CSI by looking for a lower rank or lower NNZC. Another reason is that if CSI omission is achieved by UE implementation, the network would not know whether the lower rank/NNZC is due to CSI omission or a UE preferred CSI. Thus, it is important to have non-transparent CSI omission so that the network is able to schedule more UL resource for the next CSI report trigger. Based on the discussion, we believe CSI omission rule is needed to be defined in the spec.

Proposal 13: CSI omission rule for Rel-16 Type II codebook needs to be studied and defined in the spec.

Unlike Rel-15 Type II codebook wherein the linear coefficients are reported per subband, any single coefficient in Rel-16 type II is related to the PMI of all subbands. Hence, it is impossible to reuse Rel-15 CSI omission by subband decimation, and new CSI omission rule is needed. In high level aspect, since there would be very little time left for UE to transmit the CSI reports on PUSCH, the CSI omission shall only comprise simple operations like adjusting/removing (portion of) the UL payload to meet the target coding rate, but not including any operation requiring CSI recalculation. To this end, since CSI omission would change the PMI and lead to a mismatched CQI (if not updating the CQI), CSI omission rule should consider designs that have minimal impact to CQIs/PMIs.

Proposal 14: CSI omission rule for Rel-16 Type II codebook shall not require CSI recalculation, especially for CQI. The CSI omission rule should have minimal impact to CQIs/PMIs.

To achieve the minimal impact to CQIs/PMIs, a possible scheme is layer-based CSI omission. That is, the CSI omission starts from omitting all the components of the layer with the lowest priority order to all the components of the layer with the highest priority order. In this scheme, at least some complete layers would be reported to the network. Besides, the reported CQI (calculated based on the CSI without omission) is underestimated with the respect to the reported PMI. The reason is that the CQI supported by a higher rank (after omission) can be supported by a lower rank due to the higher power allocated on each layer. Based on the discussion, we propose

Proposal 15: Support layer-based CSI omission for Rel-16 Type II codebook.

## UCI packing order based on CSI omission rule

Another issue related to CSI omission is the packing order of CSI components. In general, there are two alternatives as shown in Figure.

- Alt1 (CSI component first, layer second): UE firstly packs all the CSI components including FD bases, bitmap for coefficient indication, SCI and quantization of NZCs of layer 0, secondly packs all the CSI components of layer 1, etc.

- Alt2 (layer first, CSI component second): UE firstly packs the FD bases of layer 0 to RI-1, secondly packs the bitmaps of layer 0 to RI-1, etc.

Figure 5. Alternatives of UCI packing order

In Rel-15 Type codebook, UE firstly packs all the CSI components associated with even subband, and secondly packs all the CSI components associated with odd subband. The motivation of this packing order is due to the CSI omission rule, as CSI omission in Rel-15 is based on subband decimation. In our view, we believe same philosophy should be applied to Rel-16. Hence, layer-based CSI omission rule is preferred, we support Alt1 (CSI component first, layer second) for UCI packing order.

Proposal 16: In Rel-16 Type II CSI, the CSI components should be packed into UCI part 2 first within a layer, then across layers.

# Number of FD Compression Units

Another remaining issue lies in the definition of N3 value (number of FD compression units) when the number of PMI SBs is greater than 13. In this case, to ease the computation efficiency, it is preferred the value of N3 being always powers of 2, 3, or 5 only, i.e., N3=2a⋅3b⋅5c, where a, b and c are non-negative integers. To this end, there are two alternatives illustrated in Figure 1 (padding/extrapolation) and 2 (segmentation).

Figure 6. Illustration of extrapolation

Figure 7. Illustration of segmentation

Although extrapolation/segmentation yields N3 being N3=2a⋅3b⋅5c, these two alternatives enhance the UE implementation and spec complexity as either of them provides a different implementation compared to the case where number of PMI subbands is smaller than or equal to 13. From this perspective, these two alternatives yield neither good performance gain nor simplified design. Hence, unified framework with N3=NSB×R regardless of its value should be studied as baseline.

Moreover, since the BWP may start at any RB, the edge CQI subbands may have smaller number of RBs than the nominal CQI subband size. This issue is fine with Rel-15 Type II codebook as the CSI is reported per subband. However, for Rel-16 Type II codebook with frequency domain compression, following issues need to be considered.

- The PMI computed at edge subbands (before compression/quantization) may degrade the continuity of the PMIs across frequency domain. This would impact the performance after FD compression, as the discontinuity at edge subband may cause large ripple in the compressed domain (e.g., time domain).

- When R=2, some edge CQI subband may have only one PMI subband, thus resulting in an odd number for the total number of PMI subbands. In this case, extrapolation is needed to meet N3=NSB×R as N3 is an even number when R=2. This adds extra UE implementation complexity.

To address above two issues, any CQI subband configured for Rel-16 Type II CSI shall be always a nominal subband which contains a number of RBs equal to the configured CQI subband size. Based on the above discussion, we propose

Proposal 17:  Support a unified framework to determine the value of N3 regardless its value, i.e., N3=NSB×R, where any CQI subband configured for Rel-16 Type II CSI shall be always a nominal subband which contains a number of RBs equal to the configured CQI subband size.

# Conclusion

In this contribution, we discuss the open issues related to type II CSI enhancement. Based on the following observation,

Observation 1: Alt2B and Alt3C with M2=M/2 achieves similar performance with Alt6E.

Observation 2: Alt1 with K0 max NNZC per layer achieves 1~2% loss compared to Alt0 with no constraint on max NNZC per layer.

Observation 3: UE may employ a layer-specific starting point in calculating the W2 matrix and the window-based intermediate set, but always report the intermediate set and FD basis associated with a window with FD basis 0,1,⋯,N3'-1.

Observation 4: Window-based approach achieves similar performance as the combinatorial-based approach.

Observation 5: Combinatorial-based intermediate set may need more overhead than one-stage FD basis report.

We propose,

Proposal 1: For the setting of number of FD basis selection, support Alt2B (first preference) or 3C (second preference), and FFS the value of M2.

Proposal 2: In Rel-16 Type II codebook, support Alt0 where UE freely assign 2K0 coefficients across all layers with no restriction on per-layer NNZC.

Proposal 3: For RI={3,4} design of Rel-16 type II codebook with FD compression, further study schemes that exploit orthogonality among layers and unit norm of each layer.

Proposal 4: Support two-stage FD basis report for Rel-16 Type II codebook with FD compression, wherein

- The size of the intermediate set is RI-common and layer-common

- The intermediate set is RI-common and layer-common

Proposal 5: For the size of the intermediate set used in the 1st stage of two-stage FD bases report, support

- UE reporting intermediate set N3' (1st preference); or

- The intermediate set size N3' being derived from the max number of FD bases across all layers of all ranks, e.g., N3'=α×M (2nd preference).

Proposal 6: Support window-based intermediate set for the two-stage basis report, wherein the starting index of the window is fixed.

Proposal 7: For the layer-specific FD basis report in the second stage, the downselection between bitmap and combinatorial indicator should consider the value of N3'.

Proposal 8: For the layer-specific FD basis report in the second stage,

- Prefer bitmap over combinatorial indicator

- Support the indicator selecting Ml basis from the N3' basis specified by the intermediate set, e.g., N3'-bit bitmap or log2N3'Ml-bit combinatorial indicator.

Proposal 9: For R16 Type II codebook with FD compression, support a basis sufficiency indication in UCI part 1 with one of the following alternatives:

- Implicit indication via reporting NNZC=0 or N3'=0 in UCI part 1. In this case, UE may only report W1 matrix in UCI part 1 or drop UCI part 2.

- Explicit indication via a dedicated 1-bit field in UCI part 1. In this case, UE may only report W1 matrix in UCI part 1, or drop UCI part 2, or report a full CSI with UCI part 1 and part 2 following the configured values of L, p and β.

Proposal 10: For Rel-16 Type II codebook, support a per-layer NNZC indication without RI (Alt1.2), wherein the NNZC of each layer and the rank are jointly encoded via combinatorial indication, and the bitwidth is log2K0+r=2RImax2K0r.

Proposal 11: For Rel-16 Type II codebook, support SCI indicator based on per-layer NNZC (Alt3.1) and the bitwidth for layer-i is log2KNZ,i.

Proposal 12: For Rel-16 Type II codebook, support using size-2LMi bitmap (Alt2.1) to indicate the coefficients indices for layer-i.

Proposal 13: CSI omission rule for Rel-16 Type II codebook needs to be studied and defined in the spec.

Proposal 14: CSI omission rule for Rel-16 Type II codebook shall not require CSI recalculation, especially for CQI. The CSI omission rule should have minimal impact to CQIs/PMIs.

Proposal 15: Support layer-based CSI omission for Rel-16 Type II codebook.

Proposal 16: In Rel-16 Type II CSI, the CSI components should be packed into UCI part 2 first within a layer, then across layers.

Proposal 17:  Support a unified framework to determine the value of N3 regardless its value, i.e., N3=NSB×R, where any CQI subband configured for Rel-16 Type II CSI shall be always a nominal subband which contains a number of RBs equal to the configured CQI subband size.

# Appendix

Table 6, Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Number of RB per subbands |  | 8 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Maximum 12 layers for MU-MIMO |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50% |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |

# References

- Chairman’s notes RAN1#96bis Final

- R1-1905629 Feature lead summary for MU-MIMO CSI – revision on selected issues, Samsung.

- R1-1905741 Feature lead summary for MU-MIMO CSI offline email discussion on N3, Samsung.

- R1-1905724 Feature lead summary for MU-MIMO CSI Tuesday offline session, Samsung.

- R1-1906966 R16 summary MUCSI offline email – UCI final, Samsung.

- R1-1906967 R16 summary MUCSI offline email – FD basis final, Samsung.