3GPP TSG-RAN WG1 #97	R1-1907303

May 13th – 17th, 2019

Reno, Nevada, U.S.A

Agenda item:	7.2.13.1

Source: 	Qualcomm Incorporated

Title: 	Uplink Power Control Solutions for NR-NR Dual Connectivity

Document for:	Discussion/Decision

# Introduction

In RAN-P #81, the work item on multi-RAT dual-connectivity and carrier aggregation enhancements was approved. One of the objectives of this work item is to devise uplink power control schemes to support synchronous as well as asynchronous NR-NR dual connectivity (NN-DC):

- Support of asynchronous and synchronous NR-NR Dual Connectivity [RAN1, RAN2, RAN4]

  - UE power control [RAN1]

  - RRC signalling to support of enhanced NR-NR DC [RAN2]

  - Core requirements to support enhanced NR-NR DC [RAN4]

Note: Synchronous DC enhancements in this WID considers only cases not covered in Rel-15 exception sheet for NR WI NR_newRAT-Core.

In this section, we first briefly summarize the CA and DC power control schemes adopted in NR Rel. 15, and mention their core differences with the scenario of interest in Rel. 16 dual connectivity. The next section then presents one viable solution for managing uplink power control for NN-DC in Rel. 16 NR.

In Rel. 15 NR, power control schemes for the following deployment scenarios were introduced:

- NN-DC with one cell group fully in FR1 and one cell group fully in FR2

- NR carrier aggregation

- EUTRA and NR dual connectivity (EN/NE-DC)

As mentioned, the Rel. 15 NN-DC deployment considers the case where all the serving cells of one cell group are fully in FR1, while all the serving cells of the other cell group are fully contained in FR2. Since there is no maximum power limit defined across the cells in FR1 and FR2, the UE performs uplink power control independently across the two cell groups. However, the scenario of interest for NR Rel. 16 includes the deployments, where both cell groups contain only serving cells in FR1. Hence, joint maximum power limitation may be considered.

For NR-CA, the uplink power control is decided on a per occasion basis. At each occasion, the UE considers the overlapping channels, their requested powers and their priorities. If the sum power across all serving cells is beyond the maximum allowed power, the uplink power for some channels, based on the priority, should be scaled down. Hence, some uplink channels may experience phase discontinuity. However, it should be noted that whether the channels overlap or experience phase discontinuity is controlled by the gNB; all serving cells are managed by a single gNB. On the contrary, in a dual-connectivity deployment, the gNBs may not be aware of each other’s scheduling decisions; hence, uplink channels might be interrupted even without the associated gNB knows about it or has an option to avoid it. Taking this key difference between CA and DC into account, the uplink power control for Rel. 16 NN-DC should guarantee that the uplink transmissions for each cell group will not be interrupted due to a transmission in another cell group at all times.

Finally, the uplink power control schemes devised for EN-DC and NE-DC in NR Rel. 15 rely on a fact that LTE has a longer processing delay than NR. Hence, for EN-DC (where LTE has a higher priority), the NR transmission power should be controlled to meet the max. allowed power limit when colliding with an LTE channel. For NE-DC (where NR has a higher priority), the LTE maximum allowed power is controlled semi-statically depending on whether collision can potentially take place or not; however, when collision happens and the required power across the serving cells is larger than the joint power constraint, the power of the NR channels should be scaled down. In Rel. 16 NN-DC, on the other hand, the UE has a comparable processing latency on both cell groups in case both cell groups are used for uplink data transmissions. In such a case, it is not possible to assume that uplink transmissions in one cell group can always be dynamically power controlled to ensure meeting the maximum power limitation.

Table 1 below summarizes the power control schemes of Rel. 15 CA/DC and their differences with the framework needed for NN-DC under NR Rel. 16.

Table 1: Summary of the NR Rel. 15 CA/DC power control schemes and key differences with Rel. 16 NN-DC.

| Power control for … | NR Rel. 15 Procedure | Differences with Rel. 16 NN-DC |
| --- | --- | --- |
| NR-CA | At each occasion, the UE scales down its power based on the priority rule to ensure Pmax is satisfied. All cells are controlled by the same gNB; hence, interruptions are manageable/avoidable. | In DC, cells do not know about each other’s scheduling decisions. Following the CA behavior with no modification leads to interrupting the uplink transmission of one CG without the gNB’s knowledge. |
| EN-DC | Semi-static and dynamic power control schemes are adopted. Under the dynamic scheme, a UE is configured with P_LTE and P_NR. If P_LTE+P_NR > P_ENDC, the UE has to scale down its NR power when collision happens. The SCG has a lower priority, and it is faster; hence, it can adapt its power based on the LTE decision. | In Rel. 16 NN-DC, a UE has comparable processing latency on both cell groups. It is not possible to always adapt one cell group’s power based on the decision made by the other one. |
| NE-DC | NR has a higher priority; P_LTE is modified depending on whether collision may happen or not. If P_NEDC is not satisfied, NR power should be scaled down. |  |
| NN-DC | NN-DC using FR1+FR2 with independent power control across the cell groups. | Rel. 16 NN-DC assumes cell groups with FR1+FR1 or FR2+FR2; power control decisions are dependent across the cell groups. |

In the subsequent sections, we first discuss the benefits and drawbacks of the semi-static power sharing and dynamic power sharing, and then considering the points raised here, we propose a promising solution for uplink power control of Rel. 16 NN-DC.

# Semi-Static Versus Dynamic Power Sharing for NN-DC

Under a semi-static power sharing scheme, the total allowed power across all the serving cells is semi-statically split between the two cell groups such that the total UE’s transmit power is always below or equal to the total allowed transmit power. Hence, it is always guaranteed that at no time, the uplink transmission in one cell group is interrupted by another uplink transmission, dynamically scheduled, in the other cell group. As a consequence,power scaling due to simultaneous transmissions in the other cell group is not needed, and each transmission’s phase continuity can be preserved. This is an essential feature of a semi-static power sharing scheme for dual-connectivity deployment; due to the absence of coordination across the gNBs, with semi-static power sharing, the network operation is predictable. Another important aspect to consider is that under a semi-static power sharing, uplink link adaptation can be performed efficiently. A gNB decides the uplink MCS based on its local information about the UE’s channel conditions and available power. With such information, uplink MCS can be set accordingly.

An alternative solution for uplink power control in a dual-coonectivity deployment is to allow for dynamic power sharing across the cell groups. Under this scheme, the total allowed power can be shared across the two cell groups dynamically. As an example, each cell group may be assigned a minimum reserved power to protect, e.g., PUCCH carrying HARQ-ACK. This power is guaranteed for each cell group, and cannot be recycled by the other cell group. As long as the uplink transmission power remains within this boundary, the transmission is not impacted by the concurrent transmission in the other cell group. However, if the gNB decides to request for a larger amount of power, it is not guaranteed whether the transmission remains uninterrupted. As an example, assume that PUSCH A is scheduled with uplink power P_A > P_res,2 in cell group 2. Another uplink transmission, PUSCH B, with a higher priority is then triggered in the other cell group, and is requested to be sent with uplink power P_B > P_res,1 such that P_A+P_B > P_tot. In such a case, the power of PUSCH A needs to be scaled down in the middle of the transmission. Any power re-scaling in the middle of the transmission can cause phase discontinuity, i.e., the DMRS and data symbols may be out of phase. In such a case, the uplink transmission cannot be decoded. As another example, the minimum reserved power could be sufficient for the transmission of PUCCH carrying HARQ-ACK. However, the gNB may schedule a PUSCH overlapping with PUCCH. In such a case, HARQ-ACK should be multiplexed on PUSCH. Then, the required power for PUSCH carrying HARQ-ACK might be above the minimum reserved power. Since allocating a power above the minimum reserved power increases the risk of experiencing phase discontinuity, the HARQ-ACK reliability may be impacted. The main drawback of the dynamic power sharing is that the network opration is not predictable, the interruptions are not under the full control of the scheduling gNB, and cannot be avoided or planned. Hence, adopting any dynamic power sharing scheme for NN-DC is not desirable.

Additionaly, asynchronous DC is supportable with an architecture that has distinct hardware blocks mappable to the two cell groups. It must be a fundamental assumption that the two hardware blocks used for asynchronous DC are clearly separable, i.e., there is no cross-CG HARQ dependency, no cross-CG max data rate sharing, no cross-CG CSI triggering, etc. This means that in the asynchronous DC mode, there is no pre-existing demand to have a fast interface across the blocks carrying dynamic information. Given that the gains of dynamic power sharing are unknown (and in fact, it can degrade system performance), adding this functionality is not justifiable.

Proposal 1: Due to the extra complication for UE implementation, dynamic power sharing should not be supported for Rel. 16 NN-DC.

# Uplink Power Control Solution for Rel. 16 NN-DC

As mentioned in the preceding section, a practical scheme for uplink power control should allow a gNB to make scheduling decisions independently. Satisfying this condition is desirable due to the fact that the gNBs are not necessarily aware of each others scheduling decisions (as discussed in the preceding section, this is the key difference between CA and DC.) Under condition (2), each gNB can make its scheduling decisions independently, and is certain that the UE’s uplink transmissions are not interrupted unless controlled by the same gNB. The following solution allows for predictable network operation:

Under the proposed solution, from the network side, the following step should be taken for each UE:

- A UE should be configured with a set of maximum allowed powers for each cell group such that P_NR1 + P_NR2 <= P_NN.

- Additionally, for synchronous deployments, the UE can be configured with P_NR1’ and P_NR2’, which can each be equal to P_NN. In other words, P_NR1’+P_NR2’ < = > P_NN.

From the UE side, the following behavior is expected:

- In synchronous DC, For the uplink transmission in cell group 1, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of cell group 2. If the direction of all the symbols is downlink, then the UE is certain that there will be no overlapping uplink transmission on the other cell group. Hence, it chooses its maximum power associated with the serving group from set 1. Otherwise, if at least one symbol of any of the serving cells of the other serving group is configured as `uplink’ or `flexible’, then the UE selects the maximum power from set 2.

- In asynchronous DC, the UE uses the maximum allowed power per group and manages its uplink transmission within each group similar to Rel. 15 CA.

Figure 1: An illustration of the proposed uplink power control scheme in synchronous DC.

In the example shown in Figure 1, the semi-statically configured slot formats on serving cells of the first cell group are illustrated. Then, on CC3 and for PUSCH1, the direction of all the overlapping symbols of serving cells in CG1 is set to downlink. Hence, at the time of transmission of PUSCH1, the UE is certain that PUSCH1 does not collide with any uplink channel on CG1; thus, its uplink transmission power could be up to P_NR2’. Unlike PUSCH1, PUSCH2 collides with some symbols that are set as flexible on CC2 of CG1. Hence, collision could potentially takes place. For this reason, the maximum allowed power at the time of PUSCH2 transmissioncould be only up to P_NR2..

As evident from the description of the proposed scheme, for setting the maximum power per cell group, the UE can only rely on the semi-static configuration of the other serving cell in the synchronous DC and does not need any information about the other cell group in the asynchronous DC; thus, a tight interaction between the two serving cells at the UE side is not required. From the gNB’s point of view, the scheduling decisions and power settings are similar to NR CA, i.e., collisions and the resulting power scaling/dropping of the uplink channel in each group is comepletely under the control of the associated gNB. For these two reasons, the proposed scheme is favorable from both the UE and gNB’s point of view. Further, the proposed scheme provides sufficient flexibility for the network to balance the priority of the two groups by appropriately setting the maximum powers in each group.

Based on the above discussions, we propose:

Proposal 2: For uplink power control of synchronous NN-DC, the UE is configured with:

- A UE should be configured with a set of maximum allowed powers for each cell group such that P_NR1 + P_NR2 <= P_NN.

- Additionally, for synchronous deployments, the UE can be configured with P_NR1’ and P_NR2’, which can each be equal to P_NN. In other words, P_NR1’+P_NR2’ < = > P_NN.

From the UE side, the following behavior is expected:

- In synchronous DC, For the uplink transmission in cell group 1, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of cell group 2 and vice versa. The UE chooses the maximum power allowed per cell group based on whether its uplink transmission collides with only semi-static ‘DL’ symbols, or with semi-static ‘UL’ and ‘flexible’ symbols.

- In asynchronous DC, the UE uses the maximum allowed power per group and manages its uplink transmission within each group similar to Rel. 15 CA.

# Analysis of Dynamic Power Control Schemes

In RAN1 #96b, the proposed dynamic power control schemes were captured in [1], and copied below:

If supported, the following is considered for dynamic power sharing schemes for NR DC: - Alt.1: - If a total UE transmit power for UL transmissions exceeds the maximum transmission power determined above, UE allocates power to UL transmissions according to the following priority order - Alt.1-1: Reuse the channel priority order defined for Rel-15 CA case 2. - Alt.1-2: Introducing new priority scheme(s) - Candidate-1: To maintain a constant transmission power of a channel/signal in a CG. - Candidate-2: To additionally consider the URLLC (or such high reliability) or random-access procedure, different PUCCH formats or PUSCH types - Alt.1-3: The sum transmit power per cell group is calculated independently. In case the UE is power limited, the sum power of SCG is scaled down. The sum power of MCG and SCG is then distributed across their channels by following the Rel. 15 NR CA within each CG independently. - FFS: UE is configured with maximum available power for each cell group, e.g., P_max_MCG, P_max_SCG and P_max_MCG + P_max_SCG > P_tal_max. - FFS how to handle the case where the UE is not power-limited for transmitting PUCCH, but it is limited for transmitting PUSCH with UCI. - Alt.2: - UE is configured with transmit power limits P_limit_CG1 applicable to CG1 and P_limit_CG2 applicable to CG2. - To compute the transmit power for CG1 UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping CG2 UL transmission, and if such PDCCH(s) are detected, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1; - UE also checks (based on RRC parameters) if any possible overlapping CG2 UL transmissions can be triggered by PDCCH(s) after T0-T_offset, and if such overlapping CG2 UL transmissions are possible, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1; - Otherwise, P_limit_CG1 is not applied and UE transmit power for the CG1 UL transmission can be up to ‘full power’ allowed by RAN4 requirements - T_offset can be similar to the T_proc specified in Rel15 for UCI multiplexing related UE procedures. - If NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2 > P_limit_total, and if pwr_CG1+ pwr_CG2> P_limit_total, UE scales down transmission power of SCG transmission(s) such that UE transmission power across MCG and SCG does not exceed P_limit_total. - Note: if NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2<= P_limit_total, the above SCG scaling behavior is not required. - P_limit_total is the power limit for all transmissions in both CG1 and CG2 and will be defined in RAN4 - Alt.3: - UE is configured with a minimum guaranteed power for MCG and SCG separately. - To compute the transmit power for CG1 UL transmission starting at time T0, a UE considers a power for later scheduled overlapping transmissions T0-T_offset, - FFS on T_offset (e.g. subject to the same timelines as for determining a channel among overlapping channels for UCI multiplexing in Rel-15) - Alt.3-1: - A power limited UE applies Rel-15 CA power allocation prioritization rules for transmissions in different CGs. In case of a same prioritization, power allocation is prioritized for transmissions on the MCG. - FFS for URLLC, different PUCCH formats or PUSCH types - Alt.3-2: - For power determination of any given UL transmission on a CG in NN-DC, the UE does not adjust power to “past” UL transmission, and allocates remaining power for the higher-priority “concurrent” UL transmissions, while respecting the minimum guaranteed power (MGP) limits. Once the transmit power for a transmission is decided, the UE will not re-calculate/re-adjust the transmit power based on upcoming transmission (regardless of priority levels). - Alt.4: - Two types of power ratios can be configured for each CG - Low power ratio L is used for guaranteeing the minimum residual power for each CG. - High power ratio H is used for restricting the maximum power portion when both CGs are requiring more power than its own high ratio - Defining a “cut-off” time of look-ahead operation as an offset before the first symbol T0 of one uplink transmission - Determine the CG prioritization by RRC time pattern or by the dynamic UL grant information. - If MCG is prioritized, the high power ratio for MCG is max{H_SCG , 1-H_SCG } and the high power ratio for SCG is min{H_SCG , 1-H_SCG }; - If SCG is prioritized, the high power ratio for MCG is min{H_MCG , 1-H_MCG } and the high power ratio for SCG is max{H_MCG , 1-H_MCG }; - Alt.5: - Reuse the principle of the uplink power sharing scheme of EN-DC: - MCG has a higher priority than SCG - FFS: Support DPS in both synchronous and asynchronous NR-DC cases or synchronous NR-DC only. - FFS: Maintain a constant transmission power of a channel/signal in a CG in power limited case. - FFS: Account for the URLLC (or such high reliability) or random-access procedure, different PUCCH formats or PUSCH types for defining priority order for power limited case.

In the remainder of this section, we provide some considerations on the proposed dynamic power control schemes.

Alt1-1, Alt1-2 and Alt1-3: Under Alt1-1 and Alt1-2, the UE has to check the type of the channels and their contents on both CGs, and go through a list of priority rules that alternates between the two CGs. This approach will incur a ping-pong impact between the two cell group and may complicate the UE implementation. Alt1-3, however, circumvent this issue since there is only a one-time exchange of information is expected between the two cells.

Alt2, Alt3 and Alt4: These options enable a look-ahead operation by defining timeline rules similar to those of the Rel. 15 UCI multiplexing scheme. In addition to the fact that the performance gains from allowing the look-ahead operation are unknown, these approaches complicate the UE implementation unnecessarily. Additionally, in order to keep the phase continuity across the overlapping channels, a certain timeline should be satisfied. However, since the uplink transmissions are scheduled by different gNBs, it is not always possible to ensure the timeline; in other words, the entire operation is opportunistic.

Alt5: This approach is proposed under the assumption that the main use case of NR-NR DC is to use the MCG for control and monibility, while the SCG is used for data transmission. In such a case, it should be possible to “slow down” the UE processing on the MCG such that its uplink power decisions are available at the SCG when the UE has to allocate the power to SCG’s uplink transmissions. In particular, this approach is similar to Rel. 15 EN-DC power control design.

The UE’s processing time on the MCG can be slowed down by increasing the its minimum K1/K2 values. These values can be exchanged bwtween the two gNBs semi-statically and used as an input to their scheduling algorithm.

This approach also requires enabling information exchange between the two cell groups; however, the information can be exchanged at a lower rate as compared to the previous schemes depending on the processing timeline of the MCG.

# Conclusion

This contribution paper discussed a power control scheme applicable to Rel. 16 NR-NR dual connectivity under the synchronous and asynchronous deployments. Based on the discussions in the paper, the following proposals are made:

Proposal 1: Due to the extra complication for UE implementation, dynamic power sharing should not be supported for Rel. 16 NN-DC.

Proposal 2: For uplink power control of synchronous NN-DC, the UE is configured with:

- A UE should be configured with a set of maximum allowed powers for each cell group such that P_NR1 + P_NR2 <= P_NN.

- Additionally, for synchronous deployments, the UE can be configured with P_NR1’ and P_NR2’, which can each be equal to P_NN. In other words, P_NR1’+P_NR2’ < = > P_NN.

From the UE side, the following behavior is expected:

- In synchronous DC, For the uplink transmission in cell group 1, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of cell group 2 and vice versa. The UE chooses the maximum power allowed per cell group based on whether its uplink transmission collides with only semi-static ‘DL’ symbols, or with semi-static ‘UL’ and ‘flexible’ symbols.

- In asynchronous DC, the UE uses the maximum allowed power per group and manages its uplink transmission within each group similar to Rel. 15 CA.

# References

[1] R1-1905839, “Feature lead summary on uplink power control for supporting NR-NR dual-connectivity,” Intel Corp.