3GPP TSG RAN WG1 #97	R1-1907312

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.7.2

Source:	ITRI

Title:	TDD and FDD Evaluation results of Dense Urban-eMBB for IMT-2020 self-evaluation

Document for:	Discussion and Decision

Introduction

NR supports both FDD and TDD duplex modes in FR1 and only TDD mode in FR2. In FDD system, UE should feedback the DL channel state information, e.g. RI, PMI, CQI, for gNB to determine the DL precoding matrix and MCS. The DL transmission for FDD system is usually a PMI based transmission. However due to the channel reciprocity of unpaired spectrum, TDD system can obtain DL channel information by using uplink SRS signal. Compared with the FDD PMI based DL transmission, TDD system can determine the non-PMI based DL precoder via the uplink SRS signal, which can capture the more accurate DL channel information without doing channel quantization. In this contribution, we provide the spectral efficiency results of Dense Urban-eMBB scenario for both TDD and FDD systems and compare the SINR performance difference of PMI and non-PMI based transmission.

Non-PMI and PMI based Transmission

When FDD system uses PMI based transmission, gNB periodically transmits non-precoded CSI-RS for UE to estimate DL channel. Then UE derives RI, PMI and CQI based on the estimated channel and feedbacks the corresponding CSI. The gNB can determine the PDSCH MCS based on CQI and precoding matrix based on RI and PMI. The DL PMI based transmission procedure for FDD system is illustrated in Figure 1(a). The detailed procedure steps is listed below:

Step 1: BS periodically transmits non-precoded CSI-RS.

Step 2: UE periodically measures non-precoded CSI-RS and derived RI, PMI for CSI feedback.

Step 3: UE derives the corresponding CQI based on the RI, PMI.

Step 4: BS transmits PDSCH based on the UE feedback information RI, PMI and CQI.

When TDD system uses Non-PMI based transmission, UE will transmits non-precoded SRS to gNB. gNB uses the channel reciprocity to get DL precoder by doing SVD to the DL channel matrix. Then gNB transmits the precoded CSI-RS for UE to estimate RI and CQI. The gNB can determine the PDSCH MCS based on CQI and transmitted layers by RI. The DL Non-PMI based transmission procedure for TDD system is illustrated in Figure 1(b). The detailed procedure steps is listed below:

Step 1: UE periodically transmits non-precoded SRS.

Step 2: BS periodically measures non-precoded SRS and get DL channel matrix information (Using UL/DL channel reciprocity).

Step 3: According DL matrix, BS can determine precoding matrix.

Step 4: BS transmits precoded CSI-RS using N “eigen-vector”.

Step 5: UE measures on N CSI-RS port(s) to derive equivalent channel, and then derive rank L (L≤N) and corresponding CQI.

Step 6: BS transmits PDSCH based on the information of RI, precoding and CQI.

Figure 1 (a) DL PMI based transmission for FDD system. (b) DL Non-PMI based transmission for TDD system.

Evaluation Results

In this section, we provide our evaluation results. There are two types of antenna configurations are considered in our simulation cases:

- Case 1 : 32Tx/4Rx

- Case 2 : 4Tx/4Rx

The number of antenna elements for both cases are the same 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1). Only the antenna elements to TXRU mapping are different. The assumptions of antenna configuration case 2 (4Tx/4Rx) is based on the Dense Urban config. A in calibration phase [1]. It should be noted that there are total Tx. 12 beams (6 horizontal beam and 2 vertical beams) considered in case 2 and only one fixed beam in case 1. When UE attaches to the gNB with analog beam selection, the best beam pair link is selected to receiver and transmission for each UE. The detailed beam direction information of case 1 and 2 can be found in Annex.

The updated evaluation results of average spectral efficiency and 5th percentile user spectral efficiency are showed in Table 1 and 2. Both NR FDD and NR TDD are evaluated. The detailed evaluation assumptions can be found in Annex.

Table 1. DL Spectral Efficiency

| Test Environment |  | Antenna configuration | Average spectral efficinency (bps/Hz/TRxP) |  | 5th percentile user spectral efficiency (bps/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Result | Req. | Result | Req. |
| Dense Urban – eMBB Config A | FDD | 32Tx/4Rx | 10.6232 | 7.8 | 0.2414 | 0.225 |
|  |  | 4Tx/4Rx | 15.3431 |  | 0.3378 |  |
|  | TDD | 32Tx/4Rx | 15.2403 |  | 0.2895 |  |
|  |  | 4Tx/4Rx | 14.4995 |  | 0.3618 |  |

Table 2. UL Spectral Efficiency

| Test Environment |  | Antenna configuration | Average spectral efficinency (bps/Hz/TRxP) |  | 5th percentile user spectral efficiency (bps/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Result | Req. | Result | Req. |
| Dense Urban – eMBB Config A | FDD | 4Tx/32Rx | 10.6196 | 5.4 | 0.3815 | 0.15 |
|  |  | 4Tx/4Rx | 9.9777 |  | 0.2338 |  |
|  | TDD | 4Tx/32Rx | 9.4917 |  | 0.3579 |  |
|  |  | 4Tx/4Rx | 9.6008 |  | 0.1712 |  |

Observation 1: Both FDD and TDD fulfill the minimum average spectral efficiency requirements and 5th percentile user spectral efficiency requirements for Dense Urban-eMBB Configuration A.

5dB

Figure 2. Dense Urban-eMBB Downlink SINR

Figure 2 shows the downlink post SINR for both 4Tx/4Rx and 32Tx/4Rx cases. Comparing the DL SINR of 4Tx/4Rx and 32Tx/4Rx, it can be seen that antanna configuration with multiple beam can effectively increase DL SINR. Thus the result of average spectral efficiency in Table 3 has a better performance. Comparing the DL SINR of TDD and FDD, the SINR of TDD non-PMI based transmission has significant gain especially in 32Tx/4Rx case.

Based on the discussion in section 2, we have the following observations:

Observation 2: The DL SINR of TDD non-PMI based transmission has about 5dB gain than FDD PMI based transmission.

Observation 3: The more digital antenna ports (e.g. 32), the SINR gain of TDD mode will be more significant.

In addition, Table 3 and Table 4 shows the details of number of DL transmission layer and UL transmission layer, respectively.

Table 3. Number of DL transmission layer

| Test Environment |  | Antenna configuration | Layer 1 | Layer 2 | Layer 3 | Layer 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban – eMBB Config A | FDD | 32Tx/4Rx | 12.807 % | 28.421 % | 58.772 % | 0 % |
|  |  | 4Tx/4Rx | 13.859 % | 39.474 % | 46.667 % | 0 % |
|  | TDD | 32Tx/4Rx | 5.964 % | 10.878 % | 81.929 % | 1.229 % |
|  |  | 4Tx/4Rx | 18.597 % | 45.965 % | 35.438 % | 0 % |

Table 4. Number of UL transmission layer

| Test Environment |  | Antenna configuration | Layer 1 | Layer 2 | Layer 3 | Layer 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban – eMBB Config A | FDD | 4Tx/32Rx | 10.351 % | 33.508 % | 56.141 % | 0 % |
|  |  | 4Tx/4Rx | 24.736 % | 61.755 % | 13.509 % | 0 % |
|  | TDD | 4Tx/32Rx | 14.386 % | 35.087 % | 50.527 % | 0 % |
|  |  | 4Tx/4Rx | 36.666 % | 52.982 % | 10.352 % | 0 % |

In NR TDD downlink, more than 81% users use layer 3 and layer 4 that cause the result of average spectral efficiency with antenna configuration 32Tx/4Rx is better than 4Tx/4Rx. Spectral efficiency increase when the percentage of high layer is increasing. It can also be observed in NR FDD UL.

From this perspective, the following observation is obtained.

Observation 4: The rank adaption could have impacts on the performance of spectral efficiency.

Summary

In this contribution, we provide our updated evaluation results for Dense Urban-eMBB Configuration A. We have the following observations:

Observation 1: Both FDD and TDD fulfill the minimum average spectral efficiency requirements and 5th percentile user spectral efficiency requirements for Dense Urban-eMBB Config. A.

Observation 2: The DL SINR of TDD non-codebook based transmission has about 5dB gain than FDD codebook based transmission.

Observation 3: The more digital antenna ports (e.g. 32), the SINR gain of TDD mode will be more significant.

Observation 4: The rank adaption could have impacts on the performance of spectral efficiency.

References

- RP-180524, “Summary of calibration results for IMT-2020 self evaluation” , Huawei, March 2018

- Annex

| Dense Urban – eMBB |  | Config. A (4 GHz); 4T4R | Config. A (4 GHz); 32T4R |
| --- | --- | --- | --- |
| Carrier frequency for evaluation |  | 1 layer (Macro) with 4 GHz |  |
| Numerology |  | 15kHz SCS |  |
| Bandwdith |  | FDD: 10 MHz TDD: 20 MHz |  |
| Total transmit power per TRxP |  | 41 dBm |  |
| UE power class |  | 23 dBm |  |
| Percentage of high loss and low loss building type |  | 20% high loss, 80% low loss (applies to Channel model B) |  |
| Inter-site distance |  | 200 m |  |
| Number of antenna elements per TRxP |  | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |  |
| Number of TXRU per TRxP |  | 4TXRU, (Mp,Np,P,Mg,Ng)= (2,1,2,1,1) | 32TXRU, (Mp,Np,P,Mg,Ng)= (2,8,2,1,1) |
| Number of UE antenna elements |  | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |  |
| Number of TXRU per UE |  | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |  |
| Device deployment |  | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |  |
| UE speeds of interest |  | Indoor users: 3 km/h Outdoor users (in-car): 30 km/h |  |
| Traffic model |  | Full buffer |  |
| UE density |  | 10 UEs per TRxP |  |
| UE antenna height |  | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |
| Electronic tilt |  | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) | For direction of TRxP analog beam steering (in LCS): Zenith angle θj = [105°] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |
| Power control |  | α=0.9， P0=-86 dBm | α=0.9， P0=-105.4 dBm |
| UT attachment |  | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |  |
| Wrapping around method |  | Geographical distance based wrapping |  |
| Polarized antenna model |  | Model-2 in TR36.873 |  |
| Scheduler |  | DL: SU PF UL: SU PF |  |
| Duplexing |  | FDD / TDD |  |
| Slots structure |  | TDD : DDDSU |  |
| Modulation |  | Up to 256 QAM |  |
| Downlink Overhead | SS/PBCH block | 1 SS/PBCH block per 20ms |  |
|  | PDCCH | 2 OFDM symbols per slot |  |
|  | DMRS (Type II) | 4, 8, 12 ports |  |
|  | CSI-RS for CM | 4, 8, 16, 32 ports with periodicity of 5ms |  |
|  | CSI-RS for IM | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots |  |
|  | TRS | 20ms period; maximal bandwidth with 52 PRB; burst length with 2 slots 12 RE/PRB/20ms |  |
| Uplink Overhead | PUCCH | 2 PRBs and 14 OS for the slots without SRS transmission; 2 PRBs and 12 OS for the slots with SRS |  |
|  | DMRS | Type II, 2 complete symbols |  |
|  | SRS | 2 OFDM symbols per 5 slots |  |
| Receiver |  | MMSE-IRC |  |
| CSI feedback |  | PMI, CQI: every 5 slot; RI: every 5 slot; Subband based |  |
| Channel estimation |  | Realistic channel estimation |  |