3GPP TSG RAN WG1 #97	R1-1907313

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.7.2

Source:	ITRI

Title:	TDD and FDD Evaluation results of Rural-eMBB for IMT-2020 self-evaluation

Document for:	Discussion and Decision

Introduction

NR supports both FDD and TDD duplex modes in FR1 and only TDD mode in FR2. In FDD system, UE should feedback the DL channel state information, e.g. RI, PMI, CQI, for gNB to determine the DL precoding matrix and MCS. The DL transmission for FDD system is usually a PMI based transmission. However due to the channel reciprocity of unpaired spectrum, TDD system can obtain DL channel information by using uplink SRS signal. Compared with the FDD PMI based DL transmission, TDD system can determine the non-PMI based DL precoder via the uplink SRS signal, which can capture the more accurate DL channel information without doing channel quantization. In this contribution, we provide the spectral efficiency results of Rural-eMBB scenario for both TDD and FDD systems and compare the SINR performance difference of PMI and non-PMI based transmission.

Non-PMI and PMI based Transmission

When FDD system uses PMI based transmission, gNB periodically transmits non-precoded CSI-RS for UE to estimate DL channel. Then UE derives RI, PMI and CQI based on the estimated channel and feedbacks the corresponding CSI. The gNB can determine the PDSCH MCS based on CQI and precoding matrix based on RI and PMI. The DL PMI based transmission procedure for FDD system is illustrated in Figure 1(a). The detailed procedure steps is listed below:

Step 1: BS periodically transmits non-precoded CSI-RS.

Step 2: UE periodically measures non-precoded CSI-RS and derived RI, PMI for CSI feedback.

Step 3: UE derives the corresponding CQI based on the RI, PMI.

Step 4: BS transmits PDSCH based on the UE feedback information RI, PMI and CQI.

When TDD system uses Non-PMI based transmission, UE will transmits non-precoded SRS to gNB. gNB uses the channel reciprocity to get DL precoder by doing SVD to the DL channel matrix. Then gNB transmits the precoded CSI-RS for UE to estimate RI and CQI. The gNB can determine the PDSCH MCS based on CQI and transmitted layers by RI. The DL Non-PMI based transmission procedure for TDD system is illustrated in Figure 1(b). The detailed procedure steps is listed below:

Step 1: UE periodically transmits non-precoded SRS.

Step 2: BS periodically measures non-precoded SRS and get DL channel matrix information (Using UL/DL channel reciprocity).

Step 3: According DL matrix, BS can determine precoding matrix.

Step 4: BS transmits precoded CSI-RS using N “eigen-vector”.

Step 5: UE measures on N CSI-RS port(s) to derive equivalent channel, and then derive rank L (L≤N) and corresponding CQI.

Step 6: BS transmits PDSCH based on the information of RI, precoding and CQI.

Figure 1 (a) DL PMI based transmission for FDD system. (b) DL Non-PMI based transmission for TDD system.

Evaluation Results

In this section, we provide our evaluation results. The updated evaluation results of average spectral efficiency and 5th percentile user spectral efficiency are showed in Table 1 and 2. Both NR FDD and NR TDD are evaluated. The detailed evaluation assumptions can be found in Annex.

Table 1. DL Spectral Efficiency Evaluation

| Test Environment |  | Antenna configuration | Average spectral efficinency (bps/Hz/TRxP) |  | 5th percentile user spectral efficiency (bps/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Result | Req. | Result | Req. |
| Rural-eMBB Config A | FDD | 8Tx/2Rx | 6.891 | 3.3 | 0.139 | 0.12 |
|  | TDD | 8Tx/2Rx | 8.661 |  | 0.189 |  |

Table 2. UL Spectral Efficiency Evaluation

| Test Environment |  | Antenna configuration | Average spectral efficinency (bps/Hz/TRxP) |  | 5th percentile user spectral efficiency (bps/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Result | Req. | Result | Req. |
| Rural-eMBB Config A | FDD | 2Tx/8Rx | 9.119 | 1.6 | 0.106 | 0.045 |
|  | TDD | 2Tx/8Rx | 8.509 |  | 0.097 |  |

Observation 1: Both FDD and TDD fulfill the minimum average spectral efficiency requirements and 5th percentile user spectral efficiency requirements for Rural-eMBB Config A.

Figure 2. Dense Urban-eMBB Downlink SINR

Based on the procedure in section 2, the DL SINR comparison of TDD Non-PMI based transmission and FDD PMI based transmission are presented in Figure 2.

The DL SINR of TDD Non-PMI based transmission is higher than FDD PMI based transmission. The main reason is TDD system can use the channel reciprocity to capture more accurate DL channel information without doing channel quantization. It can also be observed that the result of DL average spectral efficiency in Table 1 has a better performance.

Observation 2: The SINR of TDD Non-PMI based transmission is higher than FDD PMI based transmission.

In addition, Table 3 and Table 4 shows the details of number of DL transmission layer and UL transmission layer, respectively.

Table 3. Number of DL transmission layer

| Test Environment |  | Antenna configuration | Layer 1 | Layer 2 |
| --- | --- | --- | --- | --- |
| Rural-eMBB Config A | FDD | Tx/Rx | 96.667 % | 3.333 % |
|  | TDD | Tx/Rx | 97.719 % | 2.281 % |

Table 4. Number of UL transmission layer

| Test Environment |  | Antenna configuration | Layer 1 | Layer 2 |
| --- | --- | --- | --- | --- |
| Rural-eMBB Config A | FDD | Tx/Rx | 96.667 % | 3.333 % |
|  | TDD | Tx/Rx | 99.122 % | 0.878 % |

Summary

In this contribution, we provide our updated evaluation results for Rural-eMBB Config. A. We have the following observations:

Observation 1: Both FDD and TDD fulfill the minimum average spectral efficiency requirements and 5th percentile user spectral efficiency requirements for Rural-eMBB Config. A.

Observation 2: The SINR of TDD Non-PMI based transmission is higher than FDD PMI based transmission.

- Annex

| Rural – eMBB |  | Config A |
| --- | --- | --- |
| Carrier frequency for evaluation |  | 700 MHz |
| Total transmit power per TRxP |  | 46 dBm for 10 MHz bandwidth |
| UE power class |  | 23 dBm |
| Percentage of high loss and low loss building type |  | 100% low loss (applies to Channel model B) |
| Inter-site distance |  | 1732 m |
| Number of antenna elements per TRxP |  | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP |  | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements |  | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) , (dH,dV) = (N/A, N/A)λ 0°,90° polarization |
| Number of TXRU per UE |  | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) |
| Device deployment |  | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest |  | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| Traffic model |  | Full buffer |
| UE density |  | 10 UEs per TRxP |
| UE antenna height |  | 1.5 m |
| Electronic tilt |  | [100°] in LCS |
| UT attachment |  | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method |  | Geographical distance based wrapping |
| Polarized antenna model |  | Model-2 in TR36.873 |
| Scheduler |  | DL: SU PF UL: SU PF |
| Duplexing |  | FDD / TDD |
| Slots structure |  | TDD : DDDSU |
| Power control |  | α=0.8， P0=-76 dBm |
| Modulation |  | Up to 256 QAM |
| Downlink Overhead | SS/PBCH block | 1 SS/PBCH block per 20ms |
|  | PDCCH | 2 OFDM symbols per slot |
|  | DMRS (Type II) | 4, 8, 12 ports |
|  | CSI-RS for CM | 4, 8, 16, 32 ports with periodicity of 5ms |
|  | CSI-RS for IM | ZP CSI-RS with 5 slots period; 4 RE/PRB/5 slots |
|  | TRS | 20ms period; maximal bandwidth with 52 PRB; burst length with 2 slots 12 RE/PRB/20ms |
| Uplink Overhead | PUCCH | 2 PRBs and 14 OS for the slots without SRS transmission; 2 PRBs and 12 OS for the slots with SRS |
|  | DMRS | Type II, 2 complete symbols |
|  | SRS | 2 OFDM symbols per 5 slots |
| Receiver |  | MMSE-IRC |
| CSI feedback |  | PMI, CQI: every 5 slot; RI: every 5 slot; Subband based |
| Channel estimation |  | Realistic channel estimation |