3GPP TSG RAN WG1 Meeting #97		R1-1907319

Reno, USA, May 13th – May 17th, 2019

Agenda item:       7.2.8.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On UCI reporting of SCI and FD basis

Document for:     Discussion

# 1	UCI overhead reduction for SCI and FD basis

This document is intended to describe in more detail how the cyclic shift property of the Rel-16 DFT codebook can be exploited to reduce the UCI overhead for reporting of SCI and FD basis in Rel-16 CSI feedback framework. In particular, we will specifically refer to Alt3.4 for SCI reporting, and on all the free selection and two-step selection procedures proposed for the FD basis selection indicator, as described in [1].

We focus on a given layer l, for simplicity. The reported precoder vectors, w0,l,w1,l,…,wN3-1,l, for all N3 PMI sub-bands are reconstructed at the gNB according to the agreed matrix structure

| Wl=W1,lW2,lWf,lH | (1) |
| --- | --- |

where:

- Wl=w0,l,w1,l,…,wN3-1,l;

- W1,l=v0,l,v1,l,…,vLl-100v0,l,v1,l,…,vLl-1 is the SD basis formed by N1N2×1 orthogonal vectors, vn,ln=0Ll-1, selected from a DFT codebook, with Ll as the number of SD beams in the basis of layer l;

- W2,l is the 2Ll×Ml matrix of LC coefficients {cil,ml}, obtained after FD compression, coefficient selection and quantization;

- Wf,l=fk0,fk1,…, fkMl-1 is the FD basis formed by N3×1 orthogonal vectors, fkmlml=0Ml-1, selected from a DFT codebook.

Let (il*,ml*) be the coordinates of the strongest coefficient in W2,l and k a fixed reference FD component index, known to both gNB and UE. Any integer value in the range [0,N3-1] may be used, for instance k=0, for simplicity.

In our proposal, a UE would then report in UCI

- the N3-modulo-shifted FD basis: Wf,l(s)=fk0',fk1',…, fkMl-1', with shift (kml*-k)=kml*, and indices given by k'ml=modkml-kml*,N3ml=0Ml-1;

- the non-zero LC coefficients in W2,l ordered according to a new Ml-modulo shifted indexing:

c'nl,ml=cnl,mod(ml+ml*,Ml);

- the bits of the bitmap ordered according to the same Ml-modulo shifted indexing used for the non-zero LC coefficients;

In other words, the indices shifts described above model a reordering of the columns of W2,l (and related bitmap) at a UE according to a modulo-Ml function, before reporting the bitmap and values of non-zero LC coefficients, such that the new equivalent column ordering reads: ml*,ml*+1,…,Ml-1,0,1,…, ml*-1. Let us call this column-shifted matrix W2,l(s).

Furthermore, thanks to the indices shifts, the index of the strongest coefficient of W2,l(s)(and corresponding bit in the bitmap) is now k=0, i.e., it is always found in the first column of W2,l(s). In practice, this limits the range of values for the SCI, which can only be an integer within the range [0,2L-1]. Therefore, the SCI can be encoded with log22L bits (see Alt 3.4 in [1]).

We now switch the focus to the reconstruction operation performed by gNB upon reception of the PMI to demonstrate that no performance degradation occurs when the proposed approach is adopted. For the sake of compactness in the notation, we define a permutation matrix of size M×M:

| Pl=0Iml*IMl-ml*0 | (2) |
| --- | --- |

where In is an n×n identity matrix, and an N3×N3 diagonal matrix of phase rotations, defined as follows:

| Rl=100000ej2πkml*N300000ej2π2kml*N300000⋱00000ej2π(N3-1)kml*N3. | (3) |
| --- | --- |

Hence, one can rewrite Wf,l(s) as a function of Wf,l,

| Wf,l(s)=RlWf,lPl | (4) |
| --- | --- |

and W2,l(s) as a function of W2,l,

| W2,l(s)=W2,lPl. | (5) |
| --- | --- |

We note that the Rl performs the modulo-shift operation, whilst Pl the index reordering operation described before. Without knowing the index ml*or the shift value kml*, the gNB reconstructs the precoder matrix as

| Wl'=W1,lW2,lsWf,lsH=W1,lW2,lPlPlTWf,lHRlH=W1,lW2,lWf,lHRlH=WlRlH | (6) |
| --- | --- |

where the last identity follows from (1), and Wl is the precoder the gNB would reconstruct if the UE reported the PMI without applying any modulo-shift. Therefore, we can conclude that the reconstructed precoder vectors, w0,l',w1,l',…,wN3-1,l', differ from the original ones, w0,l,w1,l,…,wN3-1,l, only by a phase factor, which is irrelevant for the precoder performance.

Note that this approach can be applied independently for each layer.

Observation 1. The adoption of Alt3.4 can be supported without mandating any specific UE behavior, that is by simply specifying that the indices of selected FD basis, LC coefficients and bitmap elements are mapped to UCI fields according to the following:

- the N3-modulo-shifted FD basis is mapped as: Wf,l(s)=fk0',fk1',…, fkMl-1', with shift (kml*-k)=kml*, and indices given by k'ml=modkml-kml*,N3ml=0Ml-1;

- the set of non-zero LC coefficients in W2,l are ordered and mapped according to a Ml-modulo shifted indexing, such that:

c'nl,ml=cnl,mod(ml+ml*,Ml);

- the set of bits of the bitmap ordered and mapped according to the same Ml-modulo shifted indexing used for the non-zero LC coefficients;

Observation 2. Thanks to the indices shifts, the index of the strongest coefficient of the LC matrix is always found in a fixed reference column of the LC matrix itself. This limits the range of values for the SCI, which can only be an integer within the range [0,2L-1]. Therefore, the SCI can be encoded with log22L bits.

Observation 3. The adoption of Alt3.4 does not incur any performance degradation due to Fourier transform properties.

# 2	Advantages of the proposed solution

Thanks to this approach the following two results are achieved:

- Because the index of the FD component where the strongest coefficient is located is now known by design, i.e., k, the SCI per layer can be signalled using log22L bits, to indicate its SD beam location. Note that this method is applicable regardless of how the NNZC is signalled (per layer or across layers)

- The second advantage can be appreciated if we consider the signalling of the SCI for RI=1, i.e., by using log2KNZ bits. This is done by counting the number of ones in the bitmap indicating the location of the NZ coefficients to the gNB. In particular, the gNB counts the ‘1’s either by row or column and places the strongest coefficient in the position of the ‘1’ corresponding to the SCI indicator value. Now, any error in the bitmap demodulation may cause an incorrect positioning of the strongest coefficient by a gNB, in the LC coefficient matrix, with significant degradation in the PMI reconstruction. This problem is made worse by the fact that UCI Part 2 is encoded with higher code rate than Part 1. Conversely, Alt3.4, does not have this problem because the SCI is encoded/decoded independently of the bitmap.

# 3 	Advantages for FD basis selection indicator

As a further advantage, it is important to note that the modulo shift operation also allows to achieve UCI overhead savings for FD basis selection indicator, especially when free selection or two-step selection procedures are adopted. The advantage in this sense is three-fold:

- After the modulo shift, the first FD component index per layer, k0'=0, does not need to be reported because it coincides with the fixed reference FD component, hence the FD basis indication only requires log2N3-1M-1 bits (for free selection procedures) or log2N3'-1Mi-1 for two-step procedures.

- In case of two-step procedures, a further overhead savings can be achieved for signaling the choice of the intermediate set of size N3'. In this case, in fact, the intermediate set choice can be signaled using log2N3-1N3'-1 bits.

- Always in case of two-step procedures, if N3' and/or Minitial are reported in UCI part 1, additional UCI overhead savings can be achieved by using the same reference FD component for each layer, i.e., k as defined in Section 1. In this case, in fact:

  - The overlap between the FD bases of the layers can be maximized, and N3' minimized;

  - Minitial does not need to be reported since it coincides with k.

Observation 4. The modulo shift operation allows to achieve UCI overhead savings for the FD basis selection indicator, especially when free selection or two-step selection procedures are adopted.

Observation 5. If the modulo shift operation is adopted, the FD basis indication only requires log2N3-1M-1 bits (for free selection procedures) or log2N3'-1Mi-1 for two-step procedures.

Observation 6. If the modulo shift operation is adopted a further overhead savings can be achieved for signaling the choice of the intermediate set of size N3' when two-step procedures for FD basis selection indication are used. In this case, the intermediate set choice can be signaled using log2N3-1N3'-1 bits.

Observation 7. In case of two-step procedures for FD basis selection indication, additional UCI overhead savings can be achieved if N3' and/or Minitial are reported in UCI part 1 and the same reference FD component is used for the modulo shift in each layer. In this case, in fact:

- The overlap between the FD bases of the layers can be maximized, and N3' minimized;

- Minitial does not need to be reported since it coincides with the fixed reference FD component.

# 4 	Conclusion

Observation 1. The adoption of Alt3.4 can be supported without mandating any specific UE behavior, that is by simply specifying that the indices of selected FD basis, LC coefficients and bitmap elements are mapped to UCI fields according to the following:

- the N3-modulo-shifted FD basis is mapped as: Wf,l(s)=fk0',fk1',…, fkMl-1', with shift (kml*-k)=kml*, and indices given by k'ml=modkml-kml*,N3.ml=0Ml-1;

- the set of non-zero LC coefficients in W2,l are ordered and mapped according to a Ml-modulo shifted indexing, such that:

c'nl,ml=cnl,mod(ml+ml*,Ml);

- the set of bits of the bitmap ordered and mapped according to the same Ml-modulo shifted indexing used for the non-zero LC coefficients;

Observation 2. Thanks to the indices shifts, the index of the strongest coefficient of the LC matrix is always found in a fixed reference column of the LC matrix itself. This limits the range of values for the SCI, which can only be an integer within the range [0,2L-1]. Therefore, the SCI can be encoded with log22L bits.

Observation 3. The adoption of Alt3.4 does not incur any performance degradation due to Fourier transform properties.

Observation 4. The modulo shift operation allows to achieve UCI overhead savings for the FD basis selection indicator, especially when free selection or two-step selection procedures are adopted.

Observation 5. If the modulo shift operation is adopted, the FD basis indication only requires log2N3-1M-1 bits (for free selection procedures) or log2N3'-1Mi-1 for two-step procedures.

Observation 6. If the modulo shift operation is adopted a further overhead savings can be achieved for signaling the choice of the intermediate set of size N3' when two-step procedures for FD basis selection indication are used. In this case, the intermediate set choice can be signaled using log2N3-1N3'-1 bits.

Observation 7. In case of two-step procedures for FD basis selection indication, additional UCI overhead savings can be achieved if N3' and/or Minitial are reported in UCI part 1 and the same reference FD component is used for the modulo shift in each layer. In this case, in fact:

- The overlap between the FD bases of the layers can be maximized, and N3' minimized;

- Minitial does not need to be reported since it coincides with the fixed reference FD component.

# References

| [1] | 3GPP, «RAN1 Chairman's Notes RAN1#96bis,» Xi'an, People's Republic of China, 8 April - 12 April 2019. |
| --- | --- |