3GPP TSG-RAN WG1 #97		R1-1907333

Reno, USA, May 13-17, 2019

Source:	Ericsson

Title:	Reduced latency Scell management for NR-NR CA

Agenda Item:	7.2.13.4

Document for:	Discussion and Decision

# Introduction

Reduced latency Scell management was discussed in RAN1#96bis [1],[2] and an LS was sent to RAN4 [3] requesting RAN4 input on feasibility of reducing maximum allowed activation delay requirements.

In [4] we proposed some approaches for faster SCell management. One approach is to improve the Scell activation latency by moving to L1 based activation/deactivation signalling. Another approach is to retain the existing MAC CE based Scell activation/deactivation framework but provide additional L1 based signalling to control PDCCH monitoring on activated Scells.

In this document we further discuss details.

# Discussion

## L1 mechanisms for fast Scell management

Here we discuss the case when the UE is not configured with cross-carrier scheduling (i.e., UE is not configured with CIF).

To start transmitting a data burst on an Scell,

- UE should be monitoring PDCCH on the Scell using which PDSCH carrying the data burst can be scheduled

- CSI needed for efficient scheduling of the data burst on the SCell should be available to the network

NR provides multiple approaches for CSI reporting for Scells (as shown in the Table from 38.214 in Annex A). For the UE to measure and report valid CSI for an Scell, the Scell needs to be activated. If the Scell is activated, the UE also monitors PDCCH on that Scell using which PDSCH carrying the data burst can be scheduled.

Ideally, if the Scell activation delay is small (e.g. a few slots), the Scell can be quickly activated at the onset of data burst arrival, the data burst is then scheduled and the Scell is then deactivated to provide UE power saving.

However, if the Scell activation delay is not small and due to long activation delays, it is risky for the NW to frequently deactivate the Scell since bringing the UE back to Scell activated state can take anywhere from a minimum of ~5ms to maximum allowed RAN4 values of tens or hundreds of milliseconds depending on the specific scenario and UE implementation (as discussed in [4]).

One approach to retain the ability to quickly schedule data bursts while still providing UE power saving is to keep the Scell activated (so that CSI measurement on the Scell and reporting for the Scell is still possible) but adapt the PDCCH monitoring on the activated Scell using an additional L1 mechanism. Some L1 mechanisms related to this approach were discussed in RAN1#96bis [2].

One possible L1 mechanism is to use a dedicated DCI format to control PDCCH monitoring on/off for Scells. As discussed in [4], this can be done by having a bitmap in the DCI indicating the PDCCH on/off status for each of the configured SCell(s) for the UE. Since up to 16 DL CCs can be configured for NR, this would need up to 15 bits in the DCI (one per Scell). For the case when an Scell is configured with multiple BWPs, PDCCH monitoring is turned off/restarted on the current active BWP of that Scell.

Figure 1 below illustrates this approach. Unlike L1 based Scell activation/deactivation, the L1 on/off signaling does not impact CSI reporting. Given this, if PDCCH monitoring on/off is indicated for an Scell it is less essential to wait till reception of an ACK from the UE (i.e., in response to the L1 on/off command) before starting/stopping PDCCH monitoring on that Scell.

Figure 1 – Additional L1 based on/off for NR Rel16 Scell management

Another L1 mechanism is to configure each Scell with a ‘dormant BWP’ that has no PDCCH monitoring candidates and to turn off PDCCH monitoring on the Scell, switch the UE to the dormant BWP of that Scell. This was discussed e.g. in [5][6]. Rel15 supports fast BWP switching via DCI formats used for scheduling PDSCH/PUSCH. However, the Rel15 mechanism cannot be used for switching the UE from dormant BWP to ‘regular BWP’ (i.e., BWP with PDCCH monitoring used for transmitting data bursts to the UE) as there will be no PDCCH for the UE to monitor on the dormant BWP. Given this, the dormant BWP approach would need some additional mechanism compared to Rel15 (i.e., to switch from dormant BWP to regular BWP). The explicit approach to support this would be to use a dedicated DCI format (e.g. sent on Pcell) to switch the UE from dormant BWP to ‘regular BWP’ on the Scells. Again, this can be done by a bitmap with 1 bit per configured Scell. 0 would mean UE switches to ‘dormant BWP’ for the Scell and ‘1’ would mean UE either continues operating on existing regular BWP if operating on regular BWP, or if operating on dormant BWP, switch to previous active regular BWP. In [5],[6], an implicit  approach was also discussed where BWP switch on Pcell from ‘regular’ to ‘power savings’ BWP is linked to BWP switch on all Scells from ‘regular’ to ‘dormant BWP’.

From a CSI measurement/reporting perspective, Rel15 NR already provides enough flexibility to adapt the CSI reporting for activated Scells. For aperiodic CSI reporting, gNB can trigger an aperiodic CSI report for an activated Scell from the Pcell with the desired reporting configuration (e.g. frequent/infrequent) and this can be used regardless of whether PDCCH is monitored on Scell or not. For semi-persistent CSI reporting, Rel15 signaling allows activation/deactivation of SP-CSI reporting on SCells via MAC CE based mechanism. Periodic CSI reporting on activated Scells is RRC configured and there is no need for fast adaptation of this reporting type as SP-CSI reporting can be used for such situations.

In our view, the approach of using dedicated DCI to control PDCCH monitoring on/off for Scells provides the desired benefits for fast Scell management. For Scells configured with multiple BWPs, the ‘dormant BWP’ approach is also applicable. However, this approach also requires a dedicated DCI to switch the UE from ‘dormant’ to ‘regular’ BWPSs on the Scells. The implicit approach of linking BWP switch between different cells is not flexible enough as Scells cannot be managed independently. From CSI reporting perspective, as discussed above, NR Rel15 signaling already provides enough adaptability and the additional benefit provided by the ‘dormant BWP’ mechanism for this aspect is unclear.

Considering the above, we propose the following

Proposal

- Support dedicated DCI format using which PDCCH monitoring on activated SCell(s) can be turned on/off

  - 1 bit per Scell is used in the DCI

    - 0 indicates PDCCH monitoring is stopped on the corresponding activated Scell,

    - 1 indicates continue/restart PDCCH monitoring on the corresponding activated Scell

  - When UE is configured with multiple BWPs for the Scell, PDCCH monitoring is stopped/restarted on current active BWP at the time when the DCI format is received.

# Conclusions

In this document, we discuss improving the latency needed for Scell activation and data transmission and propose the following.

Proposal

- Support dedicated DCI format using which PDCCH monitoring on activated SCell(s) can be turned on/off

  - 1 bit per Scell is used in the DCI

    - 0 indicates PDCCH monitoring is stopped on the corresponding activated Scell,

    - 1 indicates continue/restart PDCCH monitoring on the corresponding activated Scell

  - When UE is configured with multiple BWPs for the Scell, PDCCH monitoring is stopped/restarted on current active BWP at the time when the DCI format is received.

# References

[1] R1-1905900 – “Summary#2 of Efficient and low latency serving cell configuration/activation/setu”, Ericsson, RAN1#96bis, Xi’an, China, April 2019.

[2] R1-1905915 – “Summary of email discussion [96b-NR-11]”, Ericsson, RAN1#96bis, Xi’an, China, April 2019.

[3] R1-1905912 – “LS on maximum allowed SCell activation delay for Rel16 CA”, RAN1  RAN4 LS, RAN1#96bis, Xi’an, China, April 2019.

[4] R1-1905144 – “Reduced latency Scell management for NR-NR CA”, Ericsson, RAN1#96bis, Xi’an, China, April 2019.

[5] R1-1905281 – “Fast SCG and Scell activation”, Qualcomm Incorporated, RAN1#96bis, Xi’an, China, April 2019.

[6] R1-1904491 – “Efficient SCell Access Switching for NR”, MediaTek Inc., RAN1#96bis, Xi’an, China, April 2019.

# Annex A – Rel15 spec excerpts

## TS 38.214

## TS 38.321