3GPP TSG RAN WG1 #97		                               R1-1907341

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.8.1

Source:	Apple

Title:	Considerations on CSI enhancement for MU-MIMO support

Document for:	Discussion/Decision

# Introduction

Rel-16 MIMO enhancement WI was approved with the following scope [1]. Among the scope, Type II CSI feedback enhancement for MU-MIMO support is one of the main topics.

- Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support: - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2 - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul: - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission - Enhancements on multi-beam operation, primarily targeting FR2 operation: - Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection - Specify a beam failure recovery for SCell based on the beam failure recovery specified in Rel-15 - Specify measurement and reporting of either L1-RSRQ or L1-SINR - Perform study and make conclusion in the first RAN1 meeting after start of the WI, and if needed, specify CSI-RS and DMRS (both downlink and uplink) enhancement for PAPR reduction for one or multiple layers (no change on RE mapping specified in Rel-15) - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

Rel-15 NR type II CSI reporting has the potential to provide more accurate CSI information in order to facilitate MU-MIMO pairing at the gNB scheduler side. However, it also suffers from some limitation

- Rel-15 NR type II CSI reporting supports maximum of 2 layers

- Rel-15 NR type II CSI reporting incurs large overhead especially when the number of subbands are large

Significant progress has already been made in RAN1 #95 meeting in the area of reducing the type II CSI feedback overhead. The frequency domain DFT based approach was agreed with details to be finalized in the coming meetings [2]

Agreement For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002 - Note: The same DFT-based compression scheme is extended for Type II port selection codebook - Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme - FFS: detailed signaling mechanism - Note: Additional compression scheme(s) are not precluded

The following equation illustrates the general ideal of CSI overhead reduction with DFT based frequency domain compression.

Whereas N3 is the number of subbands, M is the number of frequency (DFT) basis used for compression

In the previous RAN1 meeting, further agreement has been reached in terms of the detailed design on how to compress feedback coefficients based on the frequency basis [3, 4, 5]

Agreement On FD compression unit, agree on Alt1 (PMI subband size = CQI subband size) as the default, along with Alt2.2 (PMI subband size = CQI subband size / R) as secondary - The value of R is fixed to 2 - FFS: Whether secondary implies a separate UE capability or restricted use cases - Include issues such as limitation on the number of FD compression units, CPU occupation, latency constraint and/or BW constraint - FFS: Whether FD compression unit is higher-layer configured or reported by the UE Agreement On basis/coefficient subset selection for the first layer, the following is supported: - Common selection for all beams with size-K0 subset of 2LM reported - The value of K0 is configured via higher-layer signaling - The number of reported non-zero coefficients can be smaller than or equal to K0 - FFS: Whether the value of M is configurable Agreement On the choice of oversampling factor O3, agree on O3=1 Agreement On subset selection for layer 0, agree on the following: - Unrestricted (polarization-independent) subset selection which requires a size-2LM bitmap in UCI part 2 - FFS: Further down selection of supported combinations of FD compression parameters Agreement Two values of M are supported. In RAN1#96, down select between the following alternatives (): - Alt1. - Alt2. - FFS: support for p=1/8 and/or p=3/4 in addition to 1/4 and 1/2 Agreement Values of N3: For and NSB is # CQI subbands, when , Values of N3: For and NSB is # CQI subbands, when , downselect among the following alternatives in RAN1#96 - Alt1: N3 is smallest multiple of 2, 3, or 5 which is - Alt2: N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5 Agreement For RI=2, the following is supported - Layer-independent FD basis subset selection - Layer-independent coefficient subset selection Agreement On RI=3-4 extension: - K0 setting: agree on supporting Alt1, i.e. total max # NZ coefficients across all layers ≤ 2K0 where the K0 value set for RI{1,2} - FD basis subset selection: agree on layer-specific subset selection - Coefficient subset selection: agree on layer-specific subset selection Agreement SD basis subset selection is layer-common

In this contributions, we provide our proposals for the details on how to compress the combination coefficients based on DFT base matrix.

# Number of Frequency Basis for Compression

Clearly the number of frequency basis, i.e. M, directly impact the compression gain as well as the error after compression. In fact, the compression gain is, roughly, linearly proportional to the number of frequency basis.

The optimal, or desirable, number of frequency basis depends on the UE antenna configurations as well as the radio channels, and certainly replies on the configured number of subbands. To achieve the best trade-off between compression gain and compression error, system should adapt the configuration of M based on multiple factors discussed above.

For example, when the channel is less frequency selective, it is expected that the number of frequency basis needed for compression is very limited, or mainly the basis near DC. Hence the M can be selected to be very small. Similarly, when the number of antenna elements is very large (i.e. N1*N2 large), the beam is typically very narrow and the beam formed channel tends to be less frequency selective, hence M can be chosen to be small too.

It is also important to notice that the beam selection and reporting is mostly UE implementation dependent. Furthermore, UE has the direct access to the full channel estimates and other useful information such as sensor etc. Therefore, it is beneficial to allow UE to suggest the desirable number of frequency basis to the NW to achieve the best adaption to the variable deployment conditions.

Similarly, the actual number of coefficients that need to be reported, aka k0, also highly depends on the actual channel which is best known at the UE side, therefore, it is also more reasonable for UE to indicate the k0 explicitly or implicitly, in order to maximize the compression

Proposal 1: The choice of number of frequency basis used for Type II CSI compression, i.e. M, needs to consider multiple factors, including the UE antenna configuration (N1, N2), and the configurated number of subbands (N3), etc.

Proposal 2: Rel-16 NR considers to support UE to report the suggested number of frequency basis used for Type II CSI compression, i.e. M, and the actual number of coefficients that need to be reported, i.e. k0, to the NW.

# Codebook Subset Restriction Enhancement

Codebook Subset Restriction (CBSR) is needed for multiple reasons. First of all, for MU-MIMO, gNB may request two UE to report their precoding matrices in mutually orthogonal directions. Secondly, for reducing UE CSI computation complexity, a gNB may prune some unlikely beams based on UL measurement, so that UE doesn’t have to test the precoders formed by those pruned beams.

However, with the frequency based compression, a new CBSR design is needed since, after combination of the DFT basis, wideband amplitude cannot easily be regulated per space basis. One solution is to introduce frequency basis restriction in addition to spatial basis restriction.

Proposal 3: Rel-16 NR considers to introduce frequency basis restriction in addition to codebook (spatial basis) restriction.

For frequency basis restriction, A UE is restricted from reporting CSI based on a subset of frequency basis per gNB configuration, in addition to spatial basis restriction. The max allowed amplitude for spatial basis and the max allowed amplitude for frequency basis can be separately configured. Furthermore, the maximum allowed amplitude may be layer specific, i.e., for different ranks, each layer may be configured with a different max allowed amplitude. A UE may be configured with restriction to spatial basis dependent amplitude, but no restriction to frequency basis dependent amplitude. Vice versa, A UE may be configured with restriction to frequency basis dependent amplitude, but no restriction to spatial basis dependent amplitude. In addition, A UE may be configured with both restriction to spatial basis dependent amplitude and restriction to frequency basis dependent amplitude.

# Remaining Issue from RAN1#96bis Meeting

In RAN1#96 meeting, there are some remaining issues, namely, [5]

Agreement The scheme for indicating the number of NZ coefficients (NZC) will be chosen from the following alternatives in RAN1#97 (Reno): - Alt1.1: RI + # NZC summed across layers where # NZC = {0, 1, 2, …, 2K0} (if sufficiency indicator is supported) or {1, 2, …, 2K0} - Alt1.2: Per-layer # NZC without RI where # NZC = {0, 1, 2, …, K0} - Alt1.3: RI + differential of # NZC summed across layers - Differential means fraction of 2K0 with smaller number of possible values compared to the regular # NZC (in Alt1.1) - Alt1.4: RI + per-layer differential # NZC - Differential means fraction of K0 with smaller number of possible values compared to the regular # NZC (in Alt1.2)

With the bit map per layer, we do not see the need of #NZC since bit map informs gNB about both the number of and the location of the NZC. In consideration of the support of RI of 3 and 4, we slight prefer Alt 1.1

Proposal 4: For the scheme for indicating the number of NZC, we slightly prefer Alt1.1: RI + # NZC summed across layers where # NZC = {0, 1, 2, …, 2K0} (if sufficiency indicator is supported) or {1, 2, …, 2K0}

Agreement For RI=3-4, the bitmap design will be chosen from the following alternatives in RAN1#97 (Reno): - Alt2.1: 2LMi bits per layer, i=0, 1, …, (RI-1) - Alt2.2: One joint bitmap 1 for all layers, where an indicator bit is 1 if at least one of the RI layers has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1) - Alt2.2B: Bitmaps 1 for each layer, where an indicator bit is 1 if at least one of the RI beams has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1) - Alt2.3: LMi bits for the layer in which the weaker polarization is dropped (else 2LMi bits) + up to 4-bit bitmap to indicate the layer where the weaker polarization is dropped (UCI part 1); i=0, 1, …, (RI-1)

Proposal 5: For RI=3-4, the bitmap design, we slightly prefer Alt2.1: 2LMi bits per layer, i=0, 1, …, (RI-1)

# Conclusion

Rel-15 NR type II CSI reporting has the potential to provide more accurate CSI information in order to facilitate MU-MIMO pairing at the gNB scheduler side. However, it also suffers from some limitation

- Rel-15 NR type II CSI reporting supports maximum of 2 layers

- Rel-15 NR type II CSI reporting incurs large overhead especially when the number of subbands are large

In terms of Type II CSI reporting overhead reduction. Frequency domain DFT based approach was agreed with details to be finalized in the coming meetings [2]. In this contributions, we provide our proposals for the details on how to compress the combination coefficients based on DFT base matrix. We have the following proposals

Proposal 1: The choice of number of frequency basis used for Type II CSI compression, i.e. M, needs to consider multiple factors, including the UE antenna configuration (N1, N2), and the configurated number of subbands (N3), etc.

Proposal 2: Rel-16 NR considers to support UE to report the suggested number of frequency basis used for Type II CSI compression, i.e. M, and the actual number of coefficients that need to be reported, i.e. k0, to the NW.

Proposal 3: Rel-16 NR considers to introduce frequency basis restriction in addition to codebook (spatial basis) restriction.

Proposal 4: For the scheme for indicating the number of NZC, we slightly prefer Alt1.1: RI + # NZC summed across layers where # NZC = {0, 1, 2, …, 2K0} (if sufficiency indicator is supported) or {1, 2, …, 2K0}

Proposal 5: For RI=3-4, the bitmap design, we slightly prefer Alt2.1: 2LMi bits per layer, i=0, 1, …, (RI-1)

# References

- RP-181453, "WI Proposal on NR MIMO Enhancements", 3GPP TSG RAN Meeting #80 La Jolla, USA, June 11-14, 2018

- Chairman notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, 12th – 16h November 2018

- Chairman notes, 3GPP TSG RAN WG1 Meeting #AH1, Taipei, Taiwan, 21st – 25th January, 2019

- Chairman notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019

- Chairman notes, 3GPP TSG RAN WG1 Meeting #96b, Xian, China, April 8th – April 12th, 2019