3GPP TSG RAN WG1 #97		                               R1-1907342

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.8.2

Source:	Apple

Title:	Considerations on enhancements on multi-beam operation

Document for:	Discussion/Decision

# Introduction

As part of Rel-16 MIMO enhancement [1], Non-Coherent Joint-Transmission (NCJT) is one of the design targets for multi-TRP/panel operation enhancement

- Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support: - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2 - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul: - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission - Enhancements on multi-beam operation, primarily targeting FR2 operation: - Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection - Specify a beam failure recovery for SCell based on the beam failure recovery specified in Rel-15 - Specify measurement and reporting of either L1-RSRQ or L1-SINR - Perform study and make conclusion in the first RAN1 meeting after start of the WI, and if needed, specify CSI-RS and DMRS (both downlink and uplink) enhancement for PAPR reduction for one or multiple layers (no change on RE mapping specified in Rel-15) - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

NCJT transmission is beneficial, especially for the UEs in the overlapping coverage of two TRPs. With NCJT, UE can enjoy higher PDSCH reliability via multi-TRP diversity. More importantly, NCJT also allows the UE to receive two PDSCH simultaneously, with each PDSCH independently from different TRP. This operation is useful especially when single TRP cannot schedule enough number of layers to fully exploit the UE Rx capability. Similarly, UL NCJT allows the UE to either transmit two independent PUSCHs to two TRPs in order to increase UE UL throughput, or selectively point UL transmission to the best TRP in order to improve reliability

In the previous RAN1 meetings, further agreement has been reached in terms of the detailed design on how to support multi-TRP transmission and reception [2, 3, 4, 5]

Agreement For multi-TRP/panel transmission, both multiple PDCCH and single PDCCH designs are supported in Rel-16. Applies for eMBB Agreement For multiple-PDCCH based multi-TRP/panel transmission for eMBB, for the purposes of PDCCH detection, UE does not assume any dependency amongst the multiple PDCCHs Agreement TCI indication framework shall be enhanced in Rel-16 at least for eMBB: - Each TCI code point in a DCI can correspond to 1 or 2 TCI states - When 2 TCI states are activated within a TCI code point, each TCI state corresponds to one CDM group, at least for DMRS type 1 - FFS design for DMRS type 2 - FFS: TCI field in DCI, and associated MAC-CE signaling impact Agreement For TCI state configuration in order to enable one or two TCI states per a TCI code point, - MAC-CE enhancement to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2. - FFS whether increasing the number of bits of TCI field in DCI Agreement To support multiple-PDCCH based multi-TRP/panel transmission with intra-cell (same cell ID) and inter-cell (different Cell IDs), following RRC configuration can be used to link multiple PDCCH/PDSCH pairs with multiple TRPs - one CORESET in a “PDCCH-config” corresponds to one TRP - FFS whether to increase the number of CORESETs per “PDCCH-config” more than 3 FFS: UE monitoring/decoding behavior for multiple PDCCHs. Agreement For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used - Support TDMed PUCCH transmission within a slot to convey, at least separate ACK/NACK only feedback, with separated HARQ-ACK codebook for two TRPs - FFS: Details on how this feature is supported in the specifications (for examples, introduction of restrictions and/or further enhancements) Above applies at least for FR1

In the contribution, we consider the PDCCH design for NCJT in the following areas

- DL DCI PDCCH design for NCJT

- UL DCI PDCCH design for NCJT

- Management of NCJT operation

# DL DCI PDCCH Design for NCJT

We first consider the scenario where UE receives two PDSCHs, one PDSCH from each TRP. We need to consider the DL DCI PDCCH design considering multiple factors, including the backhaul capability, DCI overhead, etc.

For two stage or two DCI design, reducing the UE DCI decoding complexity is very important. It is beneficial to include the assistance scheduling information in one DCI to help UE decoding the other DCI. The assistance can include but not limited to the following

- The existence of the other DCI

- The time/frequency domain configuration of the second DCI

The concept is illustrated as below in Figure 1

Figure 1 One DCI to aid the second DCI decoding

Proposal 1: For two DCI design, considering to introduce assistance field in one DCI to aid the decoding of the other DCI, such as existence of the second DCI and resource allocation of the second DCI

Fourthly, we also need to consider the CORESET configuration of the DCI in order to reduce the UE DCI detection complexity. Consider the scenario when UE is expecting two DCIs for NJCT, UE may have to exhaust all the DCI decoding hypothesis in order to determine if NCJT scheduling exists or not. However, if there is fixed relationship between two DCIs, UE could save power at the expense of reduced NW scheduling flexibility. There are a few options

- Single CORESET design with fixed time relationship between two DCIs

- Single CORESET design with fixed frequency relationship between two DCIs

- Two CORESETs design with each DCI in one CORESET with fixed time/frequency relationship. This allows the NW to activate and deactivate NCJT dynamically via CORESET configuration.

The concept is illustrated as below in Figure 2

Figure 2 Fixed time/frequency relationship between two DCIs for UE complexity saving

Proposal 2: For two DCI design, considering to introduce fixed time/frequency relationship between two DCIs, and/or independent CORESET configuration, to reduce UE complexity

Last but not the least, it is also important to consider the scenario when UE is only capable of receiving one PDSCH due to either hardware limitation or other issues such as UE power consumption, thermal limitation etc. Even in this case when UE only support single PDSCH decoding, NCJT can still provide performance benefit by dynamic TRP pointing. There are a few things to consider

- The TRP index or extended HARQ processing ID can be signaled in the DCI. This is very useful for dynamic single/dual streams switching and cross TRP retransmission

- For designs that require two DCI, certain reservation field such as reserved MCS/RV/NDI etc. can be used to signal that the corresponding PDSCH is disabled.

Proposal 3: NR to consider PDCCH design to support dynamic TRP pointing, including but not limited to, assistance information in DCI for TRP indication and TRP disabling via reserved DCI field

# UL DCI PDCCH Design for NCJT

UL NCJT also requires the gNB to schedule two PUSCH or schedule one PUSCH for dynamic TRP pointing. Therefore, all the above DCI discussion for DL NCJT largely applies for UL NCJT design. In this section, we consider design that is especially important for FR2.

Current Rel-15 has certain discrepancy in terms of the beam configuration for PUSCH/PDSCH. PDSCH beam is indicated via TCI while PUSCH beam is indicated via SRI. It is possible that SRI can be QCL’ed to CSI-RS or SSB. However, it is still preferable to have clean design by introducing TCI in UL DCI to indicate the Tx beam for UEs with beam correspondence.

Proposal 4: NR to consider to introduce TCI in UL DCI to configure UL beam for UEs with beam correspondence

# Management of NCJT Operation

Even though NCJT can potentially improve UE performance such as throughput and reliability, it also introduces constrains at both the UE and NW side

- NCJT requires UE to monitor multiple DCI and decode multiple PDSCH simultaneously which increase the UE complexity and power consumption

- NCJT may require UE to support multiple bean simultaneously in FR2 subject to UE hardware limitation and power consumption

- NCJT can potentially complicates the gNB scheduling and demand higher backhaul requirement.

Based on the above discussion, it is reasonable for UE to request activation and deactivation of NCJT due to various reason such as (1) UE uplink buffer is lower or higher than a threshold for certain period of time (2) UE battery status and/or thermal status, etc.

Proposal 5: NR to consider to allow UE to request activation and deactivation of NCJT

Similar consideration also applies for NW, it is also reasonable for NW to activate and deactivate NCJT dynamically which can be done via RRC/MAC-CE/DCI. The NCJT configuration can be independent for each BWP considering that some BWP may target low date rate, low power consumption operation, while some BWP may target high data rate, high power consumption operation.

Proposal 6: NR to consider to allow NW to activate and deactivate NCJT operation, preferably independently for each BWP

NCJT is most useful when two TRPs are close in signal quality, while NCJT becomes useless when one TRP is significantly weaker than the other TRP. Therefore, it is necessary to introduce measurement report to support adaptive NCJT operation as illustrated in Figure 3

Figure 3 Mobility report for adaptive NCJT operation

Measurement can be done at L3 or L1. The measurement can include but not limited to the following,

- When two TRP measurement difference is within a threshold (hysteresis) for certain period of time (Time to Trigger)

- When two TRP measurement difference exceeds a threshold (hysteresis) for certain period of time (Time to Trigger)

- UE buffer is lower or higher than a threshold for certain period of time

Proposal 7: NR to consider to introduce measurement report to support adaptive NCJT operation

# Conclusion

In this contribution, we discuss the control channel design for NCJT including the following areas

- DL DCI PDCCH design for NCJT

- UL DCI PDCCH design for NCJT

- Management of NCJT operation

We have the following proposals

Proposal 1: For two DCI design, considering to introduce assistance field in one DCI to aid the decoding of the other DCI, such as existence of the second DCI and resource allocation of the second DCI

Proposal 2: For two DCI design, considering to introduce fixed time/frequency relationship between two DCIs, and/or independent CORESET configuration, to reduce UE complexity

Proposal 3: NR to consider PDCCH design to support dynamic TRP pointing, including but not limited to, assistance information in DCI for TRP indication and TRP disabling via reserved DCI field

Proposal 4: NR to consider to introduce TCI in UL DCI to configure UL beam for UEs with beam correspondence

Proposal 5: NR to consider to allow UE to request activation and deactivation of NCJT

Proposal 6: NR to consider to allow NW to activate and deactivate NCJT operation, preferably independently for each BWP

Proposal 7: NR to consider to introduce measurement report to support adaptive NCJT operation

# References

- RP-181453, "WI Proposal on NR MIMO Enhancements", 3GPP TSG RAN Meeting #80 La Jolla, USA, June 11-14, 2018

- Chairman notes, 3GPP TSG RAN WG1 Meeting #95, Spokane, USA, 12th – 16h November 2018

- Chairman notes, 3GPP TSG RAN WG1 Meeting #AH1, Taipei, Taiwan, 21st – 25th January, 2019

- Chairman notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 25th – March 1st, 2019

- Chairman notes, 3GPP TSG RAN WG1 Meeting #96b, Xian, China, April 8th – April 12th, 2019