3GPP TSG RAN WG1 #97	R1- 1907345

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.9.1

Source:	Apple Inc.

Title:	Design of PDCCH-based Power Saving Signal and Channel

Document for:	Discussion, Decision

# Introduction

During the RAN1#96b, following agreements were made regarding PDCCH-based power saving signal/channel.

Agreements: - The PDCCH-based power saving signal/channel is UE-specifically configured. - The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - Alt 1: triggering a single UE only - Alt 2: triggering UE(s) within a group - FFS whether to always trigger all UEs in a group or a subset of it - Alt 3: Alt 1 & Alt 2 Agreements: The assumptions of the DCI design of the PDCCH-based power saving signal/channel include: - No increase of DCI format size budget - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time - Working assumption: no increase of UE BD/non-overlapping CCE limit Agreements: Possible candidates of DCI format design for the PDCCH-based power saving signal/channel (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - New DCI format(s) - The size of new DCI format may or may not be the same size as the existing DCI size - Enhancement of existing DCI. E.g.,: - Additional new field(s) - Using the existing DCI format for the power saving purpose - Re-purpose field(s) in the DCI - The detection of existing DCI format as the indication for the power saving technique Agreements: The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide: - Indication of one or more power saving associated operations. - Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format Agreements: Potential DCI contents in DCI format(s), to be further investigated: - Power saving technique associated with C-DRX– - Essential for UE function for the C-DRX - Wakeup – - UE is indicated to transition from outside Active Time to Active Time - UE is indicated to stay at Active Time - Go to sleep– - UE is indicated to transition from Active Time to outside Active Time. - UE is indicated to stay outside Active Time - FFS: The time of receiving the wakeup and go-to-sleep indication inside or outside Active Time. - Cross-slot scheduling - Triggering RS transmission - CSI report - Single vs. multi-cell operation - BWP /SCell - BWP & SCell together - BWP and SCell have separated fields - MIMO layer adaptation/number of Anenna adaptation - May further depend on RAN4’s work - Indication of CORESET/search space/candidate of subsequent PDCCH decoding - PDCCH monitoring periodicity - PDCCH skipping (skipping duration)- - PDCCH skipping – UE is indicated to skip number of the PDCCH monitoring occasions and stays in the Active Time Note that - For the bullets in italic, there are concerns that some of which may have dependence on the ongoing SI in RAN2. - For the last two bullets, there are additional concerns that these are deemded by some companies to be not in the scope of the power saving WI approved so far The following candidates may be discussed after RAN2’s SI is completed: - Skipping number of DRX monitoring - SPS activation - DRX configuration Agreements: - The design of the DCI format(s) and size needs to account for one or more of the following aspects - Within or outside Active Time - DCI format size for the power saving signal/channel to fit the DCI format size budget - Including aspects whether or not it is necessary to align it with existing DCI format size - Other aspects are not precluded Agreements: - Support at least one CORESET with configured search space(s) for the power saving signal/channel - FFS separate vs. shared with a CORESET (and/or search space(s)) configured for other purposes (when applicable) Agreements: - For PDCCH-based power saving signal/channel, - The set of AL(s) is configured - The number of PDCCH candidate(s) for each AL is configured

In this contribution, we discuss the design on power saving signal/channel to support selected functionality. We discuss the aspect of physical channel support, DCI format design, search space of the PDCCH monitoring skipping and wake up signal.

# Power Saving Signal/Channel

## Relationship with PDCCH

During the RAN1#96b, there was quick discussion on the power saving signal/channel from specification point of view. Companies showed following two different understandings on the power saving signal/channel.

- Option 1) From specification point of view, power saving signal/channel is a new physical channel based on PDCCH structure.

- Option 2) From specification point of view, DCI(s) for power saving signal are specified and existing PDCCH is used for carrying the DCI(s).

We think Option 2) is viable choice considering that most of the design effort so far (e.g., DCI) in RAN1 is based on PDCCH, which makes the amount of work/time needed for Option 2) much smaller than that of Option 1). The problem of Option 1) is that there is significant specification work to be done not only in terms of the amount of new text but also in terms of number of specifications affected since it is a new physical channel although it is PDCCH-based.

Since RAN1 design effort so far has been very much aligned with/based on PDCCH design and surrounding infrastructure to support PDCCH (such as CORESET/search space), the most time efficient viable approach would Option 2).

Proposal 1

- From specification point of view, DCI(s) to support power saving features is/are specified and existing PDCCH is used for carrying the DCI(s).

## DCI Information and DCI Format

During RAN1#96b, we made following two agreements regarding DCI information and DCI format design.

Agreements: - The PDCCH-based power saving signal/channel is UE-specifically configured. - The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - Alt 1: triggering a single UE only - Alt 2: triggering UE(s) within a group - FFS whether to always trigger all UEs in a group or a subset of it - Alt 3: Alt 1 & Alt 2 Agreements: Possible candidates of DCI format design for the PDCCH-based power saving signal/channel (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - New DCI format(s) - The size of new DCI format may or may not be the same size as the existing DCI size - Enhancement of existing DCI. E.g.,: - Additional new field(s) - Using the existing DCI format for the power saving purpose - Re-purpose field(s) in the DCI - The detection of existing DCI format as the indication for the power saving technique

The two aspects (DCI information and DCI format design), as also noted in the agreements, are closely related to power saving techniques and scenarios. The design choice should be made considering the nature of power saving techniques to be supported. We consider PDCCH monitoring skipping as one of the feature considered in DCI design.

Scheduling DCI

The main use case of PDCCH monitoring skipping is to take advantage of gap between dynamically arriving grants. In such case, the PDCCH monitoring skipping signal needs to be transmitted together with grant to indicate skipping duration until next grant arrival time. Figure 1 shows the case of using scheduling DCI to indicate PDCCH monitoring skipping.

Figure 1 Scheduling DCI based scheme for power saving signal as PDCCH monitoring skipping

The potential concern of using scheduling DCI could be the increased size of DCI. However, the additional field for supporting power saving is UE-specific information of which length should be short enough to minimize impact in performance/complexity. From signaling design point of view, the PDCCH monitoring skipping is the best feature of which signal could be carried in scheduling DCI. Considering the length of bits required to support PDCCH monitoring skipping, we see that using scheduling DCI is viable approach.

Observation 1

- Scheduling DCI (format 0_1 and 1_1) could be used for carrying power saving signal(s) at least including PDCCH skipping monitoring.

Note that with this design choice of scheduling DCI, the DCI information is UE-specific and triggering single UE only. Thus, we make following proposal.

Observation 2

- NR supports DCI triggering a single UE only.

New DCI

The other use case is when there is a chance for PDCCH monitoring skipping with no immediate data transfer during Active Time. In such case, gNB should be able to send a PS signal without grant. A new DCI format can serves this purpose. Figure 2 shows the case of new DCI to support PDCCH monitoring skipping.

Figure 2 A new DCI for indicating PDCCH monitoring skipping

Note that the new DCI can carry not only the PDCCH monitoring skipping signal but also other power saving signal as well. Having a new DCI format could introduce additional resource usage from network point of view, however it provides increased flexibility by allowing gNB to apply power saving features irrespective of grant transmission.

Observation 3

- NR can support a new DCI for PDCCH monitoring skipping.

Based on above proposal, we make a following proposal in DCI format choice.

Proposal 2

- NR supports that both new DCI and scheduling DCI (0_1 and 1_1) can carry power saving signal.

Carrying multiple PS signals for multiple UEs

The need for triggering multiple UEs arises from the need of efficient signaling. The current RAN1’s understanding is that power saving signal/channel DCI is designed to indicate one or more PS operations supporting configurability.

Agreements: The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide: - Indication of one or more power saving associated operations. - Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format

This implies that the length of configured power saving information for each UE is variable depending on the configured PS features. Another factor to consider is the DCI format size budget. During last RAN1#96b meeting, we agreed not to increase of DCI format size budget mainly motivated by not to increase UE BB/CCE limit.

Agreements: The assumptions of the DCI design of the PDCCH-based power saving signal/channel include: - No increase of DCI format size budget - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time - Working assumption: no increase of UE BD/non-overlapping CCE limit

To achieve the goal, the size of a new DCI for power saving needs to be increased to match the size of any of existing DCIs. This approach could avoid increasing the number of BB/CCE limit. The increased DCI could carry many more bits than required for supporting single UE’s PS feature. In such case, instead of padding it with known bits, other UE’s PS signal could be multiplexed.

Observation 4

- For a new DCI format for supporting power saving, group signaling could be used in case DCI size is large enough to carry more than one UE’s PS bits.

RNTI

For scheduling DCI, C-RNTI is to be used. Whereas, for new DCI for power saving, a new RNTI (e.g., power saving RNTI or PS-RNTI) needs to be introduced to differentiate it from the existing DCI with the same size.

Table 1 shows the design choice of DCI Format and DCI information. Considering the use case of considered power saving feature, both scheduling DCI and new DCI format need to be supported. One thing to note is that the single UE triggering is a special case of group triggering.

Table 1 Design choice of DCI Format and DCI Information

|  | Enhancement of scheduling DCI | New DCI format |
| --- | --- | --- |
| Single UE-triggering (single DCI triggers only one UE) | Supported (C-RNTI) | Supported (PS-RNTI) Note that This is special case of Group triggering |
| Group-triggering (single DCI can trigger multiple UEs) | Not supported | Supported (PS-RNTI) |

Proposal 3

- To differentiate new DCI format from existing DCI formats, new RNTI needs to be introduced. The new RNTI could be assigned to one or more of UEs.

## DCI Format Design

As a first step toward the configurable power saving channel, the RAN1 made a following agreement.

Agreements: The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide: - Indication of one or more power saving associated operations. - Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format

### Scheduling DCI

The configurability of DCI is already part of design principle of NR R15. A field of DCI could be present or absent depending on the RRC configuration. That is, the RRC configuration determines the way that UE interpret the DCI content. Same principle is applied to enhanced scheduling DCI for power saving. During the last RAN1#96b meeting, two approaches were discussed.

- …

- Additional new field(s)

- Using the existing DCI format for the power saving purpose

  - Re-purpose field(s) in the DCI

  - The detection of existing DCI format as the indication for the power saving technique

Addition new field could be easily configured for DCI format 0_1 and 1_1 without affecting existing fields. The second approach is to repurpose a certain field in existing DCI. This requires the change of interpretation of the field. Consequently, the UE behavior also needs to be modified. And furthermore this approach could be applied only selected power saving features which could have some relation with existing R15 feature.

For PDCCH monitoring skipping purpose, having additional new field(s) is better approach. Repurposing a certain field is not a viable approach since there is no similar R15 feature or DCI field which is relevant to PDCCH monitoring skipping. As a simple approach, we consider adding a separate field for each supported power saving scheme as shown in Figure 3. Depending on the need, the length for each field could be configured differently.

Figure 3 The scheduling DCI with additional power saving (PS) fields configured to support power saving feature(s)

Proposal 4

- NR supports adding RRC configuration of additional bit field(s) in scheduling DCI (0_1 and 1_1) at least for PDCCH skipping purpose.

### New DCI

The same configurability principle applies to new DCI as well. The difference from scheduling DCI is that the new DCI’s CRC is scrambled with PS-RNTI and potentially multiple UE’s (or more precisely multiple groups) power saving signal could be carried in a single DCI.

Note that since each UE could be assigned same or different PS-RNTI, the PS-RNTI itself could be used to specify a set of UEs which is supposed to receive the DCI. Then, each UE could be configured to monitor certain sets of fields in a DCI. (Note that if two UEs are configured to monitor the same field of DCI, then, the signal is destined to the two UEs  from gNB point of view. But from a UE’s point of view this could be UE specific information. This is second level grouping, which could be used if needed.)

We provide an example of grouping for network operation. In this example, gNB could divide e.g., 6 UEs in a cell into two groups of UEs; 3 UEs in group A and other 3 UEs in group B. gNB can assign PS-RNTI of e.g., 0xAAAA to UEs in group A and PS-RNTI of e.g., 0xBBBB to UEs in group B. Figure 4 shows the two DCIs for power saving scrambled with different PS-RNTIs. UE1, 2, and 3 can receive the first DCI and UE4, 5, and 6 could receive the second DCI. Each UE monitors the assigned UE specific field to monitor as noted in the figure.

Figure 4 Example of PS DCIs with group based on UEs

Observation 5

- gNB could assign one or more PS-RNTI(s) to each UE to monitor DCIs for power saving purpose.

gNB packet scheduler could take turns between two groups A and B for data scheduling. While one group is being scheduled, the other group could be in PDCCH monitoring skipping (or even enter to DRX OFF state). The single DCI could be used to make a group of UEs skip PDCCH monitoring for a specified time. OF course, since the DCI include UE specific fields, it could initate other power saving features separately for each UE.

Figure 5 PS DCI differentiated by PS-RNTI indicating PDCCH skipping to groups of UEs

The UE specific PDCCH monitoring skipping could be still used by power saving field in the scheduling DCI.

Proposal 5

- For new DCI for power saving purpose, each UE is assigned PS-RNTI and UE specific bit field location of the DCI to monitor.

## PDCCH Monitoring Skipping Signal

Considering the fact that PDCCH monitoring without grant is the biggest reason for UE’s high power consumption, one can easily expect that the skipping PDCCH monitoring signal in RRC_CONNECTED mode would have the highest impact on UE battery life. Through extensive study during study item, it has been shown that significant amount of power (up to 85%) could be saved [1].

Observation 6

- Reducing PDCCH-only monitoring has the highest impact on UE power consumption.

### Skipping Duration

Considering the fact that the power saving signal is to be used in RRC_CONNECTED mode where packets arrive dynamically. The inter-arrival time of packet/grant not only depends on actual data arrival at RAN from application server (in Internet) but also depends on packet scheduler behavior to serve multiple UEs. So, for most applications, inter-arrival times are not usually deterministic. To fully utilize such opportunity between grants, it is necessary to support various PDCCH monitoring skip durations such as 10ms, 20ms, etc which are RRC configurable. One code point for “no skipping” is also required to indicate continuous monitoring according to search space configuration.

Proposal 6

- The PDCCH skipping indication in power saving signal/channel supports various lengths of PDCCH monitoring skipping, which could be RRC configured.

In addition to that, it is useful to have indication for skipping until the start of next DRX ON duration. In terms of functionality this could be similar to DRX command MACCE. One can argue that having it is redundant but supporting those functionality with L1 signal can greatly increase its usability in the situation with dynamic packet arrival. Note that MACCE processing time is longer than that of L1 signal, thus MACCE based approach may not be adequate in dynamic packet arrival scenario. Another benefit for having it is possibility of extension to multi-cell scenario. Currently DRX operation is defined per UE, meaning that both PCell and SCells have the same DRX state. In this case DRX Command MACECE would have impact to all cells simultaneously. Whereas, PDCCH skipping could be potentially extended per cell basis. In this sense, it is more useful than DRX Command MACCE.

Table 2 Example of PDCCH Monitoring Skipping Signal Encoding of DCI monitored in Active Time

| PDCCH monitoring skipping signal | Skipping duration (RRC configurable) |
| --- | --- |
| 00 | No skipping, i.e., keep monitoring according to search space configuration |
| 01 | 10ms |
| 10 | 20ms |
| 11 | Skip until the start of next DRX ON duration |

Observation 7

- PDCCH monitoring skipping is faster than DRX Command MACCE and could be extended to per-cell basis signaling.

Proposal 7

- The DCI for power saving purpose can be configured with PDCCH monitoring skipping signal of two bits.

- PDCCH monitoring skipping is extended to per cell-basis.

### Outside of Active Time

PDCCH monitoring skipping signal could be transmitted both inside Active Time and outside of Active Time. The use cases discussed above is relevant to Active Time. For outside of Active Time, a UE could be configured to monitor PDCCH before DRX ON duration (in the context of wake up signal). The signal carried in DCI transmitted during this time occasion could include three different types of signals; wake up, go-to-sleep, PDCCH monitoring skipping. The wake up use case is further discussed later. The difference of go-to-sleep and PDCCH monitoring skipping is a bit discussed during last RAN1, but not clearly defined. We think it is beneficial to differentiate them for discussion / and design.

- If go-to-sleep is applied, UE skips entire upcoming ON duration. Technically, it keeps UE stay in Non Active Time during the time otherwise would be DRX ON duration. Therefore, UE behavior is changed; while UE is skipping the DRX ON duration, UE is not supposed to transmit SRS/CSI. Note that this operation has direct impact on RAN2 DRX operation.

- If PDCCH monitoring skipping is applied, UE literally skips PDCCH only. Other RRC configured activities should happen as scheduled.

Proposal 8

- In outside of Active Time, both go-to-sleep and PDCCH monitoring skipping are supported to be indicated.

Table 3 Example of PDCCH Monitoring Skipping Signal Encoding for DCI monitored outside of Active Time

| PDCCH monitoring skipping signal | Skipping duration (RRC configurable) |
| --- | --- |
| 00 | No skipping, i.e., keep monitoring according to search space configuration (UE moves to Active Time) |
| 01 | PDCCH monitoring skipping for X slots (UE moves to Active Time) |
| 10 | PDCCH monitoring skipping for upcoming DRX ON duration (UE moves to Active Time) |
| 11 | Go-to-sleep for upcoming DRX ON duration (UE stays in non Active Time) |

### DCI carrying PDCCH Monitoring Skipping Signal

PDCCH monitoring skipping could be signaled both in Active Time and outside of Active Time. During Active Time, scheduling DCI and a new DCI could be used for signaling. During outside of Active Time, new DCI could be used. The new DCI in Active Time could be potentially different from the DCI monitored outside of Active Time.

## Wake-up Signal

Wake-up signal could also be considered as a part of power saving signal content. The purpose of this signal to wake UE up to monitor PDCCH during upcoming DRX ON duration. The operation of “waking up UE” implies the change of the behavior of UE monitoring wake up signal, which is strongly related to RAN2. Key part of design would be UE behavior and wake up signal transmission from gNB.

### UE’s Understanding on gNB Behavior on Wake up Signal Transmission

There could be at least two ways of supporting wake up signal in gNB side.

- The first approach is to always send wake up signal on configured wake up signal transmission occasion and indicate whether to wake up or not. This approach is based on explicit signaling and UE, once configured, is expected to receive signal during every monitoring occasion and will decide what to do after receiving the signal. So, the signal should include both “wake-up”, “sleep”, and other indication (such as PDCCH monitoring skipping). Note that “sleep” indication makes UE skip corresponding DRX ON duration. In terms of resource usage, it is not so efficient since the power saving signal needs to be sent always every monitoring occasion.

- The second approach is to send wake up signal only when gNB wants to wake UE up. This approach is more efficient in terms of resource usage. Note that the signal transmission itself indicates wake-up. The absence of signal could indicate something else, which is discussed below.

- The third approach is to send go-to-sleep signal only when gNB wants to make UE skip upcoming DRX ON duration. If UE detects no signal, then default UE behavior is to wake up.

Due to radio resource utilization perspective, the first approach is the worst choice since it requires gNB to sends PS signal before every DRX ON duration. The second approach is most power efficient since it requires no signal during CDRX operation.

Proposal 9

- If wake up signal is introduced in power saving signal, the wake up signal is sent only when network wants to wake UE up.

### UE Behavior when No PS signal Detection

Under the second approach, when UE detects no PS signal before DRX ON duration, UE could behave two different ways as follows.

- Option 1) UE stays outside of Active Time (DRX OFF) until next DRX ON duration start time. UE is not supposed to transmit SRS/CSI during this time.

- Option 2) UE skips PDCCH monitoring only. This allows UE to perform RRC configured activities such as SRS/CSI transmission.

Clearly Option 1 has higher impact to R15 UE behavior. Option 2) has no impact to those scheduled activities but still allow UE to take advantage of power saving opportunity. If UE has no RRC configured activity and receives no wake up signal, then UE can enter sleep state while it is in Active Time. Thus, Option 2 has no impact to DRX operation.

Proposal 10

- If UE detects no PS signal before DRX ON duration, UE skips PDCCH monitoring during DRX ON duration and performs RRC scheduled activities such as CSI/SRS transmission, if needed.

### DCI carrying Wake Up Signal

The wake up signal is transmitted only before DRX ON duration. Thus, new DCI can carry wake up signal.

## Search Space Configuration

The search space configuration includes many parameters for PDCCH monitoring such as periodicity, offset, pattern, duration, aggregation level(AL), number of PDCCH candidates, search space type, and DCI Formats to monitor.

### Transmission Time Occasion for PDCCH Monitoring Skipping Signal

Inside Active Time

Note that power saving signal should be able to be transmitted to trigger supported power saving schemes in different time occasions. When power saving signal is transmitted with scheduling DCI and new DCI in Active Time, it goes though UE-specific Search Space (USS) which is configured to support PDCCH transmissions. The search space configured for data scheduling could be directly used. In terms of signaling opportunity for triggering power saving feature, this provides highest flexibility which is as flexible as data scheduling flexibility.

Observation 8

- Power saving signal for PDCCH monitoring skipping in Active Time is be sent though USS.

Outside Active Time

As discussed above, the PDCCH monitoring skipping could be also transmitted in the time occasion for wake up signal transmission. In this case, there should be corresponding search space configured. This search space could be just one of many search spaces which are configured for data scheduling. However, in R15, UE is not supposed to monitor the configured search space outside of Active Time. So, in order to support wake up signal feature, it is necessary to allow UE to monitor search space before DRX ON duration.

Observation 9

- For power saving purpose, UE should be able to monitor PDCCH outside Active Time.

One approach for realizing this is to change the definition of Active Time. Currently in 38.321, Active Time is defined as follows.

When a DRX cycle is configured, the Active Time includes the time while: - drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

However, this approach is also problematic since once the Active Time is extended to include the time occasion for PS signal transmission before DRX ON duration, UE by configuration could monitor data scheduling DCI as well in that time occasion. This may need to be avoided since then UE has to prepare receiving PDSCH, which UE wants to avoid for power saving. To avoid this, a restriction for data scheduling needs to be introduced. This approach may introduce an exceptional UE behavior in Active Time.

Another approach is to allow UE monitoring PDCCH for power saving purpose outside of Active Time without changing the definition of Active Time. Defining a new type of Active Time, say Active Time for Power Saving, could make this problem simple from spec point of view. For example, Active Time for Power Saving could be defined as the time occasion where UE monitors PS DCI only (i.e., a dedicated UE-specific search space where only DCI scrambled with PS-RNTI is monitored).

The second approach is more clean from spec development point of view.

Proposal 11

- To support wake up signal monitoring before DRX ON duration, a dedicated UE specific search space could be defined where only DCI format with CRC scrambled with PS-RNTI is monitored.

Aggregation Level and the Number of PDCCH Candidates

The AL and number of PDCCH candidates just follows that of regular USS configuration. Since we consider UE specific approach for power saving signal, all of existing AL should be supported, which can support various UEs with diverse channel conditions. Considering the impact to hardware complexity, it may not be desirable to increase maximum number of number of blind decoding per CC.

Proposal 12

- Support all the aggregation levels defined in R15 for PDCCH based power saving signal/channel.

DCI format size budget and Blind Decoding

In R15, there is 3+1 DCI format size budget, which basically says that 3 different DCI format sizes with CRC scrambled with C-RNTI and one other size with different RNTI could be monitored per slot. This restriction indirectly limit the number of blind decoding UE needs to perform. To not increase the limit, this restriction needs to be preserved. This is achieved by aligning the size of new DCI for PS with other DCI.

Proposal 13

- Align the size of new DCI with that of other existing DCI.

# Conclusions

In this contribution, we have provide detailed design of power saving signal/channel. We see there is clear benefit of supporting both scheduling DCI and new DCI for power saving signals such as PDCCH monitoring skipping. Based on above discussion, we make following observations and proposals.

Proposal 1

- From specification point of view, DCI(s) to support power saving features is/are specified and existing PDCCH is used for carrying the DCI(s).

Observation 1

- Scheduling DCI (format 0_1 and 1_1) could be used for carrying power saving signal(s) at least including PDCCH skipping monitoring.

Observation 2

- NR needs to support of DCI triggering a single UE only.

Observation 3

- NR needs the support of a new DCI to support PDCCH monitoring skipping.

Proposal 2

- NR supports that both new DCI and scheduling DCI (0_1 and 1_1) can carry power saving signal.

Observation 4

- For a new DCI format for supporting power saving, group signaling could be used in case DCI size is large enough to carry more than one UE’s PS bits.

Proposal 3

- To differentiate new DCI format from existing DCI formats, new RNTI needs to be introduced. The new RNTI could be assigned to one or more of UEs.

Proposal 4

- NR supports adding RRC configuration of additional bit field(s) in scheduling DCI (0_1 and 1_1) at least for PDCCH skipping purpose.

Observation 5

- gNB could assign one or more PS-RNTI(s) to each UE to monitor DCIs for power saving purpose.

Proposal 5

- For new DCI for power saving purpose, each UE is assigned PS-RNTI and UE specific bit field location of the DCI to monitor.

Observation 6

- Reducing PDCCH-only monitoring has the highest impact on UE power consumption.

Proposal 6

- The PDCCH skipping indication in power saving signal/channel supports various lengths of PDCCH monitoring skipping, which could be RRC configured.

Observation 7

- PDCCH monitoring skipping is faster than DRX Command MACCE and could be extended to per-cell basis signaling.

Proposal 7

- The DCI for power saving purpose can be configured with PDCCH monitoring skipping signal of two bits.

- PDCCH monitoring skipping is extended to per cell-basis.

Proposal 8

- In outside of Active Time, both go-to-sleep and PDCCH monitoring skipping are supported to be indicated.

Proposal 9

- If wake up signal is introduced in power saving signal, the wake up signal is sent only when network wants to wake UE up.

Proposal 10

- If UE detects no PS signal before DRX ON duration, UE skips PDCCH monitoring during DRX ON duration and performs RRC scheduled activities such as CSI/SRS transmission, if needed.

Observation 8

- Power saving signal for PDCCH monitoring skipping in Active Time is be sent though USS.

Observation 9

- For power saving purpose, UE should be able to monitor PDCCH outside Active Time.

Proposal 11

- To support wake up signal monitoring before DRX ON duration, a dedicated UE specific search space could be defined where only DCI format with CRC scrambled with PS-RNTI is monitored.

Proposal 12

- Support all the aggregation levels defined in R15 for PDCCH based power saving signal/channel.

Proposal 13

- Align the size of new DCI with that of other existing DCI.

# References

- TR38.840 Study on UE Power Saving

- RP-190727 New WID: UE Power Saving in NR

- R1-1714092 Control Information for UE Power Saving, Apple

- TS38.321 Medium Access Control (MAC) protocol specification