3GPP TSG RAN WG1 #97				R1-1907382

Reno, USA, May 13th – 17th, 2019

Source:	WILUS Inc.

Title:	Enhancement to initial access procedure for NR-unlicensed

Agenda item:	7.2.2.2.2

Document for:	Discussion/Decision

- Introduction

In the RAN1#96 and previous RAN1#96bis meeting, there was a discussion on initial access and mobility procedure for NR-U operation and the followings were agreed [1], [2].

Agreement at RAN1#96 meeting - At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported - FFS: o Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario o Reporting of a new medium contention/load metric other than channel occupancy o Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum Agreement at RAN1#96 meeting - For a given cell, UE may assume SS/PBCH blocks in the same candidate position within the DRS transmission window are QCL across DRS transmission windows o Alt1: The PBCH DMRS sequence index is also the same o Alt2: The PBCH DMRS sequence index may be different o Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame - FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows Agreement at RAN1#96 meeting - An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations. o FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window o FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations. o FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window. - FFS: Mechanism to handle missing RLM-RS due to LBT failure Agreement at RAN1#96bis meeting The maximum DRS transmission window duration is 5 ms. - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS. - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions. - FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values. Agreement at RAN1#96bis meeting For a given cell, the UE may assume that the PBCH DMRS sequence index is the same for SS/PBCH blocks that are transmitted at the same candidate positions across DRS transmission windows. Agreement at RAN1#96bis meeting UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

In this contribution, we provide our view on enhancement to initial access procedure for NR-U operation and discuss SS/PBCH block reception in DRS. We also further discuss LBT failure and SS/PBCH transmission with equal transmission opportunities for each SS/PBCH block index across different DRS transmission windows.

- Discussion

On SS/PBCH block reception due to LBT failure

Similar to DRS in LTE-LAA having multiple opportunities within a configured DMTC window, it was agreed that multiple SS/PBCH block transmission opportunities can set up within DRS transmission window, i.e., 5ms  for NR-U operation. Figure 1 shows the candidate slot positions for SS/PBCH block transmission in a given time window at 30 kHz subcarrier spacing. As shown in Figure 1, the transmission opportunity for maximum L SS/PBCH blocks can be repeatedly allowed to the next slots for the SS/PBCH block transmission due to LBT failure within a given time window for SS/PBCH block. From the UE’s perspectives, it may be assumed that the UE receives the same SS/PBCH block index once in multiple opportunities within a given time window from one SS/PBCH block burst set to the next SS/PBCH block burst set.

- Maximum L=4 # of SS/PBCK blocks in case of 30kHz subcarrier spacing

- Maximum L=8 # of SS/PBCK blocks in case of 30kHz subcarrier spacing

Figure 1. Candidate slot positions for SS/PBCH block transmission for a given time window

- Proposal 1: From the UE’s perspectives, a UE can assume to receive the same SS/PBCH block index once in multiple opportunities within a given time window from one SS/PBCH block burst set to the next SS/PBCH block burst set.

Also, the UE assumes that DRS transmission periodicity should be 20ms for initial access similar to SS/PBCH block reception in Rel-15 NR. The periodicity 40ms similar to the DRS in LTE-LAA can be further considered to make the Cat-2 LBT possible prior to DRS transmission within 1ms and the network can configure flexibly one value within a set of multiple of 40ms for measurement purpose.

On SS/PBCH transmission to provide equal transmission opportunity for each SS/PBCH block index across different DRS transmission windows

During NR-U study item phase, it was discussed to determine how SS/PBCH block transmissions are performed after LBT success. Several alternatives have been identified the shift granularity for candidate time domain SS/PBCH block positions/candidate groups of SS/PBCH blocks, duration of DRS transmission window, and duration of the transmitted DRS within the window including SS/PBCH blocks and other multiplexed signals/channels. One of alternatives is to cyclically wrap around the SS/PBCH block index dropped due to LBT failure at the end of the burst set transmission as shown in Figure 2 [3].

Figure 2. The cyclically wrapping around between candidate SS/PBCH block location index and actually transmitted SS/PBCH block index for a given time window [3]

This alternative seems to be advantageous due to the fact that SS/PBCH blocks with the same SS/PBCH block index can be assumed to be QCL-ed as same as in NR Rel-15 and also it may not be necessary to specify the QCL assumption within a given time window. However, mapping fixed SSB block index into multiple candidate SS/PBCH block locations over different turn-around time window cycle results in unequal number of opportunities for SS/PBCH block transmissions, depending on the SS/PBCH block indices over a longer term. Therefore, we propose to consider SS/PBCH block transmissions with equal transmission opportunities for each SS/PBCH block indices over consecutive turn-around time window cycles. Figure 3-(a) and 3-(b) exemplify the 30kHz subcarrier spacing for SS/PBCH block transmissions (e.g. L=8) with equal transmission opportunities over multiple of 5ms given DRS transmission time windows. Also, Figure 3-(a) and 3-(b) show how to make the equal TX opportunities for each SS/PBCH block indices over multiple DRS TX time windows. To achieve this, we propose re-ordering methods for the SS/PBCH block indices between 2 different DRS TX windows by using a block-shifted order or a reverse order. It seems beneficial in terms of having the same transmission opportunities for each SS/PBCH block index as well as no ambiguity of SS/PBCH index at the candidate SS/PBCH block locations, i.e., SS/PBCH blocks with the same SS/PBCH block index can be assumed to be QCL-ed as same as in NR Rel-15. In addition, considering the fact that the LBT attempt always starts from the first slot in a given window for SS/PBCH block transmission, it may be further considered that a mapping between the candidate position and the actually transmitted SS/PBCH block indexes is performed in reverse order of SS/PBCH block index from the x-th to the (x+1)-th time window as shown in Figure 3-(b).

- Proposal 2: We propose to consider SS/PBCH block transmissions with equal transmission opportunities for each SS/PBCH block indices over consecutive DRS transmission time windows.

- Mapping of shifted order between candidate SS/PBCH block location index and actually transmitted SS/PBCH block index in a different time window cycle

- Mapping of reverse order between candidate SS/PBCH block location index and actually transmitted SS/PBCH block index in a different time window cycle

Figure 3. Under the cyclic wrapping around, SS/PBCH block transmissions as equal transmission opportunity as possible over different turn-around time window cycle

Regarding the derivation of frame timing from SSBs, one of multiple candidate SS/PBCH positions with same SS/PBCH index within a configured DRS transmission window can be indicated by PBCH without any extension of PBCH DMRS sequence. If PBCH DMRS sequence can be extended, it can be further considered that the detected position of SS/PBCH block can be indicated by PBCH DMRS sequence.

- Proposal 3: One of multiple candidate SS/PBCH positions with same SS/PBCH index within a configured DRS transmission window can be indicated by PBCH without consideration of any extension of PBCH DMRS sequence

Conclusion

In this contribution, we have discussed SS/PBCH block reception in DRS and SS/PBCH transmission with equal transmission opportunities for each SS/PBCH block index across different DRS transmission windows. We summarize our view as follows.

- Proposal 1: From the UE’s perspectives, a UE can assume to receive the same SS/PBCH block index once in multiple opportunities within a given time window from one SS/PBCH block burst set to the next SS/PBCH block burst set.

- Proposal 2: We propose to consider SS/PBCH block transmissions with equal transmission opportunities for each SS/PBCH block indices over consecutive DRS transmission time windows.

- Proposal 3: One of multiple candidate SS/PBCH positions with same SS/PBCH index within a configured DRS transmission window can be indicated by PBCH without consideration of any extension of PBCH DMRS sequence

References

[1] Draft Report of 3GPP TSG RAN WG1 #96bis v0.1.0

[2] Final Report of 3GPP TSG RAN WG1 #96 v1.0.0

[3] R1-1813906, “Feature lead summary #1 of initial access and mobility”	Charter Communications