3GPP TSG RAN WG1 #97						  R1-1907387

Reno, USA, May 13th – 17th, 2019

Source:	WILUS Inc.

Title:	On UL cancellation scheme for NR URLLC

Agenda item:	7.2.6.5

Document for:	Discussion/Decision

- Introduction

In RAN#83 plenary meeting, the WID on physical layer enhancements for NR URLLC was approved [1]. Regarding inter UE Tx prioritization/multiplexing, the following scope is defined.

- Specification of enhanced inter UE Tx prioritization/multiplexing [RAN1]

  - UL cancelation scheme (see section 7.2.1 in TR 38.824)

  - Enhanced UL power control scheme (see section 7.2.2 in TR 38.824)

Similarly, as in Rel-15 DL eMBB/URLLC multiplexing, reserving UL time-frequency resources to facilitate the potential UL transmission of sporadic URLLC traffic would reduce network resource efficiency. Thus, UL inter-UE multiplexing with dynamic indication (referred to as UL cancelation indication) will be a key solution for multiplexing UL transmissions with various latency and reliability requirements. Regarding UL cancelation scheme, the following agreements are made in the last RAN1#96bis meeting

Working assumption: - PDCCH is used for UL cancelation indication - The Working assumption can be revisited if the DCI for cancelation indication only carry very small number of information bits, e.g. 1 bit. Agreements: - Upon detecting an UL cancelation indication, at least stop without resuming is supported - FFS whether and how to support stop with resume Agreements: - Further discuss which UL transmissions that can potentially be cancelled by the UL cancelation indication, including - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS - PRACH Agreements: - Further discuss, aiming for down-selection, the group common DCI and UE-specific DCI for UL cancelation indication - For group common DCI (different from Rel-15 SFI) - UE is configured to monitor a group common DCI which indicates the time/frequency region on which the UL cancellation indication applies - For UE specific-DCI - When applicable, UE is configured to monitor a second UL grant for the same TB as an earlier PUSCH indicating UL cancellation before the end of the earlier PUSCH transmission. In this case, the UE follows the UL cancellation indication.

In this contribution we provide our views on the UL cancelation scheme.

- UE-specific PDCCH vs Group-common PDCCH

It was agreed in the last RAN1#96bis meeting to down-select one format either a UE-specific PDCCH or a group-common PDCCH. Generally, the UE-specific PDCCH can be suitable for UL cancellation indication when PUSCH for a single UE is cancelled. On the other hand, the group-common PDCCH can be suitable for UL cancelation indication when PUSCHs for multiple UEs are cancelled. Since it cannot be confirmed that there is only one UE transmitting PUSCH, it may be necessary to support UE behaviors for the case that PUSCHs for multiple UEs are cancelled. If the DL control resources are enough to transmit separate UE-specific PDCCHs to multiple UEs, the UE-specific PDCCH can be used in the case the PUSCHs for the multiple UE are cancelled. In fact, the similar issue was already discussed in Rel-15 to support DL preemption indication and there is no big difference between DL cancelation indication and UL cancelation indication. Thus, similarly as in Rel-15, it seems beneficial to support UL cancelation indication via the group-common PDCCH in terms of the efficient DL control channel resource utilization. Additionally, the UE-specific PDCCH is an UL grant for the same TB as an earlier PUSCH indicating UL cancelation before the end of the earlier PUSCH transmission. If RAN1 tends to support cancelation of semi-persistent or periodic UL signals, such as PUCCH, configured grant PUSCH, or SRS, a new UE-specific PDCCH should be introduced because there are no UL grant corresponding the semi-persistent or periodic UL signals. Due to the new UE-specific PDCCH, a UE complexity of blind decoding may increase, and it has negative impacts on PDCCH blocking probability.   Thus, our preference is to use a group common PDCCH format.

- Proposal 1: Support a group-common PDCCH format for the UL cancelation indication.

- Resume for PUSCH with UCI

During the study item phase, several options for the UL cancelation indication were discussed. Here, we investigate the case that the PUSCH containing UCI such as HARQ-ACK information is cancelled. Typically, the impact of cancellation of the PUSCH without UCI may be marginal since the cancelled PUSCH can be rescheduled and re-transmitted with a single UL grant. The DL control resource for the UL grant and the UL data resource for retransmission of the cancelled PUSCH are additionally needed. However, if the UCI such as HARQ-ACK information multiplexed in the PUSCH is cancelled, then gNB will have to reschedule multiple PDSCHs, which would significantly increase the waste of DL resource (both DL control resource and data resource). This problem becomes even more serious in a carrier aggregation case, where the cancelled UCI may contain HARQ-ACK information for PDSCHs in multiple serving cells. Based on observations, it can be desirable to distinguish between UL-SCH cancelation case and UCI cancelation case.

- Observation 1: It is necessary to prevent the HARQ-ACK information in the PUSCH from being cancelled as much as possible since it is required to reschedule and retransmit all cancelled PDSCHs.

Figure 1. Examples of UL cancelation indication

If a part of the PUSCH with the UL-SCH is cancelled, the question is whether or not to transmit the non-cancelled part of the PUSCH with UL-SCH. If the cancelled resource is very small, it would be better to transmit the non-cancelled part of the PUSCH transmission. However, in order to provide high reliability to the URLLC UE, almost all resource elements in the symbol may be cancelled and thus it may not be possible to decode the PUSCH only with the non-cancelled part. When considering the PUSCH with the UL-SCH rescheduling by gNB, it seems beneficial to drop the PUSCH and waits a UL grant rescheduling the PUSCH. However, if the PUSCH includes HARQ-ACK information and resources mapping to the HARQ-ACK information are not cancelled, then the UE does not need to drop resources including the HARQ-ACK information. As shown in Figure 1-(a), if the cancelled resources overlap with the resources mapping UL-SCH only and do not overlap with the resources mapping UCI and DMRS, then the UE drops the resources mapping UL-SCH and transmits the DMRS and UCI. Conversely, if the cancelled resources overlap with either of the DMRS or UCI, then the UE drops PUSCH on the scheduled resource.

- Proposal 2: When a PUSCH is cancelled, then the following UE behaviours can be considered:

  - If at least one resource element mapping to UCI or DMRS in PUSCH is cancelled, then drop PUSCH on the scheduled resource.

  - If all of resource elements mapping to UCI or DMRS in PUSCH are not cancelled, then the DMRS and UCI are transmitted on that resources and the UE does not cancel to transmit the DMRS and UCI.

As described above, the UCI cancelation would result in a significant waste of DL resource. In order to avoid wasting such resources, the information related to resuming the cancelled UCI can be contained in the UL cancelation indication. In Figure 2, the UL cancelation indication also indicates a time domain resource for resuming the cancelled UCI. When the UE detects the UL cancelation indication, the UE first checks whether the DMRS and UCI are cancelled or not. If it is not cancelled, then the UE transmits the DMRS and UCI. On the other hand, if the DMRS or UCI is cancelled, the UE can retransmit the DMRS and UCI according to the information for resuming the cancelled UCI as in the UL cancelation indication. From this UCI resuming mechanism, it can be expected to improve the DL resource utilization.

Figure 2. UL cancelation indication with UCI rescheduling information

If the UL cancelation indication is carried via a group-common PDCCH, it is necessary to discuss how to transmit the information for resuming the cancelled UCI to each impacted UEs. One simple rule is to configure a fixed interval without dynamic indication. If the UCI on a PUSCH is cancelled, the UE retransmits the cancelled UCI after the configured interval. If the fixed interval is configured to 0, then the cancelled UCI can be transmitted right after the cancelled resource indicated by the UL cancelation indication. Additionally, a dynamic indication can be further considered to enhance UCI transmission flexibility. For example, the number of symbols or the number of slots/sub-slots can be dynamically indicated along with the UL cancelation indication.

- Proposal 3: Consider a UL cancelation indication that includes information for resuming the cancelled UCI.

  - FFS: detail indication designs

- Conclusion

In this contribution, we discussed the UL cancelation indication and the followings are proposed

- Proposal 1: Support a group-common PDCCH format for the UL cancelation indication.

- Observation 1: It is necessary to prevent the HARQ-ACK information in the PUSCH from being cancelled as much as possible since it is required to reschedule and retransmit all cancelled PDSCHs.

- Proposal 2: When a PUSCH is cancelled, then the following UE behaviours can be considered:

  - If at least one resource element mapping to UCI or DMRS in PUSCH is cancelled, then drop PUSCH on the scheduled resource.

  - If all of resource elements mapping to UCI or DMRS in PUSCH are not cancelled, then the DMRS and UCI are transmitted on that resources and the UE does not cancel to transmit the DMRS and UCI.

- Proposal 3: Consider a UL cancelation indication that includes information resuming the cancelled UCI.

  - FFS: detail indication designs

References

- RP-190726, “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)”, RAN#83

- TR38.824 v16.0.0, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”

- [3] RAN1 Chairman’s Notes, RAN WG1 Meeting #96bis