3GPP TSG RAN WG1 Meeting #97	R1-1907397
Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.7.2

Source:	Ericsson

Title:	IMT2020 self-evaluation - On eMBB spectral efficiency with NR

Document for:	Discussion

# Introduction

This contribution contains results on average spectral efficiency and 5th percentile user spectral efficiency for NR for eMBB.

The results show that the IMT-2020 requirements [1] can be met.

# Model and assumptions

Models and assumptions are aligned with the guidelines in [2]. For the Dense Urban scenario, in addition to the default setting with 80% low-loss buildings and 20% high-loss buildings, for the 30GHz case a scenario with 100% low-loss buildings is studied.

# Simulation results

Simulation results are presented in the tables below.

## Indoor Hotspot

Table 1: Indoor Hotspot – eMBB TDD DL
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | BW= 80MHz [OH 34.5%] | BW= 200MHz [OH 26.8%] | BW= 400MHz [OH 23.2%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 13.86 | 15.49 | 16.25 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 | 0.48 | 0.53 | 0.56 |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 13.76 | 15.38 | 16.13 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 | 0.49 | 0.55 | 0.58 |

Table 2: Indoor Hotspot – eMBB FDD DL
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | BW= 40 MHz [OH 32.2%] |
| --- | --- | --- | --- | --- |
| 32x8 MU-MIMO Type II Codebook; (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 120 | Average [bit/s/Hz/TRxP] | 9 | 12.27 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 | 0.44 |

Table 3: Indoor Hotspot – eMBB TDD DL
(Evaluation configuration C, CF=70 GHz, for 12TRxP)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | BW= 80MHz [OH 34.5%] | BW= 200MHz [OH 26.8%] | BW= 400MHz [OH 23.2%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 12.77 | 14.27 | 14.97 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 | 0.45 | 0.50 | 0,53 |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 12.63 | 14.11 | 14.81 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 | 0.44 | 0.50 | 0.52 |

Table 4: Indoor Hotspot – eMBB FDD DL
(Evaluation configuration C, CF=70 GHz, for 12TRxP)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | BW= 40 MHz [OH 32.2%] |
| --- | --- | --- | --- | --- |
| 32x8 MU-MIMO Type II Codebook; (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 120 | Average [bit/s/Hz/TRxP] | 9 | 11.42 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 | 0.41 |

## Dense Urban

Table 5: Dense Urban – eMBB TDD DL
(Evaluation configuration A, CF=4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | BW= 20MHz [OH 36.2%] | BW= 40MHz [OH 28.0%] | BW= 100MHz [OH 23.2%] | BW= 20MHz [OH 36.2%] | BW= 40MHz [OH 28.0%] | BW= 100MHz [OH 23.2%] |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 13.34 | 15.05 | 16.05 | 13.45 | 15.17 | 16.19 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.48 | 0.54 | 0.58 | 0.49 | 0.55 | 0.59 |
|  |  |  |  |  | BW= 20MHz [OH 38.8%] | BW= 40MHz [OH 30.8%] |  | BW= 20MHz [OH 38.8%] | BW= 40MHz [OH 30.8%] |  |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 13.58 | 15.36 |  | 13.82 | 15.62 |  |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.49 | 0.55 |  | 0.49 | 0.55 |  |

Table 6: Dense Urban – eMBB FDD DL
(Evaluation configuration A, CF=4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BW= 10MHz [OH 33.8%] | BW= 20MHz [OH 26.0%] | BW= 40MHz [OH 22.2%] | BW= 10MHz [OH 33.8%] | BW= 20MHz [OH 26.0%] | BW= 40MHz [OH 22.2%] |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 10.49 | 11.72 | 12.32 | 10.18 | 11.38 | 11.97 |
|  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.40 | 0.45 | 0.47 | 0.38 | 0.43 | 0.45 |

Table 7: Dense Urban – eMBB TDD DL
(Evaluation configuration B, CF=30 GHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | BW= 80MHz [OH 34.5%] | BW= 200MHz [OH 26.8%] | BW= 400MHz [OH 23.2%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;4,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 19.3 | 21.5 | 22.6 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.05 | 0.06 | 0.06 |
| 32x8 MU-MIMO, ‘wideband’; / type I (2 panel@UE) gNB Config = (8,16,2,1,1;4,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 15.3 | 17.1 | 18.0 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.05 | 0.06 | 0.06 |

Table 8: Dense Urban – eMBB TDD DL
(Evaluation configuration B, CF=30 GHz, 100% low-loss buildings)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | BW= 80MHz [OH 34.5%] | BW= 200MHz [OH 26.8%] | BW= 400MHz [OH 23.2%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;4,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 21.6 | 24.2 | 25.4 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.58 | 0.65 | 0.68 |
| 32x8 MU-MIMO, ‘wideband’; / type I (2 panel@UE) gNB Config = (8,16,2,1,1;4,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 16.9 | 18.9 | 19.8 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 | 0.44 | 0.49 | 0.51 |

Table 9: Dense Urban – eMBB TDD UL
(Evaluation configuration B, CF=30 GHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | BW= 80MHz [OH 19.4%] | BW= 200MHz [OH 19.1%] | BW= 400MHz [OH 19.0%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8x32 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;4,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 10.1 | 10.2 | 10.2 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 | 0.06 | 0.06 | 0.06 |

Table 10: Dense Urban – eMBB TDD UL
(Evaluation configuration B, CF=30 GHz, 100% low-loss buildings)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | BW= 80MHz [OH 19.4%] | BW= 200MHz [OH 19.1%] | BW= 400MHz [OH 19.0%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8x32 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;4,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 11.3 | 11.4 | 11.4 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 | 0.41 | 0.41 | 0.42 |

## Rural

Table 11: Rural – eMBB TDD DL
(Evaluation configuration A, CF=700 MHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | BW= 20MHz [OH = 34.0%] | BW= 40MHz [OH = 26.0%] |
| 8x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;1,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 6,61 | 7.41 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 | 0,19 | 0.21 |
|  |  |  |  |  | BW= 20MHz [OH = 34.9%] | BW= 40MHz [OH = 26.9%] |
| 8x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;1,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 6,25 | 7.02 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 | 0,16 | 0.18 |

Table 12: Rural – eMBB FDD DL
(Evaluation configuration A, CF=700 MHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BW= 10MHz [OH = 30.9%] | BW= 20MHz [OH = 23.2%] | BW= 40MHz [OH = 19.4%] |
| 8x2 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 5,16 | 5.74 | 6.02 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 | 0,15 | 0.17 | 0.17 |

Table 13: Rural – eMBB TDD DL
(Evaluation configuration B, CF=4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | BW= 20MHz [OH 36.2%] | BW= 40MHz [OH 28.0%] | BW= 100MHz [OH 23.2%] |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 15.02 | 16.95 | 18.08 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 | 0,45 | 0,51 | 0.54 |
|  |  |  |  |  | BW= 20MHz [OH 38.8%] | BW= 40MHz [OH 30.8%] |  |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 14.44 | 16.32 |  |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 | 0.42 | 0.47 |  |

Table 14: Rural – eMBB FDD DL
(Evaluation configuration B, CF=4 GHz)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BW= 10MHz [OH 33.8%] | BW= 20MHz [OH 26.0%] | BW= 40MHz [OH 22.2%] |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 12.17 | 13.60 | 14.30 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 | 0.39 | 0.44 | 0.46 |

Table 15: Rural – eMBB TDD DL
(Evaluation configuration C, LMLC)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | BW= 20MHz [OH = 34.0%] | BW= 40MHz [OH = 26.0%] |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 6.22 | 6.97 |
|  |  |  | 5th-tile [bit/s/Hz] | (0.12) | 0.19 | 0.21 |
|  |  |  |  |  | BW= 20MHz [OH = 34.9%] | BW= 40MHz [OH = 26.9%] |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 6.26 | 7.02 |
|  |  |  | 5th-tile [bit/s/Hz] | (0.12) | 0.19 | 0.22 |

Table 16: Rural – eMBB FDD DL
(Evaluation configuration C, LMLC)

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BW= 10MHz [OH = 30.9%] | BW= 20MHz [OH = 23.2%] | BW= 40MHz [OH = 19.4%] |
| 8x4 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 5.56 | 6.18 | 6.49 |
|  |  | 5th-tile [bit/s/Hz] | (0.12) | 0.18 | 0.20 | 0.21 |

As it can be seen from the tables, for all the three scenarios the IMT-2020 requirements are fulfilled. This holds for both average and 5th percentile SE. For 30GHz in the Dense Urban scenario, the average requirements are reached but not the 5th percentile requirements if 80% low-loss and 20% high-loss buildings are assumed. If 100% low-loss buildings are assumed, also the 5th percentile requrements are reached.

IMT-2020 requirements on spectral efficiency can be fulfilled.

# Conclusion

In this contribution, we have provided input to the Study Item on Self Evaluation towards IMT-2020 Submission. It has been shown that for both downlink and uplink the IMT-2020 requirements can be fulfilled in terms of both average and 5th percentile spectral efficiency for the test environments specified by ITU-R.

The following observations have been made:

Observation 1	IMT-2020 requirements on spectral efficiency can be fulfilled.

# References

Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s),” 2017.

Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020,” 2017.