3GPP TSG RAN WG1 #97			R1-1907421

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.8.5

Source:	Ericsson

Title:	On the number of TRPs for high reliability at 4 GHz

Document for:	Discussion and Decision

# Introduction

For high reliability, diversity is an important enhancement for NR Rel-16 using Multi-TRP transmission strategies. Multi-TRP diversity helps both in terms of macro diversity to handle blocking and mobility situations; but also in terms of mitigating fast-fading. In this contribution, it is investigated how the diversity and power gains scale for 2 TRPs up to 4 TRPs in a cooperating cluster. One diversity strategy studied is the FDM scheme with a single RV (Scheme 2a) [1], in which a PDSCH  is sent from different TRPs on different PRB-bundles in the frequency domain and perform power boosting accordingly, i.e., 3 dB for 2 TRPs, 6 dB for 4 TRPs.. The link level results are based upon the URLLC Macro scenario at 4 GHz with 4 TX-port TRPs and a 4 RX-port UE and 256 bits packet size.

# Performance evaluation of Multi-TRP diversity for reliability

In this section, the link level simulation performance results are presented, and observation and conclusions are drawn from these results.

## Multi-TRP cooperating cluster for 1-4 TRP transmissions

The present contribution considers downlink transmission at 4 GHz carrier frequency and 30 kHz sub-carrier frequency spacing. The channel model is CDL_A with a Multi-TRP setup as depicted in Figure 1.

The clusters used for joint transmission are TRP 0 for a single TRP, TRPs 0 and 1 for 2 TRPs etc. The results assume that all TRPs in the cluster are connected with an ideal backhaul as latency on the backhaul is not acceptable with a 1 ms latency requirement. The coordination between the TRPs is only in the scheduling, i.e., all TRPs schedule the same PDSCH and on same time-frequency resource for the data transmission but pre-coder selection is done independently per TRP according to UE reported PMI.

The precoding codebook is a 2 times spatially oversampled DFT-codebook. The CSI-RS and synchronization signals measurements are modelled as ideal, and the synchronization of the TRPs is also ideal. Frequency time allocation is 16 PRBs and 2 OFDM-symbols giving a code rate of about 0.44 for a single layer allocation using a single DMRS.

Figure 1: Multi-TRP clusters used for link-level evaluations

In Figure 2 the resource allocations for two cases are shown when Scheme 2a with frequency interleaving either 2 or 4 TRPs using the PRB bundle structure in frequency domain.

Figure 2: Frequency interleaving for 2 TPRs and 4 TRPs in Scheme 2a.

In order to check that the code rate is sufficiently low to harvest the Multi-TRP diversity gain, a 32 PRB frequency allocation was also evaluated giving a code-rate of less than ¼. To see the sensitivity to varying path-gain to the different TRPs , both the case when all TRPs are at 0 dB relative SNR and the case when TRP 0, 1, 2 and 3 are at 0 dB, -3 dB, -6 dB and -9 dB respectively were evaluated.

## Simulation results

In these simulation results it is investigated how increasing the number of TRPs provides benefit in a high reliability scenario. The frequency interleaving strategy is easy to specify for any number of TRPs. It is here considered using 1-4 TRPs as described above, see Figure 1. In all the simulation settings there is a significant diversity gain on top of the power gain using a second TRPs, see Figure 4 or Table 1.

Figure 4: Multi-TRP performance gains using 1-4 TRPs and Scheme 2a with frequency interleaving

The diversity gains seem quite independent upon if 16 or 32 PRBs is used and hence even at a code-rate close to ½ multi-TRP seems like a good solution even for 4 TRPs, this is probably due to the additional frequency diversity given by the 32 PRB allocation. Multi-TRP diversity gains are especially good with 1 additional TRP at a relative power -3 to 0 dB compared to serving, this case seems very relevant for many deployments and scenarios. Observe that the gains here are at 10-4, the gain would be even larger moving at smaller BLER values such as 10-5 and 10-6.

In some scenarios when reception from all 4 TRPs are close to each other in power it is also well motivated utilizing all 4 TRPs to transmit to the UE. Considering some URLLC scenarios, such as factory automation, traffic safety and drones, this type of scenarios also seems quite relevant in 5G.

Supporting up to 4 TRPs is well motivated as significant Multi-TRP reliability gains are observed in some scenarios using up to 4 TRPs

Table 1 shows that going from 2 to 4 TRPs can bring an additional 2-4 dB SNR gain to the link reliability.

Table 1: Multi-TRP gains for 10-4 BLER using Scheme 2a with frequency interleaving

|  | 1 to 2 TRPs | 2 to 3 TRPs | 3 to 4 TRPs |
| --- | --- | --- | --- |
| 16 PRBs, TRPs all at 0 dB | 5.3 dB | 2.6 dB | 2.0 dB |
| 16 PRBs, TRPs at 0, -3, -6, -9 dB | 3.7 dB | 1.1 dB | 0.2 dB |
| 32 PRBs, TRPs all at 0 dB | 5.4 dB | 2.6 dB | 2.0 dB |
| 32 PRBs, TRPs at 0, -3, -6, -9 dB | 3.9 dB | 1.3 dB | 0.7 dB |

# Conclusion

In this contribution, we have investigated FDM Scheme 2a with 2 to 4 TRPs for URLLC.  We made the following observation:

Observation 1	Supporting up to 4 TRPs is well motivated as significant Multi-TRP reliability gains are observed in some scenarios using up to 4 TRPs

# References

Chairman’s Notes, RAN1#96.

# Appendix: Simulation assumptions

| Parameter |  |
| --- | --- |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 20 MHz |
| Subcarrier spacing | 30 kHz |
| Cell layout | Hexagonal, 500m ISD |
| Channel model | CDL_A 300ns |
| BS antenna configuration | 4 TX: 2 x-pols horizontal 0.5 lambda separation |
| BS antenna height | 25m |
| UE antenna configuration | 4 RX: 2 x-pols horizontal 0.5 lambda separation |
| UE antenna height | 1.5m |
| DMRS channel estimation | Realistic |
| UE receiver | MMSE-IRC |
| Channel Estimation CSI-RS and Feedback assumptions | Ideal channel estimation, PMI reporting |
| Packet size | 256 bits |
| Allocation size | 16 PRBs x 2 OFDM-symbols or 32 PRBs x 2 OFDM-symbols |
| PRB bundle size | 2 PRBs |
| BS synchronization | Ideal (e.g. no propagation delay difference) |
| Code-rate | 1 Layer: 16 PRBs 0.44; 32 PRBs 0.22 |
| Channel coding | LDPC |