3GPP TSG RAN WG1 Meeting #97	R1-1907425

Reno, USA, May 13th-17th, 2019

Source:	Ericsson

Title:	Additional evaluation results on multi-TRP PDSCH schemes for URLLC

Agenda Item:	7.2.8.5

Document for:	Discussion and Decision

# Introduction

In RAN1#96bis, it was agreed that Scheme 1a, Scheme 3 and Scheme 4 for multi-TRP based URLLC will be supported.  The remaining issue is whether Schemes 2a and 2b should also be supported and if supported, one or both should be supported.

In post RAN1#96bis email discussion, some evaluation assumptions were further agreed for evaluating Schemes 2a,2B and any standard transparent schemes.  In [xxx], we have presented our results based on the evaluation assumption with code rate s of 0.12 and 0.44.  In this contribution, we present some additional results on Schemes 2a/2b and SFN with CDD with different pathloss offset between TRPs and an additional code rate of 0.27.

# Simulation Assumptions

The simulation setup is shown in Figure 1, where two TRPs are used. Downlink transmission at 4 GHz carrier frequency with 10MHz BW and 15 kHz sub-carrier frequency spacing is considered. The channel model is CDL-A. It is assumed that the two TRPs are connected with an ideal backhaul and under the control of a single scheduler. Two cross polarized antennas and thus 4 antenna ports are used in each TRP and at the UE.

Figure 1: Simulation setup

The schemes evaluated are illustrated in Error! Reference source not found., they are:

- SFN with CDD: PDSCH transmission over two TRPs with a single layer, where separate precoders are applied to each TRP based on PMI feedback and CDD is applied over the second TRP

- Scheme 2a: FDM with a single CW and thus single-RV over two TRPs, each allocated with half of the scheduled RBs

- Scheme 2b: FDM with two RVs over two TRPs, each allocated with half of the scheduled RBs

(a) SFN with CDD

(b) FDM single RV

- FDM multi-RV

- Figure 2: The schemes evaluated

Two resource allocations, i.e., 8RBs by 4 OFDM symbols and 40RBs by 2 OFDM symbols, are used, they are shown in Figure 3. A single symbol DMRS of type 1 was configured without data multiplexing. For 8RB allocation, code rate R= 0.44 was evaluated and for 40RB allocation, R=0.27 was evaluated. The code rates are for SFN with CDD and Scheme 2a with a single spatial layer.   The code rates for Scheme 2b are doubled due to the fact that only half of the RBs are available for each codeword.   Other simulation assumptions can be found in the Appendix.

Figure 3: Resource allocations.

# Evaluation Results

Three different pathloss (PL) offsets between the two TRPs, i.e., PL1 -PL2= 0dB, 3dB and 6dB, were simulated.  The results are shown in Figure 4 to Figure 6, respectively. It can be seen that Scheme 2a performs better than Scheme 2b in the case of 8RB allocation and slightly worse in the case of 40RB allocation.  Both schemes outperform SFN with CDD, particularly at PL offset = 0dB. The perform gap shrinks as the PL offset increases, likely because the second TRP contributes less to the decoding performance as its pathloss increases.

Figure 4:  0dB PL offset.

Figure 5: 3dB PL offset.

Figure 6: 6dB PL offset.

Scheme 2a and scheme 2b are quite close, they outperform SFN with CDD in most cases.

# Conclusions

In this contribution, we have presented addition evaluation results for Schmes 2a/2b and SFN with CDD for URLLC at 4GHz. Based on the results, we have the following observation:

Observation 1	Scheme 2a and scheme 2b are quite close, they outperform SFN with CDD in most cases.

# References

R1-1907418, “On multi-TRP and multi-panel”, Ericsson, RAN1#97.

# Appendix:  Simulation Assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| BW | 10MHz |
| Subcarrier spacing | 15kHz |
| Channel model | CDL-A with 300ns delay spread |
| UE mobility | 3km per hour |
| Number of TRPs | 2 |
| Pathloss offset of 2nd TRP | 0dB, 3dB and 6dB |
| Number of Tx antenna ports per TRP | 4 |
| Number of UE Rx antennas | 4 |
| Number of layers per TRP | 1 |
| DMRS configuration | One symbol, type 1, no data and DMRS multiplexing |
| Number RBs | 8, 40 |
| Number of OFDM symbols | 8 RBs: 4 40RBs: 2 |
| Code rate | 0.44, 0.27 |
| Modulation | QPSK |
| Channel estimation | Practical |
| PRG size | 2 RBs |
| Receiver | MMSE-IRC |
| Precoding | Wideband PMI |