3GPP TSG-RAN WG1 Meeting #97	 R1-1907455

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.2.2.2

Source:	Ericsson

Title:	Enhancements to initial access procedure

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss further details on procedures for initial access, RRM measurements and radio link monitoring.

# 2	Initial access

## 2.1	Timing and QCL determination

In NR Rel-15, cell search is accommodated by the UE reading at least one SS/PBCH block that provides time and frequency synchronization and the physical layer cell ID. The gNB can transmit up to L SS/PBCH blocks in a half frame, where L depends on the frequency range. The candidate SS/PBCH blocks in a half frame are indexed in ascending order in time from 0 to . The UE determines the SS/PBCH block index based on the detected PBCH DM-RS sequence when L ≤ 8, or a combination of the detected PBCH DM-RS sequence and the PBCH payload when 8 < L ≤ 64. Based on the SS/PBCH index, candidate positions as defined in [1], and the PBCH payload, the UE can establish cell timing.

In RAN1#96bis it was agreed to extend the number of SS/PBCH block candidate positions to Y=20 and Y=10 for 30 kHz and 15 kHz sub-carrier spacing respectively:

Agreement:
The maximum DRS transmission window duration is 5 ms.

- The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

  - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

Agreement:

UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

What is still not agreed is the mechanism to signal the SS/PBCH block candidate position and how the UE determines that two SS/PBCH blocks are QCL’d even if they are transmitted in different candidate positions.

Timing determination

In the RAN1#96bis feature lead summary [7], three groups (Alt-1, Alt-2 and Alt-4) of proposed mechanisms for timing determination are listed. Further Alt-1 is divided into sub-alternatives. Here we will focus on two of those sub-alternatives, namely Alt-1a and Alt-1b. Both options can be expressed as that the UE determines the timing (SS/PBCH candidate position) as t=Stx*c+s, where c is the cycle index indicated in the PBCH payload (analogous to NR-Rel15 in FR2), Stx is the number of cycled/transmitted DM-RS sequences and s denotes the DM-RS sequence index. The main difference between the alternatives is that in Alt-1a, Stx is fixed and equal to 8 as in Rel-15 and in Alt-1b, Stx depends on the maximum number of candidate beams Q (discussed further below).

For the UE to determine the timing t, Stx needs to be known. In Alt-1a, Stx is fixed and thus known to the UE. In Alt-1b, Stx, is not fixed and thus it needs to be signaled to the UE. Following the Rel-15 principle that the UE should be able to determine the cell timing after (at most) reading an SS/PBCH block, Stx would need to be signaled in either PBCH payload or MIB. Both options (PBCH payload or MIB) would imply changes to fundamental Rel-15 functionality and should in our view be avoided if possible. Therefore, we make the following proposal:

The UE determines the frame timing in the same way as in NR Rel-15 FR2 based on the decoded SS/PBCH block index (combination of 8 PBCH DM-RS sequences and 3 values (2 bits) in PBCH payload) and the half-frame indicator. Note that this implies that the number of PBCH DM-RS sequences is fixed to 8 as in Rel-15 (i.e. Alt-1a).

QCL determination

In Rel-15 NR the UE may assume that SS/PBCH blocks transmitted with the same SS/PBCH block index (and same SS/PBCH block position) on the same center frequency location are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and, when applicable, spatial Rx parameters. Conversely, the UE cannot assume quasi co-location for any other SS/PBCH block transmissions. This implies that SS/PBCH blocks with different indices (i.e. positions) shall be treated independently by the UE.

When the SS/PBCH blocks are allowed to be transmitted within a DRS transmission window based on LBT outcome, the mapping between SS/PBCH block candidate position (t) and QCL relation (b = beam index) needs to be decoupled.

In the RAN1#96bis feature lead summary [7], several options for how to determine the QCL relation were summarized. In common for most options is that they can be formulated as b = mod(t, Q), where Q denotes the maximum number of candidate beams, t the SS/PBCH block candidate position and b the beam index. The terminology “candidate beam” is used to convey the notion that the beam index b is related to a candidate position t in the DRS transmission window. The actual number of beams transmitted by the gNB may be less than Q. Note that even Rel-15 NR behavior can be formulated in the same way, in which case Q = L (maximum number of transmitted SS/PBCH blocks), even if t would never exceed L in Rel-15.

In previous meetings it has been highlighted that it is beneficial if the UE doesn’t have to read PBCH when performing RRM measurements of neighbor cells. Thus, before going further into the different options for QCL determination, two cases need to be discussed, namely RRM measurements with and without beam index reporting. Below we formulate two design goals and analyze Rel-15 NR and the proposed options for NR-U considering these.

- When performing RRM measurements without beam index reporting, the UE is not required to read PBCH payload of neighbor cells.

- When performing RRM measurements with beam index reporting, the UE is not required to read PBCH payload of neighbor cells

Our understanding is that in Rel-15 NR 1) is always fulfilled, because the UE can always determine that two SS/PBCH blocks are QCL’d using the relative timing between them.

For FR1 in Rel-15 2) is always fulfilled because the SS/PBCH index is encoded solely in the PBCH DM-RS sequence and thus reading PBCH is not necessary. For FR2 in Rel-15, the SS/PBCH index is encoded by a combination of PBCH DM-RS and PBCH payload. However, for intra frequency measurements, the UE can assume that the SS/PBCH block index can be derived based on the serving cell timing (deriveSSB-IndexFromCell = TRUE) and thus PBCH reading is not required. On the other hand, for inter-frequency measurements this is not always the case and thus PBCH reading is needed when deriveSSB-IndexFromCell = FALSE.

Because 1) is always fulfilled in Rel-15 NR, it seems reasonable to adhere to the same principle also for NR-U. According to our understanding, both Alt-1a and Alt-1b fulfils 1) as long as Q is known to the UE. In fact, all methods (including Alt-1a and Alt-1b) that can be formulated as b = mod(t, Q) where Q (maximum number of candidate beams) is known to the UE and t is the SS/PBCH candidate position, fulfils 1). The reason being that if the relative SS/PBCH block position (t2-t1) of two SS/PBCH blocks are separated by k*Q, where k is an integer, the two SS/PBCH blocks are QCL’d with each other. This was also discussed in [8].

Fulfilling 2) can be achieved if b = mod(t,Q) = mod(s,Q), where t=Stx*c+s, i.e. the UE can derive the beam index b from the detected DM-RS sequence (s) and knowledge of Q. One straightforward way to achieve this is restricting Q to 1,2,4,8 while still keeping Stx,constant equal to 8 as in Rel-15 NR.

To allow a more flexible Q, fundamental changes to Rel-15 NR behavior are needed by allowing the number of DM-RS sequences (Stx) to depend on Q and as a consequence also require that  Stx or Q is signaled in PBCH payload or MIB. In our view, such a change is not motivated by being able to configure Q=3,5,6,7 in addition to Q=1,2,4,8. It is far from clear why these additional values are needed given typical DRS compositions.

Based on the discussion above we make the following proposal:

The UE may assume that a detected SS/PBCH block in an SS/PBCH block burst set is QCL’d with a detected SS/PBCH block in a previous SS/PBCH block burst set if the detected SS/PBCH block index modulo Q is the same for both detected SS/PBCH blocks. Q is the maximum number of candidate beams. The parameter Q can have the value 1, 2, 4, or 8.

In Figure 1, several examples are shown of detected SS/PBCH block index and beam index (b) for Q=4.

Figure 1: Examples of different time shifted SS/PBCH block positions (SSBx) and the effective SS/PBCH block index QCLx or b. The effective SS/PBCH index, or QCL index, is given by QCLx = SSBx modulo Q.

## 2.2	Signaling of Q

Next, we turn to how to signal Q. The initial access procedure in principle consists of the three steps:

- Cell search with frame timing determination

- Reading of RMSI (including RACH configuration)

- Random access

Assuming that the number of PBCH DM-RS sequences are fixed (to 8) as in Rel-15, the UE can determine the frame timing (as in Proposal 1) without knowledge of Q.

To read RMSI the UE needs to monitor PDCCH in the Type0-PDCCH CSS. The location of the Type0-PDCCH CSS in relation to an SS/PBCH block is signaled in MIB. Thus, also for this step, knowledge of Q is not required, even if we recognize that some optimizations with regards to Type0-PDCCH CSS monitoring could be possible if Q would have been known to the UE at this stage.

In the next step the UE maps the detected SS/PBCH block to a RACH resource/preamble. For this step Q is needed, since the mapping must be based on the beam index determined as the detected SS/PBCH block index modulo Q.

Because Q is strictly only needed after RMSI has been read, and since modifying the MIB content and possibly changing the PBCH payload size is highly undesirable, we propose to signal Q in SIB1.

Support signaling of Q in SIB1, where Q is the number of non-QCL’d SS/PBCH blocks in a burst.

## 2.3	Random access

Support for RACH is relevant not only for the NR-U stand-alone scenario, but also for interworking (dual-connectivity) scenarios such as EN-DC.

In NR Rel-15, a mapping between transmitted SS/PBCH block(s) and a combination of ranges of preambles and random-access occasions (RO) is configured using ssb-perRACH-OccasionAndCBPreamblesPerSSB. This mapping is used for the UE to implicitly indicate to the gNB which SS/PBCH block that had the best reception conditions.

Fixed time relations between different signals/channels where LBT between the signals is required should not be assumed. If the mapping of SS/PBCH block index is done so that the UE indicates the best SS/PBCH block by transmitting Msg1 in a specific time domain RO it might happen that the UE cannot access the channel for that specific time occasion, thus it would have to delay Msg1 to the next matching time occasion. This would introduce unnecessary delays in the RACH procedure. Thus, it is beneficial if the mapping between transmitted SS/PBCH blocks and RO is done so that the best SS/PBCH block is not indicated by which time occasion the UE transmits Msg1 in, but rather by used frequency resource and/or preamble.

This can be achieved already in NR Rel-15 using appropriate configuration of ssb-perRACH-OccasionAndCBPreamblesPerSSB and msg1-FDM. For example, if 4 SS/PBCH blocks are transmitted, setting msg1-FDM equal to 4 and ssb-perRACH-Occasion to 1 or msg1-FDM equal to 2 and ssb-perRACH-Occasion to 2 will achieve time-independent mapping. Where the former would allow a maximum of 64 contention-based preambles per SS/PBCH block and the latter 32. Note that this assumes that the UE uses the beam index (as defined in Section 2.1) in the selection of PRACH occasion for preamble transmission.

Rel-15 NR already supports configurations so that the mapping of transmitted SS/PBCH blocks to RACH occasions is robust to LBT failures.

NR Rel-15, currently allows msg1 of a contention free RACH procedure to be transmitted on SCells, with the corresponding random-access response (RAR) transmitted on the PCell. To increase the robustness of the RACH procedure against LBT failures it is beneficial to allow both contention free and contention-based RACH procedures on SCells.

Two motivations for supporting CBRA on an SCell are as follows: (1) due to LBT failure, it may be more likely in unlicensed operation that the UE reaches the maximum number of PUCCH-SR failures, meaning a CBRA-SR would be triggered. In this case it is beneficial to allow the CBRA on SCells to enhance the number of RA opportunities. (2) Due to LBT failure, a CFRA triggered by a PDCCH order on an SCell may be blocked. It is beneficial if the UE is allowed to fallback to CBRA on the same or different SCell in such a case.

Support transmission of random access response (RAR) on the SCell where MSG1 was transmitted.

In addition to contention free RACH, support also contention-based RACH (CBRA) procedures on SCells.

According to the NR-U TR [2] Section 7.2.1.3.2, several potential RACH enhancements beyond the functionality supported in Rel-15 have been identified for further study:

For potential RACH resource enhancements, the following options have been identified for NR-U, beyond the flexibility already available in Rel-15, but consensus was not achieved. These options may be further considered when specifications are developed:

-	Frequency-domain enhancement: Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA

-	Time-domain enhancements:

-	For connected mode UE, scheduling of PRACH resources via DCI.

-	Triggered PRACH within gNB acquired COT can use a new resource indicated by the DCI

-	For idle mode UE, scheduling of PRACH resources via paging

-	Note: potential inefficiency in network resource due to paging across multiple cells

-	Additional, new RACH resources are used immediately following detection of DRS transmission

-	Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

-	Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

-	Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

The first listed enhancement is to use multiple PRACH resources across multiple LBT sub-bands/carriers. It should be noted that on per-carrier level, this is already supported for initial access, as further discussed in [3], and with Proposal 5 this would be allowed also for connected mode. In contrast, multiple PRACH resources across LBT sub-bands within the same carrier should not be supported, since it is not clear yet from a RAN4 perspective what potential restrictions on contiguous or non-contiguous LBT sub-bands will be introduced.

Multiple PRACH resources across multiple carriers are already supported for initial access, and will with Proposal 5 also be supported for connected mode.

Multiple PRACH resources across multiple LBT sub-bands should not be supported due to potential RAN4 restrictions on contiguous and non-contiguous LBT-subbands within a wideband carrier.

A second listed enhancement is scheduling of PRACH resources via DCI. This is, however, a potential latency optimization, and our view is that it is not critical for Rel-16. It can instead be considered for a later release in a backwards-compatible manner.

Scheduling of PRACH resources via DCI is an optimization and is not critical for the first release of NR-U. It can be considered for a later release in a backwards-compatible manner.

Scheduling of PRACH resources via DCI should not be supported in Rel-16.

A third listed enhancement is multiple PRACH transmissions before Msg2 reception in RAR window for initial access. This feature was proposed already in Rel-15 standardization, but it was noted that it would create additional interference, and it was not included for CBRA in Rel-15. Note that the potential benefit of this feature would only be in scenarios where LBT failures and/or PRACH collisions are significant issues, but in such scenarios, one should be extra careful not to create additional interference and hence should not use the feature.

Multiple PRACH transmissions before Msg2 reception in RAR window creates additional interference and hence makes the LBT/collision problems it is intended to solve even worse.

Multiple PRACH transmissions before Msg2 reception in RAR window should not be supported.

A fourth listed enhancement is group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain. The motivation is to provide multiple transmit opportunities in time domain for a PRACH transmission. However, multiple PRACH transmit opportunities can be obtained already in Rel-15 based on cyclic repetition of the SSB-to-RO mapping. See example and further discussion above in conjunction with Observation 1.

Mappings similar to group-wise SSB-to-RO mapping seem already to be supported in NR Rel-15.

## 2.4	UL Transmissions in Serving Cell SMTC Window

In RAN1#96bis, the following agreement was made with respect to the DRS transmission:

Agreement:

The maximum DRS transmission window duration is 5 ms.

- The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

  - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

As discussed further in Section 3.2, we propose to name this window “Serving Cell SMTC window.” The distinction “Serving Cell” is used since it is used for both RLM and RRM measurements based on serving cell DRS transmissions. In this window, the UE can expect (pending LBT outcome) the DRS to be transmitted. To avoid the gNB and UE competing for transmission resources in this window, it is judged valuable to restrict unscheduled UE transmissions in this window, such as PRACH, SR, and CG transmissions. In our companion paper [5], we include a proposal on how to avoid CG transmissions in the Serving Cell SMTC window, but for PRACH and SR a solution is needed.

PRACH and SR transmissions are given preconfigured transmission opportunities and it might be difficult with existing Rel-15 signaling to avoid configuring PRACH and SR resources in the Serving Cell SMTC window. Rather than modifying the configuration of such resources, it is proposed to specify new behavior such that the UE would simply avoid autonomous transmissions in the Serving Cell SMTC window. Hence, we propose the following

Support UE behavior to suppress PRACH and SR transmissions within the Serving Cell SMTC window. FFS: Optimizations such as allowing transmissions in slots not containing SS/PBCH, based on UE detecting one or more successful SS/PBCH block transmissions within the window.

## 2.5	Rate Matching Mechanisms for Reception of PDSCH

In this section we discuss rate matching mechanisms for SS/PBCH Block(s) for reception of PDSCH in the “Serving Cell SMTC window”.

In Rel-15 NR when receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, the REs corresponding to the configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 of [6] are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

Thus, to summarize in Rel-15 NR a UE uses a combination of ssb-PositionsInBurst and configured or dynamically indicated resources for determining which resources that are not available for PDSCH.

When the SS/PBCH Block(s) are allowed to shift in time in a window, relying only on ssb-PositionsInBurst is not sufficient. Instead we observe that the mechanism of dynamically indicating resources not available for PDSCH can be used as a rate matching mechanism for SS/PBCH Block(s) in the “Serving Cell SMTC window”. The Rel-15 mechanisms (configured in RateMatchPattern) are already very flexible with RB and symbol level bitmaps (resourceBlocks and symbolsInResourceBlock). In addition, periodicityAndPattern provides the possibility to configure a slot level repeating pattern, covering the “Serving Cell SMTC window” and slots in between.

The Rel-15 mechanism of dynamically indicating resources not available for PDSCH can be used as a rate matching mechanism for SS/PBCH Block(s) in the “Serving Cell SMTC window”.

Next turning back to the ssb-PositionsInBurst IE. Even if the SS/PBCH Block(s) can shift in a window, ssb-PositionsInBurst can still provide useful information to the UE. Thus, we propose to configure ssb-PositionsInBurst as in Rel-15 NR, i.e. assuming no shift of the SS/PBCH Block(s). E.g. for a DRS with one SS/PBCH Block in the first SS/PBCH Block position ssb-PositionsInBurst would be configured as 10000000.

The ssb-PositionsInBurst IE can be configured as in Rel-15 NR, i.e. assuming no shift of the SS/PBCH Block(s).

It should be noted that in many cases rate matching PDSCH around SS/PBCH Block(s) are not possible anyway due to that resources in the DRS are used to transmit e.g. RMSI. Thus, in general we think that optimizations on top of what is supported in Rel-15 NR in this area are not needed.

# 3	RRM and RLM

## 3.1	RSSI and Channel Occupancy reporting

During the SI phase [2] it was agreed to support RSSI reporting and define a metric to measure channel occupancy or medium contention and its corresponding reporting. To that end we start by revisiting the RSSI and channel occupancy procedure in Rel-13 LAA. In Rel-13 LAA the UE performs RSSI measurements using an RMTC configuration

RMTC-Config-r13 ::=	CHOICE {

release							NULL,

setup							SEQUENCE {

rmtc-Period-r13					ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r13			INTEGER(0..639)					OPTIONAL,		-- Need ON

measDuration-r13				ENUMERATED {sym1, sym14, sym28, sym42, sym70},

...

}

}

In addition, the UE can be configured with a channel occupancy threshold given by an RSSI value

MeasRSSI-ReportConfig-r13 ::=	SEQUENCE {

channelOccupancyThreshold-r13			RSSI-Range-r13				OPTIONAL	-- Need OR

}

Based on these configurations, the UE periodically reports the following

MeasResultForRSSI-r13 ::=			SEQUENCE {

rssi-Result-r13							RSSI-Range-r13,

channelOccupancy-r13					INTEGER (0..100),

...

}

where the RSSI result is the average RSSI within the reporting period and channel occupancy is the rounded percentage of samples (OFDM symbols) which are above the channel occupancy threshold during the reporting period.

In RAN1#96, the following was agreed

Agreement:

- At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported

- FFS:

  - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario

  - Reporting of a new medium contention/load metric other than channel occupancy

  - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

In our view the Rel-13 configuration provides enough flexibility also for NR-U and thus we propose to use the Rel-13 framework also in Rel-16.

Support RSSI and channel occupancy measurement and reporting using the Rel-13 LAA RSSI and channel occupancy framework without any further enhancements.

## 3.2	 Measurement Windows for RRM and RLM

In the SI phase [2] it was agreed that it is beneficial to configure windows in which UEs can perform measurements. Further it was agreed beneficial that both DRS-based RRM and RLM measurements are performed inside of the (potentially different) measurement windows. Any similarities with Rel-15 SMTC was FFS.

In RAN1#96, the following was also agreed related to measurement windows:

Agreement:

- An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

  - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window

  - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations.

  - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

- FFS: Mechanism to handle missing RLM-RS due to LBT failure

In RAN1#96bis, the following related agreement was made regarding the DRS transmission window

Agreement:
The maximum DRS transmission window duration is 5 ms.

- The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

  - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

We propose to name the RLM measurement window “Serving Cell SMTC window.” The distinction “Serving Cell” is used since it is used for RLM measurements based on serving cell DRS transmissions. In this window, the UE can expect (pending LBT outcome) the DRS to be transmitted.

One of the FFS items in the above agreements is with regards to how the Serving Cell SMTC window is related to the DRS transmission window. Our understanding is that SMTC(s) are conceptually different from the DRS transmission window. In our view, the DRS transmission window is a concept that exists at the gNB and describes the set of possible candidate DRS positions. The SMTC(s) on the other hand is a configuration given to the UE where it should search for and/or measure on the transmitted DRS. Naturally they are tightly connected, but nevertheless not the same.

For DRS-based RLM measurements, the Serving Cell SMTC window will need to cater for LBT failures when the serving gNB attempts to transmit DRS in the DRS transmission window. Thus, the optimal length of the serving cell SMTC window will depend on the LBT success rate of the serving gNB.

For DRS-based RRM measurements of neighbor cells, an SMTC window will need to account for both LBT failures at the neighbor gNBs when they attempt DRS transmission as well as any timing difference between the neighbor cells and the serving cell. Note that RRM measurements of the serving cell can be done in the same Serving Cell SMTC window as RLM measurements.

The Serving Cell SMTC window for RLM measurements (and serving cell RRM) needs to account for LBT failures at the serving gNB, whereas the SMTC window for RRM measurements (for neighbor cells) needs to account for both LBT failures in neighboring gNBs and timing differences between neighbor and serving gNBs. Hence, the RLM and RRM measurement windows should naturally be different.

### 3.2.1	Relationship to NR Rel-15 SMTC

Another open item relates to the similarities of the measurement windows with the Rel-15 NR SMTC window. In NR-Rel-15, an SMTC window for the purposes of RRM measurements is configured through the following SSB-MTC Information Element:

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                               INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

Evidently, the periodicity and offset are configurable with periodicities ranging from 5 ms to 160 ms, and the duration field is configurable in a range of 1 ms to 5 ms.

In RAN1#96b it was agreed that the DRS transmission window is 5 ms. Given that, and that the SSB-MTC Information Element is already available, we think it is best to reuse it “as-is” for RRM measurements based on SS/PBCH. Adding a new information element would affect a number of other IEs such as e.g. intraFreqCellReselectionInfo, InterFreqCarrierFreqInfo and MeasObjectNR. Thus, we make the following proposal:

The Rel-15 SMTC window can be used “as is,” for configuring RRM measurement windows even when the SS/PBCH blocks shift in time due to LBT failures.

Turning back to RLM, we note that in NR Rel-15 there is no existing Serving Cell SMTC window for RLM measurements. It was not necessary to define one, since in licensed operation the SS/PBCH blocks are transmitted at fixed locations in time; they do not shift due to LBT. In NR Rel-15, the information that is signalled with respect to the serving cell is the candidate positions of the SS/PBCH block transmissions and the SS/PBCH block burst periodicity given by the parameters ssb-PositionsInBurst and by ssb-PeriodicityServingCell, respectively. These are signalled either in in RMSI within the IE ServingCellConfigCommonSIB or by dedicated signalling within the IE ServingCellConfigCommon.

In order to define a Serving Cell SMTC window for RLM measurements for unlicensed operation, the enhancement that is needed on top of ssb-PositionsInBurst and by ssb-PeriodicityServingCell is the duration of the window. Since the first candidate position of the DRS transmission window is agreed to be located in the first half slot of a half frame, there is no need to configure an offset. One option for configuration of the duration is to reuse the SSB-MTC information element since it includes a duration field; however, the offset field is unnecessary, and the periodicity field is redundant since the SS/PBCH block burst is configured already with ssb-PeriodicityServingCell. Hence it makes more sense to add a separate duration field along with the configuration of the existing serving cell related parameters ssb-PositionsInBurst and ssb-PeriodicityServingCell.

Support explicit configuration of the Serving Cell SMTC window duration associated with the serving cell parameters ssb-PositionsInBurst and ssb-PeriodicityServingCell.

The Serving Cell SMTC window duration can be added to ServingCellConfigCommonSIB and ServingCellConfigCommon. For SCell, the window duration can be added to SCellConfig.

## 3.3	RLM/RLF

In Rel-15, an RRC_CONNECTED UE in NR is provided with up to X configurable and frequency-dependent reference signals that can be used for radio link monitoring (RLM). Both SS/PBCH block and CSI-RS can be used as RLM reference signals. The RLM-RS(s) can be explicitly configured to the UE. If they are not explicitly configured, then the UE uses the (periodic) CSI-RS contained in the TCI state that indicates the QCL source for PDCCH.

Hypothetical block error rate (BLER) is used to determine in-sync (IS) or out-of-sync (OOS) conditions for either of the reference signal types (SS/PBCH block or CSI-RS). A UE assumes to be IS, if the hypothetical BLER estimated on at least one out of the X configured RLM reference signals is below a configurable threshold. On the other hand, the UE assumes to be OOS, if the hypothetical BLER estimated on all configured RLM reference signals is above yet another configurable threshold.

Upon reception of enough OOS indications, the UE starts a timer, denoted by T310. If the radio link problem is not resolved upon T310 expiry, the UE declares radio link failure (RLF). Moreover, a failure in random access and/or RLC failure can also result in RLF. After the UE declares an RLF, it attempts an RRC Connection Re-establishment by trying to find a suitable cell. If that process is not successful within a certain time, the UE will enter RRC_IDLE state.

The intention of the RLM/RLF procedures is to prevent the UE from becoming unreachable by the network. Since hypothetical BLER estimation is performed on each RLM resource to indicate in-sync (IS) or out-of-sync (OOS), we need to consider how this should be reflected when an RLM reference signal is not transmitted due to LBT.

In the agreement from RAN1#96 in Section 3.2, the following FFS item is identified:

- FFS: Mechanism to handle missing RLM-RS due to LBT failure

An inherent problem for the UE is to reliably distinguish between missing DRS transmissions and not detecting the DRS due to low SINR. Another aspect is whether missing DRS transmissions require explicit notification to the higher layer for specific actions.

Our view is that the UE should tolerate a certain amount of DRS detection failures but at some limit, declare RLF. An attractive way forward is that test cases be added in RAN4 which tests the UE tolerance to missing DRS and declaring OOS when a certain limit is reached. Further, our view is that there are RRC procedures in place which allow the gNB to redirect the UE if this is a persistent problem.

RAN4 shall introduce test cases to show that the UE declares RLF when it encounters a certain amount of missed DRSes while being tolerant to a limited number of missed DRSes.

In the agreement from RAN1#96 in Section 3.2, the following FFS item is identified:

  - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

Compared to Rel-15, NR-U operation will likely have fewer DRS transmissions due to LBT failures and/or longer configured DRS periods. As proposed in our companion contribution [4], we don’t see a benefit of periodic CSI-RS for tracking (periodic TRS) in unlicensed systems. Use of aperiodic TRS, and aperiodic CSI-RS in general, is a more natural fit for unlicensed operation. For RLM, it makes sense that the UE could make use of aperiodic CSI-RS when triggered. Furthermore, the aperiodic CSI-RS would most likely be triggered inside a gNB acquired COT, which has no relationship to the Serving Cell SMTC window. Hence, the UE should be able to make use of ap-CSI-RS for RLM outside the Serving Cell SMTC window when needed. Increasing the number of samples used in RLM algorithms will invariably increase the measurement accuracy.

Detected DL RLM reference signals (including aperiodic CSI-RS, for example) outside of the Serving Cell SMTC window are used by the UE for both out-of-sync and in-sync evaluations.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Rel-15 NR already supports configurations so that the mapping of transmitted SS/PBCH blocks to RACH occasions is robust to LBT failures.

Observation 2	Multiple PRACH resources across multiple carriers are already supported for initial access, and will with Proposal 5 also be supported for connected mode.

Observation 3	Scheduling of PRACH resources via DCI is an optimization and is not critical for the first release of NR-U. It can be considered for a later release in a backwards-compatible manner.

Observation 4	Multiple PRACH transmissions before Msg2 reception in RAR window creates additional interference and hence makes the LBT/collision problems it is intended to solve even worse.

Observation 5	Mappings similar to group-wise SSB-to-RO mapping seem already to be supported in NR Rel-15.

Observation 6	The Rel-15 mechanism of dynamically indicating resources not available for PDSCH can be used as a rate matching mechanism for SS/PBCH Block(s) in the “Serving Cell SMTC window”.

Observation 7	The ssb-PositionsInBurst IE can be configured as in Rel-15 NR, i.e. assuming no shift of the SS/PBCH Block(s).

Observation 8	The Serving Cell SMTC window for RLM measurements (and serving cell RRM) needs to account for LBT failures at the serving gNB, whereas the SMTC window for RRM measurements (for neighbor cells) needs to account for both LBT failures in neighboring gNBs and timing differences between neighbor and serving gNBs. Hence, the RLM and RRM measurement windows should naturally be different.

Observation 9	The Rel-15 SMTC window can be used “as is,” for configuring RRM measurement windows even when the SS/PBCH blocks shift in time due to LBT failures.

Observation 10	The Serving Cell SMTC window duration can be added to ServingCellConfigCommonSIB and ServingCellConfigCommon. For SCell, the window duration can be added to SCellConfig.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE determines the frame timing in the same way as in NR Rel-15 FR2 based on the decoded SS/PBCH block index (combination of 8 PBCH DM-RS sequences and 3 values (2 bits) in PBCH payload) and the half-frame indicator. Note that this implies that the number of PBCH DM-RS sequences is fixed to 8 as in Rel-15 (i.e. Alt-1a).

Proposal 2	The UE may assume that a detected SS/PBCH block in an SS/PBCH block burst set is QCL’d with a detected SS/PBCH block in a previous SS/PBCH block burst set if the detected SS/PBCH block index modulo Q is the same for both detected SS/PBCH blocks. Q is the maximum number of candidate beams. The parameter Q can have the value 1, 2, 4, or 8.

Proposal 3	Support signaling of Q in SIB1, where Q is the number of non-QCL’d SS/PBCH blocks in a burst.

Proposal 4	Support transmission of random access response (RAR) on the SCell where MSG1 was transmitted.

Proposal 5	In addition to contention free RACH, support also contention-based RACH (CBRA) procedures on SCells.

Proposal 6	Multiple PRACH resources across multiple LBT sub-bands should not be supported due to potential RAN4 restrictions on contiguous and non-contiguous LBT-subbands within a wideband carrier.

Proposal 7	Scheduling of PRACH resources via DCI should not be supported in Rel-16.

Proposal 8	Multiple PRACH transmissions before Msg2 reception in RAR window should not be supported.

Proposal 9	Support UE behavior to suppress PRACH and SR transmissions within the Serving Cell SMTC window. FFS: Optimizations such as allowing transmissions in slots not containing SS/PBCH, based on UE detecting one or more successful SS/PBCH block transmissions within the window.

Proposal 10	Support RSSI and channel occupancy measurement and reporting using the Rel-13 LAA RSSI and channel occupancy framework without any further enhancements.

Proposal 11	Support explicit configuration of the Serving Cell SMTC window duration associated with the serving cell parameters ssb-PositionsInBurst and ssb-PeriodicityServingCell.

Proposal 12	RAN4 shall introduce test cases to show that the UE declares RLF when it encounters a certain amount of missed DRSes while being tolerant to a limited number of missed DRSes.

Proposal 13	Detected DL RLM reference signals (including aperiodic CSI-RS, for example) outside of the Serving Cell SMTC window are used by the UE for both out-of-sync and in-sync evaluations.

# 5 References

3GPP TS 38.213 v15.5.0, “Physical layer procedures for control”, March 2019.

3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum,” v16.0.0, December 2018.

R2-1907591, “Discussions on RACH enhancements for NR-U”, Ericsson, RAN2#106.

R1-1907461 , “On aperiodic reference signals for NR-U”, Ericsson, RAN1#97, May 2019.

R1-1907457 , ”Configured grant enhancement”, Ericsson, RAN1#97, May 2019.

3GPP TS 38.214 v15.5.0, “Physical layer procedures for data”, March 2019.

R1-1905786, “Feature lead summary #2 of Enhancements to initial access procedure”, Qualcomm, RAN1#96b, April 2019

R1-1904286, “Enhancements to initial access and mobility for NR-unlicensed”, Intel Corporation, RAN1#96b, April 2019