3GPP TSG RAN WG1 Meeting #97 	R1-1907481

Reno, NEVADA (US), May 13th – 17th, 2019

Source:           ESA

Title: 	Beam size computation and alternative satellite specifications

Agenda Item:  7.2.5.1

Document for: Discussion and Approval

# Introduction

The SI on “Study on Solutions for NR to Support Non-Terrestrial Networks” was approved in RAN#80 meeting [1] and further updated in RAN#82 meeting [2]. The RAN1 objectives have been reported below.

- Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]:

  - Physical layer control procedures (e.g. CSI feedback, power control)

  - Uplink Timing advance/RACH procedure including PRACH sequence/format/message

  - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

- Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

In RAN1 #96bis meeting, some satellite payload parameters (from Table 2 and Table 3 in [5]) have been agreed for calibration in NTN performance evaluation.

However, during the meeting, some questions about were raised the satellite beam diameter computation, and the request for a second set of satellite configuration was considered beneficial for alternative analysis on the NTN performance.

In this contribution, we review the beam diameter computation and clarify the values obtained in the first set [5]. In addition, another set of satellite parameters has been added, aiming at larger beam diameters.

# Satellite Beam Size Computation

In [5], the satellite beam diameter have been derived from a typical approximation of the 3 dB beamwidth generated by a satellite parabolic antenna. The formula is:

3dB-beamwidth in degrees=2∙θ3dB=70λD	(1)

where λ is the satellite carrier wavelength and D is the antenna diameter. The 3 dB beamwidth is twice the angular value θ3dB.

However, since the satellite antenna gain pattern is agreed and is given in section 6.4.1 of TR 38.811, it recommended to compute the angular value θ3dB directly from the generated radiation pattern.

Proposal 1: As far as the beam size computation is concerned, it is recommended to compute the θ3dB value directly from the generated radiation pattern.

Once derived the angular value, the satellite beam radius (or diameter) generated on-ground can be computed looking at the geometry reported in Figure 1.

Figure 1: Satellite geometry for computing the beam radius

In particular, the specific formulas for deriving the satellite beam radius on ground are given in the following.

sinρ= RERE+H

cosϵ=sinηsinρ

λ+ε+η=π2

RB=RE∙λ	(2)

When choosing η=θ3dB, the Eq. (2) provides the typical beam radius (i.e., 3dB contour).

Based on the described procedure, the updated values of the beam diameters generated by the agreed Set-1 in RAN1 #96bis meeting have been reported in the table below. It shall be noted that these diameters referred to the beam generated around the sub-satellite point (i.e., Nadir pointing of the satellite). Of course, decreasing the elevation angle, ϵ, from which the satellite is seen on-ground, the beam diameter might increase up to a factor of two (or even three times for very low elevation values).

Table 1: Satellite beam diameter on-ground (i.e., 3dB contour) generated with the Set-1 configuration.

| Frequency band | S-band | S-band | S-band | Ka-band | Ka-band | Ka-band |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude, H [km] | 35786 | 1200 | 600 | 35786 | 1200 | 600 |
| Satellite antenna aperture [m] | 22 | 2 | 2 | 5 | 0.5 | 0.5 |
| Carrier frequency [GHz] | 2 | 2 | 2 | 20 | 20 | 20 |
| θ3dB [degrees] (from the satellite antenna pattern and then equation 2) | 0.20 | 2.20 | 2.20 | 0.09 | 0.88 | 0.88 |
| Satellite beam diameter [km] | 250 | 90 | 50 | 110 | 40 | 20 |

# Set-1 Errata Corrige

After reviewing the values reported in [5], it has been noted a typo in the G/T for LEO S-band. In fact, for both LEO-1200 and LEO-600, the correct G/T value is 1.1 dB K-1.

Proposal 2: Due to a typo, it is recommended to amend the G/T to 1.1 dB K-1 for both LEO S-band altitude.

In addition, it has been noted that the wrong antenna size has been used in the GEO Ka-band G/T computation, leading to a drop in the receiver antenna gain of 6 dB. In fact, for a 5 meter antenna aperture in Ka-band, the typical G/T is 28 dB K-1.

Proposal 3: Due to a mistake, it is recommended to amend the G/T to 28 dB K-1 for the GEO Ka-band case.

# Set-2 Satellite parameters

During the RAN1 #96bis it was discussed that a second set of typical satellite payload characteristics might be useful to address different deployment scenarios and to target several use cases (from eMBB to mMTC). From this point of view, the agreed Set-1 can be associated to e-MBB services.

In the following, another set of downlink and uplink parameters are listed in Table 2 and Table 3, respectively. The spirit followed in this design has been driven by the necessity to decrease the power on-ground in the order of 6-8 dB and to enlarge the satellite beam diameter. The straightforward result has been the reduction of the equivalent satellite antenna aperture for all configurations.

Table 2: Example of satellite parameters and payload characteristics for downlink transmissions

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [3] | Section 6.4.1 in [3] | Section 6.4.1 in [3] |
| Equivalent satellite antenna aperture (Note 1) | S-band | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of [3]. Note 2: As reported in the previous section, this beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table 3: Example of satellite parameters and payload characteristics for uplink transmissions

| Parameters/Scenarios |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite antenna pattern |  | Section 6.4.1 in [3] | Section 6.4.1 in [3] | Section 6.4.1 in [3] |
| Equivalent satellite antenna aperture (Note1) | S-band | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Equivalent satellite antenna aperture (Note1) | Ka-band | 2 m | 0.2 m | 0.2 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of [3]. |  |  |  |  |

Proposal 4: As far as the alternative set (i.e., Set-2) is concerned, it is recommended to adopt Table 2 and Table 3 to describe the main satellite parameters for downlink and uplink transmissions, respectively.

# Satellite Payload Impairments

Since during the RAN1 #96bis meeting there was no time to discuss and to agree on the satellite impairments, these table below are reported again for completeness.

Table 4: Satellite downlink impairments.

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Max Satellite Doppler shift (Note 1) | S-band | up to 300 Hz [ ~ 0.15 ppm] | 40 kHz [ 20 ppm] | 48 kHz [ 24 ppm] |
| Max Satellite Doppler rate (Note 1) |  | N/A | 180 Hz/s [ 0.09 ppm/s] | 544 Hz/s [ 0.27 ppm/s] |
| Satellite amplifier |  | Table 9 in Annex 2 | Table 9 in Annex 2 | Table 9 in Annex 2 |
| Satellite phase noise |  | Table 6 in Annex 1 | Table 6 in Annex 1 | Table 6 in Annex 1 |
| Max Satellite Doppler shift (Note 1) | Ka-band | Up to 3 kHz [ ~ 0.15 ppm] | 400 kHz [ 20 ppm] | 480 kHz [ 24 ppm] |
| Max Satellite Doppler rate (Note 1) |  | N/A | 1.80 kHz [ 0.09 ppm/s] | 5.44 kHz [ 0.27 ppm/s] |
| Satellite amplifier |  | Table 8 in Annex 2 | Table 9 in Annex 2 | Table 9 in Annex 2 |
| Satellite phase noise |  | Table 7 in Annex 1 | Table 7 in Annex 1 | Table 7 in Annex 1 |
| Note 1: These values (Doppler shift and Doppler rate) refer to the scenario without pre-compensation at the transmitter side. |  |  |  |  |

Table 5: Satellite uplink impairments.

| Parameters/Scenarios |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite amplifier |  | N/A (linear region) | N/A (linear region) | N/A (linear region) |
| Max Satellite Doppler shift (Note 2) | S-band | up to 300 Hz [ ~ 0.15 ppm] | 40 kHz [ 20 ppm] | 48 kHz [ 24 ppm] |
| Max Satellite Doppler rate (Note 2) |  | N/A | 180 Hz/s [ 0.09 ppm/s] | 544 Hz/s [ 0.27 ppm/s] |
| Max Satellite Doppler shift (Note 2) | Ka-band | N/A | 600 kHz [ 20 ppm] | 720 kHz [ 24 ppm] |
| Max Satellite Doppler rate (Note 2) |  | N/A | 2.7 kHz [ 0.09 ppm/s] | 8.16 kHz [ 0.27 ppm/s] |
| Note 2: These values (Doppler shift and Doppler rate) refer to the scenario without pre-compensation at the transmitter side. |  |  |  |  |

Proposal 5: As far as the satellite payload impairments are concerned, it is recommended to adopt Table 4 and Table 5 for downlink and uplink transmissions, respectively.

# Conclusions

In this document, the following proposals have been presented in the context of the definition of typical satellite parameters and payload characteristics:

Proposal 1: As far as the beam size computation is concerned, it is recommended to compute the θ3dB value directly from the generated radiation pattern.

Proposal 2: Due to a typo, it is recommended to amend the G/T to 1.1 dB K-1 for both LEO S-band altitude.

Proposal 3: Due to a mistake, it is recommended to amend the G/T to 28 dB K-1 for the GEO Ka-band case.

Proposal 4: As far as the alternative set (i.e., Set-2) is concerned, it is recommended to adopt Table 2 and Table 3 to describe the main satellite parameters for downlink and uplink transmissions, respectively.

Proposal 5: As far as the satellite payload impairments are concerned, it is recommended to adopt Table 4 and Table 5 for downlink and uplink transmissions, respectively.

# References

- RP-181370, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#80, La Jolla, USA, June 11-14, 2018.

- RP-182444, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#82, Sorrento, Italy, December 10-13, 2018.

- TR 38.811, “Study on NR to support non terrestrial networks”, 3GPP source.

- TR 38.821, “Solutions on NR to support non-terrestrial networks”, 3GPP source.

- R1-1905216, “Key satellite parameters and simulation assumptions for NTN”, ESA.

- ETSI TS 102 584 v1.1.1, “DVB-SH Implementation Guidelines”, December 2008.

- ETSI TR 102 376-2 v1.1.1, “Digital Video Broadcasting (DVB): Implementation guidelines for the second generation system for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 2 - S2 Extensions (DVB-S2X)”, November 2015.

- ETSI TR 101 545-4 v1.1.1 “Guidelines for the implementation and use of EN 301 545-2 (DVB-RCS2 Lower Layer)”, April 2014.

# Annex A.1: Satellite phase noise models

Hereafter, typical phase noise masks for satellite payloads in S- and Ka- bands are reported, respectively. The measurements have been extracted from [6] and [7]. These tables refer to downlink transmissions, however without loss of precision, these masks can be adopted also for uplink analysis.

Table 6: S-band satellite phase noise mask [6]

| S-Band | 10 Hz | 100 Hz | 1 kHz | 10 kHz | 100 kHz | 1 MHz | 10 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| dBc/Hz (SSB) | -29 | -59 | -69 | -74 | -83 | -95 | -101 |

Table 7: Ka-band satellite phase noise mask [7]

| Ka-Band | 100 Hz | 1 kHz | 10 kHz | 100 kHz | 1 MHz | 10 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| dBc/Hz (SSB) | -62 | -80 | -90 | -95 | -106 | -116 | -118 |

# Annex A.2: Satellite on-board amplifier model

In the following, typical AM/AM and AM/PM characteristics for on-board amplifiers are shown. Aiming at state-of-the-art devices, the reported characteristics belong to linearized amplifiers. In particular, the non-linear characteristics have been differentiated between Travelling Wave Tube Amplifier (TWTA) and Solid State Power Amplifier (SSPA). Both input/output measurements of the amplifier have been normalized.

Table 8: Typical AM/AM and AM/PM characteristics for linearized TWTA, extracted from [7].

| IBO [dB] | OBO [dB] | Phase [deg] |
| --- | --- | --- |
| -20 | -18 | 0 |
| -18 | -16 | 0 |
| -16 | -14 | -0.5 |
| -14 | -12 | -0.5 |
| -12 | -10 | -0.5 |
| -10 | -8 | -1 |
| -8 | -6 | -1 |
| -6 | -4 | -1 |
| -4 | -2 | -2.5 |
| -3 | -1 | -3 |
| -2 | -0.5 | -4 |
| -1 | -0.1 | -6 |
| 0 | 0 | -8 |
| 1 | -0.1 | -7 |
| 2 | -0.5 | -6 |
| 3 | -1 | -4 |
| 4 | -2 | -2 |

Table 9: Typical AM/AM and AM/PM characteristics for SSPA, extracted from [8]

| IBO [dB] | OBO [dB] | Phase [deg] |
| --- | --- | --- |
| -18 | -16.7 | 0 |
| -16 | -14.7 | 0.5 |
| -14 | -12.7 | 1 |
| -12 | -10.7 | 2.5 |
| -10 | -8.6 | 3.5 |
| -8 | -6.5 | 5 |
| -6 | -4.5 | 6 |
| -4 | -2.6 | 7 |
| -3 | -1.9 | 7 |
| -2 | -1.2 | 7 |
| -1 | -0.6 | 6.5 |
| 0 | 0 | 6 |
| 1 | 0.5 | 5 |
| 2 | 0.9 | 4 |
| 3 | 1.1 | 3.5 |
| 4 | 1.3 | 3 |

The θ3dB value is the numerical solution for which the antenna gain pattern is equal to 0.5, i.e., the power drops of 3 dB with respect to the maximum value (the boresight direction).

For parabolic antennas, the following relationship is valid: reducing the antenna diameter of a factor k (i.e., D2 = D1/k), this yields to decrease of a factor 20*log10(k) the EIRP value.