3GPP TSG RAN WG1 Meeting #97	R1-1907486

Reno, USA, May 13 –17, 2019

Agenda Item:	7.2.2.3

Source:	Huawei, HiSilicon

Title:	SLS evaluation of LBT for short control signaling

Document for:	Discussion and decision

# Introduction

In RAN#96b, following agreement was approved [1]

- The maximum DRS transmission window duration is 5 ms.

  - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

    - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

  - FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

- Agreement from the SI is updated as shown:

Table 7.2.1.3.1-1: Channel access schemes for gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

In this contribution, we mainly discuss various types of LBT for NR-U DRS transmission and its impact on Wi-Fi system throughput.

# LBT type of DRS transmission

As the agreement approved in RAN#96b, the maximum DRS transmission window duration is limited in 5ms. According to the agreement updated from SI [2], when the DRS duty cycles ≤1/20, and the total duration is up to 1ms, 25 µs Cat 2 LBT is used (as in LAA), if the DRS duty cycle is > 1/20, or total duration > 1ms, Cat4 with any channel access priority class value can be used if DRS is alone or multiplexed with non-unicast data.  Besides, according to specification from IEEE [3], Cat4 with AC_VO access category is used for beacon transmission in Wi-Fi system. To estimate the coexistence between NRU and Wi-Fi with different LBT methods for NRU DRS transmission, we evaluate three different LBT methods as below:

Case 1: DRS with Cat2

Case 2: DRS with Cat4

Case 3: DRS with Cat2 and Cat4

For case 2 and 3, Cat4 assumes the highest priority class.

For Case 1, Cat2 LBT is used before each candidate SSB position if the previous Cat2 LBT fails in the DRS windows. For example, if the DRS windows is 5ms with 30 kHz SCS, total 20 Cat2 LBTs are allowed.

In Case 2, Cat4 LBT is used for DRS transmission, which is same as Wi-Fi system. In the evaluation, post back off is adopted. According to EN 301 893, if the back off counter decrease to negative, when the channel becomes busy, the channel access engine should initiate a new back off procedure with least CW value after an idle DIFS period.

Case 3 is a combination LBT method of Cat2 and Cat4, for example, maximum 5 or 6 Cat2 LBT is allowed in the initial part of the DRS windows. If LBT failed within the allowed opportunities, Cat4 LBT is used for the rest of the DRS window.

# Simulation results

Assuming DRS transmission periodicity is 20 ms, DRS transmission duration is 1 ms with 30 kHz SCS. The traffic arrival rates of 0.2/0.4/0.6 file/sec are selected such that the mean buffer occupancies (BO) of Wi-Fi coexist with Wi-Fi operators is about 20%/50%/80% corresponding to low/medium/high traffic load. If gNB acquires DL COT for unicast data with CAT4 LBT before the DRS transmission window, DRS burst is multiplexed with unicast data in the same COT. The impact of missing DRS at UE is not considered in the SLS. The system-level simulation of UPT results in 5%, 50% and 95% are shown in the figure below. The detail of simulation assumption can be found in the appendix. It can be observed that there is not obvious difference of system performance for Wi-Fi network under DRS transmission with Cat2 LBT, Cat4 LBT, or a mixture of Cat2 and Cat4 LBT with a limitation to the number of Cat2 LBT attempts within a DRS window.

Observation 1: There is not obvious difference of system performance for Wi-Fi network in coexistence scenario under DRS transmission with Cat2 LBT, Cat4 LBT, or a mixture of Cat2 and Cat4 LBT with a limitation to the number of Cat2 LBT attempts within a DRS window.

Observation 2: Cat2 LBT for NRU DRS transmission will not lead to Wi-Fi system throughput loss.

- Low traffic load

- Medium traffic load

- High traffic load

Figure 1.  5%, 50%, 95% throughput of Wi-Fi network in coexistence when NR-U using different LBT types for DRS transmission in low/medium/high traffic load

# Conclusions

In this contribution, we mainly discussed different LBT types for NR-U DRS transmission and its impact on Wi-Fi system throughput. Based on the discussion and evaluations, we have made the following observations:

Observation 1: There is not obvious difference of system performance for Wi-Fi network in coexistence scenario under DRS transmission with Cat2 LBT, Cat4 LBT, or a mixture of Cat2 and Cat4 LBT with a limitation to the number of Cat2 LBT attempts within a DRS window.

Observation 2: Cat2 LBT for NRU DRS transmission will not lead to Wi-Fi system throughput loss.

# References

Draft Report of 3GPP TSG RAN WG1 #96b v0.1.0

TR 38.889 V16.0.0

- [3] IEEE 802.11-2016

# Appendix I: system level simulation assumptions

Table A: Summary of simulation assumptions for NR-U

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 60KHz 30KHz(DRS) |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms for NRU and WiFi |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #1 |
| Link adaptation | CQI feedback + OLLA |
| Scheduling | Proportional Fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | DL and UL |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

Table B: Summary of simulation assumptions for 802.11ac

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| MCS |  |  | 802.11ac MCS with 256QAM |
| Carrier Frequency |  |  | 5GHz |
| Carrier Channel Bandwidth |  |  | 20MHz |
| Number of carriers |  |  | 1 |
| Number of users per operator |  |  | Exactly 5 per gNB per 20MHz |
| MCOT |  |  | 8 ms |
| Frame aggregation |  |  | A-MPDU |
| MPDU size |  |  | 1500B MSDU + 14 B header |
| Guard interval |  | 11ac | 0.8us |
| Channel Model |  |  | NR UMi street canyon |
| AP Tx Power |  |  | 23dBm |
| STA Tx Power |  |  | 18dBm |
| MAC | Coordination |  | DCF |
|  | SIFS/DIFS |  | 16us/43us |
|  | Detection |  | Energy detection & preamble detection |
|  | RTS/CTS |  | No |
|  | Contention |  | EDCA |
| CCA-PD |  |  | -82dbm for 11ac |
| CCA-ED |  |  | -62dbm for 11ac |
| ACK Modeled |  |  | Immediate ACK |
| OFDM symbol length |  |  | 4 us |

# Appendix II: system level simulation results

Table 1: Throughput of Wi-Fi system in coexist with NR-U with different LBT types for NR-U DRS transmission in low traffic load

| Reported parameters |  | DRS windows = 5ms |  |  |
| --- | --- | --- | --- | --- |
|  |  | Wi-Fi in coex (Case 1) | Wi-Fi in coex (Case 2) | Wi-Fi in coex (Case 3) |
| DL UPT CDF [Mbps] | 5% | 26.04 | 26.19 | 26.05 |
|  | 50% | 67.56 | 66.78 | 66.92 |
|  | 95% | 76.94 | 76.91 | 76.87 |
|  | Mean | 59.21 | 59.07 | 58.77 |
| DL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.06 | 0.06 | 0.06 |
|  | 95% | 0.15 | 0.15 | 0.15 |
|  | Mean | 0.08 | 0.08 | 0.08 |
| UL UPT CDF [Mbps] | 5% | 22.69 | 24.18 | 23.65 |
|  | 50% | 60.36 | 60.12 | 59.30 |
|  | 95% | 76.43 | 76.56 | 76.43 |
|  | Mean | 55.37 | 55.48 | 55.38 |
| UL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.06 | 0.07 | 0.07 |
|  | 95% | 0.16 | 0.16 | 0.15 |
|  | Mean | 0.08 | 0.08 | 0.08 |
| ρDL |  | 99.47% | 99.44% | 99.44% |
| ρUL |  | 99.10% | 99.14% | 99.12% |
| Mean BO |  | 13.72% | 13.70% | 13.77% |
| λ |  | 0.2 |  |  |

Table 2: Throughput of Wi-Fi system in coexist with NR-U with different LBT types for NR-U DRS transmission in medium traffic load

| Reported parameters |  | DRS windows = 5ms |  |  |
| --- | --- | --- | --- | --- |
|  |  | Wi-Fi in coex (Case 1) | Wi-Fi in coex (Case 2) | Wi-Fi in coex (Case 3) |
| DL UPT CDF [Mbps] | 5% | 13.15 | 12.90 | 12.77 |
|  | 50% | 44.65 | 44.55 | 44.32 |
|  | 95% | 78.45 | 78.29 | 78.63 |
|  | Mean | 46.05 | 45.82 | 45.98 |
| DL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.09 | 0.09 | 0.09 |
|  | 95% | 0.30 | 0.29 | 0.30 |
|  | Mean | 0.12 | 0.12 | 0.12 |
| UL UPT CDF [Mbps] | 5% | 8.96 | 9.12 | 9.55 |
|  | 50% | 38.15 | 38.29 | 39.21 |
|  | 95% | 78.14 | 78.17 | 78.18 |
|  | Mean | 41.10 | 41.17 | 41.71 |
| UL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.10 | 0.10 | 0.10 |
|  | 95% | 0.42 | 0.42 | 0.41 |
|  | Mean | 0.15 | 0.16 | 0.15 |
| ρDL |  | 98.90% | 98.79% | 98.83% |
| ρUL |  | 99.06% | 99.13% | 99.12% |
| Mean BO |  | 36.3% | 36.7% | 36.0% |
| λ |  | 0.4 |  |  |

Table 3: Throughput of Wi-Fi system in coexist with NR-U with different LBT types for NR-U DRS transmission in high traffic load

| Reported parameters |  | DRS windows = 5ms |  |  |
| --- | --- | --- | --- | --- |
|  |  | Wi-Fi in coex (Case 1) | Wi-Fi in coex (Case 2) | Wi-Fi in coex (Case 3) |
| DL UPT CDF [Mbps] | 5% | 6.63 | 7.19 | 7.05 |
|  | 50% | 30.50 | 31.73 | 32.28 |
|  | 95% | 73.71 | 73.70 | 74.13 |
|  | Mean | 34.36 | 35.30 | 35.73 |
| DL Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 |
|  | 50% | 0.13 | 0.12 | 0.12 |
|  | 95% | 0.58 | 0.53 | 0.52 |
|  | Mean | 0.19 | 0.18 | 0.18 |
| UL UPT CDF [Mbps] | 5% | 4.64 | 4.83 | 5.21 |
|  | 50% | 24.61 | 25.69 | 25.55 |
|  | 95% | 69.78 | 71.21 | 71.72 |
|  | Mean | 28.80 | 29.80 | 29.74 |
| UL Delay CDF [s] | 5% | 0.06 | 0.06 | 0.06 |
|  | 50% | 0.16 | 0.15 | 0.15 |
|  | 95% | 0.75 | 0.76 | 0.66 |
|  | Mean | 0.25 | 0.24 | 0.23 |
| ρDL |  | 98.63% | 98.74% | 98.50% |
| ρUL |  | 97.77% | 97.84% | 97.89% |
| Mean BO |  | 60.47% | 60.48% | 59.48% |
| λ |  | 0.6 |  |  |