3GPP TSG RAN WG1 Meeting #97	R1-1907499

# Reno, USA, May 13-17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | CRNum | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on OFDM baseband signal generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For OFDM baseband signal generation, it is said that “μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList” in the current specification. The description is not clear that whether μ0 is the largest μ value of both UL and DL or μ0 is the largest μ value of UL for UL transmission and of DL for DL transmission, respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “for UL and DL, respectively” for OFDM baseband signal generation for non-PRACH case and add “for UL” for that for PRACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification for OFDM baseband signal generation. UE and gNB may have different understanding for the reference μ0 for baseband signal generation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact. This CR impacts OFDM baseband signal generation.The CR does not impact functionality as it is the common understanding. Inter-operability analysis: If the UE is implemented in accordance to this CR and the gNB is not, or the gNB is implemented in accordance to this CR and the UE is not, there may be an issue that UE and gNB refer to different SCSs when generating baseband signal, if different max SCS configurations are provided for UL and DL. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

## 5.3	OFDM baseband signal generation

### 5.3.1	OFDM baseband signal generation for all channels except PRACH

The time-continuous signal  on antenna port p and subcarrier spacing configuration μ for OFDM symbol  in a subframe for any physical channel or signal except PRACH is defined by

where  is the time within the subframe,

and

-	 is given by clause 4.2;

-	 is the subcarrier spacing configuration;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList for UL and DL, respectively.

The starting position of OFDM symbol  for subcarrier spacing configuration in a subframe is given by

### 5.3.2	OFDM baseband signal generation for PRACH

The time-continuous signal  on antenna port p for PRACH is defined by

sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA
K=ΔfΔfRA
k1=k0μ+NBWP,istart-Ngridstart,μNscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB2
k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ

where  and

-	 is given by clause 6.3.3;

-	 is the subcarrier spacing of the initial active uplink bandwidth part during initial access. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList for UL;

-	 is the lowest numbered resource block of the initial active uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP during initial access. Otherwise,  is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

-	 is the frequency offset of lowest PRACH transmission occasion in frequency domain with respect to PRB 0 of the initial active uplink bandwidth part given by the higher-layer parameter msg1-FrequencyStart during initial access associated with the initial active uplink bandwidth part. Otherwise,  is the frequency offset of lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part given by the higher-layer parameter msg1-FrequencyStart associated with the active uplink bandwidth part;

-	 is the PRACH transmission occasion index in frequency domain for a given PRACH transmission occasion in one time instance as given by clause 6.3.3.2;

-	 is the number of resource blocks occupied and is given by the parameter allocation expressed in number of RBs for PUSCH in Table 6.3.3.2-1.

-	 and  are given by clause 6.3.3

-	NCP,lRA=NCPRA+n∙16κ where

-	for ,

-	for ,  is the number of times the interval  overlaps with either time instance 0 or time instance  in a subframe

The starting position tstartRA of the PRACH preamble in a subframe (for ) or in a 60 kHz slot (for ) is given by

where

-	the subframe or 60 kHz slot is assumed to start at  ;

-	a timing advance value NTA=0 shall be assumed;

-	Nuμ and NCP,l-1μ are given by clause 5.3.1;

-	 shall be assumed for , otherwise it is given by  and the symbol position  is given by

l=l0+ntRANdurRA+14nslotRA

where

-	 is given by the parameter "starting symbol" in Tables 6.3.3.2-2 to 6.3.3.2-4;

-	 is the PRACH transmission occasion within the PRACH slot, numbered in increasing order from 0 to  within a RACH slot where  is given Tables 6.3.3.2-2 to 6.3.3.2-4 for  and fixed to 1 for ;

-	 is given by Tables 6.3.3.2-2 to 6.3.3.2-4;

-	 is given by

-	if , then

-	if  and either of "Number of PRACH slots within a subframe" in Tables 6.3.3.2-2 to 6.3.3.2-3 or "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then

-	otherwise,

If the preamble format given by Tables 6.3.3.2-2 to 6.3.3.2-4 is A1/B1, A2/B2 or A3/B3, then

-	if ntRA=NtRA,slot-1, then the PRACH preamble with the corresponding PRACH preamble format from B1, B2 and B3 is transmitted in the PRACH transmission occasion;

-	otherwise the PRACH preamble with the corresponding PRACH preamble format from A1, A2 and A3 is transmitted in the PRACH transmission occasion