3GPP TSG-TSG RAN WG1 Meeting #97	 R1-1907506

# Reno, USA, May 13th-17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | DRAFT | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRS request field for SRS Carrier Switching in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#90bis meeting, we have following agreements for SRS carrier switching: Agreement: - Specify NR SRS switching among CCs similar to Rel-14 LTE SRS carrier-based switching design including - Periodic/aperiodic/semi-persistent SRS on a CC without PUCCH/PUSCH configured - TA (through PRACH) on TAG without PUSCH/PUCCH configured - Power control separated from that of PUSCH - Group common DCI for aperiodic SRS triggering and TPC - DL/UL interruptions and collision handling due to SRS switching For SRS carrier switching in LTE, in case of triggering type 1 in DCI format 3B, SRS request value is specified in Clause 8.2 of TS36.213 as following: Table 8.2-0C: SRS request value for trigger type 1 in DCI format 3B and for UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission Value of SRS request field; Description. '00'; No type 1 SRS trigger for a 1st set of serving cells configured by higher layers. '01'; Type 1 SRS trigger for a 2nd set of serving cells configured by higher layers. '10'; Type 1 SRS trigger for a 3rd set of serving cells configured by higher layers. '11'; Type 1 SRS trigger for a 4th set of serving cells configured by higher layers. Similar with 36.213, the SRS request value highlighted in bold font shall be also captured in 38.212 according to identify the maximal four sets of serving cells configured by higher layer as shown in 38.331, i.e . SRS-CC-SetIndex ::= SEQUENCE { cc-SetIndex INTEGER (0..3) OPTIONAL, -- Need M cc-IndexInOneCC-Set INTEGER (0..7) OPTIONAL -- Need M } However Table 7.3.1.1.2-24 from current specification of 38.212 only refers to the first/second/third sets of serving cells so that it is ambiguous about how/where to apply the fourth set of serving cells configured by higher layer, or whether the first/second/third set in DCI description of SRS request field refers to cc-SetIndex 0/1/2 respectively. Therefore the ambiguity may lead to different understanding between gNB and UE about specific codepoint of SRS request field. Moreover, it is also specified in LTE that “For the 2-bit SRS request field in Table 8.2-0C and DCI format 3B, and UE configured with more than 5 TDD serving cells without PUSCH/PUCCH transmission, if the UE receives an SRS request field with value '00', the DCI does not indicate a type 1 SRS trigger, but the UE shall apply the power control commands received in the DCI format 3B.” Similar with 36.213, for the codepoint “00” of SRS request field for Format 2_3 specified in section 7.3.1.3.4, 38.212, it shall also be clarified that TPC commands are still valid, even though there is no SRS triggering, so that the UE behaviours can be clarified and follow TPC properly. Otherwise it is unclear for the NW whether the UE may or may not follow TPC commands and lead to ambiguous UE behaviour. |
| Summary of change: |  | SRS request field in Table 7.3.1.1.2-24 in 38.212 has been clarified to identify the maximal 4 sets of serving cells of DCI format 2_3 configured by high layers. The codepoint “00” in DCI format 2_3 is also clarified that TPC commands are still valid for the UE even without aperiodic SRS triggering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous between gNB and UE about the understanding of codepoints of SRS request field with regarding to the configuration of sets of serving cells. Moreover it is also unclear whether the UE may or may not follow TPC command for the codepoint “00” in DCI format 2_3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2, 7.3.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is to clarify the SRS request field for DCI format 2_3 configured by higher layer. It is expected that UEs have implemented this CR accordingly to avoid such an ambiguity of how to handle the 4th set of serving cells and follow TPC commands in the codepoint “00”. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3.1.2	DCI formats for scheduling of PDSCH

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

< Unchanged parts are omitted >

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 1_0, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered for a 1st set of serving cells configured by higher layers |
| 01 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 | SRS resource set(s) configured with higher layer parameter SRS-SetUse set to 'antenna switching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 | SRS resource set(s) configured with higher layer parameter SRS-SetUse set to 'antenna switching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 | SRS resource set(s) configured with higher layer parameter SRS-SetUse set to 'antenna switching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 4th of serving cells configured by higher layers |

< Unchanged parts are omitted >

#### 7.3.1.3	DCI formats for other purposes

##### 7.3.1.3.1	Format 2_0

DCI format 2_0 is used for notifying the slot format.

The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI:

-	Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N.

The size of DCI format 2_0 is configurable by higher layers up to 128 bits, according to Subclause 11.1.1 of [5, TS 38.213].

##### 7.3.1.3.2	Format 2_1

DCI format 2_1 is used for notifying the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE.

The following information is transmitted by means of the DCI format 2_1 with CRC scrambled by INT-RNTI:

-	Pre-emption indication 1, Pre-emption indication 2, …, Pre-emption indication N.

The size of DCI format 2_1 is configurable by higher layers up to 126 bits, according to Subclause 11.2 of [5, TS 38.213]. Each pre-emption indication is 14 bits.

##### 7.3.1.3.3	Format 2_2

DCI format 2_2 is used for the transmission of TPC commands for PUCCH and PUSCH.

The following information is transmitted by means of the DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI:

-	block number 1, block number 2,…, block number N

The parameter tpc-PUSCH or tpc-PUCCH  provided by higher layers determines the index to the block number for an UL of a cell, with the following fields defined for each block:

-	Closed loop indicator   – 0 or 1 bit.

-	For DCI format 2_2 with TPC-PUSCH-RNTI, 0 bit if the UE is not configured with high layer parameter twoPUSCH-PC-AdjustmentStates, in which case UE assumes each block in the DCI format 2_2 is of 2 bits; 1 bit otherwise, in which case UE assumes each block in the DCI format 2_2 is of 3 bits;

-	For DCI format 2_2 with TPC-PUCCH-RNTI, 0 bit if the UE is not configured with high layer parameter twoPUCCH-PC-AdjustmentStates, in which case UE assumes each block in the DCI format 2_2 is of 2 bits; 1 bit otherwise, in which case UE assumes each block in the DCI format 2_2 is of 3 bits;

-	TPC command –2 bits

The number of information bits in format 2_2 shall be equal to or less than the payload size of format 1_0 monitored in common search space in the same serving cell. If the number of information bits in format 2_2 is less than the payload size of format 1_0 monitored in common search space in the same serving cell, zeros shall be appended to format 2_2 until the payload size equals that of format 1_0 monitored in common search space in the same serving cell.

##### 7.3.1.3.4	Format 2_3

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24. For the 2-bit SRS request field in Table 7.3.1.1.2-24, if the UE receives an SRS request field with value '00', the UE shall apply TPC commands received in the DCI format 2_3.

-	TPC command number 1, TPC command number 2, ..., TPC command number N, where each TPC command applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeB for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block or more blocks is configured for the UE by higher layers where each block applies to an UL carrier, with the following fields defined for each block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command –2 bits

The number of information bits in format 2_3 shall be equal to or less than the payload size of format 1_0 monitored in common search space in the same serving cell. If the number of information bits in format 2_3 is less than the payload size of format 1_0 monitored in common search space in the same serving cell, zeros shall be appended to format 2_3 until the payload size equals that of format 1_0 monitored in common search space in the same serving cell.

< Unchanged parts are omitted >