3GPP TSG RAN WG1 Meeting #97	R1-1907535

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.8.5

Source:	Huawei, HiSilicon

Title:	Evaluation results of panel-specific beam selection

Document for:	Discussion and Decision

Introduction

Rel-16 MIMO is tasked to enhance various aspects of multi-beam operation in FR2, including to specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection [1]. In last meeting [2], the following agreement was reached:

Agreement

In Rel-16, only introduce specification enhancement for MPUE-Assumption3

- MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission.

  - Note that this does not require a UE to always activate multi-panels simultaneously

  - Note: UE can control the panel activation/deactivation

- Possible use cases at least include

  - (General) UL coverage enhancement for FR2 considering the UE power consumption

- Discussion topics in Rel-16 include:

  - Details on the identification for a panel and corresponding panel definition

- Any enhancement introduced in Rel-16 should take further enhancement of simultaneous transmission across multiple panels for future releases into account.

This is a UE optional feature

To understand the gain of potential enhancements, quantitative performance comparisons with what was specified in Rel-15 are highly recommended. In this paper, we provide some results on performance gain on UL coverage of panel-specific beam selection.

Panel-specific UL beam selection

Panel-specific UL beam selection is in the scope of Rel-16 multi-beam enhancements. Detailed discussions on panel activation/deactivation, panel ID and usage can refer to our companion papers [3][4]. In this section, we provide evaluation results to show the potential gain on UL coverage by selecting the best panel. Figure 1 shows the distribution of the RSRP difference across two UE panels, which is quite large (with a probability of ~70% of being larger than 5dB and a probability of ~40% of being larger then 10dB). With this results, we can expect several dB gain if gNB and UE can always communicate via the best panel, compared with a random panel selection.

Figure 1. Distribution of difference between RSRPs received from different UE panels

Figure 2 provides initial SLS results on the possible UL coverage gain of supporting panel-specific beam selection. It shows that by selecting the best Tx beam across 2 UE panels, the probability of UL RSRP larger than -100dBm is increased by ~10%, compared with using a single/fixed panel, which suggests the potential gain on the UL coverage. Moreover, it is also observed that the medium RSRP is increased by ~5dB, which suggests the potential gain on the UL throughput. To achieve the observed benefit, having aligned panel management between gNB and UE is one of the keys.

Figure 2. Gain on UL RSRP from using best UL beam across 2 UE panels

Observation 1: Panel-specific UL beam selection can bring substantial benefits on both UL coverage and UL throughput.

Summary

The observations in this paper are summarized as follows.

Observation 1: Panel-specific UL beam selection can bring substantial benefits on both UL coverage and UL throughput.

- References

RP-182863, “Revised WID: Enhancements on MIMO for NR”, Sorrento, Italy, December 10th – 13th, 2018

3GPP, “RAN1 Chairman’s Notes”, Xi’an, China, April 8th – 12th, 2019

Huawei, HiSilicon, R1-1907531, “Discussion on panel activation/deactivation status”, Reno, USA, 13th – 17th, 2019

Huawei, HiSilicon, R1-1907553, “Discussion on panel ID and usage”, Reno, USA, 13th – 17th, 2019

- Appendices

Table 1 SLS simulation assumptions in Figure 1

| Parameters | Values |
| --- | --- |
| Scenarios | Dense Urban Macro layer only Option 1: 2 tier (7 sites with 21 cells) |
| Simulation bandwidth | 80MHz (DL+UL), TDD |
| Subcarrier Spacing for data | 60kHz |
| Channel Model | Following related assumption in TR 38.802/38.901 |
| TXRU mapping to antenna elements | 2D DFT |
| TXRU mapping weights | 2D DFT |
| Criteria for selection for serving TRP | RSRP-based |
| Criteria for beam selection for serving TRP | RSRP-based |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 2; 1 , 1). ((dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| BS antenna element radiation pattern | According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1); (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (0, 0) λ. *Θmg,ng=90°; Ω0,1=Ω0,0+180°; |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration for UE | N.A. |
| Beam correspondence | Full correspondence |
| UE receiver type | MMSE-IRC |
| BF scheme | PMI-based |
| UE mobility feature | UE mobility 30km/h |