3GPP TSG RAN WG1 Meeting #97	R1-1907536

Reno, USA, May 13th–17th, 2019

Agenda Item:	7.2.8.5

Source:	Huawei, HiSilicon

Title:	Evaluation results of full power transmission for UL MIMO with multiple PAs

Document for:	Discussion and Decision

Introduction

In the last meeting, UE PA architecture 1, 2 and 3 are supported for full power transmission and detailed transmission scheme will be discussed in the next meeting [1][2].

Agreement

Supported UE capabilities and supported scheme for UE capability 1

- Option 3

- FFS: Whether to additionally support Option 1-2

Agreement

RAN1 will select one of the alternative solutions below to support UE capability 2. Further clarification or details are needed for Alt1, Alt3-1, Alt3-2, and Alt5. Email discussion by 17th of April for companies to provide clarification on Alt1, Alt3-1, Alt3-2, and Alt5. To be coordinated by Rakesh (vivo).

- Alt1: Option1-1 (Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs, e.g. for 2Tx the new codeboookSubset is all non-antenna selection TPMIs or with only TPMI [1 1] for rank 1)

- Alt3-1: Option3+Option2 (Multiple SRS resources with different number of SRS port(s) in each resource)

  - FFS: Whether to additionally support Option 1-2

- Alt3-2: Option3+Option2+ Option1-1 (Multiple SRS resources with different number of SRS port(s) in each resource)

- Alt5: FDM multi-port simultaneous transmission

Offline agreement

Supported scheme for UE capability 3:

- Alt1: Option1-1

- Alt3: Option 3 or Option 3 + Option 1-2

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

- This specification support is a UE optional feature

- FFS: Whether this applies for the entire codebook or subset of codebook

Several options or combination of the options from the last two meetings listed for further discussion:

- Option 1: Refinement/adjustment of UL codebook is supported

  - 1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

  - 1-2: Introduce additional scaling factor for uplink codebook

- Option 2: UE transparently apply a small cyclic or linear delay

- Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

  - Note: Full rated PA refers to a PA having power not lower than that of the power class

- Option 4: Up to UE implementation with UE capability signalling of full power transmission in UL

- Option 5: For the precoders with 0 entries, the linear value PPUSCH,b,f,ci,j,qd,l of a PUSCH transmission power is scaled by a ratio Rel-16. The value of Rel-16 is selected up to UE implementation within the range of [Rel-15, 1],  where Rel-15 is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of configured antenna ports for the PUSCH transmission scheme as defined in NR Rel-15 specification.

  - UE is required to maintain consistent Rel-16 value on different occasions of PUSCH transmissions with the same precoder for PUSCH

In this paper, we compared potential solutions and UE capabilities.

Performance on potential solutions

For UE capability 3

In order to let partially-coherent / non-coherent UE achieves the same maximum transmit power as fully-coherent UE, full power transmission should be supported for partially-coherent / non-coherent UE especially for the coverage limited cases.

Based on the offline discussion of different alternatives in the last meeting, we provide the performance comparison between Alt1 and Alt3 which may need to be down selected in the next meeting for UE capability 3. The simulation assumes that UE has 2Tx with non-coherent capability and PUSCH transmission is fixed to rank-1, and more detailed simulation assumptions can be found in the appendix. And we also provide the evaluations for the case considering the modelling of UE’s one antenna blockage.

In Figure 1 which is based on the case without antenna blocking, we can see that both of Alt1 and Alt3 provide substantial gain over current Rel-15 non full power PUSCH transmission mechanism since UE can use more power than Rel-15 for PUSCH transmission via either Alt1 or Alt3.

In the case that both of the antennas are not blocked, Alt3 with full power transmission in single antenna is slightly better than Alt1 with small delay CDD on two antennas. The reason is that all the transmission power is concentrated to the optimal antenna which is selected by gNB based on the SRS measurement for Alt3, while for Alt1, the transmission power is scaled equally to all the antennas to support full power transmission.

Figure 1. Performance of the candidate alternatives without blocking.

Observation 1: For UE capability 3, Alt3 with full power transmission in single antenna is slightly better performance than Alt1 with small delay CDD on two antennas, and both them much better than non-full power case.

However, as shown in Figure 2 for UE capability 3, in the case that one antenna is blocked, there is obvious performance loss of Alt1 compared to Alt3. The reason is that each of the antennas cannot utilize the maximum power in Alt1, when one antenna is blocked, only half of the maximum power can be achieved. Meanwhile, for Alt3-2, the antennas that are not blocked for the PUSCH transmission can be selected for PUSCH transmission and full power on the selected antennas can still be utilized. Therefore, Alt3-2 outperforms Alt1. Based on the evaluation results and discussion, at least for the UE with full power related PAs, Alt3-2 should be supported to enhance UL coverage.

Figure 2. Performance of the candidate alternatives with blocking.

Observation 2: For UE capability 3, Alt3 with full power transmission in single antenna is much better performance than Alt1 with small delay CDD on two antennas in the blockage cases, i.e., 2dB difference.

For UE PA architecture 2

Furthermore, for the UE PA architecture-2, we also provide the simulation results with 2T4R as follows. Where the simulation for capability-2 is based on the full power transmission solution Alt3-1, i.e., Option 3+Option 1-2. The baseline here is Rel-15, non-full power transmission for non-coherent UE. As shown in Fig.3, we can see significant gain can be obtained with PA architecture-2 UEs with Alt3-1 solution. The difference is that Alt.3-1 (i.e., Option 3 + Option 2+ Option 1-2) is with a new scaling factor for precoders on the exist precoders, and Alt.3-2 (i.e., Option 3 + Option 2+ Option 1-1)  is with new added codewords such as with scaling factor equal to 1. So, Alt.3-1 and Alt.3-2 is different on the codebook size, but the same as performance.

Figure 3. Full power transmission of the UEs with Capability 2

Observation 3: UE capability 2 with full power transmission scheme Alt3-1 or Alt3-2 can obtain obvious performance gain.

Summary of discussions

Based on the above discussion, we have the following observations:

Observation 1: For UE capability 3, Alt3 with full power transmission in single antenna is slightly better performance than Alt1 with small delay CDD on two antennas, and both them much better than non-full power case.

Observation 2: For UE capability 3, Alt3 with full power transmission in single antenna is much better performance than Alt1 with small delay CDD on two antennas in the blockage cases, i.e., 2dB difference.

Observation 3: UE capability 2 with full power transmission scheme Alt3-1 or Alt3-2 can obtain obvious performance gain.

- References

3GPP, “RAN1#95 Meeting Chairman’s Notes”, Spokane, USA, November 12th – 16th, 2018.

3GPP, “RAN1#96bis Meeting Chairman’s Notes”, Xi’an, China, April 8th – 12th, 2019.

R1-1906031, “UL MIMO full power transmission with multiple PAs, Huawei, HiSilicon”, Reno, USA, May 13th – 17th, 2019.

- Appendix A. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz |
| Subcarrier spacing | 15kHz |
| Delay spread | 100ns |
| System band | 10MHz |
| Channel model | TDL |
| UE speed | 3km/h |
| UE antenna configurations | 2Tx |
| UE antenna pattern | Omnidirectional |
| BS antenna configurations | 2Rx/4Rx |
| Link Adaptation | Enable |
| Receiver | MMSE |