3GPP TSG RAN WG1 #97	R1-1907545

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.10.5

Source:	Huawei, Hisilicon

Title:	Discussion on multi-path DAoD

Document for:	Discussion and decision

# Introduction

At RAN1#96bis, an agreement on UE RX beamforming behavior for DL PRS reception was approved:

Agreement:

For positioning purposes, to assist UE to perform Rx beamforming, consider one or more of the following options:

- Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

  - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

- Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

- Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

  - FFS: Whether this option can be achieved by Option 1

- FFS on specification impacts.

In this agreement, DAoD estimation in multi-path environment was not considered.

In this contribution, we further discuss the measurements reporting for DAoD positioning in FR2. In particular, we focus on the necessary enhancements on UE measurement and reporting to improve the DAoD performance in multi-path scenarios.

# DAoD estimation in multi-path channel

For DAoD positioning, the gNBs transmit PRS resources in a beam-sweeping manner, and configure the UE to report the beam/RS measurement results (e.g. RSRP). The gNB can then estimate the DAoD with a maximum likelihood estimation algorithm. More detail can be found in our companion contribution [1].

In the presence of multi-path environment, some companies proposed that the UE should report the first received path or the strongest path detected for each PRS resource. In a LOS scenario, before RX beamforming, the first received path is likely to be largest path. If the first paths from different Tx beams are detected and reported correctly, the UE’s DAoD can be estimated accurately.

However, due to the beam gain, noise impact, and power loss in signal propagation, whether to report the largest path or first received path is actually a tradeoff between robustness and accuracy. In particular, consider the case shown in Figure 1:

- The first received path, if it is not the strongest path, is vulnerable to noise and interference, which could lead to miss detection and false alarm

- The strongest path, if it is not the first received path,  is not a LOS path, which could lead to severe DAoD estimation error;

- Even in a LOS-dominant channel, the first received path and the largest path may not always be the same path over all TX-RX beam pairs.

Observation 1: Reporting either the first received path or the strongest path is a tradeoff between robustness and accuracy.

gNB

UE

1st path

2nd path

PRS resource #0

PRS resource #1

PRS resource #2

1st path reporting

with correct detection

Strongest path

reporting

1st path reporting

with missed detection

Figure 1: Impact of multi-path effect on DAoD estimation

To resolve this issue, both the largest path and first received path should be reported to the gNB to guarantee estimation accuracy. In LTE, multi-path reporting is already supported for RSTD reporting. It is natural to support multi-path power reporting to improve the DAoD estimation. Unlike TDOA estimation, the DAoD estimation requires the power of the same path (i.e. with the same path delay) transmitted from different TX beams. Therefore, the association between the measurements and corresponding path should be indicated in the UE measurements report. The report should contain a set of measured values for each reported path so that the impact of false/miss detected path can be alleviated. Consequently, we propose delay grouped reporting of the path received power (PRP) as follows:

| Path index | relative delay | resource #1 | resource #2 | resource #3 |
| --- | --- | --- | --- | --- |
| 0 | -3Ts | PRP 1-1 | PRP 1-2 | PRP 1-3 |
| 1 | 0Ts | PRP 2-1 | PRP 2-2 | PRP 2-3 |
| 2 | 4Ts | PRP 3-1 | PRP 3-2 | PRP 3-3 |

Proposal 1: Support delay and path power reporting in case of multi-path scenarios for DL-based positioning.

Proposal 2: Path received power measurements contain a set of measured values for each reported path.

# Conclusions

In this contribution, we present our views the enhancement in UE measurement and reporting to improve the DAoD performance in multi-path scenarios. Based on the discussion, we have the following observations and proposals:

Observation 1: Reporting either the first received path or the strongest path is a tradeoff between robustness and accuracy.

Proposal 1: Support delay and path power reporting in case of multi-path scenarios for DL-based positioning.

Proposal 2: Path received power measurements contain a set of measured values for each reported path.

# References

3GPP R1-1901574, Remaining issues on DL based positioning, Huawei, HiSilicon, RAN1#96, Athens, Greece, Feb. 25 – Mar. 1, 2019.