3GPP TSG-RAN WG1 #97	R1-1907568

Reno, USA, May 13th – 17th, 2019

Agenda item:    6.2.1.1

Source:             Qualcomm Incorporated

Title:                  UE-group wake-up signal for MTC

Document for:  Discussion and Decision

# Introduction

In RAN1#96bis meeting, we have the following agreements for UE-group WUS:

Agreement

For evaluation purpose and down-selection on multiplexing schemes in RAN1#97:

- UE assumes the same WUS max duration and same transmit power for Rel-15 WUS and Rel-16 WUS

Agreement

Down-select one of the following options until RAN1#97 based on evaluation results including power saving gain, usage of resources, etc

- Up to 2 orthogonal WUS resources may be configured in time domain

- Up to 2 orthogonal WUS resources may be configured in frequency domain

- Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

- Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

Determine in RAN1#97 whether legacy WUS resource is counted as one of the configured WUS resource(s).

Agreement

Rel-15 WUS and Rel-16 WUS can be configured on the same legacy WUS resource via SI

- FFS explicitly or implicitly

- Same WUS parameters are assumed for both Rel-15 and Rel-16 WUS in case both are on the same legacy WUS resource

Agreement

- Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation.

- Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

In this contribution, we further discuss UE-group MWUS from the following aspects:

- Multiplexing of WUS

  - Up to 2 or up to 4 total WUS resources

  - Number of UE groups

- Configuration of Rel16 WUS

  - Narrowbands for Rel16 WUS

  - Weighting factors for WUS resources

  - Other parameters

# Multiplexing of WUS

If we only configure TDM or FDM WUS resources, there are up to 2 WUS resources. If we allow TDM and FDM, there are up to 4 WUS resources in total, as illustrated in Figure 1. The legacy WUS resource is one of the WUS resource. The 4 WUS resources are consecutive in time domain or frequency domain.

Figure 1 Up to 4 WUS resources for legacy and UE-group WUS

For the simulation of power saving gain, we compare the following 4 cases, where the new UEs are distributed by equally balancing in each new WUS resource to achieve the best performance [2]:

- Case1:	N=2 WUS resources include legacy WUS resource not sharing for with new UEs

- Case 2:	N=2 WUS resources include legacy WUS resource shared by legacy UE0 and new UE1

- Case 3:	N=4 WUS resources include legacy WUS resource not sharing for with new UEs

- Case 4: 	N=4 WUS resources include legacy WUS resource shared by legacy UE0 and new UE1

Table 1 UE ratio of legacy UE0/new UE1/new UE2 for Case 1/2/3/4

|  | UEs monitor legacy WUS resource |  | New UEs monitor i-th new WUS resource |
| --- | --- | --- | --- |
|  | Legacy UE0 | New UE1 | New UEi+1 in i-th WUS resource |
| Case 1 | R0 | R1=0 | R2=1 with i=1 only |
| Case 2 | R0 | R1=max⁡(1/2-R0,0) | R2=1-R0-R1 with i=1 only |
| Case 3 | R0 | R1=0 | Ri+1=1/3 with i=1, 2 or 3 |
| Case 4 | R0 | R1=max⁡(1/4-R0,0) | Ri+1=(1-R0-R1)/3 with i=1, 2 or 3 |

Table 1 summarizes of the distribution of UEs monitoring different WUS resource. We assume the legacy UE0 with ratio of R0 only monitor the 0th WUS resource, e.g., legacy WUS resource. The new UE1 if configured with ratio of R1>0 will monitor legacy WUS resource too and the remaining new UEs will be equally distributed into 1 or 3 new WUS resources. The probabilities of WUS to be detected by legacy UE0, new UE1 and remaining new UEs in i-th WUS resource are

pUE0=1-1-piNUE01-piNUE1+G11-piNUE1G1(G1-1)(1-1-piNUE1G1)

pUE1=1-1-piNUE01-piNUE1+(G1-1)1-piNUE1G1(G1-1)(1-1-piNUE1G1)

pUEi+1=1-1-piNUEi+1+(Gi+1-1)1-piNUEi+1Gi+1(Gi+1-1)(1-1-piNUEi+1Gi+1)

where p is the total paging probability and pi is the paging probability per UE with relationship of  p=1- 1-piNUE . The UE ratio R0=NUE0/NUE , R1=NUE1/NUE, and Ri+1=NUEi+1/NUE with total number of UEs as NUE=NUE0+NUE1+NUE2…. In simulation, we assume NUE=100 per PO (but similar results for different NUE if assuming same p). If legacy WUS resource is only used for legacy UEs, R1=0 and all the new UEs will only monitor one of new WUS resource(s). If the legacy UE ratio R0 is large and R1=0, no new UEs will monitor legacy WUS resource. In each WUS resource, subgrouping by further divide NUEi into Gi subgroups can further help power saving.

The legacy UE0’s average power is PUE0=(pUE01-pMISS+pFA+β), which may be increased if there is new UE1 in G1. While, the new UEi’s average power is PUEi=pUEi1-pMISS+1-1-pFAminGi,2+β. β is the scaling factor for WUS maximum duration relative to Rmax. Assuming β=1/16, pFA=1%, pmiss=1%, we have compared the normalized average power consumption per UE for different ratio R0 of legacy UEs. The average power consumption of all UEs, including legacy UE0, new UE1, new UE2,…., as P=i=0,…RiPUEi.

In Figure 2, we compare Case 1/2/3/4 with different paging probability per PO, e.g., p=0.1, 0.2 or 0.5. For TDMed and/or FDMed WUS resources, we assume same power boosting as that of legacy WUS for the new WUS resource. By using more than one WUS resource and UE subgrouping G=4 per WUS resource for new UEs, there is a substantial power saving gain of UE-group WUS with respect to that of no UE grouping, especially in case of high paging probability. In summary, we have the following observations:

Observation 1: UE grouping by using Case 1/2/3/4 achieve a substantial power saving gain with respect to that of no UE grouping, summarized in Table 2.

Table 2 Normalized average UE power consumption for Case 1/2/3/4 with G=4 per WUS resource

| Paging probability per PO | Normalized average UE power consumption (e.g., ~1% legacy UE) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Rel-15 WUS only (no grouping) | Rel-16 UE-group WUS |  |  |  |
|  |  | Case 1 | Case 2 | Case 3 | Case 4 |
| p=0.1 | 0.17 (0%) | 0.11 (-35%) | 0.096 (-44%) | 0.09 (-47%) | 0.09 (-47%) |
| p=0.2 | 0.27 (0%) | 0.14 (-48%) | 0.11 (-59%) | 0.10 (-63%) | 0.096 (-64%) |
| p=0.5 | 0.57 (0%) | 0.30 (-47%) | 0.18 (-68%) | 0.15 (-73%) | 0.13 (-77%) |

Observation 2: Comparing UE-group WUS of Case 1/2/3/4, it improves power saving by sharing the legacy WUS by legacy WUS and new WUS.

- Case 2 further reduces the power consumption and achieves substantial gain than Case 1 by 9%/11%/21% for paging probability of p=0.1/0.2/0.5.

- Case 4 with 4 WUS resources shows better performance than Case 2 but the additional gain is limited to 3%/5%/9% for paging probability of p=0.1/0.2/0.5.

- Case 4 further improves power saving gain relative to Case 3 but gets saturated if there are more than 10% legacy UE.

- p=0.1					(b) p=0.2

(c) p=0.5

Figure 2 Average UE power consumption for Case 1/2/3/4
with G=4 subgroups per WUS resource for new UEs

Figure 3 investigates the performance by using different number G of subgroups per WUS resource with different paging probability. Assuming there are N=1, 2, or 4 WUS resources, one of WUS resources coincides with the legacy WUS resource. The total number of groups will be equal to (NxG).

- p=0.1					(b) p=0.2

- p=0.5

Figure 3 Average power consumption per UE with 1 or 2 WUE resources 
and different number of UE groups per WUS resource

Table 3a summarizes the power saving gain relative to no grouping (Rel15 WUS only) assuming no legacy UEs for p=0.1, 0.2 or 0.5, respectively. In case of no legacy UEs, the legacy WUS resource will be fully used by new UEs and all the new UEs will be distributed equally into N=1, 2 or 4 WUS resources.

Table 3a Power saving gain of different number of UE groups per WUS resource (No legacy UEs)

- p=0.1

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | 0% | +22% | +36% | +42% | +46% |
| Total number of groups (2xG) | 2G=2 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +28% | +37% | +44% | +48% | +49% |
| Total number of groups (4xG) | 4G=4 | 4G=8 | 4G=16 | 4G=32 | 4G=64 |
| Gain of N=4 WUS resources | +43% | +44% | +48% | +50% | +51% |

- p=0.2

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | 0% | +31% | +47% | +54% | +58% |
| Total number of groups (2xG) | 2G=2 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +35% | +50% | +59% | +62% | +65% |
| Total number of groups (4xG) | 4G=4 | 4G=8 | 4G=16 | 4G=32 | 4G=64 |
| Gain of N=4 WUS resources | +53% | +59% | +64% | +67% | +68% |

- p=0.5

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | 0% | +34% | +48% | +54% | +56% |
| Total number of groups (2xG) | 2G=2 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +36% | +58% | +68% | +73% | +75% |
| Total number of groups (4xG) | 4G=4 | 4G=8 | 4G=16 | 4G=32 | 4G=64 |
| Gain of N=4 WUS resources | +59% | +71% | +77% | +80% | +82% |

Table 3b summarizes the power saving gain relative to no grouping (Rel15 WUS only) assuming 25% legacy UEs for p=0.1, 0.2 or 0.5, respectively. Legacy WUS is shared by legacy WUS and new WUS and 25% legacy UEs and new UE1 of 75%-(N-1)/N are allocated to monitor legacy WUS resource, and the remaining 1/N new UE2 monitor the new group WUS resource with N=1, 2 or 4 WUS resources.  Here we assume the common WUS for new UE1 is the legacy WUS in G1 and another common WUS for new UEs is a non-legacy WUS in other WUS resources.

Table 3b Power saving gain of different number of UE groups per WUS resource (25% legacy UEs)

- p=0.1

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | +11% | +22% | +29% | +33% | +35% |
| Total number of groups (2xG) | 2G=2 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +32% | +37% | +41% | +43% | +44% |
| Total number of groups (4xG) | 4G=4 | 4G=8 | 4G=16 | 4G=32 | 4G=64 |
| Gain of N=4 WUS resources | +43% | +44% | +48% | +48% | +49% |

- p=0.2

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | +13% | +29% | +38% | +42% | +44% |
| Total number of groups (2xG) | 2G=2 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +39% | +48% | +54% | +56% | +58% |
| Total number of groups (4xG) | 4G=4 | 4G=8 | 4G=16 | 4G=32 | 4G=64 |
| Gain of N=4 WUS resources | +53% | +58% | +62% | +63% | +64% |

- p=0.5

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | +15% | +31% | +38% | +41% | +43% |
| Total number of groups (2xG) | 2G=2 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +42% | +55% | +61% | +64% | +65% |
| Total number of groups (4xG) | 4G=4 | 4G=8 | 4G=16 | 4G=32 | 4G=64 |
| Gain of N=4 WUS resources | +59% | +68% | +73% | +75% | +76% |

Observation 2: More total number (NxG) of UE groups achieves larger power saving gain until (NxG)=16 achieving almost saturated gain.

- G: number of subgroups per WUS resource

- N: number of WUS resources

Observation 3: Assuming NxG=16 UE groups (to see saturated power saving gain), the larger number N of WUS resources the better power saving gain.

- In case of no legacy UEs, N=1 is sufficient for p=0.1, 0.2 but N=2 is much better for p=0.5. N=4 performs similarly as N=2.

  - p=0.1: N=1 +46%  N=2 +48% (+2%)  N=4 +48% (+2%)

  - p=0.2: N=1 +58%  N=2 +62% (+4%)  N=4 +64% (+6%)

  - p=0.5: N=1 +56%  N=2 +73% (+17%)  N=4 +77% (+21%)

- In case of 25% legacy UEs, N=2 could be ok for p=0.1 but N=4 is much better for p=0.2, 0.5.

  - p=0.1: N=1 +35%  N=2 +43% (+8%)  N=4 +48% (+13%)

  - p=0.2: N=1 +44%  N=2 +56% (+12%)  N=4 +62% (+18%)

  - p=0.5: N=1 +43%  N=2 +64% (+21%)  N=4 +73% (+30%)

Observation 4: (NxG)=16 groups in total requires 16 WUS sequences plus N common WUS sequence for N WUS resources if configured.

- For TDMed WUS resources, the WUS sequences should be different to avoid time ambiguity and improve interference randomization.

- For FDMed WUS resources, the WUS sequences should be also different to reduce PAPR (4-PRB repetition results in 6dB higher PAPR).

Based on the above analysis, we propose

Proposal 1: Up to 2 orthogonal WUS resources per dimension can be configured

- Legacy WUS resource is counted as one of up to 4 configured orthogonal WUS resources.

- eNB can keep same power boosting on the FDMed and/or TDMed WUS resources

Proposal 2: The number of WUS resources and the number of UE groups per WUS resource can be configured based on the paging probability.

- Number of WUS resources: N=1, 2, or 4

- Number of UE groups per WUS resource: G=1, 2, 4, 8, or 16

- Total number of UE groups: NxG<=16

# Configuration of UE-group WUS

3.1 Common WUS configuration

In previous meetings, we have the following agreements related to common WUS:

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

Agreement

If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to additionally support the feature of waking up a subset of all WUS UE groups

When the eNB can schedule the paging of legacy and new UEs in different POs, a common WUS can be configured as a non-legacy WUS for the new UEs. If there are more than one group per WUS resource, we prefer to let eNB schedule the paging with minimal latency and use a common WUS for all the group WUS UEs. From the Table 3a and 3b in Sect. 2, the configuration of more than one groups significantly saving the power even in case of low paging probability.

Proposal 3: If more than one UE group is configured in the same WUS resource, a common WUS is configured.

The UE only monitors one WUS resource. Per WUS resource, there is only one sequence sent from eMB at one time, e.g., a UE group WUS, a common WUS for all group WUSs. If a common WUS is configured, the UE will detect two WUS sequences. More than two WUS sequence detection will increase UE complexity as well as the false detection probability.

We have investigated the potential power saving gain by using a common WUS for a subset of groups per WUS resource. Assuming all new UEs with one WUS resource configured, there are 4 groups in total. The number of UEs per group is equal to NUE/4. We have compared the power consumption for the following two cases in Figure 4.

- Case 1: Rel16 WUS UEs with 2-sequence detection

  - A group WUS is configured per group

  - A common WUS0 is configured to wake up all 4 groups #1,2,3,4

  - The total wake-up probability is pUE_case1,  and fake wake-up probability is pUE_case1_fake

pUE_case1=1-1-piNUE+31-pi3NUE4(1-1-piNUE4)

pUE_case1_fake=pUE_case1-(1-1-piNUE4

- Case 2: Rel16 WUS UEs with 3-sequence detection

  - A group WUS is configured per group

  - A common WUS0 is configured to wake up all 4 groups #1,2,3,4

  - In addition, a common WUS1 is configured to wake up group#1 and #2 only, and a common WUS2 is to wake up group#3 and #4 only.

  - The total wake-up probability is pUE_case2,  and fake wake-up probability is pUE_case2_fake

pUE_case2=pUE_case1-1-pi2NUE4(1-1-piNUE4)2

pUE_case2_fake=pUE_case2-(1-1-piNUE4

Here, pUE_case2 is less than pUE_case1, since UEs in group#1 or #2 will not be waken up when common WUS2 is sent to wake up group #3 and #4, and UEs in group#3 or #4 will not be waken up when common WUS1 is sent to wake up group #1 and #2. However, the difference between Case 1 and Case 2 is marginal. When UE paging rate is pi=0.01 and NUE=16 (i.e.,, total paging rate will be p=0.15), the difference between Case 1 and Case 2 is only 0.0014, which is marginal relative to the total wake-up rate of 0.044. Looking at the total wake-up probability, Case 1 and Case 2 are almost same.

Figure 4 Wake-up probability comparison

Observation 5: The configuration of a common WUS for a subset of UE groups increases the UE complexity but achieves marginal power saving gain.

Proposal 4: Each UE monitors up to 2 WUS sequences.

- Configuration of a common WUS for a subset of UE groups is not supported.

3.2 WUS configuration on narrowbands

In MTC, there can be multiple narrowbands configured for paging. The eNB uniformly distributes the paging for narrowbands based on the UE ID as:

PNB = floor(UE_ID/(N*Ns)) mod Nn

The paging probability is similar per narrowband due to equal number of UEs.

If narrowband-specific on/off is supported for Rel16 WUS, it may be beneficial to change the way a Rel16 UE selects its paging narrowband. For the UEs using group WUS, the number Nn of paging narrowbands is the number of narrowbands that support group WUS.

For example, there are narrowband #0-5 for Rel-15 WUS UEs and Nn=6 for legacy UEs. When there are less new UEs, only the limited number of narrowbands are used for new UEs, e.g., the narrowband #0-2 are used to send UE-group WUS. Therefore, Nn=3 for new UEs. It could be a possible way to change the relative ratio between legacy UEs and new UEs by narrowband selection.

Proposal 5: Consider narrowband selection for group WUS configuration.

3.3 Weighting factors amongst group WUS resources

Another aspect is how to load balancing amongst the different WUS resources. From the observation in [2], load balancing to uniform UE distribution maximize the power saving gain. We could balance the number of UEs per WUS resource, e.g., in a manner similar to that developed in Rel-14 for paging in NB-IoT carriers. For the distribution of new UEs in the two WUS resources, we can set weighting factors for group WUS resources as WG(i,G1) and WG(i,G2) and WG(i)=WG(i,G1)+WG(i,G2) on i-th paging narrowband supporting group WUS. The new UEs of WG(i,G1)/WG(i) are allocated to monitor the legacy WUS resource and remaining new UEs of WG(i,G2)/WG(i) belong to the new WUS resource.

For example,

- If only 1/4 UEs are legacy, the weighting factors can be WG(i,G1)=1, WG(i,G2)=2 and WG(i)=WG(i,G1)+WG(i,G2)=3. The new UEs of WG(i,G1)/WG(i)=1/3 are allocated to monitor the legacy WUS resource and remaining new UEs of WG(i,G2)/WG(i)=2/3 belong to the new WUS resource. In total, there are 1/2 UEs per WUS resource.

- If 1/2 UEs are legacy, the legacy WUS resource is not used for group WUS. The weighting factors can be WG(i,G1)=0, WG(i,G2)=1 and WG(i)=WG(i,G1)+WG(i,G2)=1. No new UEs, i.e., WG(i,G1)/WG(i)=0, are allocated to monitor the legacy WUS resource and all the new UEs, i.e., WG(i,G2)/WG(i)=1 are belong to the new WUS resource. In total, there are 1/2 UEs per WUS resource.

Proposal 6: Support weighting factor configuration for UE distribution amongst the different WUS resources configured for a narrowband.

3.4 Power and max WUS duration

In RAN2 agreements, coverage-based grouping is not supported. The legacy WUS can be shared by Rel15 WUS and Rel16 WUS. Therefore, the parameters for legacy WUS, i.e., the max duration for new WUS sequence and the transmit power, should be applied to Rel16 WUS sequence as well.

However, there is concern on the miss detection/false alarm performance of Rel16 WUS, if a UE has to monitor more than one WUS sequence. As shown in Figure 5, we have evaluated the miss detection by assuming same transmit power and same total false alarm (FA) of 1% for a UE to detect 1/2/3 WUS sequences. A legacy UE will only detect one WUS sequence. However, a new UE with 3-sequence detection, e.g., a group WUS plus a common WUS for all groups, needs to reduce the threshold by setting FA per WUS sequence to around 0.5% so as to keep the total FA=1%. A new UE with 3-sequence detection will set FA per WUS sequence as small as 0.3%. Compared with the case of one sequence detection, the miss detection of more than one sequence per UE is degraded. However, the required WUS duration is same for all three cases.

- MCL=154dB					(b) MCL=164dB

Figure 5 Miss detection performance of WUS sequence(s) per UE

Observation 6: If UE assume same transmit power, the required WUS duration is 16 subframes for MCL=154dB (without tx diversity) and 32 subframes for MCL=164dB (with tx diversity), same for 1/2/3-sequence detection.

Proposal 7: UE assumes transmit power for Rel16 WUS sequence is same as legacy WUS sequence.

Proposal 8: Max WUS duration for Rel16 WUS sequence is same as legacy WUS sequence.

# Summary

The proposals made in this contribution are summarized below.

For WUS multiplexing:

Proposal 1: Up to 2 orthogonal WUS resources per dimension can be configured

- Legacy WUS resource is counted as one of up to 4 configured orthogonal WUS resources.

- eNB can keep same power boosting on the FDMed and/or TDMed WUS resources

Proposal 2: The number of WUS resources and the number of UE groups per WUS resource can be configured based on the paging probability.

- Number of WUS resources: N=1, 2, or 4

- Number of UE groups per WUS resource: G=1, 2, 4, 8, or 16

- Total number of UE groups: NxG<=16

For Rel16 WUS configuration:

Proposal 3: If more than one UE group is configured in the same WUS resource, a common WUS is configured.

Proposal 4: Each UE monitors up to 2 WUS sequences.

- Configuration of a common WUS for a subset of UE groups is not supported.

Proposal 5: Consider narrowband selection for group WUS configuration.

Proposal 6: Support weighting factor configuration for UE distribution amongst the different WUS resources configured for a narrowband.

Proposal 7: UE assumes transmit power for Rel16 WUS sequence is same as legacy WUS sequence.

Proposal 8: Max WUS duration for Rel16 WUS sequence is same as legacy WUS sequence.

References

- 3GPP RAN1 Chairman’s Notes, RAN1#96bis, Xi’an, China, April 8th – 12th, 2019.

- R1-1907569, “UE-group wake-up signal for NB-IoT”, Qualcomm, Reno, USA, May 13th – 17th, 2019