3GPP TSG-TSG RAN WG1 Meeting #97	R1-1907582

# Reno, USA,  13th - 17th May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on timing relationship for UE configured with EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-5-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - “else if…” sentence has an indentation error. - For the UE is configured with higher layer parameter edt-parameters and IMCS>2, the timing relationship of NPUSCH and NPDCCH is not specified if a NPUSCH transmission is not for Msg3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - to de-indent “else if” sentence - For timing relationship “if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3. ”, adding the case the UE is configured with higher layer parameter edt-parameters but NPUSCH transmission is not for Msg3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The indentation error may cause misunderstanding. The timing relationship of NPUSCH and NPDCCH is missing if the UE is configured with higher layer parameter edt-parameters but NPUSCH tranmission is not for Msg3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1-1906494 |  |  |  |  |  |  |  |  |

16.6   Narrowband physical downlink control channel related procedures

<Unchanged parts are omitted>

If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3;

else if the UE is not configured with higher layer parameter edt-parameters or if the UE is configured with higher layer parameter edt-parameters and or if the UE is configured with higher layer parameter edt-parameters but NPUSCH transmission is not for Msg3

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n’+1 to subframe n+3.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

<Unchanged parts are omitted>