3GPP TSG-RAN WG1 Meeting #97 	R1-1907589

Reno, USA, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0321 | rev |  | Current version: | 14.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial bundling for PUCCH format 3 with more than 5 CC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5C-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 36.213 clarifies that the UE should follow the PUCCH format 4 procedures when using PUCCH format 3 with more than 5 CC: If a UE is configured with more than 5 serving cells and is configured with PUCCH format 3 and not configured with PUCCH format 4/5: - The UE can assume that the total number of bits in a given subframe corresponding to HARQ-ACK (if any), SR (if any), and periodic CSI (if any) is not larger than 22. - For calculating the HARQ-ACK bits to be transmitted, the UE shall follow the procedure in subclauses 10.1.2.2.3, 10.1.2A, 10.1.3.2.3, 10.1.3A by assuming that PUCCH format 4 is configured. 36.212, however, has a differentiation of applicaiblity of spatial bundling (only for format 4/5). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that spatial bundling also applies for the case of PUCCH format 3 with >5CC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.3.1	Channel coding for UCI HARQ-ACK

<Unchanged parts are omitted>

For FDD, the sequence of bits  is the result of the concatenation of HARQ-ACK bits for different cells according to the following pseudo-code:

Set c = 0 – cell index: lower indices correspond to lower RRC indices of corresponding cell

Set j = 0 – HARQ-ACK bit index

Set  to the number of cells configured by higher layers for the UE

while c <

if transmission mode configured in cell  -- 1 bit HARQ-ACK feedback for this cell

HARQ-ACK bit of this cell

j = j + 1

else

if the UE is configured with spatial bundling on PUCCH by higher layers and if the UE is configured with PUCCH format 4 or PUCCH format 5 or PUCCH format 3 with more than 5 serving cells

binary AND operation of the HARQ-ACK bits corresponding to the first and second codewords of this cell

j = j + 1

else

HARQ-ACK bit corresponding to the first codeword of this cell

j = j + 1

HARQ-ACK bit corresponding to the second codeword of this cell

j = j + 1

end if

end if

c = c + 1

end while

For the aggregation of more than one DL cell including a primary cell using FDD and at least one secondary cell using TDD, the bit sequence  is the result of the concatenation of HARQ-ACK bits for one or multiple DL cells. Define  as the number of cells configured by higher layers for the UE and  as the number of subframes for which the UE needs to feed back HARQ-ACK bits in UL subframe n for the c-th serving cell. For a cell using TDD, the subframes are determined by the DL-reference UL/DL configuration if the UE is configured with higher layer parameter eimta-HARQ-ReferenceConfig, and determined by the UL/DL configuration otherwise. For a cell using TDD,  if subframe n-4 in the cell is a DL subframe or a special subframe with special subframe configurations 1/2/3/4/6/7/8/9/10 and normal downlink CP or a special subframe with special subframe configurations 1/2/3/5/6/7 and extended downlink CP, and  otherwise. For a cell using FDD, .

The sequence of bits  is performed according to the following pseudo-code:

Set c = 0 – cell index: lower indices correspond to lower RRC indices of corresponding cell

Set j = 0 – HARQ-ACK bit index

while c <

if

if transmission mode configured in cell 	– 1 bit HARQ-ACK feedback for this cell

HARQ-ACK bit of this cell

j = j + 1

else

if the UE is configured with spatial bundling on PUCCH by higher layers and if the UE is configured with PUCCH format 4 or PUCCH format 5 or PUCCH format 3 with more than 5 serving cells

binary AND operation of the HARQ-ACK bits corresponding to the first and second codewords of this cell

j = j + 1

else

HARQ-ACK bit corresponding to the first codeword of this cell

j = j + 1

HARQ-ACK bit corresponding to the second codeword of this cell

j = j + 1

end if

end if

c = c + 1

end while

<Unchanged parts are omitted>

For PUCCH format 3, the number of HARQ-ACK feedback bits  is computed as follows:

If the UE is configured with more than 5 serving cells and spatial bundling on PUCCH is configured, then:

-	.

Else, set  when TDD is used in all the configured serving cell(s) of the UE and  when FDD is used in at least one of the configured serving cells with TDD primary cell.

If  and , or if  and , or if  and , then

-

else,

-	.

For PUCCH format 4 or PUCCH format 5,  if the UE is not configured with spatial bundling on PUCCH by higher layers; otherwise .