3GPP TSG RAN WG1 Meeting #97	R1-1907612

Reno, USA, 13th – 17th May, 2019

Agenda Item:	6.2.3.1

Source:	Huawei, HiSilicon

Title:	Feature summary on LTE DL MIMO efficiency enhancement

Document for:	Discussion and decision

Introduction

In this contribution, the discussion on enhancing SRS capacity and coverage for LTE [2-12] are summarized. Some suggested proposals are also provided.

WID objectives and previous agreements

At RAN#80 meeting, the following objectives were agreed for enhancement in LTE Rel-16 [1].

The work item aims to specify the enhancements identified for further improving network performance. The detailed objectives are as follows.

- Enhance SRS capacity and coverage [RAN1]

  - Introduce more than one symbol for SRS for one UE or for multiple UEs on a UL normal subframe

      - Baseline: the minimum SRS resource allocation granularity for a cell is one slot, when more than one symbol in a normal subframe is allocated for SRS for the cell

      - Enhancements on PUCCH and PUSCH are not in scope

  - Introduce virtual cell ID for SRS

- Specify higher layer support of enhancements listed above [RAN2]

- Specify RF requirement of UE for the enhancements listed above [RAN4]

The agreements achieved in previous meetings are listed in the appendix.

The agreements achieved in RAN1#97 so far are listed as below:

Agreement

A guard period can be configured for frequency hopping and antenna switching of additional SRS symbols. FFS whether any restriction is needed for guard period (such as only when repetition is not configured).

- If guard period is configured, it is 1 OFDM symbol

Agreement

When intra-subframe frequency hopping/repetition and intra-subframe antenna switching are concurrently configured, frequency hopping should be performed before antenna switching.

Conclusion

Legacy SRS symbols follow the legacy configuration

Agreement

Aperiodic additional SRS can only be triggered for transmission in any subframe belonging to the legacy UE-specific SRS subframe configuration

Agreement

At least for the case where there no legacy SRS transmission on the subframe, at least independent open loop power control of additional SRS symbols from legacy SRS symbols is supported.

- Further study the power control for SRS symbols when additional and legacy SRS symbols are transmitted in the same subframe.

- FFS: independent closed loop

Summary

Remaining issues of SRS resources in a cell

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Observation 1: Since all symbols in one subframe can be used for SRS from cell perspective, the collision between SRS symbols and PUCCH would impact the scheduling of PDSCH and the feedback of HARQ-ACK, and further degrade the system performance significantly, especially for downlink heavy UEs. Observation 2: 1-13 symbols configured for SRS in one subframe would also impact the scheduling of PUSCH, and consequently degrade the system performance. Proposal 1: Necessary specification support should be introduced to handle the collision of SRS and PUCCH/PUSCH transmission. The following possible enhancements can be FFS - Introduce sPUSCH capability for Rel-16 UEs to enable sPUSCH transmission in the SRS subframe. - Note: the introduction of the sPUSCH capability has no RAN1 impact. - Introduce HARQ reference configuration, similar to eIMTA and SRS carrier based switching, for HARQ feedback - Enhanced scheduling |
| Vivo | Proposal2: multiplexing of sTTI on same subframe and same PRBs with additional SRS on symbols where SRS is not transmitted is supported. Proposal3: dynamic time and frequency domain resource allocation for additional SRS is supported. Proposal4: additional SRS dropping mechanism should be specified in the case where scheduled PUSCH and triggered additional SRS fall in the same subframe. |
| LG Electronics | Observation 1: Rel-13 additional SRS symbols for special subframe can be configured with separate higher layer parameters from those of legacy SRS symbols on special subframe. Proposal 1: New set of RRC parameter values for Rel-16 cell-specific/UE-specific SRS can be configured separately from the legacy SRS configuration. Proposal 2: It should be clarified how to use the remaining symbols in terms of resource utilization of the network for other UL usages, when the agreed cell-specific region as 1 to 13 symbols spans below the maximum possible number, e.g., only 7 symbols in the cell perspective. Proposal 5: DL HARQ reference configuration can be reused for handling collision of SRS and the other uplink channels. |
| Samsung | Proposal 1: Define the baseline by using a combination of Rel-14 SRS enhancements (i.e. up to 6 symbols in UpPTS + SRS carrier switching). Proposal 2: Cell-specific configuration of SRS resources in normal subframes is not specified in Rel-16. Observation 1: Introducing new UE capability on shortened PUSCH/PUCCH is not in the work scope of this WI. |
| QC | Observation 4: Subframe-based SRS creates issues of collision between SRS and HARQ-ACK. Observation 5: Collision between legacy PUSCH and subframe-based SRS in different CCs results in spectrum utilization degradation. Proposal 12: Further study UE-specific slot-level PUSCH/PUCCH in the same subframe of additional SRS symbols. |
| Nokia, NSB | Observation 1: If simultaneous transmission of additional SRS symbols and PUCCH is not supported, the only way to transmit PUSCH/PUCCH and additional SRS symbols in the same subframe is to use sTTI operation. Proposal 1: Consider supporting simultaneous transmission of additional SRS symbols and PUCCH in a carrier. Observation 2: In case of UL CA when carriers belong to the same TAG, configuring additional SRS symbols so that they span over the whole slot can be used to reduce the need to drop SRS symbols Proposal 2: In case of UL CA when carriers belong to different TAG or additional SRS symbols are in a carrier not configured for PUSCH/PUCCH, UE shall not drop additional SRS symbol transmissions unless UE is power limited. |

On whether to introduce specification support to handle collision of SRS and PUCCH/PUSCH transmission, the inputs are summarized as below:

- Support:

  - Huawei, HiSilicon, Vivo, LG Electronics, QC, Nokia, NSB

- Not support:

  - ZTE, Samsung, Ericsson

The proposals of supporting companies are summarized as below:

- Introduce HARQ reference configuration, similar to eIMTA and SRS carrier based switching, for HARQ feedback

- Enhanced scheduling and retransmission

- Use sPUSCH

  - Introduce sPUSCH capability for Rel-16 UEs to enable sPUSCH transmission in the SRS subframe.

  - Multiplexing of sTTI on same subframe and same PRBs with additional SRS on symbols where SRS is not transmitted is supported

- UE-specific slot-level PUSCH/PUCCH in the same subframe of additional SRS symbols

- Supporting simultaneous transmission of additional SRS symbols and PUCCH in a carrier

- Drop rules

  - UE drops SRS transmission in the additional symbols if the SRS collides with PUCCH/PUSCH/PRACH in the same carrier

Based on the input, the following is proposed:

Proposal 1: Down-selection on whether to introduce specification support to handle collision of SRS and PUCCH/PUSCH transmission.

Periodic and aperiodic of additional SRS symbols

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 2: Support periodic SRS transmission for additional SRS symbols. - FFS semi-persistent SRS to achieve the tradeoff between overhead and performance |
| Vivo | Proposal1: periodic transmission of additional SRS symbol(s) is not supported. |
| Lenovo, Moto | Proposal 1: Periodic SRS transmission for additional SRS symbol(s) should also be supported. Proposal 2: Frequency hopping and repetition should be supported in the same subframe for periodic SRS transmission for additional SRS symbol(s). |
| Samsung | Proposal 3: Periodic SRS transmission in the additional SRS symbols is not supported in Rel-16. - No new priority/dropping rules between aperiodic SRS and PUSCH/PUCCH in Rel-16. |
| QC | Proposal 11: Support periodic SRS transmission in additional SRS symbols. - Intra-subframe antenna switching/frequency hopping/repetition are supported for periodic SRS in additional symbols. |
| Nokia, NSB | Proposal 4: Periodic SRS transmission is supported in the additional SRS symbols Proposal 5: UE drops SRS transmission in the additional symbols if the SRS collides with PUCCH/PUSCH/PRACH in the same carrier |
| Ericsson | - Periodic additional SRS is not supported in LTE Rel-16 |

On the support of periodic SRS transmission for additional SRS, the views of companies are summarized as below:

- Periodic:

  - Support: Lenovo, Moto, QC, Nokia, NSB, Huawei, HiSilicon

  - Not support: Vivo, Samsung, Ericsson

The reasons to support periodic SRS transmission include further delay with only AP SRS in frequency hopping or antenna switching, configuration flexibility, DL intermittent traffic UEs. The reasons to not support periodic SRS transmission include impacts to uplink transmission, usage of legacy SRS, spec complexity. Based on the inputs, the following observation is made:

Proposal 2: Further study on periodic SRS triggering for additional SRS considering the gain and impacts to DL/UL performance.

Guard period for frequency hopping, antenna switching

The views of companies are listed as below

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: The eNB is able to configure a guard period for antenna switching and frequency hopping of additional SRS symbols. |
| ZTE | Proposal 2: If the one symbol guard period is introduced in LTE, when both SRS antenna switching and frequency hopping are configured within one subframe, the gap should be minimized. |
| Intel | Proposal: SRS configuration of antenna switching and frequency hopping should support the following cases - For additional SRS with and without repetition, SRS configuration should ensure one OFDM symbol guard period to provide “clean” SRS transmission - For additional SRS with repetition, SRS configuration may allow SRS transmission without guard symbol |
| QC | Observation 1: From RAN4 requirements, the transient period is the same for SRS frequency hopping, SRS antenna switching and power change between SRS symbols. Proposal 5: Configuration of intra-subframe SRS frequency hopping/antenna switching should consider the max number of subband changes/antenna changes/power changes. - FFS whether max number of subband changes/antenna changes/power changes within a subframe is based on UE capability. Proposal 6: Configuration of intra-subframe SRS frequency hopping/antenna switching should consider the transient period for power change due to subband hopping/antenna switching. Proposal 7: Support the following configurable transmission patterns for intra-subframe SRS frequency hopping/antenna switching: - Pattern 1: at least one symbol gap and no repetition (R=1) - Pattern 2: no gap but repetition number of R>1 Note: number of configured SRS symbols as well as repetition factor R does not count the gap symbol if configured. |
| Ericsson | - Introduce a network configurable SRS symbol offset to facilitate arbitrary guard period for antenna switching |

For the following FFS,

- FFS whether any limitation is needed for guard period (such as only when repetition is not configured).

there can be following scenarios:

- No guard period, when no intra-subframe repetition is configured

  - No support: QC

- A guard period, when no intra-subframe repetition is configured:

  - Support: QC, Intel

- No guard period, when intra-subframe repetition is configured:

  - Support: QC, Intel

- A guard period, when intra-subframe repetition is configured:

  - Support: Intel

Based on the inputs, the following is proposed:

Proposal 3: Guard period for frequency hopping and/or antenna switching

- is always configured when intra-subframe repetition is not configured, and

- can be configured when intra-subframe repetition is configured.

Frequency hopping, repetition and antenna switching for additional SRS

The views of companies are listed as below

| Company | Proposals and comments |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: When intra-subframe frequency hopping/repetition and intra-subframe antenna switching are concurrently configured, frequency hopping should be performed before antenna switching. Proposal 5: Support intra-subframe antenna switching across a subset of antenna ports. The following options can be for further study: - Configure subset of antenna port index to a UE in a subframe. - Support continuous counting of the parameter of nSRS between subframes. Proposal 6: Intra-subframe repetition, intra-subframe frequency hopping or intra-subframe antenna switching can only be applied to additional SRS symbols. |
| Vivo | Proposal6: intra-subframe frequency hopping and repetition transmission for additional SRS is supported, frequency hopping using both additional SRS and legacy SRS together is not supported. Proposal7: intra-subframe frequency hopping of additional SRS symbols is confined within a sub-frame. |
| ZTE | Proposal 1: For aperiodic SRS frequency hopping, to keep the same hopping pattern as legacy UE in the last symbol, Rel-16 SRS frequency hopping order can start from the last symbol to the first symbol of additional SRS symbols. |
| Lenovo, Moto | Proposal 3: Frequency hopping and repetition should performed before antenna switching when frequency hopping/repetition and antenna switching are concurrently configured. Proposal 4: The maximal number sounding symbols forantenna switching/frequency hopping that can be configured should not exceed 14 OFDM symbols in the same subframe including the antenna switching time. |
| LGE | Proposal 6: When eNB configures additional SRS symbols to a UE with antenna switching and frequency hopping/repetition concurrently in the same subframe, it should be clarified that the UE firstly performs frequency hopping/repetition before switching antenna port(s). |
| Intel | Proposal: To maximize the processing gains and minimize the number of transient periods, SRS transmission should be performed by SRS repetition from the same antenna(s) first followed by SRS antenna switching - LTE Rel-15 equations for frequency hopping can be reused with , where l∈0,1,…,NsymbSRS-1 is the SRS symbol number in the subframe, NsymbSRSis the number of antenna switches in the subframe, R is the repetition factor |
| Samsung | Proposal 4: Antenna switching should be done across all antenna ports within a subframe. Proposal 6: UE expects to transmit SRS over adjacent SC-FDMA symbols within a subframe. - FFS, the gap for antenna switching |
| QC | Observation 2: Pure SRS repetition reduces the SRS capacity with respect to more advanced techniques. Observation 3: SRS repetition with comb offset has the following desired properties: - Keep constant power/bandwidth across multiple symbols (thus no transient is needed) - Improve multiplexing capacity - Signal can be partially recovered even in case of one of the SRS symbols being dropped/interfered (unlike OCC). Proposal 4: The SRS pattern of additional SRS symbols can be configured complementary to that of legacy SRS symbol if transmitted in same subframe. Proposal 8: Support intra-subframe aperiodic SRS antenna switching across a subset of antenna ports. Proposal 9: Configure starting antenna index for intra-subframe aperiodic SRS antenna switching. Proposal 10: Support SRS repetition with flexible comb/comb offset configuration. |

On the details of frequency hopping, repetition and antenna switching of additional SRS symbols, companies have following proposals:

- UE expects to transmit SRS over adjacent SC-FDMA symbols within a subframe. FFS guard period.

  - Samsung

- Support SRS repetition with flexible comb/comb offset configuration.

  - QC

- Rel-16 SRS frequency hopping order can start from the last symbol to the first symbol of additional SRS symbols.

  - ZTE

- The SRS pattern of additional SRS symbols can be configured complementary to that of legacy SRS symbol if transmitted in same subframe.

  - QC

On whether to support intra-subframe antenna switching across a subset of antenna ports:

- Support: Huawei, HiSilicon, QC,

- Not support: Samsung

Based on companies’ inputs, we have the following proposals:

Proposal 4: Down-selection on whether to support intra-subframe antenna switching across a subset of antenna ports.

Trigger of additional and legacy SRS symbols

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Vivo | Proposal5: when additional SRS and legacy SRS symbols are configured on the same subframe and a codepoint in the same DCI triggers aperiodic SRS the UE transmits either aperiodic legacy SRS symbols or aperiodic additional SRS symbols. Certain rule on how UE determines which SRS to transmit should be defined. |
| Lenovo, Moto | Proposal 3: Rel-16 aperiodic SRS can be transmitted in both additional SRS symbol(s) and legacy SRS symbol and can be triggered by the same DCI. Proposal 5: Multiple SRS parameter sets can be configured by higher layers and an SRS Activation/Deactivation MAC CE can be introduced to activate the SRS parameter sets associated with each SRS request state. |
| LGE | Proposal 3: For Rel-16 LTE SRS enhancement, a codepoint in the SRS request field corresponding to nth SRS parameter set (n=1, 2, or 3 depending on DCI formats) can be flexibly/independently configured by eNB with enumerating SRS configuration(s) belonging to one of {legacy SRS only, additional SRS only, both legacy and aperiodic SRS}. |

On the remaining issues of triggering of additional SRS symbols, companies have following proposals:

- When both aperiodic additional SRS symbols and legacy SRS are triggered, the UE transmits either aperiodic legacy SRS symbols or aperiodic additional SRS symbols. FFS on which one to transmit.

  - Vivo

- Support SRS activation/deactivation of additional SRS configurations through MAC CE.

  - Lenovo, Moto

- A codepoint in the SRS request field corresponding to nth SRS parameter set (n=1, 2, or 3 depending on DCI formats) can be flexibly/independently configured by eNB with enumerating SRS configuration(s) belonging to one of {legacy SRS only, additional SRS only, both legacy and aperiodic SRS}

  - LGE

Based on the inputs, the following is proposed:

Proposal 5: Further study on triggering of additional SRS symbols and/or legacy SRS symbols:

- When both aperiodic additional SRS symbols and legacy SRS are triggered, the UE transmits either aperiodic legacy SRS symbols or aperiodic additional SRS symbols. FFS on which one to transmit.

- Support SRS activation/deactivation of additional SRS configurations through MAC CE.

- A codepoint in the SRS request field corresponding to nth SRS parameter set (n=1, 2, or 3 depending on DCI formats) can be flexibly/independently configured by eNB with enumerating SRS configuration(s) belonging to one of {legacy SRS only, additional SRS only, both legacy and aperiodic SRS}

Power control

The views of companies are listed as below:

| Company | Proposals and comments |
| --- | --- |
| Vivo | Proposal8: independent power control for additional SRS and legacy SRS is supported. |
| Intel | Proposal: If independent power control is used for SRS, for the subframe where legacy and additional SRS are present, all SRS symbols transmission should follow the same power control configurations corresponding to legacy SRS to facilitate joint processing at eNB. |
| Samsung | Proposal 5: A single value of PSRS,ci within a subframe is supported. |
| QC | Proposal 2: Support flexible power control configuration for additional SRS same or different with legacy SRS. Proposal 3: TPC command in the DCI triggering SRS can be configured to apply to aperiodic additional SRS. |
| Nokia, NSB | Proposal 3: Independent power control loop for additional SRS symbols should be considered so that trade-off between SRS repetitions and SRS TX power can be controlled. |
| Ericsson | Power control for additional SRS symbols is based on independently configured open and closed loop PC parameters If both legacy SRS and additional SRS symbols are present in a subframe, the power control for all SRS symbols follows that of legacy SRS |
| Huawei, HiSilicon | Proposal 1: Introduce a separate set of power control parameters for additional SRS from legacy SRS and the formula of the additional SRS power control is decoupled with PUSCH. Proposal 2: The TPC value in the Rel-15 DCI format applies to the additional SRS if the addition SRS are triggered. - Re-use the power control mechanism defined for carriers without PUSCH/PUCCH, where higher layer parametersPO_SRS,c, aSRS,c andPO_PUSCH,c, ac can be independently configured, even PUSCH/PUCCH is configured. |

If independent power control is supported for additional SRS symbols, how to perform power control when both legacy/additional SRS symbols:

- All SRS symbols transmission should follow the same power control configurations corresponding to legacy SRS to facilitate joint processing at eNB.

  - Intel

- A single value of PSRS,ci within a subframe is supported.

  - Samsung

- TPC command in the DCI triggering SRS can be configured to apply to aperiodic additional SRS.

  - QC

- The power control for all SRS symbols follows that of legacy SRS

  - Ericsson

- Re-use the power control mechanism defined for carriers without PUSCH/PUCCH

  - Huawei, HiSilicon

For the following FFS of power control:

Further study the power control for SRS symbols when additional and legacy SRS symbols are transmitted in the same subframe.

FFS: independent closed loop

Companies have diverse opinions, therefore, they need further study.

Sequence generation

And the details of group/sequence hopping of proponents are:

| Sourcing | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 7: Detailed sequence design of group hopping and sequence hopping should be further studied after the decision on whether a GP is introduced in RAN1 specifications. |
| ZTE | Proposal 3: For SRS pseudo-random sequence in additional SRS symbol(s), support the same symbol-level generation of the pseudo-random sequence for both and as NR. To be more specific, we provide the modified formula based on proposal 3 as below. Where is the symbol number within the subframe . |
| Intel | The corresponding agreement would require some modifications to Rel-15 approach of SRS sequence and group determination which is currently based on the subframe index. In particular, the following equations can be considered to facilitate per symbol group and sequence hopping for additional SRS symbols: , , where l is OFDM symbol index of SRS within a slot ns and Nsymb is the number of symbols per slot. Proposal: Adopt equations above to support sequence and group hopping for the additional SRS |
| QC | Proposal 13: The time-varying SRS sequence for multiple SRS symbols in a UL normal subframe is defined by changing (u, v) for SRS ZC roots as - u=fghl0+l',ns+nIDRSmod 30, where the group hopping pattern fghl',ns is given by fghl',ns=i=07c8(l0+l'+ns∙Nsymbslot)+i∙2imod 30if grop hopping is enabled0 if grop hopping is disabled , - with l0+l'+ns∙Nsymbslot as the index of the starting SRS symbol, and c(i) is initialized with cinit=nIDRS 30 at the beginning of each radio frame. - v=c(l0+l'+ns∙Nsymbslot if sequence hopping is enabled and grop hopping is disabled0 otherwise where c(i) is initialized with cinit=nIDRS 30∙25+nIDRS+∆ssmod30 at the beginning of each radio frame, and ∆ss could be 0 or ∆ss∈{0,1,…29} configured by higher layers. |

In general, there are mainly two options:

- Option 1:

where  is the symbol number within the slot  and Nsymb is the number of symbols per slot.

- Option 2:

u=fghl0+l',ns+nIDRSmod 30, where the group hopping pattern fghl',ns is given by

fghl',ns=i=07c8(l0+l'+ns∙Nsymbslot)+i∙2imod 30if grop hopping is enabled0                                                                                      if grop hopping is disabled ,

- with l0+l'+ns∙Nsymbslot as the index of the starting SRS symbol, and c(i) is initialized with cinit=nIDRS 30 at the beginning of each radio frame.

- v=c(l0+l'+ns∙Nsymbslot    if sequence hopping is enabled and grop hopping is disabled0                                             otherwise

where  c(i) is initialized with cinit=nIDRS 30∙25+nIDRS+∆ssmod30 at the beginning of each radio frame, and ∆ss could be 0 or ∆ss∈{0,1,…29} configured by higher layers.

Based on companies’ inputs, the following is proposed:

Proposal 6: For the sequence generation of additional SRS symbols, down-select between following options:

- Option 1:

where  is the symbol number within the slot  and Nsymb is the number of symbols per slot.

- Option 2:

u=fghl0+l',ns+nIDRSmod 30, where the group hopping pattern fghl',ns is given by

fghl',ns=i=07c8(l0+l'+ns∙Nsymbslot)+i∙2imod 30if grop hopping is enabled0                                                                                      if grop hopping is disabled ,

- with l0+l'+ns∙Nsymbslot as the index of the starting SRS symbol, and c(i) is initialized with cinit=nIDRS 30 at the beginning of each radio frame.

- v=c(l0+l'+ns∙Nsymbslot    if sequence hopping is enabled and grop hopping is disabled0                                             otherwise

Conclusion

Based on companies’ inputs, the following are proposed:

Proposal 1: Down-selection on whether to introduce specification support to handle collision of SRS and PUCCH/PUSCH transmission.

Proposal 2: Further study on periodic SRS triggering for additional SRS considering the gain and impacts to DL/UL performance.

Proposal 3: Guard period for frequency hopping and/or antenna switching

- is always configured when intra-subframe repetition is not configured, and

- can be configured when intra-subframe repetition is configured.

Proposal 4: Down-selection on whether to support intra-subframe antenna switching across a subset of antenna ports.

Proposal 5: Further study on triggering of additional SRS symbols and/or legacy SRS symbols:

- When both aperiodic additional SRS symbols and legacy SRS are triggered, the UE transmits either aperiodic legacy SRS symbols or aperiodic additional SRS symbols. FFS on which one to transmit.

- Support SRS activation/deactivation of additional SRS configurations through MAC CE.

- A codepoint in the SRS request field corresponding to nth SRS parameter set (n=1, 2, or 3 depending on DCI formats) can be flexibly/independently configured by eNB with enumerating SRS configuration(s) belonging to one of {legacy SRS only, additional SRS only, both legacy and aperiodic SRS}

Proposal 6: For the sequence generation of additional SRS symbols, down-select between following options:

- Option 1:

where  is the symbol number within the slot  and Nsymb is the number of symbols per slot.

- Option 2:

u=fghl0+l',ns+nIDRSmod 30, where the group hopping pattern fghl',ns is given by

fghl',ns=i=07c8(l0+l'+ns∙Nsymbslot)+i∙2imod 30if grop hopping is enabled0                                                                                      if grop hopping is disabled ,

- with l0+l'+ns∙Nsymbslot as the index of the starting SRS symbol, and c(i) is initialized with cinit=nIDRS 30 at the beginning of each radio frame.

- v=c(l0+l'+ns∙Nsymbslot    if sequence hopping is enabled and grop hopping is disabled0                                             otherwise

- References

RP-181485, “DL MIMO efficiency enhancements for LTE”, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11 - 14, 2018.

R1-1906069	Introduction of additional SRS symbols in normal UL subframe	Huawei, HiSilicon

R1-1906119	Discussion on remaining issues on additional SRS symbols	vivo

R1-1906256	Discussion on additional SRS symbols	ZTE

R1-1906273	Discussion of additional SRS symbols	Lenovo, Motorola Mobility

R1-1906716	Discussion on additional SRS symbols	LG Electronics

R1-1906776	Remaining details for additional SRS	Intel Corporation

R1-1906900	Discussion on additional SRS symbols	Samsung

R1-1907006	Additional SRS symbols	Qualcomm Incorporated

R1-1907072	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

R1-1907440	On additional SRS symbols for DL efficiency enhancements	Ericsson

R1-1906068	Discussion on power control for additional SRS	Huawei, HiSilicon

- Appendix: agreements

- RAN1#96bis:

Agreement

For aperiodic SRS transmission, any combination of the following features can be simultaneously configured

- Intra-subframe antenna switching

  - At least antenna switching across all antenna ports is supported

  - FFS: Antenna switching across a subset of antenna ports

- Intra-subframe frequency hopping

- Intra-subframe repetition

FFS: Whether above applies at the additional SRS symbols only or for legacy SRS symbols as well

Agreement

At least for the support of SRS repetition , where l∈0,1,…,NsymbSRS-1 is the OFDM symbol number, NsymbSRS is the number of configured SRS symbols, and R is the repetition factor for the configured UE.

Agreement

The configurable number of additional SRS repetitions can be

- {1, 2, 3, 4, 6, 7, 8, 9, 12, 13}

Above applies per antenna port and per subband

Agreement

A codepoint in the same DCI triggers SRS transmission for one of the following:

- Only aperiodic legacy SRS symbols

- Only aperiodic additional SRS symbols

- Both aperiodic legacy and aperiodic additional SRS symbols within the same subframe

The association of the codepoint and one of the above is configured by RRC signalling. A separate codepoint will be supported for the case of no SRS triggering.

Agreement

The size of the SRS request field for triggering Rel-16 SRS is the same as that of relevant Rel-15 DCI formats.

Agreement

Only Rel-15 DCI formats that support SRS triggering can be used to trigger Rel-16 SRS transmission.

Agreement

For additional SRS symbols, per-symbol group hopping and sequence hopping are supported.

- In a given time, only one of per-symbol group hopping or sequence hopping can be used by a UE

- FFS: Details such as sequence design

Agreement

In order to address at least possible power change due to frequency hopping or antenna switching for additional SRS symbols, select one of the following options:

- Option 1: A guard period of one symbol is introduced in RAN1 specifications.

- Option 2: No guard period is introduced in RAN1 specifications

  - It is up to RAN4 to introduce a transient period between additional SRS symbols in RAN4 specifications.

Send an LS to RAN4 to ask their view on transient period for frequency hopping or antenna switching (including the time duration needed for this transient period).

For further study until RAN1#97

Power control on additional SRS symbols and legacy symbols

- RAN1#96:

Agreement

For the numbers of additional SRS symbols that can be configured to a UE: 1-13

Agreement

For the time location of possible additional SRS symbols in one normal UL subframe for a cell:

- 1 to 13 symbols in one subframe can be used for SRS from cell perspective

  - FFS: How/Whether to introduce specification support to handle collision of SRS and PUCCH/PUSCH transmission

Agreement

Same power control configuration applies for all additional SRS symbols configured to a single UE

Agreement

Transmission of aperiodic legacy SRS and aperiodic additional SRS symbol(s) in the same subframes for a UE is supported

The following FFS were identified:

For further study in future meetings:

For intra-subframe frequency hopping and repetition of additional SRS symbols

- For support of repetition and frequency hopping, , where l∈0,1,…,NsymbSRS-1 is the OFDM symbol number,  NsymbSRSis the number of configured SRS symbols, and R is the repetition factor for the configured UE

  - FFS: Value of R,

  - Applies for aperiodic SRS

- Consider whether legacy SRS and additional SRS symbols have the same hopping pattern.

- Consider whether flexible configuration (e.g., comb/comb offset configuration) is supported for repetition of additional SRS symbols.

- LTE Rel-15 framework for frequency hopping and antenna switching can be reused for supporting frequency hopping and antenna switching for additional SRS symbols with appropriate changes. FFS on the appropriate changes.

For further study in future meetings:

Further study on power control enhancements of additional SRS symbols (including whether these features are needed):

- Independent power control of additional SRS symbols from legacy SRS symbols

- Reuse of SRS power control for carriers without PUCCH/PUSCH

- Consider SRS configurations to reduce the range of power variations in a subframe

For virtual cell ID of SRS, the following agreement was achieved:

Agreement

When a virtual cell ID is configured, eNB has the flexibility to apply the virtual cell ID to

- Only additional SRS symbol(s)

- Or both legacy and additional SRS symbol(s)

- If virtual cell ID is not configured, physical cell ID is used.