3GPP TSG RAN WG1 Meeting #97		R1-1907629

Reno, USA, May 13th – 17th, 2019

Source:	Sierra Wireless

Title:	LTE-M Preconfigured UL Resources Summary #2 RAN1 #97

Agenda Item:	6.2.1.2

Document for:	Discussion and Decision

# Introduction

The following tdoc provides a summary of the LTE-M PUR open issues, company positions and proposals for RAN1 #97.

Definitions

The following definitions are used in this document:

PUR Configuration:	A configuration that contains all required information for D-PUR transmissions. The configuration may include: the number of D-PUR allocations (see below), time/frequency resource(s), repetition, MCS, TBS, period, time offset, hopping, RNTI, power control, DMRS, scrambling code, timing advance validation mechanism configuration, and others.

PUR Parameter: 	A parameter which may not be included in the PUR configuration: Timing Advance

PUR Allocation: 	A single time/frequency D-PUR resource allocation.

PUR SS Window: 	The time period associated with a PUR allocation where the UE monitors MPDCCH.

# PUR Search Space

## CE Mode A and B

Issue: How CE Mode A and Mode B procedures apply to PUR the PUR SS is an open issue and how a mode changed is an open issue.

For dedicated PUR in idle mode, the CE mode is

Option 1: explicitly configured in the PUR configuration.

Option2: based on CE mode of last connection

Down select in RAN1#98

## PUR SS MPDCCH candidates

Issue: How are the PUR SS MPDCCH candidates (repetition and AL) determined is an open issue.

Agreement from RAN1 #96:

In idle mode, the PUR search space configuration includes at least the following:

- MPDCCH narrowband location

- MPDCCH repetitions and aggregation levels

- MPDCCH starting subframe periodicity (variable G)

- Starting subframe position (alpha_offset)

We are missing the configuration of the PUR search space bandwidth.

In idle mode, the PUR search space configuration includes at least one of the legacy USS MPDCCH bandwidths

Agreement from RAN1 #97:

For dedicated PUR in idle mode,

The maximum mPDDCH repetitions, rmax-mPDCCH-PUR, is included in the PUR configuration

For dedicated PUR in idle mode, the PUR search space repetitions and AL candidates are derived using legacy USS rules where rmax= rmax-mPDCCH-PUR

FFS: Initialization of Yp,k

## Monitor PUR SS Window when UE doesn’t transmit

Issue: Whether the UE must still monitor the PUR SS window when it doesn’t transmit on its PUR allocation (i.e. PUR is skipped) is an open issue.

Option 1: Not required to monitor PUR SS - Ericsson [1], Lenovo [4], Sierra Wireless [5], ZTE [6]

Option 2: Configurable to monitor PUR SS – LGE [8], Nokia [8]

For dedicated PUR in idle mode, if a UE skips a PUR transmission, it is not mandated to monitor the associated PUR SS window.

# Asynchronous Data Transmissions

Issue:  For a UE with a PUR configuration, how to support asynchronously data transmissions open issue.

For dedicated PUR in idle mode, a UE with a PUR configuration may send data via legacy EDT or RACH process at anytime

FFS: what UE behaviour is if PUR allocation collides with USS

# Appendix I: LTE-M PUR Tdocs

R1-1905957	Support for transmission in preconfigured UL resources in LTE-MTC	Ericsson

R1-1905980	UL transmission in preconfigured resource	Huawei, HiSilicon

R1-1906187	Discussion on preconfigured UL resources in MTC	NTT DOCOMO, INC.

R1-1906278	UL transmission in preconfigured resources for eMTC	   Lenovo, Motorola Mobility

R1-1906460	LTE-M Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

R1-1906496	Support for transmission in preconfigured UL resources for MTC	ZTE

R1-1906575	Discussion on transmission in preconfigured UL resources	Beijing Xiaomi Software Tech

R1-1906682	Discussion on preconfigured UL resources in MTC	LG Electronics

R1-1906702	Transmission in preconfigured UL resources	Nokia, Nokia Shanghai Bell

R1-1906769	UL transmission in preconfigured resources for eMTC	 Intel Corporation

R1-1906845	Remaining issues in PUR	Sony

R1-1906893	Discussion on transmission in preconfigured UL resources for MTC	Samsung

R1-1906992	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-1907080	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

R1-1907356	Discussion on LTE-M PUR resource configuration in respect to radio condition	ASUSTEK COMPUTER (SHANGHAI)

# Appendix II: Agreements

## RAN1 #97

For a given UE, for dedicated PUR in idle mode and for a given CE mode, the same size DCI, the same PUR M-PDCCH candidates, and the same RNTI is used for all DCI messages for unicast transmissions.

For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission

FFS: Value of x, and if x is fixed or signaled

FFS: FD-FDD UEs, TDD UEs

FFS: Support for monitoring of PUR SS Window before PUR transmission

Note: The PUR SS Window is the time period where the UE monitors the MPDCCH for at least a time period after a PUR transmission

For dedicated PUR in idle mode,

The maximum mPDDCH repetitions, rmax-mPDCCH-PUR, is included in the PUR configuration

For dedicated PUR in idle mode, the duration of the PUR SS window is configured by eNB

How the duration is configured, and the possible values, will be decided by RAN2.

## RAN1 #96bis

- Working Assumption#1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

- FFS: Which PUR configurations and PUR parameters will be signaled via L1

- FFS: Definition of PUR configurations and PUR parameters

- The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

- Working Assumption#2

- For dedicated PUR

- During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

  - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

- FFS if the UE monitors any additional RNTI which may be shared with other UEs.

- Note: The same RNTI may be used over non-overlapping time and/or frequency resources

- Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed.

- The UE monitors the MPDCCH for at least a time period after a PUR transmission

- FFS: Details of the time period

- FFS: UE behaviour if nothing is received in the time period

- FFS: If and how often UE monitors MPDCCH after a PUR allocation in which it has not transmitted

The value(s) of RSRP threshold(s) is UE specific

- The power control parameters within the PUR configuration, shall at least include:

- Target UL power level (P_0) for the PUR transmission

For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

## RAN1 #96

In idle mode, the TA validation configuration can include “PUR Time Alignment Timer”

- Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer

- Details on how to specify the “PUR Time Alignment Timer” is up to RAN2

In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes

- Above applies for the case where the UE is configured to use the serving cell change attribute

For dedicated PUR in idle mode, the Dedicated PUR ACK is at least sent on MPDCCH

- RAN2 can decide if a higher layer PUR ACK is also supported

For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration.

- PUR search space is the search space where UE monitors for MPDCCH

- FFS: Whether PUR search space is common or UE specific

When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures

- FFS whether only TA is acquired and then data sent on PUR is supported

- FFS other approaches to obtain a valid TA

When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied.

For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following

- Time domain resources including periodicity(s)

  - Note: also includes number of repetitions, number of RUs, starting position

- Frequency domain resources

- TBS(s)/MCS(s)

- Power control parameters

- Legacy DMRS pattern

In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission:

- Timing advance adjustment

- UE TX power adjustment

- FFS: Repetition adjustment for PUSCH

FFS: Whether the above update is done in L1 and/or higher layer

In idle mode, the PUR search space configuration includes at least the following:

- MPDCCH narrowband location

- MPDCCH repetitions and aggregation levels

- MPDCCH starting subframe periodicity (variable G)

- Starting subframe position (alpha_offset)

For dedicated PUR in idle mode, the PUR resource configuration includes at least the following

- A PUSCH frequency hopping indication to enable or disable legacy frequency hopping

In idle mode, a UE can be configured such that TA is always valid within a given cell.

- FFS: up to RAN2 how to implement e.g. PUR Time Alignment Timer = infinity

## RAN1 #95

For dedicated PUR in idle mode, the UE may skip UL transmissions.

- FFS: Resource release mechanism

- FFS: Whether or not to support mechanism to disable skipping by eNB

The following evaluation parameters for CFS PUR link level simulation are agreed.

| Parameter | Value/Description |
| --- | --- |
| Channel model | ETU 1Hz |
| Resource Bandwidth(s) | 1, 2, 3 or 6 PRBs (contiguous) for eMTC Proponent to provide details on resource allocation Same bandwidth for all UEs |
| Traffic Pattern | Fixed (N UEs transmit in the same resource) or (Optional) Bursty (N UEs may transmit in the same resource, with activity factor p for each of them) Note: For all traffic models, it is assumed that a UE has a dedicated DMRS Note: Final decision can be based on the results for both fixed and bursty traffic model |
| Number of repetitions in a resource | To be declared by proponent |
| Transport Block Size and Modulation Order | To be declared by proponent |
| Number of eNB Rx antennas | 2 as baseline, other values are not precluded |
| Metrics | BLER before HARQ Number of resources (PRB x subframes) used by the technique to serve a given number of UEs Baseline for comparison is dedicated PUR |
| MCL | 164, 144, 124dB MCL |
| UE grouping | All UEs in the same MCL level. |
| Processing Receiver at eNB | No SIC/joint decoding of UEs is assumed. MMSE single user decoding. Not NOMA. |
| DMRS collision | Each company to provide details on their DMRS assumption |
| Channel Estimation CFO Timing Error Power Imbalance | Realistic Uniform +/-50 Hz, +/-100 Hz (optional) +/-0.5 CP uniform, +/-1 CP uniform (optional) Uniform +/-[5 or 10] dB |

The evaluated proposals should be within the scope of the eMTC WI.

If multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process

- FFS: if multi-TB grant is enabled/supported

In idle mode, at least the following TA validation attributes are supported:

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

  - Based on RSRP measurement definition in existing Rel-15 TS36.214

Include in LS to RAN2, RAN4 to consider in their work.

The UE can be configured to use at least these TA validation attributes:

- Time Alignment Timer for idle mode

- Serving cell RSRP changes

- Note: the configuration shall support disabling of the TA validation attributes

Include in LS to RAN2, RAN4

For further study:

TA validation attributes:

- Subscription based UE differentiation (or Stationary indication in held in subscription)

- Cell specific indication where TA is valid within that cell

Include in LS to RAN2, RAN4:

RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode.

For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in MPDCCH search space

- FFS: Details on the search space (for example USS, CSS)

For dedicated PUR in idle mode, upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK

FFS: if ACK is sent on MPDCCH (layer 1) and/or PDSCH (layer 2/3)

Include in LS to RAN2, RAN4.

For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect

- an UL GRANT for retransmission on the MPDCCH, or

- FFS: a NACK, or

- FFS: no explicit ACK

Include in LS to RAN2, RAN4.

## RAN1#94bis

In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed):

- Serving cell changes (serving cell refers the cell that the UE is camping on)

- Time Alignment Timer for idle mode

- Serving cell RSRP changes (serving cell refers the cell that the UE is camping on)

- FFS Other attributes:

  - Neighbour cell RSRP change

  - TDOA of >=2 eNBs

  - TA History

  - Subscription based UE differentiation

  - Others not precluded (for example, attributes that need to be considered for high mobility UEs)

Note that UE power consumption should be taken into account for the FFS attributes

Dedicated preconfigured UL resource is defined as an PUSCH resource used by a single UE

  - PUSCH resource is time-frequency resource

  - Dedicated PUR is contention-free

Contention-free shared preconfigured UL resource (CFS PUR) is defined as an PUSCH resource simultaneously used by more than one UE

  - PUSCH resource is at least time-frequency resource

  - CFS PUR is contention-free

Contention-based shared preconfigured UL resource (CBS PUR) is defined as an PUSCH resource simultaneously used by more than one UE

  - PUSCH resource is at least time-frequency resource

  - CBS PUR is contention-based (CBS PUR may require contention resolution)

In idle mode, dedicated PUR is supported.

- Support for CFS PUR is FFS.

- Support for CBS PUR is FFS.

In IDLE mode, HARQ is supported for transmission in dedicated PUR

- A single HARQ process is supported,

  - FFS whether more than one HARQ processes are supported

- FFS: The design of the corresponding MPDCCH search space

For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported.

For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria

Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

## RAN1 #94

Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA

- FFS: Validation mechanism for TA

- FFS: How the pre-configured UL resources is acquired

For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed

Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types.

HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered:

  - Whether to support HARQ;

      - If supported, details of HARQ design including the number of HARQ processes;

  - Whether ACK/NACK is necessary

Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.