3GPP TSG-RAN WG1 Meeting #97	R1-1907657

Reno, USA, May 13th – 17th 2019

Agenda Item:	7.2.5.1

Source:	Thales

Title:	Summary on simulation assumptions for NTN

Document for:	Discussion

# Introduction

This document aims to summarize the companies’ views on :

- SLS parameters and assumptions for calibration

- SLS parameters and assumptions for performance evaluation

- LLS parameters and assumptions

- Link budget assumptions and methodology

Companies proposals and comments already in line with the conclusions of the last e-mail discussion are not reported in the document. Potential agreements are proposed in the corresponding sub-sections of each section.

The top priority for the next meeting is to reach consensus on SLS parameters and assumptions for calibration (i.e. on the first four proposals).

The proposals updated based on offline discussions are summarized in the conclusion section.

# Discussion on System Level Simulation assumptions.

## Simulations assumptions for calibration

### Frequency bands and Maximum bandwidth per beam

Note : S-band Carrier Frequency = 2 GHz for UL and DL ; K-band Carrier Frequency = 20 GHz for DL and 30 GHz for DL.

| Companies | Proposal and observation |
| --- | --- |
| Samsung | S-Band: 10 MHz / Ka band: 100 MHz |
| Ericsson | Proposal 1 TR 38.821 should avoid referring to frequency bands but could specify generic carrier frequency values (e.g. fc = 2 GHz for below 6 GHz and fc = 20 GHz for above 6 GHz) for evaluation/calibration purposes. • Below 6 GHz : DL: 10 MHz / UL: 10 MHz • Above 6 GHz : DL: 100 MHz / UL: 100 MHz |
| e-mail discussion conclusions | Potential agreement : 400 MHz system bandwidth for Ka-band and 30 MHz for S-band should be considered for both UL and DL. Potential agreement : The system bandwidth per beam should be adapted based on the previous agreement and the frequency factor considered. |

### Frequency reuse factor

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Observation 1: Full frequency reuse is adopted in NR while N-color frequency reuse is usually employed for current satellite communication. Proposal 3: A 3-color or 4-color scheme can be considered for NTN performance evaluation. Frequency re-use factor; S band: 3 or 4 for linear polarization Ka band: 4 for linear polarization and 2 for dual circular polarization. |
| e-mail discussion conclusions | Potential agreement : The system bandwidth per beam should be adapted based on the frequency factor considered. |

### Beam layout description

Note Tdoc R1-1907687 has been submitted in revision of R1-1907087.

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Proposal 4: A 19-beam layout with possibly different configurations should be considered for system level simulation in NTN. Reference satellite position; GEO: (42157,0,0) LEO-600: (6971,0,0) LEO-1200: (7571,0,0). Beam layout; Bore-sight points placed at the center of hexagonal cells as in Figure 1(a). |
| CATT | Proposal 2: Support hexagonal beam layout and rectangle beam layout for performance evaluation. |
| Intel | Proposal 3: Clarify the following details for the hexagonal beam layout for NTN evaluations - Determination of center of cells considering curvature of earth, determination of directions of beams, relation between inter site distance and half power beamwidth of the satellite antenna Proposal 6: Support 4-color frequency reuse with 19 cells for evaluations of NTN - Full frequency reuse with 7 cells can be optionally considered |
| ZTE | Proposal 5: Beam layout defined with the bore-sight direction based on the UV space can be considered for NTN.  Parameters listed in Table 10 can be considered as starting point for LEO1200 in S-band |
| Samsung | Scenario A : 57 beams, Frequency re-use factor = ¼ Scenario C2/D2 : 48 beams, Frequency re-use factor =1/4 ‘Constant inter-beam distance on the ground’ can be used. c.f. ITU-R S.2361 |
| Ericsson | • 1 satellite right above the center of simulation area (i.e., the center of hexagonal layout) • The satellite generates 19 beams • Each beam points to the center of a hexagon in the hexagonal layout |
| ESA | Proposal 1: As far as the beam size computation is concerned, it is recommended to compute the θ_3dB value directly from the generated radiation pattern. |
| Thales (from R1-1907687) | Proposal 1 Defining the beams bore-sight directions and the satellite location is sufficient to provide a complete description of the NTN beam layout. Proposal 2 The beam layout definition principles based on the 3dB contour and the hexagonal mapping on UV plane should be adopted as baseline. Proposal 3 For GEO scenarios, the beam layout should be defined based on a target service area. Proposal 4 The beam layout definition proposal for GEO scenario presented in Table 4.1-1 should be adopted. Proposal 5 The beam layout parameters for GEO scenarios presented in Table 4.1-2 should be considered as a starting point for calibration. Proposal 6 Distinct beam layouts characterized by different elevation angle variation in the coverage should be considered to model accurately LEO scenarios. Proposal 7 The beam layout definition proposal for LEO scenarios presented in Table 4.2-1 should be adopted. Proposal 8 The beam layout parameters for LEO scenarios presented in Table 4.2-2 should be considered as a starting point for calibration. |
| e-mail discussion conclusions | Potential agreement : The beam layout is fully described by the bore-sight beam directions, the antenna beam pattern and the satellite location. Potential agreement : The satellite location should be provided in commonly-used geocentric referential (.e.g. ECEF coordinates). Potential agreement : The bore sight directions should be provided at least in terms of azimuth-elevation spherical coordinates (from satellite reference). The directions described in equivalent models as u-v plane coordinates can be provided as well if beneficial. Potential agreement : Three tiers of interfering beams (.i.e. 37 satellite beams based on a hexagonal layout) should be considered as a baseline for single multi-beam satellite simulations. Potential agreement : Additional beams can be considered for multi-satellite simulations. |

### Satellite characteristics

Note : Considering the satellite characteristics set (denoted as Set 1) captured in the chairman’s notes has already been agreed last meeting.

| Companies | Proposal and observation |
| --- | --- |
| Huawei | New set of satellite characteristics provided with updated antenna diameter in Ka-band. The EIRP density and G/T values have been updated as well. |
| CATT | New set of satellite characteristics provided in Ka-band. The satellite beam diameter values have been updated. However, the equivalent satellite antenna aperture values seem to remain identical to set 1 values. The reason for this should be clarified. |
| ZTE | Proposal 2: Update the agreed satellite parameters for set-1 by values listed in Table 3. Proposal 4: Satellite parameters in in Table 5 can be considered as the baseline for set-2 simulation with reference constellation defined in Table 6.  Selection on partial of beams for each satellite can be conducted as required. |
| Samsung | No preference for Set 2. However, as this a second set of parameters, it should be optional. |
| ESA | Set 1 : Proposal 2: Due to a typo, it is recommended to amend the G/T to 1.1 dB K-1 for both LEO S-band altitude. Proposal 3: Due to a mistake, it is recommended to amend the G/T to 28 dB K-1 for the GEO Ka-band case. Set 2 : Proposal 4: As far as the alternative set (i.e., Set-2) is concerned, it is recommended to adopt Table 2 and Table 3 to describe the main satellite parameters for downlink and uplink transmissions, respectively. |
| e-mail discussion conclusions | Potential agreement : The need to define an additional set of satellite parameters should be further discussed. |

### Satellite polarization configuration

| Companies | Proposal and observation |
| --- | --- |
| Samsung | Linear Polarization |
| e-mail discussion conclusions | Potential agreement : Circular polarization should be considered for satellite antennas |

### UEs coverage distribution

| Companies | Proposal and observation |
| --- | --- |
| e-mail discussion conclusions | Potential agreement : X=10 UEs per beam (with uniform distribution in all the beams) should be considered as baseline. It is up to the companies to provide additional results based on other X values. |

### UE configurations

| Companies | Proposal and observation |
| --- | --- |
| CATT | VSAT characteristics are provided for Ka-band: Equivalent satellite antenna aperture; Ka-band; 0.5 m VSAT. GEO; LEO-1200; LEO-600. UE EIRP density; 28 dBW/MHz; 10 dBW/MHz; 4 dBW/MHz. Equivalent satellite antenna aperture (Note1); Ka-band; 0.5 m VSAT. G/T; 10 dB K-1. |
| Samsung | VSAT Ka : •VSAT: Ka band: dish antenna with radius 0.74m, linear polarization, EIRP: 42.6 dBW, noise figure: 5dB, antenna temperature 225K , G/T 18.6 dB/K |
| e-mail discussion conclusions | Potential agreement : The UE characteristics described in the following table should be considered for calibration : Characteristics; VSAT; Handheld; Other (Note 1). Frequency band; S band; Ka band; S-band; Ka band. Antenna type and configuration; Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter; Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter; 1 or 2 omni directional antennas (FFS); Directional (M,N,P,Mg,Ng) = (4,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=650, directivity 8dB). Polarisation; circular; circular; Linear : +/-45°X-pol; Linear : +/-45°X-pol. Rx Antenna gain; 19.7 dBi; 39.7 dBi; 0 dBi; FFS. Antenna temperature; 150 K; 150 K; 290 K; FFS. Noise figure; 1.2 dB; 1.2 dB; 7 dB; 9 dB. Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices.. |

### UE orientation

| Companies | Proposal and observation |
| --- | --- |
| e-mail discussion conclusions | Potential agreement : Ideal beam tracking is assumed for both VSAT and Others UE. |
| Nokia | It is better to provide the reference for definition of “Other” UE in potential agreement, or update potential agreement to “Ideal beam tracking is assumed for non-handheld UEs”. |

### Hand over margin

| Companies | Proposal and observation |
| --- | --- |
| Intel | For RSRP-based cell selection, support 0 dB handover margin for calibration, value of handover margin for performance evaluations should be reported by each company |
| Nokia | Proposal 8: Use non-zero handover margin (for example 2dB) as simulation assumption to calibration handover margin function for serving beam selection. |
| e-mail discussion conclusions | Potential agreement : 0 dB hand over margin should be considered for calibration. |

### UE attachment

| Companies | Proposal and observation |
| --- | --- |
| Intel | Proposal 5: Consider RSRP-based cell selection for handheld NTN terminals and location-based cell selection for VSAT |
| e-mail discussion conclusions | Potential agreement : RSRP should be considered for UE attachment. |

### Metrics for calibration

| Companies | Proposal and observation |
| --- | --- |
| Samsung | Proposal 3: The number of metrics to evaluate should be minimized to reduce efforts. Coupling loss, geometry SINR, RTT and residual Doppler shift should be regarded as mandatory metrics. |
| e-mail discussion conclusions | Potential agreement : Coupling loss and geometry should be adopted as baseline metrics for calibration. It is up to the companies to provide additional metrics. |

### Multi-satellite simulations :

| Companies | Proposal and observation |
| --- | --- |
| Huawei | To reduce the simulation complexity, instead of defining a complete satellite constellation for scenarios C2 and D2, it is better to define positions for up to 7 satellites (including 2 neighboring intra-orbit satellites, 4 neighboring inter-orbit satellites and the reference satellite itself). While for scenario A, it is sufficient to give positions for up to 3 satellites (including 2 neighboring intra-orbit satellites and the reference satellite itself). Proposal 2: Multiple satellites should be considered to study the inter-satellite interference for beams at low elevation angle. |
| Nokia | Observation 1: Satellite constellation or multiple neighbour satellites should be considered in the calibration for simulation platform in NTN. Proposal 1: Use inclination satellite constellation and focus on partial geographical area for simulation platform calibration and following system-level evaluation. Complete constellation descriptions are provided for each configuration scenario. Proposal 3: For LEO satellite, the neighbor satellites, which are visible by one UE, will be included for this UE’s performance statistics, including serving beam selection and interference statistics. Proposal 4: For GEO satellite, the neighbor beams, which might leak interference to one UE, will be included for this UE’s performance statistics, including serving beam selection and interference statistics. |
| Intel | Proposal 2: Support satellite constellation with one satellite for calibration and performance evaluations for NTN o Satellite constellation with multiple satellites is FFS |
| ZTE | Proposal 1: Basic principle on the satellite reference constellation design and beam layout should be agreed, e.g., minimum elevation angle is set to 30. Proposal 3: Reference constellation defined in Table 4 can be considered for set-1 simulation with single or multiple satellite(s).  Selection on partial of beams for each satellite can be conducted as required. |
| Samsung | No preferred. We should not consider multi-satellite simulations for simplicity |
| e-mail discussion conclusions | Potential agreement :The need to consider multi-satellite simulations at least for calibration should be further discussed. Potential agreement :If multi-satellite simulations are considered, the need to define reference constellations for each configuration scenario should be further discussed. |

### Path loss computation for calibration

| Companies | Proposal and observation |
| --- | --- |
| Nokia | Proposal 5: Calculate atmospheric absorption loss for calibration in Ka band but neglect it in S band with 10° as the minimal elevation angle configuration. Proposal 6: It is recommended to consider clear sky condition as baseline in calibration and neglect the path loss due to rain and cloud attenuation. Proposal 7: The value of amplitude index S_4 should be given in the simulation assumptions if the latitude is possible within ±20° and moreover the carrier frequency is below 6GHz. |

### Wrap around methodology

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Proposal 1: An extended wrap-around mechanism by adding one or several layers comprising different beam bore-sight points can be adopted for intra-satellite interference modeling. |
| Intel | Proposal 4: Consider system level evaluations without wrap around for NTN o Support system level evaluations with wrap around after the details of wrap around modelling are clearly defined |

### General agreements

Proposal : The following tables are proposed for agreement on SLS assumptions for calibration :

- Table X.1 and X.2 define the 2 sets of satellite parameters to be considered as the baseline for SLS.

- Table X.3 defines UE characteristics to be considered as the baseline for SLS.

- Table X.4 summarizes the beam layout definition principles to be considered as the baseline for SLS.

- Table X.5 summarizes the simulation assumptions to be considered for calibration.

Table X.1 : Set-1 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table X.2 : Set-2 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | TBC | TBC | TBC |
| Satellite EIRP density |  | TBC | TBC | TBC |
| Satellite beam diameter (Note 2) |  | TBC | TBC | TBC |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | TBC | TBC | TBC |
| Satellite EIRP |  | TBC | TBC | TBC |
| Satellite beam diameter (Note 2) |  | TBC | TBC | TBC |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | TBC | TBC | TBC |
| G/T |  | TBC | TBC | TBC |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | TBC | TBC | TBC |
| G/T |  | TBC | TBC | TBC |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table X.3 : UE characteristics for System Level Simulations

| Characteristics | VSAT | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 20 GHz UL and 30 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 20 GHz UL and 30 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (4,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=650, directivity 8dB) |
| Polarisation | circular | Linear : +/-45°X-pol | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi | TBC |
| Antenna temperature | 150 K | 290 K | TBC |
| Noise figure | 1.2 dB | 7 dB | 9 dB |
| Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. |  |  |  |

Table X.4 : Beam layout definition for each configuration scenario

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Beam layout definition | Baseline : Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame |  |
| Number of beams | Baseline: 37-beam layout (i.e. 36 beams surrounding the central beam and allocated on 3 distinct “tiers”) |  |
| UV plane illustration (extracted from [16]) |  |  |
| UV plane convention | U axis is collinear to Z axis in ECEF reference frame. The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) | U axis is collinear to satellite velocity vector in ECEF reference frame. The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width (HPBW) of the satellite antenna pattern : ABS = sqrt(3) x sin(HPBW/2) |  |
| Central beam bore sight direction definition | Baseline: Based on service target area | Baseline: Central beam center is considered at nadir point |

Table X.5 : SLS assumptions for calibration

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) / Ka- Band (i.e. 20 GHz DL, 30 GHz UL) |
| Maximum Bandwidth per beam (DL + UL) | S-band : DL 30 MHz and UL 30 MHz Ka-band : DL 400 MHz and UL 400 MHz The bandwidth per beam must be adapted based on the frequency factor considered. |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table X.1 and Table X.2 (optional) |
| Satellite antenna pattern | TR 38.811v15.0.0 section 6.4.1 Bessel function |
| Satellite polarization configuration | Circular |
| Beam layout definition | See Table X.4 |
| Frequency re-use factor | Base-line : - S band : Frequency re-use factor = 4 for linear polarization ; - Ka band: Frequency re-use factor = 4 for linear polarization and 2 for dual circular polarization ; Additional results with other frequency re-use factor assumptions can be provided |
| Deployment scenarios | Base-line : Rural Additional deployment scenario results can be provided |
| Fast fading model | Frequency selective channel model from TR 38.811 |
| Propagation conditions | Base-line : Clear Sky |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | Base-line : X=10 UEs per beam with uniform distribution in all the beams |
| UE configuration | S-band : - Handheld (optional for scenario A) Ka-band : - VSAT - Others (optional for scenario A) See Table X.3 |
| UE orientation | VSAT and Others : Ideal Tracking serving beam; Handheld: Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base-line : Coupling loss, Geometry |
| Note 1 : Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. |  |

Proposal : Based on the companies’ feedback, some of the beam layout parameters provided in the companies contributions (e.g. Thales contribution R1-1907687, ZTE contribution R1-1906870, …) should be endorsed as starting points for calibration and captured in the following table.

Table X.6 : Beam layout parameters

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | TBC | TBC |
| Adjacent beam spacing on UV plane | TBC | TBC |
| Satellite location in ECEF coordinates | (X, Y, Z) = TBC | (X, Y, Z) = TBC if needed The satellite location/velocity may not be needed as long as only one satellite is considered |
| Satellite normalized velocity vector in ECEF coordinates | Not needed | (Vx,Vy,Vz) = TBC if needed The satellite location/velocity may not be needed as long as only one satellite is considered |
| Central beam center location on ground in long lat coordinates | TBC | Not needed |
| Central beam bore sight direction coordinates in UV plane | TBC | TBC |
| Gateway location in long lat coordinates | Baseline : Same as central beam center location on ground | Not needed |
| Gateway direction coordinates in UV plane | Baseline : Same as central beam bore sight direction coordinates in UV plane | TBC |

Proposal : The wrap around mechanism should not be considered as a baseline . It can be introduced if needed for intra-satellite interference modeling based on additional bore-sight beam directions (which in this case should be provided with the beam layouts parameters).

Proposal : The value of amplitude index S_4 used in ionospheric scintillation loss model should be given in the simulation assumptions if the UE latitude is comprised within ±20° and the carrier frequency is below 6GHz.

## Simulation assumptions for performance evaluations.

### Fast fading model

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Frequency selective channel model from TR 38.811v15.0.0 |
| CATT | Proposal 5: Use the LOS based channel model for performance evaluation. |

### Receiver type

| Companies | Proposal and observation |
| --- | --- |
| Huawei | MMSE-IRC |
| CATT | MMSE |

### Reference signals configuration

| Companies | Proposal and observation |
| --- | --- |
| Huawei | DL; PDCCH; 2 symbols*. DMRS; Type II, 2 symbols. CSI-RS; 1 port for each beam. SSB; 1 SSB for each beam, per 20 ms*. TRS; 2 consecutive slots per 20 ms, 1 port*. PTRS; 1 port PT-RS, (L,K) = (1,4)*. UL; PUCCH; 2 RBs, 14 OFDM symbols*. DMRS; Type II, 2 symbols. SRS; 2 symbols per 5 slots,8 RBs per symbol*. PTRS; 1 port PT-RS, (L,K) = (1,4)*. Note: Parameter value with mark * is reused from ITU self-evaluation. Reference in R1-1808073.. |
| CATT | 1 port CSI-RS, period=20ms |

### Scheduler

| Companies | Proposal and observation |
| --- | --- |
| Huawei | PF |
| CATT | PF |

### Traffic model

| Companies | Proposal and observation |
| --- | --- |
| Huawei | FTP 3 for handheld, packet size 0.5Mbytes, arrival ratio TBD; TBD for VSAT; |
| CATT | FTP model 1 |

### Channel estimation

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Realistic |

### MIMO mode

| Companies | Proposal and observation |
| --- | --- |
| Huawei | IT-1R |

### CSI feedback

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Periodic |

### Metrics for performance evaluations.

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Baseline: UE throughput (5%, 50%, 95%) Optional : Packet delay |
| CATT | Baseline: UE throughput (5%, 50%, 95%) |
| Intel | Proposal 1: At least user experienced data rate defined in TR38.913 should be considered as KPI for performance assessment of NR NTN deployments |
| Nokia | Proposal 18: UE throughput (5%, 50%, 95%) and beam throughput (5%, 50%, 95%) can be used as performance metric for throughput evaluation in NTN. Proposal 19: Solutions for interference mitigation can be used for throughput evaluation in NTN. |

### General agreements

Proposal : The following skeleton tables are proposed for agreement on SLS parameters for performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.7 : SLS parameters for performance evaluation

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | Same as in Table X.5 |
| Maximum Bandwidth per beam (DL + UL) | Same as in Table X.5 |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | Same as in Table X.5 |
| Satellite antenna pattern | Same as in Table X.5 |
| Satellite polarization configuration | Same as in Table X.5 |
| Beam layout | FFS |
| Number of beams | FFS |
| Frequency re-use factor | FFS |
| Deployment scenarios | Same as in Table X.5 |
| Fast fading model | FFS |
| Propagation conditions | Same as in Table X.5 |
| UEs outdoor/indoor distribution | Same as in Table X.5 |
| UEs coverage distribution | Same as in Table X.5 |
| UE configuration | Same as in Table X.5 |
| UE orientation | Same as in Table X.5 |
| Handover Margin | FFS |
| UE attachment | FFS |
| Receiver type | FFS |
| Reference signals configuration | See Table X.8 |
| Scheduler | FFS |
| Traffic model | FFS |
| Channel estimation | FFS |
| MIMO mode | FFS |
| CSI feedback | FFS |
| Metrics for performance | Baseline: UE throughput (5%, 50%, 95%) |
| Note 1 : Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. |  |

Table X.8 : Reference signals configuration for performance evaluation

| DL | PDCCH | FFS |
| --- | --- | --- |
|  | DMRS | FFS |
|  | CSI-RS | FFS |
|  | SSB | FFS |
|  | TRS | FFS |
|  | PTRS | FFS |
| UL | PUCCH | FFS |
|  | DMRS | FFS |
|  | SRS | FFS |
|  | PTRS | FFS |

# Discussion on Link Level Simulation assumptions

## Downlink synchronization

### Assumptions

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Proposal 6: A maximum frequency offset value that reflects the typical NTN deployment scenarios should be determined for both regenerative and transparent cases. Some simulation assumptions have been provided in Table 1 (R1-1905992) |
| Intel | Proposal 7: Consider pre/post compensation of Doppler shift at the transmitter/receiver of the gNB - Values of max Doppler shift residual error due to satellite and UE mobility provided in R1-1905206 |
| ZTE | Proposal 7: Simulation assumption listed in Table 9 (R1-1906870) can be considered as the baseline for DL synchronization evaluation. |
| Ericsson | Assumption 2: The simulation focuses on the service link only. The feeder link impairments are ignored. Assumption 3: The signal received by the satellite is assumed to undergo post-compensation to correct for UL frequency shift. This correction, for example, can be performed with respect to the beam center. The signal transmitted by the satellite is assumed to undergo pre-compensation to correct for DL frequency shift. This correction, for example, can be performed with respect to the beam center. Proposal 12 RAN1 to discuss antenna configuration assumptions. Proposal 13 RAN1 to agree on the link level evaluation assumptions listed in Table 2 (R1-1907395). |
| ESA | Proposal 5: As far as the satellite payload impairments are concerned, it is recommended to adopt Table 4 and Table 5 (R1-1907481) for downlink and uplink transmissions, respectively. |
| Nokia | The simulation assumptions for UE capability about reference frequency with following 3 probabilities should be considered. - UE does not have a reference and synchronizes its frequency to observed DL SSB signal - UE has reference and reference is equal to DL reference - UE has reference but is different from DL reference This aspect may be also important in context of PRACH transmission and UE Doppler frequency estimation. For example, a UE without reference may be required to have GPS in order to estimate the Doppler frequency offset. On the other hand, a UE with its own reference may estimate the Doppler from DL synchronization signals like the PSS/SSS and thus may not need GPS. |

| Companies | Proposal and observation |
| --- | --- |
| Nokia | The definition of max Doppler rate needs to be provided. |

### Metrics

| Companies | Proposal and observation |
| --- | --- |
| Huawei | PSS detection rate for one shot FAR detection rate Residual frequency offset |
| ZTE | Joint PSS and SSS detection false alarm rate, residual timing/frequency offset |

### General agreements

Proposal : The following skeleton table is proposed for agreement to capture the impairments introduced on the RF signal due to the satellite payload and movement. Companies are encouraged to provide values to fill the table for the next meeting.

Table X.9 : Impairments due to satellite payload and satellite movement

|  | FR1 – Below 6 GHz | FR2 – Above 6 GHz |
| --- | --- | --- |
| Satellite HPA model | FFS | FFS |
| Phase noise model | FFS | FFS |
| Local oscillator drift | FFS | FFS |
| Max Doppler shift (Note 1) | Scenario A : see TR 38.811 Scenario C2/D2 : see TR 38.811 | Scenario A : see TR 38.811 Scenario C2/D2 : see TR 38.811 |
| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Max Doppler rate | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Note 1 : Min. Elevation angle for both sat-gateway and user equipment = 10 degree. |  |  |

Proposal : The following skeleton table is proposed for agreement on LLS parameters for DL synchronization performance evaluation. Companies are encouraged to provide values to fill the table for the next meeting.

Table X.10 : LLS parameters for DL synchronization evaluation

|  | FR1 - Below 6GHz | FR2 - Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| Subcarrier Spacing(s) | FFS | FFS |
| SNR range | FFS | FFS |
| Search window | FFS |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | FFS | FFS |
| Frequency Offset | UE: uniform distribution [+/- FFS ppm] BS: uniform distribution [+/- FFS ppm] Satellite : uniform distribution [+/- FFS ppm] (see Table X.7) Doppler in channel due to satellite movement: max. [+/- FFS] ppm (see Table X.7) Doppler in channel due to UE movement: max. [+/- FFS] ppm |  |
| Number of interfering beams | 0 as start point FFS interference from other beams, based on the frequency reuse pattern or constellation |  |
| Phase noise model | Table X.9 | Table X.9 |
| Satellite HPA model | Table X.9 | Table X.9 |
| Metrics | FFS |  |

## PRACH

### Assumptions

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Some simulation assumptions have been provided in Table 2 (R1-1905992) |
| Nokia | Proposal 13: Discussion on performance requirements for PRACH based on proposed assumptions and methodology. Proposal 14: Use link-level simulation assumptions in Table 4-6 (R1-1906085) for PRACH performance evaluation in NTN. Proposal 17: Use angle scaling procedure including subpaths in Section 4.4 (R1-1906085) for performance evaluation of PRACH. Consider following 3 probabilities for UE capability about reference frequency as simulation assumptions. - UE does not have a reference and synchronizes its frequency to observed DL SSB signal - UE has reference and reference is equal to DL reference - UE has reference but is different from DL reference For example, a UE without reference may be required to have GPS in order to estimate the Doppler frequency offset. On the other hand, a UE with its own reference may estimate the Doppler from DL synchronization signals like the PSS/SSS and thus may not need GPS. |
| ZTE | Proposal 8: Simulation assumption listed in Table 10 (R1-1906870) can be considered as the baseline for PRACH evaluation. |
| Samsung | Proposal 1: Use the following table table (R1-1906951) as a starting point for link-level simulation evaluation for synchronization performance. |
| Ericsson | Assumption 1: UE knows its position relative to the satellite with sufficient accuracy to perform coarse timing advance before uplink initial access. Assumption 2: The simulation focuses on the service link only. The feeder link impairments are ignored. |
| ESA | Proposal 5: As far as the satellite payload impairments are concerned, it is recommended to adopt Table 4 and Table 5 (R1-1907481) for downlink and uplink transmissions, respectively. |

| Companies | Proposal and observation |
| --- | --- |
| Nokia | Assuming a residual Doppler frequency offset after DL synchronisation is a good starting point. We should also consider the assumptions from the DL synchronisation evaluation assumptions. It will be good to consider DL synchronisation in the PRACH discussion (complete initial access), because PRACH performance may depend on Doppler pre/post compensation capabilities at satellite and/or UE. This additional effort is reasonable and can lead to more “trustful” results/conclusions on PRACH performances. |

### Metrics

| Companies | Proposal and observation |
| --- | --- |
| Huawei | PRACH detection rate FAR detection rate |
| Nokia | Proposal 15: Performance metrics for PRACH evaluation should include 1) the minimum required SNR to achieve 1% missed detection, and 2) observed missed detection probability as function of SNR with the SNR range to cover missed detection from at least 10-4 to around 1 at least. Proposal 16: False alarm probability as function of SNR and CDF of timing estimation error at minimum required SNR for 0.1 % missed detection rate can be used as optional performance metrics for PRACH evaluation. |
| ZTE | Miss detection rate |

### General agreement

Proposal : The following skeleton tables are proposed for agreement on LLS parameters for PRACH performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.11 : LLS parameters for PRACH performance evaluation

| Configurations | FR-1 | FR-2 |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| Antenna Configuration at the TRP (satellite) | 1Rx | 1Rx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | UE: value FFS, based on the residual frequency offset of DL synchronization BS : uniform distribution +/- 0.05 ppm |  |
| UE speed | FFS | FFS |
| Initial timing Offset (Note 1) | FFS |  |
| Phase noise model | Table X.9 | Table X.9 |
| PRACH design | FFS | FFS |
| Metric | FFS |  |
| Note1 : Ideal common delay compensation is assumed. |  |  |

## Data transmission / Throughput

### Assumptions

| Companies | Proposal and observation |
| --- | --- |
| Huawei | Some simulation assumptions have been provided in Table 3 |
| Fraunhofer IIS | Proposal 1: Link-level simulations considering satellite HPA non-linearity should model the ICI between multiple allocations in the same downlink beam, even under AWGN channel or ideal channel knowledge assumptions. Proposal 2: Input value range of the HPA AM/AM and AM/PM characteristics specified for NR NTN downlink simulations shall cover the high dynamic range of the OFDM waveforms: recommended range is from -30 dB to 10 dB. Proposal 3: Appropriate HPA non-linearity modeling should be included in the link- and system-level simulation assumptions for NR NTN downlink. |
| Ericsson | Proposal 12 RAN1 to discuss antenna configuration assumptions. Proposal 13 RAN1 to agree on the link level evaluation assumptions listed in Table 2. |
| ESA | Proposal 5: As far as the satellite payload impairments are concerned, it is recommended to adopt Table 4 and Table 5 for downlink and uplink transmissions, respectively. |

### Metrics

| Companies | Proposal and observation |
| --- | --- |
| Huawei | BLER or throughput |

### General agreement

Proposal : The following skeleton tables are proposed for agreement on LLS parameters for data transmission performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.12 : LLS parameters for data transmission performance evaluation

| Parameters | FR 1 | FR 2 |
| --- | --- | --- |
| Carrier frequency | 2GHz | DL 20GHz |
| Duplex Mode | FDD |  |
| Subcarrier spacing | 15kHz, 30kHz | 60kHz, 120kHz |
| CSI feedback | FFS |  |
| Modulation order, Coding rate | FFS |  |
| Channel coding scheme | NR channel coding |  |
| Link adaptation | FFS |  |
| HARQ | FFS |  |
| Channel estimation | Realistic estimation |  |
| Frequency offset model | Frequency offset in case of non-initial acquisition TRP: uniform distribution +/- FFS ppm UE: uniform distribution +/- FFS ppm |  |
| UE speed | FFS |  |
| Channel model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| TRP antenna configuration | 1Tx | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | Table X.9 | Table X.9 |
| Satellite HPA Model | Table X.9 | Table X.9 |
| Metrics | FFS |  |

# Discussion on Link budgets assumptions

## Additional contributions on link budgets

Additionally to the document submitted in agenda item 7.2.5.1, the following document submitted in agenda item 7.2.5.5 [13] [14] [15] address also link budget analysis :

R1-1906876

R1-1906090

R1-1905997

## Assumptions

| Companies | Proposal and observation |
| --- | --- |
| Nokia | Proposal 9: The values of elevation angles for link budget analysis can be 90, 30 and 10°. Proposal 10: The values of BLER targets for link budget analysis can be 10%, 0.1% and 0.001%. Proposal 11: Use AWGN model as baseline and select additional one link-level channel model in TR 38.811 for link budget analysis. Proposal 12: The simulation assumptions in Table 3-1, Table 3-2 and Table 3-3 can be considered for link budget analysis in NTN with achievable throughputs as output. |
| MediaTek | Proposal 1: Link parameters, Satellite parameters, and terminal parameters illustrated in example below are considered for evaluation of NR-NTN. |

## General agreements :

Proposal : Link budget methodology and analysis should be captured in TR 38.821

Proposal : Endorse section X.Y.1 from Nokia TP in R1-1906090 to capture link budget methodology in the TR.

Proposal : Endorse skeleton tables from section X.Y.2 from Nokia TP in R1-1906090 to capture the simulation assumptions for link budget analysis. Parameters values can be further discussed by the companies.

Table X.Y.2-1 Simulation assumptions for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| Bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Satellite altitude | 600km, 1200km, 35786km |
| SCS | 30kHz (S band), 120kHz (Ka band) |
| Elevation angle | 10, 30, 90 |
| Atmospheric loss | 0.1 dB (S band), 1 dB (Ka band) |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Average C/I within a satellite beam | 16 dB |
| Frequency reuse factor (Note 1) | 4 |
| Terminal type | - Ka band: VSAT, (M, N, P) = (4, 8, 2) - S band: (M, N, P) = (1,1,1), (M, N, P) = (1,1,2), and (M, N, P) = (1, 2, 2) |
| Terminal speed | 0 km/h |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Fast fading channel model | AWGN, NTN-CDL-D in TR 38.811 |
| Target BLER | 10%, 0.1%, 0.001% |
| Note 1: Frequency reuse factor of 1 is not precluded from simulation studies. |  |

Table X.Y.2-2 Terminal parameters

| Terminal Type | VSAT | (M, N, P) = (1,1,1) | (M, N, P) = (1,1,2) | (M, N, P) = (1, 2, 2) | (M, N, P) = (8, 4, 2) |
| --- | --- | --- | --- | --- | --- |
| Terminal receive antenna element gain (dBi) | 39.7 | 0 | 0 | 0 | 5 |
| Terminal noise figure (dB) | 1.2 | 7 | 7 | 7 | 10 |
| Terminal ambient temperature (K) | 290 | 290 | 290 | 290 | 290 |
| Terminal antenna temperature (K) | 150 | 290 | 290 | 290 | 290 |
| Terminal maximal EIRP (dBW) | 45.75 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal EIRP (dBW) | 44 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal output back off (dB) | 1.75 | 0 | 0 | 0 | 0 |
| Terminal transmit power (dBW) | 3 | -7 | -7 | -7 | -7 |
| Terminal transmit antenna element gain (dBi) | 43.2 | 0 | 0 | 0 | 5 |
| Terminal cable loss (dB) | 0.45 | 0 | 0 | 0 | 0 |

Table X.Y.2-3 Satellite parameters

|  |  | Configuration 1 |  |  | Configuration 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 |
| S band | Satellite EIRP density (dBW/MHz) | 59 | 40 | 34 |  |  |  |
|  | Satellite G/T (dB/K) | 19 | 1.2 | 1.2 |  |  |  |
| Ka band | Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |  |  |  |
|  | Satellite G/T (dB/K) | 22 | 13 | 13 |  |  |  |

Proposal : Endorse skeleton tables from section X.Y.3 from Nokia TP in R1-1906090 to capture the link budget analysis.

|  |  |  |  | Ka band |  |  |  |  |  | S band |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE type |  |  |  | VSAT |  |  | (M, N, P) = (8, 4, 2) |  |  | (M, N, P) = (1,1,1) |  |  | (M, N, P) = (1,1,2) |  |  | (M, N, P) = (1,2,2) |  |  |
| Elevation angle () |  |  |  | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 |
| Satellite configuration 1 | DL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite configuration 2 | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

Proposal #1: Both multi-satellite and single satellite simulations should be considered for calibration and performance evaluation.

Proposal #2: The following tables are proposed for agreement on SLS assumptions for calibration :

- Table X.1 and X.2 define the 2 sets of satellite parameters to be considered as the baseline for SLS.

- Table X.3 defines UE characteristics to be considered as the baseline for SLS.

- Table X.4 summarizes the beam layout definition principles to be considered as the baseline for single satellite SLS.

- Table X.5 summarizes the simulation assumptions to be considered for calibration.

Table X.1 : Set-1 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table X.2 : Set-2 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 12 m | 1 m | 1 m |
| Satellite EIRP density |  | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 12 m | 1 m | 1 m |
| G/T |  | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 2 m | 0.2 m | 0.2 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Table X.3 : UE characteristics for System Level Simulations

| Characteristics | VSAT | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 20 GHz UL and 30 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 20 GHz UL and 30 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (4,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (3dB-BW=65 deg) |
| Polarisation | circular | Linear : +/-45°X-pol | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | 8dBi per element |
| Antenna temperature | 150 K | 290 K | 290 K |
| Noise figure | 1.2 dB | 7 dB | 9 dB |
| Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. |  |  |  |

Table X.4 : Beam layout definition for single satellite simulation

| Scenario | Scenario A, C2 and D2 |
| --- | --- |
| Beam layout definition | Baseline : Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame |
| Number of beams | For single satellite simulation : Baseline: 19-beam layout considering wrap-around mechanism (i.e. 18 beams surrounding the central beam and allocated on 2 distinct “tiers”) |
| UV plane illustration (extracted from [16]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after : The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern : ABS = sqrt(3) x sin(3dB-BW [rad]) Note : 3dB-BW is the positive numerical solution for which the antenna gain pattern is equal to 0.5 based on the formula defined in section 6.4.1 of TR 38.811 |
| Central beam bore sight direction definition | Baseline : - Case 1 : Central beam center is considered at nadir point - Case 2 : Central beam boresight direction computed based on elevation angle target |

Table X.5 : SLS assumptions for calibration

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) / Ka- Band (i.e. 20 GHz DL, 30 GHz UL) |
| Maximum Bandwidth per beam (DL + UL) | S-band : DL 30 MHz and UL 30 MHz Ka-band : DL 400 MHz and UL 400 MHz The bandwidth per beam must be adapted based on the frequency factor considered. |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table X.1 and Table X.2 (optional) Note : Same satellite characteristics should be considered for both single and multi-satellite simulations |
| Satellite antenna pattern | TR 38.811v15.0.0 section 6.4.1 Bessel function |
| Satellite polarization configuration | Circular |
| Beam layout definition | For singles satellite simulation : See Table X.4 For multi satellites simulation : FFS |
| Frequency re-use factor | Base-line : - S band : Frequency re-use factor = 4 for linear polarization ; - Ka band: Frequency re-use factor = 4 for linear polarization and 2 for dual circular polarization ; Additional results with other frequency re-use factor assumptions can be provided |
| Deployment scenarios | Base-line : Rural Additional deployment scenario results can be provided |
| Fast fading model | Frequency selective channel model from TR 38.811 |
| Propagation conditions | Base-line : Clear Sky |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | Base-line : X=10 UEs per beam with uniform distribution in all the beams |
| UE configuration | S-band : - Handheld (optional for scenario A) Ka-band : - VSAT - Others (optional for scenario A) See Table X.3 |
| UE orientation | VSAT and Others : Ideal Tracking serving beam; Handheld: Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base-line : Coupling loss, Geometry |
| Note 1 : Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. |  |

Proposal #3 : The beam layout parameters captured in the following table should be adopted as a starting point for single satellite simulation.

Table X.6 : Beam layout parameters for single satellite simulation

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band : 2 GHz Ka-band : 20 GHz for DL | S-band : 2 GHz Ka-band : 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band : ABS = 0.0061 Ka-band : ABS = 0.0027 | S-band : ABS = 0.0668 Ka-band : ABS = 0.0267 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline : - Case 1 : 90 degree - Case 2 : 45 degree | Baseline : - Case 1 : 90 degree - Case 2 : FFS |
| Central beam bore sight direction coordinates in UV plane | Baseline : - Case 1 : (0,0) - Case 1 : (0.107,0) | Baseline : - Case 2 : (0,0) - Case 2 : FFS |
| Gateway direction coordinates in UV plane | Baseline : Same as central beam bore sight direction coordinates in UV plane |  |

Proposal #4 : The wrap around mechanism should be considered as a baseline for single satellite simulation for intra-satellite interference modeling based on additional bore-sight beam directions which should be computed based on the methodology captured in Table X.4.

Proposal #5 : The value of amplitude index S_4 used in ionospheric scintillation loss model should be given in the simulation assumptions if the UE latitude is comprised within ±20° and the carrier frequency is below 6GHz.

Proposal #6: The following skeleton tables are proposed for agreement on SLS parameters for performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.7 : SLS parameters for performance evaluation

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | Same as in Table X.5 |
| Maximum Bandwidth per beam (DL + UL) | Same as in Table X.5 |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | Same as in Table X.5 |
| Satellite antenna pattern | Same as in Table X.5 |
| Satellite polarization configuration | Same as in Table X.5 |
| Beam layout | FFS |
| Number of beams | FFS |
| Frequency re-use factor | FFS |
| Deployment scenarios | Same as in Table X.5 |
| Fast fading model | FFS |
| Propagation conditions | Same as in Table X.5 |
| UEs outdoor/indoor distribution | Same as in Table X.5 |
| UEs coverage distribution | Same as in Table X.5 |
| UE configuration | Same as in Table X.5 |
| UE orientation | Same as in Table X.5 |
| Handover Margin | FFS |
| UE attachment | FFS |
| Receiver type | FFS |
| Reference signals configuration | See Table X.8 |
| Scheduler | FFS |
| Traffic model | FFS |
| Channel estimation | FFS |
| MIMO mode | FFS |
| CSI feedback | FFS |
| Metrics for performance | Baseline: UE throughput (5%, 50%, 95%) |
| Note 1 : Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. |  |

Table X.8 : Reference signals configuration for performance evaluation

| DL | PDCCH | FFS |
| --- | --- | --- |
|  | DMRS | FFS |
|  | CSI-RS | FFS |
|  | SSB | FFS |
|  | TRS | FFS |
|  | PTRS | FFS |
| UL | PUCCH | FFS |
|  | DMRS | FFS |
|  | SRS | FFS |
|  | PTRS | FFS |

Proposal #7 : The following skeleton table is proposed for agreement to capture the impairments introduced on the RF signal due to the satellite payload and movement. Companies are encouraged to provide values to fill the table for the next meeting.

Table X.9 : Impairments due to satellite payload and satellite movement

|  | FR1 – Below 6 GHz | FR2 – Above 6 GHz |
| --- | --- | --- |
| Satellite HPA model | FFS | FFS |
| Phase noise model | FFS | FFS |
| Local oscillator drift | FFS | FFS |
| Max Doppler shift (Note 1) | Scenario A : see TR 38.811 Scenario C2/D2 : see TR 38.811 | Scenario A : see TR 38.811 Scenario C2/D2 : see TR 38.811 |
| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Max Doppler rate | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Note 1 : Min. Elevation angle for both sat-gateway and user equipment = 10 degree. |  |  |

Proposal #8 : The following skeleton table is proposed for agreement on LLS parameters for DL synchronization performance evaluation. Companies are encouraged to provide values to fill the table for the next meeting.

Table X.9 : Impairments due to satellite payload and satellite movement

|  | FR1 – Below 6 GHz | FR2 – Above 6 GHz |
| --- | --- | --- |
| Satellite HPA model | FFS | FFS |
| Phase noise model | FFS | FFS |
| Local oscillator drift | FFS | FFS |
| Max Doppler shift (Note 1) | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Max Doppler rate | Scenario A : FFS Scenario C2/D2 : FFS | Scenario A : FFS Scenario C2/D2 : FFS |
| Note 1 : Min. Elevation angle for both sat-gateway and user equipment = 10 degree. |  |  |

Table X.10 : LLS parameters for DL synchronization evaluation

|  | FR1 - Below 6GHz | FR2 - Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| Subcarrier Spacing(s) | FFS | FFS |
| SNR range | FFS | FFS |
| Search window | FFS |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | FFS | FFS |
| Frequency Offset | UE: uniform distribution [+/- FFS ppm] BS: uniform distribution [+/- FFS ppm] Satellite : uniform distribution [+/- FFS ppm] (see Table X.7) Doppler in channel due to satellite movement: max. [+/- FFS] ppm (see Table X.7) Doppler in channel due to UE movement: max. [+/- FFS] ppm |  |
| Number of interfering beams | 0 as start point FFS interference from other beams, based on the frequency reuse pattern or constellation |  |
| Phase noise model | Table X.9 | Table X.9 |
| Satellite HPA model | Table X.9 | Table X.9 |
| Metrics | FFS |  |

Proposal #9 : The following skeleton tables are proposed for agreement on LLS parameters for PRACH performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.11 : LLS parameters for PRACH performance evaluation

| Configurations | FR-1 | FR-2 |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| Antenna Configuration at the TRP (satellite) | 1Rx | 1Rx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | UE: value FFS, based on the residual frequency offset of DL synchronization BS : uniform distribution +/- 0.05 ppm |  |
| UE speed | FFS | FFS |
| Initial timing Offset (Note 1) | FFS |  |
| Phase noise model | Table X.9 | Table X.9 |
| PRACH design | FFS | FFS |
| Metric | FFS |  |
| Note1 : Ideal common delay compensation is assumed. |  |  |

Proposal #10 : The following skeleton tables are proposed for agreement on LLS parameters for data transmission performance evaluation. Companies are encouraged to provide values to fill the tables for the next meeting.

Table X.12 : LLS parameters for data transmission performance evaluation

| Parameters | FR 1 | FR 2 |
| --- | --- | --- |
| Carrier frequency | 2GHz | DL 20GHz |
| Duplex Mode | FDD |  |
| Subcarrier spacing | 15kHz, 30kHz | 60kHz, 120kHz |
| CSI feedback | FFS |  |
| Modulation order, Coding rate | FFS |  |
| Channel coding scheme | NR channel coding |  |
| Link adaptation | FFS |  |
| HARQ | FFS |  |
| Channel estimation | Realistic estimation |  |
| Frequency offset model | Frequency offset in case of non-initial acquisition TRP: uniform distribution +/- FFS ppm UE: uniform distribution +/- FFS ppm |  |
| UE speed | FFS |  |
| Channel model | FFS |  |
| Delay scaling | FFS | FFS |
| Channel Angle Spread Scaling | FFS | FFS |
| Channel Mean Angle Scaling | FFS | FFS |
| Elevation Angle Scaling | FFS | FFS |
| TRP antenna configuration | 1Tx | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | Table X.9 | Table X.9 |
| Satellite HPA Model | Table X.9 | Table X.9 |
| Metrics | FFS |  |

Proposal #11 : Link budget methodology and analysis should be captured in TR 38.821

Proposal #12: Endorse section X.Y.1 from Nokia TP in R1-1906090 to capture link budget methodology in the TR.

Proposal #13: Endorse skeleton tables from section X.Y.2 from Nokia TP in R1-1906090 to capture the simulation assumptions for link budget analysis. Parameters values can be further discussed by the companies.

Table X.Y.2-1 Simulation assumptions for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| Bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Satellite altitude | 600km, 1200km, 35786km |
| SCS | 30kHz (S band), 120kHz (Ka band) |
| Elevation angle | 10, 30, 90 |
| Atmospheric loss | 0.1 dB (S band), 1 dB (Ka band) |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Average C/I within a satellite beam | 16 dB |
| Frequency reuse factor (Note 1) | 4 |
| Terminal type | - Ka band: VSAT, (M, N, P) = (4, 8, 2) - S band: (M, N, P) = (1,1,1), (M, N, P) = (1,1,2), and (M, N, P) = (1, 2, 2) |
| Terminal speed | 0 km/h |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Fast fading channel model | AWGN, NTN-CDL-D in TR 38.811 |
| Target BLER | 10%, 0.1%, 0.001% |
| Note 1: Frequency reuse factor of 1 is not precluded from simulation studies. |  |

Table X.Y.2-2 Terminal parameters

| Terminal Type | VSAT | (M, N, P) = (1,1,1) | (M, N, P) = (1,1,2) | (M, N, P) = (1, 2, 2) | (M, N, P) = (8, 4, 2) |
| --- | --- | --- | --- | --- | --- |
| Terminal receive antenna element gain (dBi) | 39.7 | 0 | 0 | 0 | 5 |
| Terminal noise figure (dB) | 1.2 | 7 | 7 | 7 | 10 |
| Terminal ambient temperature (K) | 290 | 290 | 290 | 290 | 290 |
| Terminal antenna temperature (K) | 150 | 290 | 290 | 290 | 290 |
| Terminal maximal EIRP (dBW) | 45.75 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal EIRP (dBW) | 44 | -10 | -3.99 | -0.98 | 13.05 |
| Terminal output back off (dB) | 1.75 | 0 | 0 | 0 | 0 |
| Terminal transmit power (dBW) | 3 | -7 | -7 | -7 | -7 |
| Terminal transmit antenna element gain (dBi) | 43.2 | 0 | 0 | 0 | 5 |
| Terminal cable loss (dB) | 0.45 | 0 | 0 | 0 | 0 |

Table X.Y.2-3 Satellite parameters

|  |  | Configuration 1 |  |  | Configuration 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GEO | LEO-1200 | LEO-600 | GEO | LEO-1200 | LEO-600 |
| S band | Satellite EIRP density (dBW/MHz) | 59 | 40 | 34 |  |  |  |
|  | Satellite G/T (dB/K) | 19 | 11.2 | 11.2 |  |  |  |
| Ka band | Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |  |  |  |
|  | Satellite G/T (dB/K) | 22 | 13 | 13 |  |  |  |

Proposal #14 : Endorse skeleton tables from section X.Y.3 from Nokia TP in R1-1906090 to capture the link budget analysis.

|  |  |  |  | Ka band |  |  |  |  |  | S band |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE type |  |  |  | VSAT |  |  | (M, N, P) = (8, 4, 2) |  |  | (M, N, P) = (1,1,1) |  |  | (M, N, P) = (1,1,2) |  |  | (M, N, P) = (1,2,2) |  |  |
| Elevation angle () |  |  |  | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 | 90 | 30 | 10 |
| Satellite configuration 1 | DL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite configuration 2 | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | UL | CNIR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | AWGN | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 10% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.1% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NTN-CDL-D, BLER = 0.001% | Throughput (Mbps) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Reference

[1] R1-1907087, On beam layout definition for NTN system level simulations, Thales
[2] R1-1907687, On beam layout definition for NTN system level simulations, Thales
[3] R1-1906463, Link budget and Doppler Aspects for evaluation of NR NTN, MediaTek Inc.
[4] R1-1906870, Discussion on the simulation assumption for NTN, ZTE
[5] R1-1906323, Simulation assumption for NTN evaluation, CATT
[6] R1-1906802, Considerations on performance evaluation for NTN, Intel Corporation
[7] R1-1906085, Considerations on Performance Evaluation in NTN, Nokia
[8] R1-1905992, Discussion on performance evaluation for NTN, Huawei
[9] R1-1906951, Remaining issues on NTN link-level and system-level simulation assumptions, Samsung
[10] R1-1907369, On Dowlink Performance in NTN, Fraunhofer IIS
[11] R1-1907481, Beam size computation and alternative satellite specifications, ESA
[12] R1-1907395, On NTN link level and system level evaluations, Ericsson
[13] R1-1906090, TP for Link Budget Analysis in TR 38.821, Nokia
[14] R1-1905997, Discussion on link budget for NTN, Huawei
[15] R1-1906876, Preliminary link budget for NTN, ZTE
[16] Radar System Engineering, Lecture 9, Antennas, Robert M.O’Donnel, IEEE New Hampshire