Three options are given below, if there is no consensus on selecting one alternative from option A then option B can be considered, if there is not consensus on option B then option C should be default.

Option A: down select one from the following alternatives below:

- Alt1-1: Option1-1 (Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs, e.g. for 2Tx the new codeboookSubset is all non-antenna selection TPMIs or with only TPMI [1 1] for rank 1)

  - Small delay CDD, if applied, is transparent to RAN1 specification

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable, includes the TPMI precoders in codebookSubset = fullyAndPartialAndNonCoherent defined in Rel-15

    - Only a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

Supported by: Nokia/NSB, AT&T, Samsung, InterDigital, LGE, MediaTek, Sreadtrum, CATT, OPPO, DOCOMO, Qualcomm

- Alt1-2: Option1-1 (Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs, e.g. for 2Tx the new codeboookSubset is all non-antenna selection TPMIs or with only TPMI [1 1] for rank 1)

  - Small delay CDD, if applied, is transparent to RAN1 specification

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable, includes the TPMI precoders in codebookSubset = fullyAndPartialAndNonCoherent defined in Rel-15

    - All of the non-antenna selection TPMI precoders are supported

    - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

Supported by: Qualcomm, AT&T, InterDigital, LGE, MediaTek, Spreadtrum, OPPO

- Alt3-1-1: Option3+Option2 (Multiple SRS resources with different number of SRS port(s) in each resource)

  - FFS: Whether to additionally support Option 1-2

  - UE is configured with multiple SRS resources in a set each with different number of SRS ports, e.g. 1 SRS source is configured with 1 port and another SRS resource is configured with 2 ports for the UE with 2 Tx chains.

    - UE virtualizes Tx chains when configured with an SRS resource that has fewer ports than the number of Tx chains, e.g. 2 Tx chains are virtualized to transmit 1 port SRS resource

      - Small delay CDD or other virtualization mechanism is applied in spec transparent manner

    - UE may or may does not virtualize Tx chains when configured with an SRS resource with number of ports equal to the number of Tx chains, e.g. 2 Tx chains transmit 2 ports SRS resource.

Supported by: Ericsson, Intel, CATT, Huawei, HiSilicon (with the modification, since virtualization is UE implementation, which cannot force UE to do)

- Alt3-1-2: Option3+Option2 (Multiple SRS resources with different number of SRS port(s) in each resource)

    - FFS: whether to additionally support Option 1-2

  - UE is configured with multiple SRS resources in a set each with different number of SRS ports, e.g. 1 SRS source is configured with 1 port and another SRS resource is configured with 2 ports for the UE with 2 Tx chains.

    - UE virtualizes Tx chains when configured with an SRS resource that has fewer ports than the number of Tx chains, e.g. 2 Tx chains are virtualized to transmit 1 port SRS  resource

    - UE may or may not virtualize Tx chains when configured with an SRS resource with number of ports equal to the number of Tx chains, e.g. 2 Tx chains transmit 2 ports SRS resource.

    - Option 1-2 is used for updating the normalization factor for full power transmission, where normalized power should be 1 for the full power transmission.

Supported by: Huawei/HiSilicon

- Alt3-1-3: Option3+Option2 (Multiple SRS resources with different number of SRS port(s) in each resource)

  - UE is configured with multiple SRS resources with different number of SRS ports, e.g. 1 SRS source is configured with 1 port and another SRS resource is configured with 2 ports for the UE with 2 Tx chains.

    - UE may virtualizes Tx chains when configured with an SRS resource that has fewer ports than the number of Tx chains, e.g. 2 Tx chains are virtualized to transmit 1 port SRS resource

      - Small delay CDD or other virtualization mechanism is applied in spec transparent manner

      - FFS: partial or whole TPMI(s) support full Tx power transmission in such case.

Supported by: ZTE

- Alt3-2-1: Option3+Option2+ Option1-1 (Multiple SRS resources with different number of SRS port(s) in each resource)

  - UE can be configured with one SRS resource (without virtualization)

    - When configured with multiple SRS resources with different number of ports, Alt 3-1 is used instead

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable, includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - Only a subset of the non-antenna selection TPMI precoder(s) is(are) supported

  - A different SRS port is transmitted on each Tx chain (small delay CDD maybe transparently applied)

  - gNB can coherently combine channel estimates from SRS according to TPMI when determining CQI/TPMI/RI

Supported by: Ericsson, DOCOMO

- Alt3-2-2: Option3+Option2+ Option1-1 (Multiple SRS resources with different number of SRS port(s) in each resource)

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

    - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable in Rel-15

      - Alt1: Only a subset of the non-antenna selection TPMI precoder(s) in codebookSubset = partialAndNonCoherent and fullyAndPartialAndNonCoherent defined in Rel-15 is(are) supported

      - Alt2: All of the non-antenna selection TPMI precoders in codebookSubset = partialAndNonCoherent and fullyAndPartialAndNonCoherent defined in Rel-15 are supported

      - Alt.3: Introduce the precoders with normalization factor is 1, such as [1 0] or [0 1]

Supported by: Huawei/HiSilicon

- Alt5: FDM multi-port simultaneous transmission

  - The scheduled RBs are divided into several RB sets, each of which is associated with a respective antenna port or antenna port set (ports of which should be coherent)

    - Schedule RBs can be contiguous or non-contiguous

    - E.g., four PRBs are allocated to the UE for one layer PUSCH transmission. UE only use antenna port 0 for PUSCH transmission in PRB#0 and PRB#1, while only use antenna port 1 for PUSCH transmission in PRB#2 and PRB#3. It also could be considered as that precoding matrix [1, 0] is applied for PRB#0 and PRB#1 while precoding matrix [0, 1] is applied for PRB#2 and PRB#3.

  - This is applicable to both CP-OFDM and DFT-s-OFDM

    - For DFT-s-OFDM, one DFT is applied to one RB set

  - UL full power Tx is considered for rank 1 only.

  - This does not have spec impact on PRB bundling

  - The minimal size of a RB set is 1 RB

  - Note: this scheme is mainly targeting UL 2Tx case, it can also be extended for 4Tx

Supported by: CMCC, ZTE, CATT

Option B: as a compromise, support following scheme for UL full power Tx for UE capability 2:

  - When UE is configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within a SRS resource set which usage is set to ‘codebook’

    - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

    - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable, includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

      - Only a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

  - When UE is configured with more than one SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

    - UE transmits SRS and PUSCH in same manner

    - UL full power Tx is achieved for PUSCH transmission according to indicated SRI, e.g. SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted in same manner as single port SRS transmitted, if SRI indicating SRS resource with 2 ports then two layers PUSCH is transmitted in same manner as two ports SRS transmitted

| Company | views |
| --- | --- |
| Huawei, HiSilicon | - When UE is configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within a SRS resource set which usage is set to ‘codebook’ - gNB can configure the UE to use a subset of TPMIs that to produce full power transmission. - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable, includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15 - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI - When UE is configured with more than one SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’ - UE transmits SRS and PUSCH in same manner - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and TPMI, e.g. SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted in same manner as single port SRS transmitted, if SRI indicating SRS resource with 2 ports and TPMI [1 0] or [1 0; 0 1], then one or two layers PUSCH is transmitted in same manner as two ports SRS transmitted; if SRI indicate 4 ports SRS resource and TPMI [1 0 0 0; 0 1 0 0] or [1 0 0 0; 0 1 0 0; 0 0 0 1] or [1 0 0 0; 0 1 0 0; 0 0 1 0; 0 0 0 1]. |
| Intel | - When UE is configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within a SRS resource set which usage is set to ‘codebook’ - The two sentences below in Rel-15 is NOT applicable to capability2 UE for this case - “A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent'.” - “A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent'.” |
| Samsung | - When UE is configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within a SRS resource set which usage is set to ‘codebook’ - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission. - Note: here the combined ports belong to non-coherent port groups - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable, includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15 - Only a subset of the non-antenna selection TPMI precoder(s) is(are) supported - FFS: whether the “new” included TPMI(s) replace existing TPMI(s) or are added to the codebook - FFS: whether existing 3 states are used or new state(s) are added to the RRC parameter CodebookSubset - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI - When UE is configured with more than one SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’ - UE transmits SRS and PUSCH in same manner - UL full power Tx is achieved for PUSCH transmission according to indicated SRI, e.g. SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted in same manner as single port SRS transmitted, if SRI indicating SRS resource with 2 ports then two layers PUSCH is transmitted in same manner as two ports SRS transmitted |

Option C: further specification over what is supported in Rel-15 is not needed.

Note: since a CR was agreed in RAN1#96bis, the gNB can configure single port SRS or multi ports SRS by RRC, when single port SRS is configured and PUSCH is scheduled by DCI 0_1, UE doesn’t scale the power, that means full power Tx is already achieved in Rel-15.