3GPP TSG RAN WG1 Meeting #97													 		R1- 1907700

Reno, NV, USA, May 13th – 17th, 2019

Source:	Ericsson

Title:	FL summary for 7.2.10.2 UL Reference Signals for NR Positioning

Agenda item:	7.2.10.2

Document for:	Discussion and decision

Introduction

UL SRS design for positioning

SRS usage

the following proposals relate to SRS usage :

| [2] | Proposal 1: A SRS positioning usage is defined. |
| --- | --- |
| [3] | Proposal 6: SRS resource set with usage of ‘positioning’ is supported. |
| [4] | Proposal 1: Rel-16 SRS with possible enhancements for positioning can be used for multiple purposes. There is no need to define a new usage of SRS specifically for positioning. |
| [5] | Proposal 1: Introduce a new usage, i.e. ‘positioning’ per SRS resource set. |
| [7] | Proposal 7: Define a new usage of Rel-16 SRS for positioning purpose. |
| [8] | Proposal 1: An additional usage of SRS resource set needs to be defined to apply SRS enhancements for NR positioning. |
| [12] | Proposal 9: Define UEs categories according to the capabilities related to SRS-P configurations Category A: UE supporting SRS configuration according to Rel. 15 Category B: UE also supporting the enhanced SRS configuration for positioning use case |
| [13] | Proposal 4: Support a new set usage for SRS, “positioning” |
| ericsson | Proposal 12 Define a SRS usage for positioning where o the network can assume the antenna port in each resource in a resource set to be the same in all resources. o The number of SRS symbols in a slot is equal to the number of symbols in the slot o FFS other properties |
| MTK | We may consider this as the enhancement of “beamManagement”. So no need to define a new usage |

Based on the proposals, there is a majority consensus on introducing a new usage for the SRS, with positioning as an intended purpose.

- Proposal for offline consensus: a new SRS usage named “positioning” is defined.

  - FFS Scope of the usage  / impact on procedures

SRS pattern for positioning

During RAN1#96b, the following agreement was reached regarding the construction of the SRS pattern for positioning:

Agreement: Select one or both of the following options to support staggered SRS transmissions for UL SRS positioning - Staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource - Configuration of the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set.

The following proposals captured the companies opinion on the issue:

| [2] | Proposal 5: staggered SRS transmissions for UL SRS positioning is done by configuring the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set. No staggered SRS is defined in Rel-16. |
| --- | --- |
| [3] | Proposal 2: Support SRS staggered patterns with inter-symbol comb offset among SRS symbols in a single SRS resource. |
| [4] | Proposal 3: Support one of the following options of staggered SRS transmissions for positioning Define the SRS staggered pattern, i.e., the different offsets for different SRS symbols, of an SRS resource in the physical layer specification Support configuring the SRS staggered pattern, i.e., the different offsets for different SRS symbols, of an SRS resource through high-layer signaling |
| [5] | Proposal 2: Support Staggered patterns in a single SRS resource. |
| [7] | Proposal 4: We prefer option-1 of staggered patterns in a single SRS resource. |
| [8] | Proposal 2: NR positioning supports the staggered RE patterns for each SRS resource while reusing the current comb-offset configuration parameter as much as possible. |
| [11] | Proposal 3: Support configuration of the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set. |
| [12] | Proposal 8: Allow staggered pattern to be configured as a single SRS Resource within SRS-Config IE by adding parameter to allow staggered repetition. |
| Mistubishi | Proposal 3: Support staggered patterns (a collection of SRS symbols from the same antenna port with different vertical frequency offsets fro at least some symbols) in a single SRS resource |
| [15] | Proposal 1: Regarding the support of staggered SRS transmissions for UL SRS positioning, support the option with staggered patterns in a single SRS resource. |
| ericsson | Proposal 3 The SRS pattern based on comb-N contains M symbols where M need not be equal to N |
| ericsson | Proposal 4 For positioning, the network should assume the antenna port in each resource in a resource set to be the same in all resources. |
| ericsson | Proposal 5 The SRS pattern for positioning used for a given measurement is realized with one or more SRS resource set. |
| MTK | Support symmetric RS structure design between DL and UL except the sequence |

The majority of the proposals support the configuration of a staggered pattern in an SRS resource.

- Proposal for offline consensus: the SRS pattern for positioning is realised within an SRS resource.

  - FFS: construction of the pattern inside the SRS resource structure

SRS resource for positioning

Number of symbols per resource

| [2] | Proposal 2: For positioning purposes, the number of SRS symbols per resource is the same as in Rel. 15 NR. |
| --- | --- |
| [3] | Proposal 3: SRS repetition on is supported. |
| [3] | Proposal 5: SRS with 8 symbols should be additionally supported for positioning. |
| [4] | Proposal 2: Consider supporting more than 4 SRS symbols in an SRS resource to achieve positioning performance improvement. |
| [5] | Proposal 3: Support M>4 symbols in one SRS resource for positioning. M can be {1, 2, 4, 6, 8} |
| [7] | Proposal 2: Extend the number SRS symbols from {1,2,4} symbols to at least {1,2,4,6} symbols for orthogonal UE multiplexing capacity. |
| [9] | For the purpose of NR positioning, increase number of symbols for UL SRS (PRS) transmission within single UL SRS (PRS) Resource to 8 and 14. |
| [11] | Proposal 5: The number of SRS symbols per resource should not be increased beyond the NR Rel-15 allowable set {1,2,4} and the SRS location should be configured only in the last 6 symbols as in NR Rel-15. |
| [12] | Proposal 6: Support SRS symbols per resource NsymbSRS to be selected from the following set NsymbSRS {1, 2, 4, 6, 12} with staggering. |
| ITRI | Proposal 1: For the purpose of NR positioning, allow flexible configuration of symbols number for UL SRS (PRS) transmission within single UL SRS (PRS) Resource. |
| [15] | Proposal 2: For positioning, regarding time-domain SRS enhancements: Increase the number of SRS symbols per SRS resource to [14]. SRS can be configured in the last N symbols of a slot where N = [14]. |
| ericsson | Proposal 6 Allow an increase of the number of SRS symbol per resource, up to at least 6 symbols |
| MTK | We consider the symbol number: 2, 3, 4, 6, 12, 14 for comb-2, comb-2, comb-4, comb-4, comb-6 and comb-8, respectively |

Based on the proposals in the contribution, only 2 proposals did not support extending the number of symbols per resources, and the majority supports a larger number of symbols per resources. the following proposal for offline consensus is given:

- Proposal for offline consensus: For positioning, the number of consecutive OFDM symbols in an SRS resource  is configurable with one of the values in the set {1,2,4,6,8,12,14}

Number of  symbols per slot

| [2] | Proposal 3: For positioning purposes, the possible SRS symbol locations per slot are the same as in Rel. 15 NR. |
| --- | --- |
| [3] | Proposal 1: SRS resource can start on all symbols within a slot for positioning purpose. |
| [4] | Proposal 4: Configure SRS only in the last N symbols of a slot with N<=8. |
| [5] | Proposal 4: Support N > 6 SRS symbols in one slot for positioning. N can be {1, 2, 4, 6, 8, 10, 12} |
| [6] | Proposal 2-3: In DL, the multiple transmission occasions within a slot is proposed. Then in UL, we also consider that SRS can be transmitted in earlier symbols than the last 6 symbols in a slot We consider symbol number per slot as 2, 3, 4, 6, 12, 14 |
| [7] | Proposal 3: Extend Rel-15 SRS symbol allocations from the last 6-symbols to the last 12 symbols. |
| [9] | UL SRS (PRS) resource can be configured to occupy any symbols within a slot |
| [12] | Proposal 5: Increase the region where SRS can be transmitted from last 6 OFDM symbols in a slot to the last 12 OFDM symbols or all OFDM symbols in a slot for the positioning use case. |
| [13] | Proposal 1: Up to N=14 NR SRS symbol is used for positioning |
| [15] | Proposal 2: For positioning, regarding time-domain SRS enhancements: Increase the number of SRS symbols per SRS resource to [14]. SRS can be configured in the last N symbols of a slot where N = [14]. |
| [16] | Proposal 11 For positioning purpose, extend the number of possible UL SRS in a slot to the number of symbols in the slot. |

Based of the contribution proposals, there is majority support with supporting sending SRS starting from anywhere in the slot. The following offline proposals is given:

- Proposal for offline consensus: For positioning, the the starting position in the time domain for the SRS resource may be anywhere in the slot, i.e. the offset loffset  range is {0,1,…,14}

Comb size

| [2] | Proposal 4: For positioning purposes, the supported SRS transmission comb values are the same as Rel. 15 NR. |
| --- | --- |
| [3] | Proposal 4: SRS comb-8 structure should be additionally supported for positioning. |
| [5] | Proposal 5: Support comb 2, 4, 6 for SRS. |
| [7] | Proposal 1: Consider comb-6 SRS structure additionally for UL positioning. |
| [9] | For the purpose of NR positioning, additionally support comb-6 and comb-12 structures for UL SRS(PRS) configuration |
| [12] | Proposal 7: Support transmission comb size (KTC) to be selected from the following set {1, 2, 4, 6, 12} with and without staggering. |
| [16] | Proposal 7 the available comb size for SRS in positioning is extended with size 6 o FFS higher comb size |
| MTK | In above, we consider the symbol number: 2, 3, 4, 6, 12, 14 for comb-2, comb-2, comb-4, comb-4, comb-6 and comb-8, respectively So we consider comb-2, comb-4, comb-6 and comb-8 |

Based on the contributions proposals, there is a majority support for extending the number of supported comb sizes. Comb-6 seem to have the majority support, with comb-12 mentioned twice and comb 8 once.

- Proposal for offline consensus: For positioning, the SRS comb size set is extended from {2,4} to {2,4,6}

  - FFS additional comb sizes

Sequence generation

| [2] | Proposal 6: Increase the bit width of the parameter sequenceId of the SRS resource configuration to [16] bits. |
| --- | --- |
| [3] | Proposal 7: In addition to ZC sequence, support to reuse pi/2- BPSK and CGS sequences agreed in Rel-16 MIMO for SRS sequence generation for positioning. |
| [4] | Proposal 5: Reuse NR Rel-15 SRS sequence for positioning. |
| [6] | Proposal 3-2: The backward compatibility should also be considered seriously for potential development of pi/2 BPSK based SRS |
| [7] | Proposal 5: First study UE multiplexing capacity in orthogonal manners through comb and symbol structures. If finding issues regarding lack of UE capacity, then consider modifying the number of code sequences ID. |

There is no consensus among the presented proposal, however, tracks for discussion may be set. Is is thus proposed to discuss the following:

Discuss the bit width of the sequence ID of the SRS resource configuration

Discuss the use of non ZC sequences for SRS in positioning

SRS orthogonality

| Intel | Further consider to introduce time domain code multiplexing for UL SRS (PRS) in order to increase the UE multiplexing capacity for NR positioning in UL |
| --- | --- |
| [12] | Proposal 4: Users transmitting SRS for positioning use case should be configured to transmit on a different slot (‘Uplink positioning slot’) than the standard Rel. 15 SRS use cases, in order to mitigate interference due to higher transmit power. |
| [13] | Proposal 2: A slot or multiple slots dedicated for positioning are supported for NR positioning |
| [16] | Proposal 8 The possible values for the maximum number of cyclic shifts for SRS is increased to [8,12,24,48] Proposal 9 Maximum number of cyclic shifts nSRScs,max and comb size should be configured at the resource set level. Proposal 10 The actual cyclic shift of a SRS resource should be configured by the parameter nSRScs∈0,1,2,…,nSRScs,max-1. |
| [4] | Proposal 10: Support SRS resource coordination to achieve orthogonal SRS resource assignment and/or SRS interference cancellation. |

Several different proposal to generate more orthogonal UL SRS for positioning were discussed. As these are single proposal with currently no consensus, it is propose to start a discussion before reaching a potential proposal:

Discuss the extension of the use of cyclic shifts in SRS for positioning

Discuss the use of dedicated slots for UL SRS in positioning

Discuss time domain CDM for UL SRS in positioning

UL SRS configurations

RRC / LPP / NRPPa configuration of SRS

| [2] | Proposal 9: The SRS for NR positioning is configured by RRC by the serving gNB. |
| --- | --- |
| [4] | Proposal 10: Support SRS resource coordination to achieve orthogonal SRS resource assignment and/or SRS interference cancellation. |
| CATT | Proposal 8: The existing configurations for SRS transmission mode and transmission offset can also be reused for NR positioning. |
| [11] | Proposal 12: The configuration of UL PRS should be exchanged between serving gNB and neighboring gNBs via X2 interface so that the neighboring gNBs know when and where exactly to measure UL PRS.. |
| [16] | Proposal 1 The SRS for positioning is configured by the UE serving cell via RRC |
| [16] | Proposal 2 The SRS for positioning UE configuration is communicated to the UE neighbor (non-serving) cell via NRPPa from the location server |

Based on the proposals, the following consensus is proposed:

- Proposal for offline consensus: SRS is RRC configured

- Proposal for offline consensus: For coordination of resources, the following option are considered:

  - The location server coordinate the configuration of SRS resources, based on input from the gNodeB supported SRS configurations

  - The EnodeBs coordinate the SRS resources via X2

  - No coordination is expected

UL SRS procedures

Note: the following proposals have been mentioned in the UL RS design contribution (AI 7.2.10.2)  but would be more fitted for the procedures discussion in 7.2.10.4. the non-overlapping procedures will be discussed in the 7.2.10.2 session, while overlapping topics will be left to 7.2.10.4.

Beam sweeping / aligments

| [4] | Proposal 9: The beam information of the DL reference signals (e.g., SSB, CSI-RS tec.) and UL reference signals (e.g., SRS) should be included in the messages exchanged between UE, gNBs, and LMF for supporting UL positioning based on SRS signals. |
| --- | --- |
| [10] | Proposal 3: The uplink positioning reference signal (e.g. SRS) beam configuration are determined based on the UE reports on selected downlink measurement information. |
| [10] | Proposal 4: SRS transmission with UL beam sweeping is supported, particularly when the UE does not support beam correspondence. |
| [11] | Proposal 2: For AoA/AoD based positioning exploring beam information, the existing NR beam measurement, reporting and management mechanism is sufficiently flexibly to support positioning except a reference beam pointing to a pre-defined geographic direction needs to be defined. |
| [11] | Proposal 8: Only support UE Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. |
| [11] | Proposal 9: Only support UE Tx beam sweeping on UL SRS transmissions across multiple UL SRS Resources. |
| [11] | Proposal 10: For DL&UL positioning technologies, only the following reference signals should be supported for DL and UL measurements, respectively UL: SRS and PRACH; DL: CSI-RS/TRS and new PRS. |
| [11] | Proposal 11: For AoA/AoD based positioning exploring beam information, the existing NR beam measurement, reporting and management mechanism is sufficiently flexibly to support positioning with following enhancements A reference beam pointing to a pre-defined geographic direction needs to be defined; gNB derivation of AoD should be supported. |

A previous agreement from RAN1#96b in the procedure AI provides a suitable scope for this issue.

Agreement: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, consider one or more of at least the following options: - Option 1: Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS. - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS). - Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources. - Option 3: The Tx beam is fixed for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2. FFS: How the fixed beam is selected

Based on the proposals, and if the issue is not treated in AI 7.2.10.4, it is proposed to narrow the previous agreement

- Proposal for offline consensus: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, the following is supported:

  - configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

    - CSI-RS and SSB are supported for reference DL RS. Other RS FFS

  - UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

  - FFS other options

## BWP operation

| [2] | Proposal 7: The configuration of SRS for positioning per UL BWP follows the Rel. 15 NR specification. |
| --- | --- |
| [2] | Proposal 8: For positioning, the SRS transmission in the active UL BWP of an active cell and SRS carrier switching feature follow the Rel. 15 NR specification. |
| [4] | Proposal 7: No enhancement is needed in terms of the SRS transmission bandwidth for NR positioning. |
| [5] | Proposal 6: SRS is configured within the active UL BWP. |
| [11] | Proposal 4: For the BWP configuration of NR UL PRS, e.g. SRS, both solutions as follows should be supported Alt1: SRS is configured within the active UL BWP; Alt2: SRS is configured in a dedicated BWP with the largest bandwidth or subcarrier spacing. |
| [12] | Observation 2: It is not necessary for the UEs to use same SRS bandwidth for positioning with respect to other SRS cases. Proposal 3: Allow different SRS bandwidth to be configured for positioning use case compared to other use cases. |

Based on the proposals in the contribution, the following

- Proposal for offline consensus: regarding UL BWP configuration, downselect between the possible  SRS configuration:

  - Option 1: SRS may only be configured within the active UL BWP

  - Option 2: SRS may be configured in a BWP that may be different from the active BWP.

Timing advance and power control

Timing advance

| [5] | Proposal 7:Use PRACH to get multiple TAs estimation of neighbour cells. |
| --- | --- |
| [7] | Proposal 6: Study UE specific TA and TPC control mechanisms for neighbor cell hearability improvements. |
| [8] | Proposal 3: Support one or both of option 2 and option 3 of the TA agreement (FL note: from ran#196b). |
| [11] | Proposal 7: The propagation delay difference between the serving cell and the neighbor cells can be handled in the following ways. Adjustment of measurement window; Adjustment of TA value based on multiple propagation delays and such adjustment should be indicated by the network. |
| CATT | To support TA adjustment specifically for neighboring cell, the UE needs first to know beforehand what are the TAs for difference cells. The information is not available to the UE. Also, changing the TAs to fit the remote gNB may cause the interference to nearby cells/gNBs. The interference may be significant due to the UE is closer to these cells/gNBs. In addition, due to the UL power limitation, the measurement quality from remote cells very far away from a UE will be low and may not be reliable anyway. Thus, we do not see the clear need and benefits to let the UE to adjust TAs for the transmission of the SRS for remove cells/gNBs. |

A previous agreement from RAN1#96b in the procedure AI provides a suitable scope for this issue.

Agreement: For positioning purposes, with regard to the UL SRS transmission timing, at least the following options have been identified: - Option 1: No adjustment is made to the configured TA. That is, support only a TA configuration which is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols). - Option 2: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value can be provided by the UE to the network upon request. - Note: The adjustment value is not needed to be known at the measuring cell for the purpose of UL-AoA measurement - Option 3: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value is provided by the network to the UE.

Based on the proposals, it is observed that the agreement from RAN1#96b is not downscoped by the new proposals. Thus it is proposed to discuss further, in AI 7.2.10.4

Discuss TA for SRS in positioning in 7.2.10.4

Power control

| [12] | Proposal 1: Power control for positioning uses a different mechanism from other SRS use cases. |
| --- | --- |
| [12] | Proposal 2: The transmit power for UE is fixed for all UEs within a cell by the serving cell, which may be specifically configured for SRS positioning use case. |
| [11] | Proposal 6: In order to improve the hearability of UL PRS, both open loop and closed loop power control can be considered For open loop power control, UE can transmit based on the weakest link of surrounding TRPs/gNBs as long as it is lower than the maximum allowed transmit power per carrier; For closed loop power control, UE can receive multiple TPCs from multiple TRPs/gNBs and adjust its transmission power based on the received TPCs, e.g., maximum power, average power. A fall-back mechanism where UE always transmits with maximum power should also be supported. |
| [10] | Proposal 1: Define a new power control procedure of UL SRS for positioning. |
| [10] | Proposal 2: The transmit power of uplink positioning reference signal (e.g. SRS) is determined based on the downlink measurement information of one or more gNBs. |
| [8] | Proposal 4: [FL note: for power control, based on proposal of RAN1#96b) ]Our first preference is option 3 but we also support option 1 if the constant power is the maximum transmission power under the control of the gNB. |
| [5] | Proposal 8:SRS for positioning should support power control configuration per beam directing to different cells. |
| CATT | In our view, changing the SRS Tx power control based on the PL of the neighboring cells could potentially introduce large interference for the reception of other UEs in the serving and neighboring cell. Thus, NR should avoid defining new SRS power control procedure for supporting NR position, unless it is demonstrated with the clear benefit for NR positioning without adverse impact on NR data communication. |

A previous agreement from RAN1#96b in the procedure AI provides a suitable scope for this issue.

Agreement: For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified: - Option 1: The UL SRS Tx power is constant (i.e., no power control is supported). - FFS: The value of the constant Tx power and how it is determined - Option 2: The UL SRS Tx power is based on the existing power control procedure. - Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example: - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop).

Based on the new proposal, it is proposed to refine the existing agreement with the following

- Proposal for offline consensus: For positioning purposes, with regards to the UL SRS transmission power, the following is supported:

  - the UL SRS Tx power is constant (i.e., no power control is supported).

    - FFS: The value of the constant Tx power and how it is determined

  - A modified power control procedure including the neighbor cells

    - detail FFS

Others

| [6] | Proposal 2-1: Support symmetric design between DL and UL except the sequence |
| --- | --- |
| [4] | Proposal 6: Support only a single antenna port for SRS resource configuration for positioning. |
| [6] | Proposal 2-2: Support the following table for SRS transmission Row; Bt (number of transmission occasions within a slot); Sf (frequency domain RS spacing); N1 (symbol number to determine observation range); N2 (symbol number determine processing gain); N= N1+ N2 (time domain duration). 1; 1; 8; 8; 6; 14. 2; 1; 6; 6; 6; 12. 3; 2; 4; 3; 3; 6. 4; 3; 4; 3; 1; 4. 5; 4; 2; 2; 1; 3. 6; 6; 2; 2; 0; 2. Where, Sf : frequency domain RS spacing N1 : symbol number to determine the observation range without ambiguity for unknown timing delay measurement N2 : symbol number to determine the processing gain regarding the received SINR improvement Bt : number of transmission occasions within a slot |
| [6] | Proposal 3-1: Study the influence of FDSS to the timing measurement accuracy |
| [10] | Proposal 5: Aperiodic SRS transmission can be used to support on-demand positioning. |
| [11] | Proposal 1: Both SRS and PRACH should be supported for UL positioning and following principles should be considered for further enhancements NR reference signal design and configuration already provide high level of flexibility so that further enhancements to support positioning functionality should be minimized. Same reference signal should be chosen for joint measurement of both timing advance and angle when possible to reduce complexity, latency and storage size. |
| [12] | Proposal 10: Discuss within RAN1 whether LS can be sent to RAN2/3 requesting group management for interference coordination among UEs separated along the code domain. |
| [14] | Proposal 2: The mechanism to report geographical information to gNB should be FFS as part of UL PRS. |
| [2] | Proposal 10: Aperiodic SRS for positioning is only sent to the serving gNB(s) |

Conclusions

In this summary, the following proposals and offline consensus proposals have been made :

Proposal for offline consensus: a new SRS usage named “positioning” is defined.

o	FFS Scope of the usage  / impact on procedures

Proposal for offline consensus: the SRS pattern for positioning is realised within an SRS resource.

o	FFS: construction of the pattern inside the SRS resource structure

Proposal for offline consensus: For positioning, the number of consecutive OFDM symbols in an SRS resource  is configurable with one of the values in the set {1,2,4,6,8,12,14}

Proposal for offline consensus: For positioning, the the starting position in the time domain for the SRS resource may be anywhere in the slot, i.e. the offset loffset  range is {0,1,…,14}

Proposal for offline consensus: For positioning, the SRS comb size set is extended from {2,4} to {2,4,6}

o	FFS additional comb sizes

Proposal 2	Discuss the bit width of the sequence ID of the SRS resource configuration

Proposal 3	Discuss the use of non ZC sequences for SRS in positioning

Proposal 4	Discuss the extension of the use of cyclic shifts in SRS for positioning

Proposal 5	Discuss the use of dedicated slots for UL SRS in positioning

Proposal 6	Discuss time domain CDM for UL SRS in positioning

Proposal for offline consensus: SRS is RRC configured

Proposal for offline consensus: For coordination of resources, the following option are considered:

o	The location server coordinate the configuration of SRS resources, based on input from the gNodeB supported SRS configurations

o	The EnodeBs coordinate the SRS resources via X2

o	No coordination is expected

Proposal for offline consensus: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, the following is supported:

o	configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

i.	CSI-RS and SSB are supported for reference DL RS. Other RS FFS

o	UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

o	FFS other options

Proposal for offline consensus: regarding UL BWP configuration, downselect between the possible  SRS configuration:

o	Option 1: SRS may only be configured within the active UL BWP

o	Option 2: SRS may be configured in a BWP that may be different from the active BWP.

Proposal 7	Discuss TA for SRS in positioning in 7.2.10.4

Proposal for offline consensus: For positioning purposes, with regards to the UL SRS transmission power, the following is supported:

o	the UL SRS Tx power is constant (i.e., no power control is supported).

i.	FFS: The value of the constant Tx power and how it is determined

o	A modified power control procedure including the neighbor cells

i.	detail FFS

References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

- R1-1906053	SRS design for NR positioning	Huawei, HiSilicon

- R1-1906178	Discussion on UL RS for NR positioning	vivo

- R1-1906306	UL Reference Signals for NR Positioning	CATT

- R1-1906425	Discussion on positioning usage of SRS	ZTE Corporation,Sanechips

- R1-1906562	UL RS design for NR positioning	MediaTek Inc.

- R1-1906659	Views on UL reference signals for NR Positioning	Nokia, Nokia Shanghai Bell

- R1-1906720	Discussion on UL Reference Signals for NR Positioning	LG Electronics

- R1-1906822	Uplink reference signal design for NR positioning	Intel Corporation

- R1-1906853	Considerations on UL reference signals for NR positioning	Sony

- R1-1906908	UL Reference Signals for NR Positioning	Samsung

- R1-1907088	Details on Uplink PRS Signal Design	Fraunhofer IIS

- R1-1907178	Views on UL reference signal designs for NR positioning	Mitsubishi Electric Co.

- R1-1907237	Discussion on UL Reference Signals for NR Positioning	ITRI

- R1-1907297	UL Reference Signals for NR Positioning	Qualcomm Incorporated

- R1-1907509	UL Reference Signals for NR Positioning	ericsson