3GPP TSG RAN WG1#97		R1-1907705

Reno, USA, 13th– 17th May 2019

Agenda Item:	7.2.2.1.2

Source:	Motorola Mobility, Lenovo

Title:	Feature lead summary for NR-U DL Signals and Channels

Document for:	Discussion, Decision

# Introduction

This document summarizes documents submitted to RAN1#96bis on AI 7.2.2.1.2.

The following is an excerpt from the work item description (RP-182878), highlighting added by the feature leader:

- Physical layer aspects including [RAN1]: - Frame structure including single and multiple DL to UL and UL to DL switching points within a shared COT with associated identified LBT requirements (TR Section 7.2.1.3.1). - NR-U supports a mode of operation where for a carrier and at least for intra-band CA on serving cells on unlicensed bands, all DL channels / signals can be operated with the same numerology, and all UL channels / signals can be operated with the same numerology. - Subcarrier spacing for control and data channels supporting 15kHz, 30kHz, and 60kHz (air-interface perspective; optionality to be discussed separately). - Wide band operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with multiple serving cells, and wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with one serving cell with bandwidth > 20MHz with potential scheduling constraint subject to input from RAN2 and RAN4 on feasibility of operating the wideband carrier when LBT is unsuccessful in one or more LBT subbands within the wideband carrier. For all wide-band operation cases, CCA is performed in units of 20MHz (at least for 5GHz). - NR-U Discovery Reference Signal (DRS) containing at least SS/PBCH block burst set transmission and possibly CSI-RS, RMSI-CORESET(s)+PDSCH(s), OSI and paging with properties and extensions from NR Rel-15 in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2). 60kHz based SSB/PBCH block is outside the scope of the WI. - PRACH including possible extension of PRACH format(s) in line with agreements during the SI phase (TR 38.889, Section 7.2.1.2) to support minimum bandwidth requirement given by regulation. Determine the applicability of Rel-15 NR formats to NR-U operation.RAN1 should decide whether 60 kHz subcarrier spacing for PRACH is supported, based on a unified design with 15 kHz and 30 kHz PRACH for meeting occupied channel bandwidth (OCB) requirements. - UL control including extension of PUCCH format(s) to support PRB-based frequency block-interlaced transmission and use of Rel-15 NR PUCCH formats 2 and 3 for NR-U operation. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1. - UL data channel including extension of PUSCH to support PRB-based frequency block-interlaced transmission; support of multiple PUSCH(s) starting positions in one or multiple slot(s) depending on the LBT outcome with the understanding that the ending position is indicated by the UL grant; design not requiring the UE to change a granted TBS for a PUSCH transmission depending on the LBT outcome. The necessary PUSCH enhancements based on CP-OFDM. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1. - SRS including the introduction of additional flexibility in configuring/triggering SRS in line with agreements during the study phase. - For DL data channel, support of multiple PDSCH starting positions. - Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption. - DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

According to the agenda, the following is to be treated in this agenda item:

Impact to PDCCH, dynamic PDCCH monitoring, impact to PDSCH to support flexible starting point due to LBT, mechanism to detect COT start for UE power saving, COT structure indication

Agreement(s) from RAN1 AH1901 (Taipeh):

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection). - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design. - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst - FFS: Potential enhancements to DMRS design to address issues with detection probability - The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

Agreement(s) from RAN1 96bis (Xian):

Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

# PDSCH Transmission

For PDSCH transmission in the partial slot at least for the first PDSCH(s) transmitted in the DL transmission burst, the following options are still being considered:

Option 1: PDSCH(s) as in Rel-15 NR

Option 2: Punctured PDSCH depending on LBT outcome

Option 3: PDSCH mapping type B with durations other than 2/4/7 symbols

Option 4: PDSCH across slot boundary

Company views:

| Huawei, HiSilicon | Proposal 1: Considering the tradeoff between channel access opportunities and UE implementation complexity, NR-U should allow around 8 starting points within a subframe. Proposal 2: NR-U should consider the tradeoff between frequent start points and overhead introduced by scheduling large number of PDSCH with PDSCH mapping type B. • When a UE is configured with a BWP of 60 kHz SCS, it is beneficial to allow DL burst starts from every 7-OS. • When a UE is configured with a BWP of 15/30 kHz SCS, it is beneficial for a gNB to transmit a series of non-uniform mini-slot scheme with combination of maximum 3 PDSCHs with lengths of 2, 4, 7 symbols. Proposal 3: Time domain PDSCH scheduling information determination could use the starting/ending symbol position of PDCCH as the reference point. Proposal 4: The following two options to implement mini-slot/slot scheme could be considered. • Option 1: The symbols of mini slot/slot(s) is discarded if LBT fails on corresponding mini slot/slot(s). • Option 2: Mini slot/slot(s) transmission is deferred until the LBT succeeds and the symbols of mini slot(s) beyond slot boundary is (are) punctured. Proposal 5: At least 5 symbol and/or 6 symbol mapping type B PDSCH shall be introduced to support RMSI transmission in NR-U. Proposal 6: DMRS for mapping type B PDSCH will be transmitted at the first available symbol which doesn’t collide with configured CORESET. |
| --- | --- |
| ZTE, Sanechips | - Proposal 6: If Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) is considered, only limited additional symbol number should be supported for type B mapping, e.g. 9/10 symbols. |
| Vivo | Proposal 1: NR-U supports PDSCH mapping type B with durations other than 2/4/7 symbols in the subset of {5, 6, 8, 10, 12} symbols. |
| MediaTek | Proposal 1: Agree that any selected option to handle the initial partial slot of transmission in NR-U of a PDSCH shall not result in an increase to the UE’s PDSCH capability beyond what is already defined for Rel-15 NR. Proposal 2: Adopt option 1, PDSCH(s) as in Rel-15 NR, for the handling of PDSCH initial partial slot transmission. |
| Oppo | Proposal 1: Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) is not supported in Rel-16 NR-U. |
| Nokia, Nokia Shanghai Bell | Observation 1: FG3-1 and FG 3-5b (7,3) can be handled easily using Option 1, it is unclear how to handle FG 3-5b (4,3) and FG 3-5b (2,2). Observation 2: For a UE with capability FG 3-5b (4,3), Option 3+Option 1 compared to solely Option 1 has ~15% better efficiency in partial slot on average. Observation 3: For UE with capability FG 3-5b (2,2), Option 3+Option 1 compared to solely Option 1 has better efficiency. Observation 4: Independently of capability, if RAN1 supports Option 1+ Option 3, the UE’s PDCCH blind decoding effort in Partial slot phase can be kept the same (max 2 PDCCH monitoring occasions). This resulting into power efficient NR-U design. Proposal 6: For NR-U and TYPE B unicast PDSCH mapping, allow at least [L=5 or L=6] and [L=9 or L=10] |
| Intel | Proposal 3: For PDSCH partial slot supports, Option 3 is not supported. |
| LG | Proposal #4: Support Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) at least for RMSI PDSCH transmission, allowing 5 and 6 symbols PDSCH mapping type B in addition to 2/4/7 symbols. |
| Sharp | Proposal 2: • NR-U should support PDSCH mapping type B with durations other than 2/4/7 symbols (i.e. Option 3). |
| Qualcomm | Proposal 6: The current NR PDSCH transmission type B scheduling with 2/4/7 symbol duration (mini-slot based scheduling) can be reused for NR-U, to allow flexible gNB COT starting points. Proposal 12: PDSCH(s) as in Rel.15 NR (Option 1) is used to support multiple DL starting positions and other options need not be considered. |
| NTT DOCOMO | Proposal 1: PDSCH mapping type B with durations 6/9 or 6, is supported for NR-U operation. |
| Ericsson | Proposal 1 All durations for PDSCH mapping type B within a slot are supported i.e. {2, …, 14}. |
| Samsung | Proposal 1: For PDSCH transmission in the partial slot, no further enhancements are necessary for NR-U. |
| CAICT | Proposal 5: A fixed length plus flexible length structure could be used for PDSCH transmission in the first slot of a gNB’s COT. |
| Motorola Mobility, Lenovo | Proposal 1: Existing starting positions and ending positions specified for NR Rel-15 PDSCH mapping Type B are reused for NR-U. Proposal 2: Do not introduce additional starting position and duration for NR-U. Support of only PDSCH maping as in Rel-15 NR is sufficient for the basic functionality. |
| Panasonic | Proposal 6: Support Option 3: PDSCH mapping type B with durations other than 2/4/7 symbols for NR-U. |
| Xiaomi | Proposal 5: Support Alt 1 only for the PDSCH transmission in partial slot. |
| ETRI | Proposal 1: Introduction of additional Type B duration(s) for NR-U is discussed for the initial access, and if introduced, the new Type B duration(s) are applied to any PDSCH. |
| Spreadtrum | Proposal 6: Only Option 1 should be supported in PDSCH transmission in the partial slot. |
| Charter | Existing Type B durations are sufficient (2/4/7 symbols) |
| AT&T | Proposal 1: Specify PDSCH mapping type B for mini-slots of length {1,3,5-6,8-13} OFDM symbols Proposal 2: Specify the aggregation of partial slots with full slots using both PDSCH mapping type A and B |

FL Summary:

Support for only relying on Option 1:

MediaTek, Oppo, Intel, Qualcomm, Samsung, Motorola Mobility, Lenovo, Xiaomi, Spreadtrum, Charter

Support an adding other durations for PDSCH mapping type B:

Huawei, HiSilicon: at least support length 5 and/or 6 symbols

ZTE, Sanechips: 9/10 symbols

Vivo: {5, 6, 8, 10, 12}

Nokia, Nokia Shanghai Bell: [L=5 or L=6] and [L=9 or L=10]

LG: allow 5 and 6 symbols

NTT DOCOMO: support 6 or 6/9 symbols

Ericsson: all lengths {2, …, 14}

AT&T: Specify PDSCH mapping type B for mini-slots of length {1,3,5-6,8-13} OFDM symbols

Sharp, Panasonic: other than 2/4/7

FL Comment: This issue has seen progress in the online session, so it is suggested to come back to remaining issues in the next meeting.

# DL burst detection (+”Preamble”, +”WUS”, …) for power saving

Company views:

| Huawei, HiSilicon | Proposal 7: The group common PDCCH together with its DMRS could be used by UE to identify the DL burst from serving cell. • FFS: mechanism to reduce blind decoding of GC-PDCCH • FFS: wideband DMRS for GC-PDCCH Proposal 8: NR-U should support to transmit GC-PDCCH and its DMRS multiple times within a gNB’s acquired COT. Proposal 9: NR-U should support a PDCCH search space configuration allowing different PDCCH monitoring occasion configurations before and after the first slot boundary of a COT in terms of UE power consumption. |
| --- | --- |
| ZTE, Sanechips | Proposal 7: The following aspects should be considered when designing a signal for DL burst detection in NR-U: • NR-U UE is not expected to receive Wi-Fi preamble at least in 5GHz unlicensed band. • Wideband DMRS is more appropriate for used to detect gNB’s DL transmission. • No further enhancement is needed for wideband DMRS. |
| Vivo | Proposal 2: NR-U UEs could use GC-PDCCH DM-RS or CSI-RS to detect the DL burst based on configuration. |
| MediaTek | Proposal 7: For NR-U DL transmission burst detection, an UE may assume PDCCH DMRS is available for all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH. Proposal 8: The CORESET carrying PDCCH DMRS for NR-U DL burst detection is configured to occupy the full channel bandwidth of 20MHz. |
| LG | Proposal #3: During Phase A, NR-U UE determines the presence of serving gNB’s DL transmission at least based on DM-RS in GC-PDCCH together with GC-PDCCH containing the information on channel occupancy structure. • UE is not required to use signals other than DM-RS in PDCCH to detect serving gNB’s DL transmission. • FFS whether DM-RS in PDCCH together with PDCCH other than GC-PDCCH containing the information on channel occupancy structure also can be used to detect serving gNB’s DL transmission or not • FFS on minimum transmission or occupied bandwidth for DM-RS and corresponding PDCCH • FFS on UE behaviour if DM-RS in (GC-)PDCCH is detected but CRC check for (GC-)PDCCH is failed |
| Apple | Proposal 1: For detecting of presence of DL transmission burst, RAN1 to decide between the following: • Option 1: leveraging and modifying existing NR reference signal, allowing a common time & frequency resource pattern, to simplify UE detection • Option 2: Defining a new cell-independent common pre-amble. |
| Google | Proposal 3. For DL burst detection, consider the following actions: Option 1: Take it as a UE implementation feature and find an alternative to improve the coverage. Option 2: The feature is configured/activated by gNB according to the signal strength measurement. |
| Ericsson | Proposal 5 The UE may assume the presence of GC-PDCCH and its associated DMRS in a subset of configured PDCCH monitoring occasions within the DL transmission burst by the serving gNB for power saving purposes. |
| Samsung | Proposal 2: Wideband DMRS of a CORESET is used for transmission burst detection. |
| Intel | Proposal 1: For DL burst detection and PDCCH monitoring, UE behavior is composed of • Phase 1: WB DMRS detection phase • UE is not required to monitor PDCCH until WB DMRS is detected • Monitoring pattern for WB DMRS can be configured by RRC • Phase 2: PDCCH monitoring with pre-configured monitoring periodicity • If WB-DMRS is detected, UE performs PDCCH monitoring based on RRC configuration during PDCCH monitoring Phase 1, e.g., mini-slot level PDCCH monitoring • UE assumes that GC-PDCCH is transmitted with WB-DMRS • If GC-PDCCH is detected, UE can change the PDCCH monitoring based on indication of GC-PDCCH • Phase 3: PDCCH monitoring with released monitoring periodicity • UE can change the PDCCH monitoring pattern based on information of GC-PDCCH, e.g., slot level PDCCH monitoring • GC-PDCCH and WB DMRS can be transmitted even in PDCCH monitoring Phase 2 |
| Panasonic | Proposal 4: Before detection DL burst, UE only monitors GC-PDCCH. As an implementation option, UE can rely on DM-RS in GC-PDCCH to detect the DL burst for power saving. AGC/fine-frequency tuning aspect should be considered in addition if DRS is not sufficient. |
| ETRI | Proposal 2: In NR-U, DMRS in GC-PDCCH (or wideband DMRS in CORESET) together with GC-PDCCH content is used for DL burst detection. |
| Sharp | Proposal 1: • Wideband DMRS should be used in PDCCH monitoring occasions of initial partial slots. |
| NTT DOCOMO | Proposal 3: gNB informs UE whether GC-PDCCH indicating gNB’s burst structure is transmitted or not. |
| Spreadtrum | Proposal 1: Wideband DMRS should be used for DL transmission burst detection. |
| Oppo | Proposal 2: Detection of a DL burst by UE can be determined by receiving: 1) at least one DL grant; 2) at least one UL grant; 3) wideband DM-RS in CORESET. |
| InterDigitial | Proposal 1: A UE is configured with a PDCCH DMRS monitoring configuration to enable PDCCH DMRS monitoring for the purpose of COT indication (in Phase A). |
| Charter | Based on (wideband) DM-RS |
| AT&T | Proposal 3: A UE initiates decoding of a PDCCH candidate after successful correlator-based energy detection • Correlator-based energy detection could be based on the GC-PDCCH DMRS or a dedicated preamble • A dedicated preamble could also be used to serve other purposes such as coexistence as well as CSI acquisition and fine time/frequency tuning |
| Qualcomm | Proposal 1: Wideband DMRS can be used to enable UE power saving in one of the following modes: C. Spec transparent mode: UE saves power only when monitoring PDCCH in coresets where WB-DMRS is configured. For other coresets UE performs regular PDCCH monitoring. i. In this mode, gNB is oblivious to the UE power save behaviour. ii. FFS: Is any additional functionality/spec change needed for this mode. D. Spec non-transparent mode (preferable): UE monitors all PDCCH coresets only after it detects WB-DMRS i. FFS if configuration is explicit (for example, through RRC) or implicit (for example one of the coresets monitored by the UE (PDCCH/GC-PDCCH) is configured with WB-DMRS) ii. FFS if a UE capability indication is required iii. Note: In this mode the gNB is aware of UE power save behaviour, so it does not transmit a grant to the UE before transmitting WB-DMRS |

From the WID (RP-182878)

- Physical layer aspects including [RAN1]: Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption.

# Dynamic PDCCH Monitoring

Company views:

| Huawei, HiSilicon | Proposal 6: NR-U should support a PDCCH search space configuration allowing different PDCCH monitoring occasion configuration before and after the first slot boundary of a COT in terms of UE power consumption. |
| --- | --- |
| ZTE, Sanechips | Proposal 1: In Phase A (outside of gNB’s COT), no PDCCH monitoring is needed and UE only need to monitor the initial signal e.g. wideband DMRS for DL burst detection. Proposal 2: Different PDCCH monitoring patterns can be performed for Phase B (beginning of gNB’s COT) and Phase C (remainder of gNB’s COT). The PDCCH monitoring occasion granularity in the first slot in Phase B is smaller than the remaining slot(s) in Phase C. Proposal 3: UE can determine the transitions between different phases associated with different PDCCH monitoring efforts according to: • From Phase A (outside of gNB’s COT) to Phase B (beginning of gNB’s COT) based on DL burst detection (initial signal detection). • From Phase B to Phase C (remainder of gNB’s COT) based on the end boundary of the first slot of the COT. • From Phase C to Phase A based on the length or end position of the COT. |
| Vivo | Proposal 3: NR-U supports dynamic PDCCH monitoring by explicit signalling. |
| MediaTek | Proposal 3: UE should not be mandated to perform mini-slot based PDCCH monitoring outside of a serving gNB’s COT (Phase A) due to decoding complexity and power consumption. Proposal 4: If a DL Tx burst is detected, a mini-slot based PDCCH monitoring can be activated immediately within a time window for the beginning of a serving gNB’s COT (Phase B). Proposal 5: If a DL Tx burst is detected, after a time offset, a slot based PDCCH monitoring can be activated within a time window for the remainder of a serving gNB’s COT (Phase C). Proposal 6: To support dynamic PDCCH monitoring for NR-U operation, it would be necessary to introduce a type of search space set that can be activated/deactivated. • UE doesn’t perform PDCCH monitoring for a non-activated search space set. • A search space set can be activated by other signal/channel (e.g. initial signal, GC-PDCCH, etc.) immediately, or after a time offset configured/indicated by gNB or determined based on a pre-defined rule. • The duration of an activation of a search space set can be indicated/configured by gNB or determined based on a pre-defined rule. |
| Nokia, Nokia Shanghai Bell | Proposal 1: UE should be able receive DL channels/signals and transmit UL channels/signals in the gNB acquired COT also in the absence of GC-PDCCH. Proposal 2: To enable power saving by not necessitating performing blind decodes to detect the transmission burst • UE can be configured with initial search-space set(s) with to up to X PDCCH candidates. FFS on X. • UE monitors DMRS transmitted in all REGs of a CORESET associated with initial search-space set(s) Proposal 3: To enable power saving by not necessitating performing blind decodes to detect the transmission burst • In Pre-COT phase, when UE detects WB-DMRS and a DCI with CRC check, it may assume that gNB transmits a COT. • The DCI can be GC-PDCCH or unicast DCI, i.e. up to gNB configuration • UE continues monitoring initial search-space set(s) until next slot boundary • FFS whether and how to support additional indicating of monitoring in a partial slot • UE monitor non-initial search-space set(s) from the next slot boundary • Pre-COT phase (with WB DMRS detection + monitoring according to initial search-space set(s)) continues after the current COT is over. UE determines COT ending: • based on received GC-PDCCH or • in the absence of GC-PDCCH: • based on expiration of UE’s “inactivity timer” or • based on ending of UL transmission |
| Sony | Proposal 1: Dynamic switching of PDCCH monitoring periodicity should be supported in NR-U. Proposal 2: The following UE behavior should be supported. • For a slot outside gNB’s COT (Phase A), the UE both attempts to detect a signal for power saving purposes on occasions where the signal is potentially transmitted and attempts to decode GC-PDCCH in the slot-based scheduling search space. • For an initial slot in the gNB’s COT (Phase B), the UE attempts to decode PDCCH in a non-slot-based scheduling search space. • For a slot inside the gNB’s COT other than the initial slot (Phase C), the UE attempts to decode GC-PDCCH and PDCCH in the slot-based scheduling search space. • If configured, the UE can attempt to decode PDCCH in the non-slot-based scheduling search space. |
| Intel | Proposal 1: For DL burst detection and PDCCH monitoring, UE behavior is composed of • Phase 1: WB DMRS detection phase • UE is not required to monitor PDCCH until WB DMRS is detected • Monitoring pattern for WB DMRS can be configured by RRC • Phase 2: PDCCH monitoring with pre-configured monitoring periodicity • If WB-DMRS is detected, UE performs PDCCH monitoring based on RRC configuration during PDCCH monitoring Phase 1, e.g., mini-slot level PDCCH monitoring • UE assumes that GC-PDCCH is transmitted with WB-DMRS • If GC-PDCCH is detected, UE can change the PDCCH monitoring based on indication of GC-PDCCH • Phase 3: PDCCH monitoring with released monitoring periodicity • UE can change the PDCCH monitoring pattern based on information of GC-PDCCH, e.g., slot level PDCCH monitoring • GC-PDCCH and WB DMRS can be transmitted even in PDCCH monitoring Phase 2 |
| LG | Proposal #2: For a search space set configuration, at least the parameters of monitoringSlot PeriodicityAndOffset and monitoringSymbolsWithinSlot are separately configured for each phase which is defined by as follows. • Phase A: Before UE detects serving cell’s DL burst or after serving cell’s DL burst is finished • Phase B: At the beginning (e.g., the first slot) of serving cell’s DL burst • Phase C: Remaining part of serving cell’s DL burst • FFS on how UE determines phase transition, e.g., explicit signalling or implicit signalling based on channel occupancy structure indication Proposal #3: During Phase A, NR-U UE determines the presence of serving gNB’s DL transmission at least based on DM-RS in GC-PDCCH together with GC-PDCCH containing the information on channel occupancy structure. • UE is not required to use signals other than DM-RS in PDCCH to detect serving gNB’s DL transmission. • FFS whether DM-RS in PDCCH together with PDCCH other than GC-PDCCH containing the information on channel occupancy structure also can be used to detect serving gNB’s DL transmission or not • FFS on minimum transmission or occupied bandwidth for DM-RS and corresponding PDCCH • FFS on UE behaviour if DM-RS in (GC-)PDCCH is detected but CRC check for (GC-)PDCCH is failed |
| InterDigital | Proposal 2: A UE is configured with multiple PDCCH monitoring configurations per search space to enable varying PDCCH monitoring patterns for resource allocation (e.g. one per Phase). Proposal 3: A UE determines the active PDCCH monitoring configuration of a search space based on detection of the COT-indicating DM-RS, indication from the gNB, COT structure indication or slot timing. Proposal 4: A UE determines the PDCCH monitoring configuration of a set of search-spaces based on the set of acquired LBT subbands. |
| Qualcomm | Proposal 2: When the UE power save, based on spec non-transparent mode is enabled: • UE is not required to monitor PDCCH sent in a COT (i.e. it remains in a power save mode) if it has not yet detected wideband DMRS in the COT. • UE shall monitor PDCCH as configured for a duration of time, after detecting the wideband DMRS in a configured CORESET. • FFS: If this is based on only detecting the WB-DMRS even if a valid PDCCH/GC-PDCCH is not found or also on detecting of PDCCH/GC-PDCCH Proposal 3: A UE that is monitoring control channels, e.g. inside a COT, can switch back to power save mode and start looking for wideband DMRS under the following conditions: • UE did not receive a valid PDCCH or GC-PDCCH for a configured duration of time • UE received COT-SI in a GC-PDCCH, and the COT end signalled by the COT-SI is reached. Proposal 7: The frequency resources UE monitors for PDCCH may change dynamically, e.g., based on explicit signalling or determined implicitly through the gNB COT start detection. Proposal 8: In case the PDCCH monitoring configuration is changed dynamically and switching trigger is not reliable enough, NR-U should support a mechanism to provide robustness for UE if it misses the switching trigger. Proposal 9: A UE can be provided with two configurations to monitor PDCCH/GC-PDCCH (for example, first at mini-slot level and second at slot or multi-slot level). The following triggers can be considered for switching between the monitoring configurations. • The UE switches from the first to the second configuration, after detecting a common or UE specific PDCCH • FFS if explicit (e.g. bits in DCI) or implicit switch (e.g. just detecting PDCCH) • FFS delay after which the switch is applicable (e.g. next slot boundary after a minimum required processing time) • The UE switches from second to first configuration after the end of COT determined based on COT structure indication received in a common PDCCH or a timer expiry if COT-SI is not received. Proposal 10: A UE can switch from a first monitoring configuration (e.g., mini-slot level) to a second monitoring configuration (e.g., slot/multi-slot level), it is not scheduled for a specific duration of time Proposal 11: In NSA mode of operation, gNB can explicitly indicate/trigger PDCCH monitoring switch for the unlicensed carrier, using the licensed carrier. |
| Ericsson | Proposal 2 Support dynamic indication of a change in the UE configured PDCCH monitoring occasions with two different monitoring phases a. More frequent (mini-slot based) monitoring occasions b. Less frequent (slot-based) monitoring occasions FSS: whether switching between each phase is explicit or implicit Proposal 3 It is up to the gNB scheduling decision to inform the UE when to switch between different monitoring configurations. UE is always schedulable regardless of which set of PDCCH monitoring occasion is active. Proposal 4 Detection of the DL Burst and/or of a COT structure should not relax the requirements of NR Rel-15 PDCCH monitoring. |
| Samsung | Proposal 7: NR-U support adaptation of PDCCH monitoring periodicity • Mini-slot granularity outside of gNB’s COT and at least the first slot of gNB’s COT while slot-level granularity inside of gNB’s COT • Switching PDCCH monitoring can be indicated/triggered implicitly (e.g., based on detection of COT) and explicitly (e.g., common or UE specific DCI) |
| CAICT | Proposal 1: PDCCH monitoring in NR-U should be divided into outside of gNB’s COT(Phase A), beginning of gNB’s COT(Phase B), remainder of gNB’s COT(Phase C). Proposal 2: A joint design should be considered for PDCCH monitoring outside of gNB’s COT and Preamble/discovery signal. Proposal 3: PDCCH monitoring in Phase B could be bundle to the candidates starting points for the first slot within a gNB’s COT. Proposal 4: The transition from Phase B to Phase C is before or after the first slot of gNB’s COT. |
| Xiaomi | Proposal 4: UE is not required to perform PDCCH decoding outside a COT. |
| Fujitsu | Proposal 1: A UE should monitor the PDCCH in symbol-level granularity during Phase A and Phase B, and monitor the PDCCH in slot-level granularity during Phase C. • FFS: the search space configuration details on the symbol-level and the slot-level PDCCH monitoring granularity. Proposal 2: Changing the PDCCH monitoring granularity should base on the detection COT structure indication. Proposal 3: Support symbol-level search space configuration for the COT structure indication in NR-U. Observation 1: According to the SFI indication method by GC-PDCCH in Rel-15 NR, a later received SFI cannot overrides the previous received SFI if they indicate the overlapped slot(s). UE can suspend monitoring of COT structure indication during Phase C for power saving up to UE implementation. • FFS: Whether SFI as a COT structure indication need to be updated during Phase C. |
| ETRI | Proposal 3: Support dynamic PDCCH monitoring change within a COT (from Phase B to Phase C) on top of dynamic PDCCH monitoring change based on DL burst detection (from Phase A to Phase B/C). Proposal 4: To clarify the dynamic PDCCH monitoring mechanism when there are multiple DL bursts within a COT. Proposal 5: Consider PDCCH monitoring in a DL duration of a UE-initiated COT. |
| NTT DOCOMO | Proposal 2: Multiple search spaces with different monitoring periodicities are configured to UE and UE switches the search space to monitor according to the detection status of serving gNB burst. |
| Google | Proposal 2. PDCCH configuration should indicate the ability of being switched on/off by other signal (e.g. WUS, DCI, etc.). |
| Spreadtrum | Proposal 2: Different PDCCH monitoring period should be configured during Phase B and Phase C. Proposal 3: UE could implicitly switch the PDCCH monitoring based on the detection of ‘initial signal’ or deriving the information of the COT structure. |
| Oppo | Proposal 3: PDCCH monitoring efforts from time domain perspective and that from frequency domain perspective should be considered independently. |
| Panasonic | Proposal 4: Before detection DL burst, UE only monitors GC-PDCCH. As an implementation option, UE can rely on DM-RS in GC-PDCCH to detect the DL burst for power saving. AGC/fine-frequency tuning aspect should be considered in addition if DRS is not sufficient. Proposal 5: COT structure information in GC-PDCCH is used to adapt UE’s PDCCH monitoring efforts in Phases A/B/C. • UE identifies the transition from Phases A to B by DL burst detection. Before that (i.e. in Phase A), UE only monitors GC-PDCCH containing COT info. For UE capable of mini-slot monitoring, UE can be configure to monitor GC-PDCCH at mini-slot level • UE does not distinguish between Phases B and C, during which UE performs full BD for PDCCH. COT info can dynamically indicate UE to skip some PDCCH monitoring occasions. • UE identifies the transition from Phases C to A by COT info. |
| Charter | Phase A: Use Rel-15 configured periodicity Phase B: Based on GC-PDCCH Phase C: Based on DCI in either GC-PDCCH or UE-specific search space |
| AT&T | Proposal 3: A UE initiates decoding of a PDCCH candidate after successful correlator-based energy detection • Correlator-based energy detection could be based on the GC-PDCCH DMRS or a dedicated preamble • A dedicated preamble could also be used to serve other purposes such as coexistence as well as CSI acquisition and fine time/frequency tuning Proposal 4: Support dynamic switching between PDCCH monitoring case 1 and case 2. A UE monitors for PDCCH candidates according to case 2. When a DL or UL assignment or a GC-PDCCH is received, in switches to PDCCH monitoring case 1 for the COT duration. Thereafter, it resumes PDCCH monitoring according to case 2. |
| ITRI | Proposal 1: RAN1 should support that a UE stops monitoring PDCCH within a time period configured by gNB in NR-U. Proposal 2: After a COT initiated by the serving gNB, the UE stops monitoring PDCCH until the UE detects a signal for the transmission burst transmitted by the serving gNB. |

From the WID (RP-182878)

- Physical layer aspects including [RAN1]: DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

FL Summary:

The following terminology was suggested by the feature leader prior to RAN1#96bis, which should be understood as being from the UE’s perspective:

- Phase A: Outside of gNB’s channel occupancy

- Phase B: Beginning of gNB’s channel occupancy

- Phase C: Remainder of gNB’s channel occupancy

[Huawei, HiSilicon] Phase B only occurs when COT start in middle of a slot. When COT start at slot boundary, no Phase B.

[Panasonic] If explicit indication is used, there is no need for UE to distinguish between Phase B from Phase C. UE simply follows the indication.

For the BD decoding effort in case the UE has not detected a DL burst (Phase A), the following is supported by companies:

No PDCCH monitoring by UE is required: ZTE, Sanechips, MediaTek, Intel, Qualcomm (if power-save is configured), Xiaomi,

PDCCH is monitored by UE (TBD how much effort):  Nokia, Nokia Shanghai Bell, Sony, LG, InterDigital, Qualcomm (if power-save is not configured), Ericsson, Samsung, Fujitsu, Huawei, Hisilicon (only monitoring GC-PDCCH), Panasonic (at least GC-PDCCH)

Proposed Conclusion: There is no consensus to completely disable PDCCH decoding when the UE has not detected a gNB’s transmission burst

For switching the BD decoding effort dynamically, several companies expressed support of having different PDCCH monitoring configurations. The UE determines which monitoring configuration to apply as a function of an explicit indication, e.g. by DCI, or based on implicit relations to the start of a detected DL burst or configured windows.

Proposal for online discussion:

Support a mechanism for the UE to change its PDCCH (incl. GC-PDCCH) monitoring behaviour based on one or more of the following, to be further discussed:

- Indication, such as by a DCI content or DCI CRC pass

- Relation to the detection of a DL burst, such as

  - before and after the end of the first slot of a detected burst

  - before and after the end of the DL burst

  - Timer after DCI detection

- End of DL burst

- Timer after DCI detection

- Configuration-based

Note: monitoring behaviour may include CORESET and search space parameter(s)

# DMRS Sequence Generation

Company views:

| Huawei, HiSilicon | Proposal 15: The slot and symbol number used to generate DMRS sequence(s) of PDSCH(s) and PDCCH(s) in the 1st slot of a DL burst should be set to 0. The slot numbers used to generate DMRS sequences of PDSCHs and PDCCHs in the following slots of the DL burst are relative to the slot where DL burst starts. FFS: slot number configuration where additional GC-PDCCH(s) are transmitted within the DL burst. |
| --- | --- |
| Vivo | Proposal 5: Time dependent DMRS initialization for PDCCH/PDSCH should be removed to avoid regeneration of data when LBT fails. |
| Nokia, Nokia Shanghai Bell | Proposal 4: RAN1 shall optimize frequency domain properties of WB DMRS to reduce power consumption at UE. Proposal 5: In a CORESET associated with initial search-space set, for REGs of the CORESET, both wide-band or narrow-band precoding can be configured. Observation 5: Symbol-index independent structures at least in the initial-TTI of COT significantly reduce UE complexity. Proposal 7: For NR-U, remove symbol index l dependency for PDSCH/PDCCH DMRS sequence, i.e. l=0, for the DMRS sequence. |
| Oppo | Proposal 5: DMRS sharing among channels within DRS should be considered in NR-U. |
| ZTE, Sanechips | Proposal 8: No change to DMRS sequence generation is needed. |
| Qualcomm | Proposal 4: NR-U design should not preclude transmitting wideband DMRS inside COT Proposal 5: UE may assume the same precoder is used for WB-DMRS across frequency within a subband and/or a CORESET cluster |
| Intel | Proposal 4: No change to DMRS sequence generation is needed for PDSCH. |
| Charter | Support time-independent initialization of DMRS for PDCCH and PDSCH |
| Sharp | Proposal 3: • Rel-15 DMRS sequence generation should be used for NR-U. |

Offline conclusion from RAN1#96: Revisit this issue after progress on partial PDSCH transmission issue

# COT Structure Indication, GC-PDCCH Content, …

Company views:

| Huawei, HiSilicon | Proposal 10: UE could monitor DCI Format 2_0 carrying COT structure indication at each possible starting point in a slot before UE identifies the DL burst from serving cell. Proposal 11: The COT structure indication reflects slot format from the slot carrying DCI format 2_0 carrying COT structure indication no matter where the starting point in a slot. UE could obtain slot format of partial slot at the beginning of COT from the SFI corresponding to the first slot and the starting point in the slot jointly. Proposal 12: The following two method could be considered for slot format indication for the last partial slot within a COT. Option 1: introducing a reference symbols for the ending slot Option 2: introducing new entries in the SFI table including a new state indicating ‘out of COT’ Proposal 13: NR-U should support indicating “Pause” state within the COT. Proposal 14: NR-U shall support indicating actual transmission bandwidth of DL burst. |
| --- | --- |
| ZTE, Sanechips | Proposal 5: Rel-15 DCI format 2_0 can be reused to indicate the COT structure for NR-U, e.g, DL/UL/F configuration, COT duration, PDCCH monitoring indicator. |
| Vivo | Proposal 4: GC-PDCCH could deliver information on PDCCH monitoring indicator, COT information, and UL/DL configuration to help UE determine PDCCH monitoring occasions. Besides, its DM-RS could be also used as DL burst detection to trigger UE specific PDCCH monitoring. |
| Panasonic | Proposal 1: COT structure in time domain defines which symbols are DL, UL, and Flexible for one or a few slots, similar to Rel-15 Slot Format Indication. Proposal 2: COT structure indication defining DL, UL and Flexible symbols can be used to dynamically indicate whether to skip some RRC configured PDCCH monitoring occasions. Proposal 3: Group-common PDCCH based on SFI-PDCCH is used to indicate COT structure info. |
| Nokia, Nokia Shanghai Bell | Observation 6: Operation based on legacy R15 DCI format 2_0 would result in significantly increased configuration overhead as well as additional complexity. Proposal 8: In NR-U, a GC-PDCCH may indicate in addition to a legacy SFI (indicating a preconfigured combination of slot formats) also • A slot/symbol where the legacy SFI starts • A slot/symbol where the legacy SFI ends • Next/upcoming SFI Observation 7: New partial slot formats are not needed. Proposal 9: In NRU, support GC-PDCCH overriding the direction indicated in the previous GC-PDCCH Proposal 10: It should be left for corresponding features to decide whether the following content is supported by the GC-PDCCH • Occupied bandwidth (Wideband operations) • COT Sharing (Configured grant sharing) • PDCCH monitoring occasion indication (Partial slot design) |
| Sony | Proposal 3: Both implicit COT indication and explicit COT indication should be considered in NR-U. • Implicit COT indication: Once UE detects a signal for the power saving purpose or DRS, UE assumes that a duration from the detection point to the next boundary of slot is DL. • Explicit COT indication: Direction (DL, UL, or flexible), COT length, and potentially information related to occupied bandwidth is indicated to UE by GC-PDCCH. |
| Intel | Proposal 2: DCI format 2_0 in GC-PDCCH • DCI format 2_0 is future-proof design • Dynamic indication of length of the COT is indicated by defining additional field to indicate the COT length • COT structure in frequency domain is included in DCI format 2_0 for wideband operation • PDCCH monitoring can be changed depending on the indication of DCI format 2_0 |
| LG | Proposal #1: At least the following functionalities are supported by GC-PDCCH indicating channel occupancy structure. • Symbol-level dynamic DL/UL direction indication, as slot format indicator in Rel-15 • Duration of channel occupancy in time domain • Whether to allow configured grant UL transmission within gNB’s channel occupancy or not • gNB’s LBT outcome across carriers and/or LBT-BWs in a carrier |
| InterDigital | Proposal 5: NR-U should define new entries in the SFI table to account for partial slots due to the COT starting immediately after a successful LBT. |
| Sharp | Proposal 4: • An SFI for a given COT should be able to be signaled multiple times within the COT. Observation 2: • It does not make sense to indicate SFI of outside the current COT. Proposal 5: • SFI indicates DL/UL/Flexible configuration of inside the COT in which the SFI is sent. Proposal 6: • Frequency domain channel occupancy indication should be supported. |
| Qualcomm | Proposal 14: NR SFI can be enhanced to support the functionality of LTE-LAA C-PDCCH and support sub-band based channel access. The proposed enhancements include at least the following • Include a bitmap to indicate which sub-bands/carriers are acquired by the gNB • Information of remaining COT duration in number of symbols • FFS if full flexibility is provided or if it is quantized • Information of pause durations within the COT comprising a pause start offset and a pause duration in number of symbols, for each pause duration • FFS if full flexibility is provided or if this is quantized • FFS: Number of pauses supported Proposal 15: At least some COT information, like the DL/UL/pause durations can be transmitted more than once within the COT. Proposal 16: NR-U shall support configuring the COT-SI (or SFI) monitoring in the RMSI. Proposal 17: NR SFI can be enhanced to support the following additional functionalities • Information on additional dynamically triggered RACH resources. • E.g. Resource location (may be relative), enable disable flag, LBT type • Traffic priority class of the current COT. • Information related to CG-UL configured for the current COT. • E.g. Whether it is allowed, and LBT type if it is allowed. |
| Ericsson | Proposal 5 The UE may assume the presence of GC-PDCCH and its associated DMRS in a subset of configured PDCCH monitoring occasions within the DL transmission burst by the serving gNB for power saving purposes. Proposal 6 NR-U supports indication of end-of-COT and/or COT duration as an extension of SFI signaling in DCI Format 2_0. FFS: signaling details. |
| Samsung | Proposal 3: A set of symbols that is outside of gNB’s COT can be indicated as flexible. Proposal 4: At least one or more modifications on SFI can be considered to indicate COT structure by GC-PDCCH. • Introduce COT duration (or validate SFI duration) field in GC-PDCCH • Utilize a set of reserved SFI values and/or increase the number of SFI bits per cell for supporting more SFI combinations • Behavior on flexible symbol inside and outside of gNB’s COT • Etc. Proposal 5: The indication of the COT structure can be transmitted more than once in a COT. Further studies are required whether the indicated COT structure can be changed by another indication. Proposal 6: At least an indication to allow NR-U configured grant sharing should be informed together with information of COT structure for NR-U operation. |
| Asustek | Proposal 1: In NR-U, OFDM symbols outside COT are configured as ‘flexible’ via semi-static RRC signalling. Proposal 2: NR-U supports SFI to indicate slot structure in NR-U with at least one of following modifications: • supporting an aperiodic SFI, • different monitoring periodicity for outside COT and inside COT. Proposal 3: For NR-U, beside CORESET monitoring, UE is allowed to receive pre-configured resource on RRC signalled “flexible” symbols, at least for measurement, even not receiving SFI on monitoring occasion. Proposal 4: DCI format 2_0 or a signal indicating COT structure in NR-U is used to serve as an indication of start of a DL burst or COT by gNB. Proposal 5: For NR-U, how UE interprets the configuration for monitoring DCI format 2_0 is associated with the start of a DL burst or COT by gNB. |
| CAICT | Proposal 6: COT indication could be based on DCI format 2_0 with explicitly COT length indication. |
| Motorola Mobility, Lenovo | Proposal 3: The maximum number of DL/UL switching points within one COT needs further study. Proposal 4: DCI format 2_0 is used to explicitly indicate the slot format for unlicensed spectrum. |
| Apple | Proposal 2: For detecting COT length information, RAN1 to decide between the following: • Option 1: modifying existing GC-PDCCH, including • Prepend existing GC-PDCCH payload with a fixed-size header containing COT length information • Only allow a fixed size payload in DCI 2-0 for usage in unlicensed spectrum. • Specify a common CORESET and aggregation level configuration for DCI 2-0 for usage in unlicensed spectrum • Option 2: a clean slate design with new DCI format, including the following information in payload • Cell specific identifier • COT length |
| NTT DOCOMO | Proposal 5: COT structure is indicated by using SFI in DCI format 2_0. • COT end timing can be implicitly indicated by using the number of slot formats in indicated slot format combination in DCI format 2_0. Proposal 6: Whether the indicated COT structure can be updated later or not should be discussed. |
| Google | Proposal 1. COT structure indicators should be transmitted with wake-up signals. Proposal 4. Determine the timing of COT structure transmission. |
| Xiaomi | Proposal 1: UE can be informed of the slot format for a gNB’s COT at the beginning of the COT. Proposal 2: Additional SFI can be transmitted multiple times within a COT. Proposal 3: Further study how to indicate the slot format for partial slot. |
| Fujitsu | Proposal 3: Support symbol-level search space configuration for the COT structure indication in NR-U. Observation 1: According to the SFI indication method by GC-PDCCH in Rel-15 NR, a later received SFI cannot overrides the previous received SFI if they indicate the overlapped slot(s). UE can suspend monitoring of COT structure indication during Phase C for power saving up to UE implementation. • FFS: Whether SFI as a COT structure indication need to be updated during Phase C. |
| ETRI | Proposal 6: In addition to the slot structure, DCI format 2_0 (or its enhanced version) includes at least the following information: ending time of a COT, gNB’s Tx bandwidth, and COT sharing information. |
| Oppo | Proposal 4: Introduce more than one COT indications within a COT can be considered. |
| Spreadtrum | Proposal 4: It is necessary to indicate “out of COT” by DCI format 2_0. Proposal 5: Flexible symbol can be used to indicate out of COT, when there are no DL or UL symbols after flexible symbol for the whole periodicity of DCI format 2_0. |
| Charter | Indicate at least the following in addition to SFI: - Duration of COT - Allowance of CG PUSCH |
| AT&T | Proposal 5: If DCI format 2_0 is used to indicate the COT structure, additional enhancements are needed relating to the procedures according to which a UE monitors the GC-PDCCH. • A UE can be configured to monitor for a GC-PDCCH on any OFDM symbol • If a UE does not detect a GC-PDCCH where it was configured to monitor for one, it does not commence monitoring for a PDCCH instead |

# CSI-RS, CSI Reporting

Company views:

| InterDigital | Proposal 6: For aperiodic CSI-RS, NR-U should consider enabling multiple CSI-RS resource transmission opportunities in the case that LBT is needed before the CSI-RS transmission. |
| --- | --- |
| Qualcomm | Proposal 13: Periodic, semi-persistent, and aperiodic CSI-RS can be supported in NR-U. The transmission and processing of periodic and semi-persistent CSI-RS should be conditioned on the detection that the transmission opportunity falls in the DL portion of a COT. |
| Panasonic | Proposal 7: The timing of CSI-RS should be allowed to shift based on LBT outcome • Method to avoid UE blind detection of CSI-RS due to the shift should be studied Proposal 8: To postpone the aperiodic CSI reporting to the next COT could be considered. |
| Ericsson | Proposal 7 Support a “stand-alone” configuration of a set of aperiodic CSI-RS resources for tracking (ap-TRS) in which co-configuration of a set of periodic CSI-RS resources for tracking (p-TRS) is not required. The UE shall expect to be configured with the same QCL source(s) for ap-TRS as for p-TRS in NR Rel-15. |
| Huawei, HiSilicon | Proposal 17: The gNB shall also attempt to transmit periodic CSI-RS that falls outside of COT. |
| LG | Proposal #5: Support multiple transmission opportunities within a period at least for periodic or semi-persistent CSI-RS. FFS on details for sequence generation and PDSCH rate-matching |
| Charter | Increase opportunities for AP-CSI-RS to mitigate impact of DL LBT failure |

# Wideband operation

Company views:

| Huawei, HiSilicon | Proposal 16: CORESET shall be configured within the bandwidth of sub-band LBT in NR-U, e.g. 20MHz. |
| --- | --- |
| Charter | CORESET does not need to be configured within the bandwidth of sub-band LBT |
| ITRI | Proposal 3: RAN1 should support that a UE suspends the BWP inactivity timer within a time period configured by gNB in NR-U. Proposal 4: After a COT initiated by the serving gNB, the UE suspends the BWP inactivity timer until the UE detects a signal for the transmission burst transmitted by the serving gNB. |

# References

The following documents were used for the summary:

R1-1905949	Considerations on DL reference signals and channels design for NR-U	ZTE, Sanechips

R1-1906042	DL channels and signals in NR unlicensed band	Huawei, HiSilicon

R1-1906128	Discussion on physical DL channel design in unlicensed spectrum	vivo

R1-1906195	DL signals and channels for NR-U	NTT DOCOMO, INC.

R1-1906384	Discussion on DL signals and channels for NR-U	Spreadtrum Communications

R1-1906431	On DL signals and channels for NR-U	Fujitsu

R1-1906484	DL signals and channels for NR-U	OPPO

R1-1906541	DL Signals and Channels for NR-U operation	MediaTek Inc.

R1-1906636	Discussion on the DL signals and channels for NR-U	Beijing Xiaomi Mobile Software

R1-1906656	On DL signals and channels	Nokia, Nokia Shanghai Bell

R1-1906673	Physical layer design of DL signals and channels for NR-U	LG Electronics

R1-1906761	DL signals and channels for gNB initiated COT	InterDigital, Inc.

R1-1906783	DL signals and channels for NR-unlicensed	Intel Corporation

R1-1906833	DL Signals and Channels for NR-U	Sony

R1-1906918	DL signals and channels for NR-U	Samsung

R1-1907033	DL signals and channels for NR-U	PANASONIC

R1-1907035	DL signals and channels for NR-U	ETRI

R1-1907085	DL Frame Structure and COT Aspects for NR-U	Motorola Mobility, Lenovo

R1-1907120	On NR-U DL Signals and Channels	Charter Communications

R1-1907159	Design of DL signals and channels for NR-based access to unlicensed spectrum	AT&T

R1-1907198	Discussions on DL signals and channels design in NR-U	CAICT

R1-1907210	Downlink structure and procedure for NR-U operation	Sharp

R1-1907259	DL signals and channels for NR-U	Qualcomm Incorporated

R1-1907308	Discussion on PDCCH monitoring after a COT initiated by gNB in NR-U	ITRI

R1-1907334	On COT detection and structure indication for NR-U	Apple Inc.

R1-1907373	Discussion on DL signals and channels in NR unlicensed	Google

R1-1907445	Discussion on DL signals and channels	ASUSTEK COMPUTER (SHANGHAI)

R1-1907452	DL signals and channels for NR-U	Ericsson