3GPP TSG RAN WG1 #96	                 	                           R1-1907742

Reno, USA, May 13th – May 17th 2019

Source: MediaTek Inc.

Title: CSI enhancement for MU-MIMO support

Agenda Item: 7.2.8.1

Document for:  Discussion

# Introduction

From previous meetings, agreements on MU-CSI were reached. In this contribution we provide our views on a few selected topics.

# Discussion on FD components

On RI=3-4 extension, the following agreement was reached at RAN1 #96bis:

- (L,p) setting: In RAN1#97 (Reno), down select and decide from the following alternatives:

  - Alt2B, Alt3C, Alt6E (see Table 9 from R1-1905629)

As it was agreed also that SD component selection is common across layers and RI, we need to discuss the design concerning p under Alt. 2B, Alt. 3C and Alt. 6E only.

Among the alternatives, Alt. 6E provides the maximum flexibility, however the case is still be made to justify such flexibility. In terms of design complexity, Alt. 3C and Alt. 2B are similar as for both schemes one additional parameter needs to be introduced besides y0.

To that end, we collect channel realizations to study the number of FD components required at each layer and the number of non-zero coefficients required at each layer to achieve roughly the same level of quantization error. We observe that on average the number of FD components required at each layer pretty much increases monotonically with the spatial layer index; though there are also many cases the number of FD components required is more or less the same across spatial layers. We believe there are two mechanisms leading to that:

- With RI=3 or 4, precoders for spatial layers of higher indices tend to be composed from more delay taps, than precoders for spatial layer 1.

- Assume SVD or EVD with sorted eigenvalues is used to derive the precoders at each subband, for a given spatial layer, effectively we select one eigen-vector from each subband into a set of rank-1 precoders across subbands, with the ranking of the eigenvector’s corresponding eigenvalue as the selection metric.

  - For spatial layers of higher indices, due to channel fluctuation (frequency selective fading) and frequency-dependent interference, a selected eigenvector for a spatial layer of a high index (e.g. layer 3 or layer 4) can be subjected to a large rotation.

  - For a given subspace (e.g. the precoders for Layer 3 and Layer 4), there can be many equivalent bases. Which basis is selected is also subjected to the matrix manipulation procedure. In Figure 1, it shows the selected eigenvector for Layer 1 remains relatively stable across subbands; yet selected eigen-vectors for Layer 2 or Layer 3 can suffer from large rotations due to the afore-mentioned reasons.

  - It may be possible in a careful implementation the effect due to 2) can be mitigated.

Hence between Alt. 3C and Alt. 2B, as Alt. 2B is able to facilitate selecting different numbers of FD components for Layers ½ and Layers ¾, we hvae

Proposal 1: Alt2B  from R1-1905629 is selected.

Figure 1 Subspace rotation due to frequency selection fading & frequency dependent interference

# Discussion on  N3 handling

A rank R linear combination precoder can be written as

Wr,f=BBw0,r,f⋮wl,r,f⋮wL-1,r,fwL,r,f⋮wl+L,r,f⋮w2L-1,r,fWr,f(2)

Where

B=b1⋯bl⋯bL, L is the number of spatial beams per polarization, and bi, 1≤i≤L are the spatial beams selected in a wideband fashion per polarization, and in total 2L spatial beams are selected for two polarization as in Rel-15 Type II CSI feedback;

r is the spatial layer index, 1≤r≤R, and f is a frequency index (e.g. the subband index or PRB index), 1≤f≤Ns Ns is the total number of frequency units (sub-bands, PRBs etc) over which the CSI feedback is applicable, e.g. 16 or 18.

At the NR Ad Hoc meeting in Taipei (January 2019), the following agreement was reached:

Agreement

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19,

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96

- Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is  ≥NSB×R

- Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

It is clear two alternatives (i.e. padding vs segmentation) are to be downselected.

## Candidate schemes for padding

If Ns>13,  then a pre-processing step is needed to pad some frequency units to segment 0, so the resulted number of frequency units is N3 .

Here we consider a number of options for padding as illustrated in Figure 2:

- Alt 1-1: a few subband(s) at the start and the end of the reporting band are copied and placed at the original Ns subbands; more specifically ⌈N3-Ns2⌉ subbands at the start are copied and prepended to the original Ns subbands; and ⌊N3-Ns2⌋ subbands at the end are copied and appended to the subbands.

- Alt. 1-3: a few subband(s) at the start and the end of the reporting band are copied and placed, more specifically the first subband in original Ns subbands   is copied and duplicated into  ⌈N3-Ns2⌉ copies and prepended to the original Ns subbands; and the last subband in original Ns subbands is copied and duplicated into    ⌊N3-Ns2⌋ copies and appended to the subbands;

- Alt. 1-2: ⌈N3-Ns2⌉  and ⌊N3-Ns2⌋  subbands (zeros for all coefficients) are appended to the  original Ns subbands;

- Alt. 1-4: Consider when DFT is applied to a sequence with a few tones, and sudden changes in phase/amplitude (e.g. between the first element and the last element in the sequence as they are adjacent with circular indexing) may result in many non-zero frequency components in the resulted sequence, smoothing the transition from one element to another element in the original sequence may help reduce the number of non-zero frequency components. From that, the padded CSI subbands consist of weighted combinations of the last subband and the first subband of the original Ns subbands. The weighted function can a raised cosine function.

Figure 2 Padding schemes with padding from both sides

From Figure 2, it can be seen that depending the number of padded subbands, the gNB needs to discard a number of CSI subbands at the beginning of the reported CSI, so the PMI/CQI for the original Ns subbands can be identified correctly. In another word, a mapping function is needed to map the Ns subbands out of N3 subbands. In Figure 3, instead of padding CSI subbands on both ends, all the padded subbands are appended to the Ns subbands, hence the first Ns subbands out of N3 subbands of the CSI report are for the original Ns subbands.

Figure 3 Design choices with padding on a single side

Here we provide more details on Alt. 1-4b:

- Let the precoding coefficient of the first subband in the original Ns subbands be X1;

- Let the precoding coefficient of the last subband in the original Ns subbands be X2;

- Then the assumed precoding coefficient for subband k, Ns<k≤N3 is given by fk-Ns,N3-NsX2+ gk-Ns,N3-Ns)X1;

  - where fd,Δ, 1≤d≤Δ,   Δ=N3-Ns is a weighting function, with 0≤fd,Δ≤1 for d; gd,Δ, 1≤d≤Δ,   Δ=N3-Ns is a weighting function, with 0≤gd,Δ≤1, for all d;

  - fd,Δ and gd,Δ can be used to create tapering/weighting between X1 and X2.

  - One can require gd,Δ=f(Δ-d+1,Δ) to simplify the design.

  - The raised cosine function can be used for fd,Δ:

    - If d≤1-β2Δ+1, fd,Δ=1;

    - If 1-β2Δ+1< d≤1+β2Δ+1,  fd,Δ=12[1+cos⁡(πβΔ+1d-1-β2Δ+1)];

    - If d>1+β2Δ+1, fd,Δ=0.

    - If β=1, fd,Δ=12(1+cosdΔ+1×π).

Other design choices are also possible: denote fd,Δ as αd,d=1,…,Δ.  For Δ=3, for example we have

The intuition behind Alt 1-4b is that at a given spatial layer, the precoding coefficients for the Ns subbands change slowly with frequency; in another word it should have power concentrated over low frequency components (around the DC) rather than high frequency components. To facilitate discussion below, let us introduce a few notations:

Approximately at Ns subbands, the precoding coefficients hn,  1≤n≤Ns, are given by

hn≈m=1Mamej2πnωm  where am is the channel gain for delay tap m, and ωmis the phase ramp for delay tap m. And we have X1=h1, X2=hNs.

Typically ωm≠kN3  for any integer k; hence the phase/amplitude discontinuity between X1 and X2 leads to non-zero coefficients  showing up at many FD components after DFT. By using the weighting function such as the raised cosine function, then a sudden change between X1 and X2 is avoided, and the change between X1 and X2 is made gradual, and consequently the number of coefficients with significant power showing up at FD components after DFT is reduced. The contrast is illustated in Figure below.

Figure 4 (a) Abrupt change without padding (b) Gradual change with padding

As the processing for Alt 1-1b, 1-2b, 1-3b and 1-4b is simpler than Alt. 1-1, 1-2, 1-3 and 1-4, we have

Proposal 2: Candidate schemes for padding include Alt 1-1b, 1-2b, 1-3b and 1-4b.

## Candidate schemes for segmentation

With a given Ns, if Ns has a prime factor other than 2, 3 or 5, then find the least number N3 which has 2, 3 or 5 only as prime factor(s)  and is larger than or equal to  Ns2, the first segment consists of CSI subbands 1, …, N3, and the second segment consists of CSI subbands Ns-N3+1, …, Ns.

We have

Proposal 3: Consider the following scheme for segmentation:

With a given Ns, if Ns has a prime factor other than 2, 3 or 5, then find the least number N3 which has 2, 3 or 5 only as prime factor(s)  and is larger than or equal to  Ns2, the first segment consists of CSI subbands 1, …, N3, and the second segment consists of CSI subbands Ns-N3+1, …, Ns.

# Conclusions

In this contribution, we provide our views on MU CSI enhancement. We have

Proposal 1: Alt2B  from R1-1905629 is selected.

Proposal 2: Candidate schemes for padding include Alt 1-1b, 1-2b, 1-3b and 1-4b.

Proposal 3: Consider the following scheme for segmentation:

With a given Ns, if Ns has a prime factor other than 2, 3 or 5, then find the least number N3 which has 2, 3 or 5 only as prime factor(s)  and is larger than or equal to  Ns2, the first segment consists of CSI subbands 1, …, N3, and the second segment consists of CSI subbands Ns-N3+1, …, Ns.

# Appendix (simulation evaluation)

Let Ns  be the number of CSI subbands before padding. N3-Ns  subbands are padded to the original Ns subbands where N3 is the smallest integer which is a multiple of 2, 3 or 5 and is larger than  Ns .

Option 1:

N3-Ns  subbands are appended to the original Ns subbands.

Option 2:

⌈N3-Ns2⌉ subbands are prepended to the original Ns subbands; and ⌊N3-Ns2⌋ subbands are appended to the original Ns subbands.

Let the precoding coefficient of the subband k in the original Ns subbands be Xk; 1≤k≤Ns.

Let N3-Ns  be the padding size.

As for padding schemes, with Option 1:

- Padding with average:

  - with padding size at 3: XNs+1= XNs+2=XNs+3=(X1+X2+…+XNs)/ Ns

  - with padding size at 2: XNs+1= XNs+2=(X1+X2+…+XNs)/ Ns

  - with padding size at 1: XNs+1=(X1+X2+…+XNs)/ Ns

- padding with the raised cosine function:

  - with padding size at 3:

    - XNs+1=α1XNs+1-α1X1, α1=0.8536,

    - XNs+2=α2XNs+1-α2X1, α2=0.5,

    - XNs+3=α3XNs+1-α3X1, α3=0.1464.

  - with padding size at 2:

    - XNs+1=β1XNs+1-β1X1,  β1=34,

    - XNs+2=β1XNs+1-β1X1, β2=14.

  - with padding size at 1:

    - XNs+1=γ1XNs+1-γ1X1, γ1=1/2.

- Zero padding: i.e. Xk=0, Ns+1≤k≤N3

- Repetition padding,

  - with padding size at 3: XNs+1=XNs, XNs+2=X1,XNs+3=X1

  - with padding size at 2: XNs+1=XNs, XNs+2=X1

  - with padding size at 1: XNs+1=XNs

## Simulation evaluation

With resource utilization around 50%, with 16 TX and 2 RX and 30 KHz carrier spacing, we have performed system level simulations, and provide simulation results below, which actually achieve gains over the non-padding cases.

| Number of subbands | PRBs/subband | Padding size | Cell edge throughput gain over non-padding | 50% throughput gain over non-padding | Padding scheme |
| --- | --- | --- | --- | --- | --- |
| 37 | 2 | 3 | 2.17% | 1.05% | Raised cosine function |
| 21 | 4 | 3 | 0.8% | 1.53% | Raised cosine function |