3GPP TSG RAN WG1 Meeting #97	R1-1907745

Reno, USA, May 13th-17th, 2019

Source:	Ericsson

Title:	Additional results comparing rank2 vs rank1 with SDM and TDM multi-TRP schemes for URLLC

Agenda Item:	7.2.8.5

Document for:	Discussion and Decision

# Introduction

In RAN1#96bis, it was agreed that multi-TRP schemes 1a, 3, and 4 will be supported in NR Rel-16 for reliable PDSCH transmission for URLLC.   One remaining question for the different schemes is whether only rank 1 per TRP or higher rank should be supported.  In [1],  we present some results of FDM single RV (Scheme 2a) with rank 2 vs rank 1 per TRP and show that rank 2 per TRP provides better BLER performance than rank 1 per TRP.

In this contribution, we present some additional results comparing rank 2 vs rank 1 per TRP with Scheme 1a (SDM) and Scheme 3 (TDM with mini slot repetition). This is a revision of R1-1907725.

# Simulation Assumptions

Downlink transmission at 4 GHz carrier frequency with 10MHz BW and 15 kHz sub-carrier frequency spacing is considered. It is assumed that the two TRPs are connected with an ideal backhaul and under the control of a single scheduler.  CDL-A channel model with 300ns channel delay spread was used. Two cross polarized antennas (and thus 4 antenna ports) are used in each TRP and at the UE.

The schemes evaluated are:

- Scheme 1a: SDM with a single CW over 2 TRPs with either rank 1 or rank 2 per TRP

- Scheme 3: TDM repetition over two mini-slots, each over one TRP, with either rank 1 or rank 2 per TRP

They are illustrated in Figure 2.

(a) Scheme 1a with one layer per TRP

(b) Scheme 1a with 2 layers per TRP

(c) Scheme 3 with one layer per TRP

- Scheme 3 with 2 layers per TRP

Figure 2: The schemes evaluated

The resource allocation is shown in Figure 3, 8RBs by 4 OFDM symbols are allocated. A single symbol DMRS of type 1 was configured without data multiplexing.  A fixed TB size of 256bits was used, the corresponding code rates are approximately

- R = 0.44 for Scheme 3 with rank 1 per TRP

- R = 0.22 for scheme 3 with rank 2 per TRP and Scheme 1a with rank 1 per TRP.

- R=0.11 for Scheme 1a with rank 2 per TRP

0dB pathloss (PL) offset between the two TRPs was assumed.  Other simulation assumptions can be found in the Appendix.

(a) Scheme 1a

(b) Scheme 3

Figure 3: Resource allocation.

# Evaluation Results

The evaluation results Scheme 1a and Scheme 3 are shown in Figure 4 and Figure 5, respectively. It can be seen that for both schemes, rank 2 per TRP has better BLER performance than rank 1 per TRP at 10^(-4) BLER.

Figure 4:  Rank 2 vs. rank 1 per TRP:  Scheme 1a.

Figure 5: Rank 2 vs. rank 1 per TRP:  Scheme 3.

Rank 2 per TRP out performs rank 1 per TRP for both Scheme 1a and Scheme 3 in the evaluated scenario.

# Conclusions

In this contribution, we have presented addition evaluation results comparing rank 2 vs. rank 1 per TRP for multi-TRP Schemes 1a and 3 for URLLC. Based on the results, we have the following observation:

Observation 1	Rank 2 per TRP out performs rank 1 per TRP for both Scheme 1a and Scheme 3 in the evaluated scenerio.

# References

R1-1907418, “On multi-TRP and multi-panel”, Ericsson, RAN1#97.

# Appendix:  Simulation Assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| BW | 10MHz |
| Subcarrier spacing | 15kHz |
| Channel model | CDL-A with 300ns delay spread |
| UE mobility | 3km per hour |
| Number of TRPs | 2 |
| Pathloss offset of 2nd TRP | 0dB |
| Number of Tx antenna ports per TRP | 4 |
| Number of UE Rx antennas | 4 |
| Number of layers per TRP | 1 or 2 |
| DMRS configuration | One symbol, type 1, no data and DMRS multiplexing |
| Number RBs | 8 |
| Number of OFDM symbols | 4 |
| TBS size | 256 bits |
| Modulation | QPSK |
| Channel estimation | Practical |
| Receiver | MMSE-IRC |
| Precoding | Wideband PMI |