3GPP TSG-RAN WG1 Meeting #97	R1-1907770

Reno, USA, May 13th – 17th, 2019

Title:	[Draft] Reply LS on the usage of SDAP in MTSI and re-usability of delay and error profiles

Response to:	R1-1905923/S4-190555

Release:	-

Work Item:	-

Source:	Samsung [RAN WG1]

To:	SA WG4

Cc:	RAN WG2, SA WG2, GSMA 5GJA

Contact Person:

Name:	Youngbum Kim

E-mail Address:	youngbum.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 would like to thank SA4 for the LS on the usage of SDAP in MTSI and re-usability of delay and error profiles (R1-1905923/S4-190555). RAN1 has discussed the issues and provides SA4 the following feedback:

| WG | Question |
| --- | --- |
| RAN1 | 3. The traces of error-delay profiles for the support of the conversational voice services were developed for HSPA by RAN1. SA4 used them also for LTE. Can RAN1 confirm the profiles can be applicable in NR? => Answer: It is RAN1 understanding that the error-delay profiles can be also applicable in NR. |
| RAN1 | 4. The radio parameters in Table E.2 of TS 26.132 for constructing the profiles were used for acoustic testing of UEs over LTE (and Wi-Fi). Can RAN1 confirm that the same set of parameters could be applicable in NR? If not, can RAN1 confirm that the delay statistics of NR are not worse than in the LTE? => Answer: It is RAN1 understanding that the same set of parameters can be also applicable in NR. |
| RAN1/ RAN2 | 6. SA4 wonders if even in NR, there will be preferred uplink TBSs designed to offer optimized transmissions. If so, SA4 would like to be informed of these preferred TBSs. SA4 would also like to understand the potential penalty, e.g., in transmission efficiency, latency, etc., if the preferred TBSs cannot be matched. => Answer: NR uplink TBS determination is specified in section 6.1.4.2 of TS38.214 where Table 5.1.3.2-1 is referenced to be used for small TBS values. NR TBSs are defined to accommodate at least the TBSs supported for LTE VoIP |

2. Actions:

To SA WG4 group.

ACTION: 	RAN1 respectfully asks SA4 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #98	26 – 30 Aug 2019	Prague, Czech Republic

TSG-RAN WG1 Meeting #98bis	14 – 18 October 2019	China