3GPP TSG RAN WG1 Meeting #97 R1-1907778 Reno, USA, 13th – 17th May, 2019 CR-Form-v12.0. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 15.5.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on QCL indication for PDSCH. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2019-5-15. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.214, the QCL assumption condition of non-configured QCL-TypeD for PDSCH is not only related to the serving cell of scheduled PDSCH, which means that the QCL-TypeD configuration for the other CC also impacts UE behavior in the current CC when considering the cross-CC case. Consequently, for example, in FR1+FR2 CA case, there is unnecessary latency for beam switching for PDSCH.. -. Summary of change:; Adding “for the serving cell of scheduled PDSCH” to clarify that the QCL assumption condition should be that none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD'.. Consequences if not approved:; If the change is not approved, UE behavior for QCL indication for PDSCH is not correct.. Clauses affected:; 5.1.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR fixes the issue of the QCL indication for PDSCH. It is expected UEs and networks are implemented in accordance to this CR and therefore no change is required on UE and network implementation.. This CR's revision history:; This is the first version for this CR..

## 5.1.5	Antenna ports quasi co-location

<Unchanged part omitted>

If the tci-PresentInDCI is set as 'enabled', the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP and when the PDSCH is scheduled by DCI format 1_1, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling, the UE expects tci-PresentInDci is set as 'enabled' for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains 'QCL-TypeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and the corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL

For both the cases when tci-PresentInDCI is set to 'enabled' and tci-PresentInDCI is not configured in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

<Unchanged part omitted>