3GPP TSG RAN WG1 97			R1-1907783

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary on offline email discussion for N3 issue

Document for:	Discussion and Decision

Introduction

The following agreement was made online on UCI design:

Agreement

On the value of N3 for N3=NSB×R>13:

- For Alt1 (padding), consider only extrapolation-based scheme and decide on the final specific design alternatives in RAN1#97 (Reno) for down selection in RAN1#98 (Prague)

Summary

Alt1 (padding) is described as follows:

Notation: - Ns: the number of FD compression units (derived from the number of CSI subbands and R) before padding. - N3: the smallest integer which is a multiple of 2, 3 or 5 and is larger than Ns. - {Xk, 1≤k≤Ns}: The precoding coefficient of the unit k in the original Ns units. - θ=N3-Ns: extrapolation length - Y0,…,Yθ-1: padding coefficients Procedure: ⌈N3-Ns2⌉ units are prepended to the original Ns units; and ⌊N3-Ns2⌋ units are appended to the original Ns units, resulting in: - When θ=3: [Y1, Y2, X1,X2,…,XNs, Y0 ] - When θ=2: [Y1, X1,X2,…,XNs, Y0 ] - When θ=1: [Y0 , X1,X2,…,XNs] Padding signal (extrapolation): - Alt 1.1: Average: Yt= (X1+X2+…+XNs)/Ns, t=0,1,…,θ-1 - Alt 1.2: Raised cosine extrapolation: Yt= αtXNs+1-αtX1, t=0,1,…,θ-1 - When θ=3: α0=0.8536, α1=0.5, α2=0.1464 - When θ=2: α0=0.75, α1=0.25 - When θ=1: α0=0.5 - Alt 1.3: Repetition: - If θ=3 or 2, Y0=XNs, Y1=Y2=X1 - If θ=1, Y0=X1 - Alt 1.4: Zero padding: Yt= 0, t=0,1,…,θ-1

Offline agreement: On Alt1 (padding) for N3, for evaluation purposes, select one of Alt1.1, 1.2, 1.3, and 1.4 in RAN1#98 (Prague) as described in the above table.