3GPP TSG RAN WG1 Meeting #97	R1-1907790

Reno, USA, May 13th – 17th, 2019

| Source: | Nokia, Nokia Shanghai Bell, Thales |
| --- | --- |
| Title: | TP for Link Budget Analysis in TR 38.821 |
| Document for: | Discussion and Decision |
| Agenda Item: | 7.2.5.5 |

# 1 Introduction

A new Study Item on “Study on Solutions for NR to Support Non-Terrestrial Networks” was approved in RAN#80 meeting [1] and further updated in RAN#82 meeting [2] and RAN#83 meetings [3] with the considered scenarios of transparent GEO satellite and transparent/regenerative LEO satellite (moving beam on earth) for pedestrian UEs and on board vehicle UEs in NTN. This contribution provides TP for link budget analysis in TR 38.821.

# 2 Text Proposal for Link Budget Analysis in TR 38.821

Start of changes

2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)"

[3]	3GPP TS 38.401; NG-RAN; Architecture description (Release 15)

[4]	3GPP TR 38.874 NR-Study on IAB (Integrated Access and Backhaul)

[5]	3GPP TS 37.340; NR; Multi-connectivity; Overall description

[6]	RP-181370, Study on solutions evaluation for NR to support Non Terrestrial Network

[7]	3GPP TS 23.501 V15.0.0 : System Architecture for the 5G System

[8]	R3-184403, NR-NTN: Paging in NGSO Satellite Systems

[9]	3GPP TS 37.324 V15.0.0 “E-UTRA and NR; Service Data Adaptation Protocol (SDAP) specification (Release 15)”

[10]	AIAA 2006-6753, Revisiting Spacetrack Report #3 , Models for Propagation of NORAD Element Sets, David A. Vallado, Paul Crawford, Richard Hujsak, T. S. Kelso, presented at the AIAA/AAS Astrodynamics Specialist Conference, Keystone, CO, 2006 August 21–24, 2006

[z1]	3GPP TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”

[z2]	3GPP TS 38.214, “NR; Physical layer procedures for data”

End of changes

Start of changes

X.Y Link Budget Analysis

X.Y.1 Link Budget Calculation

X.Y.1.1 CNIR

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise-plus-interference-density ratio (CNIDR) and bandwidth B as follows

|  | CNIR dB=CNIDRdBHz-B(dBHz) | (X.Y-1) |
| --- | --- | --- |

where CNIDR can be obtained by carrier-to-noise-density ratio (CNDR) and carrier-to-interference-density ratio (CIDR) as follows

|  | CNIDR dB=10log1010-0.1CNDRdB+10-0.1CIDRdB | (X.Y-2) |
| --- | --- | --- |

The formula for CNDR calculation is

|  | CNDR dBHz=EIRPdBW+GT(dB/K)-k(dBW/K/Hz)-PLFS(dB)-PLA(dB)-PLS(dB)-PLAD(dB) | (X.Y-3) |
| --- | --- | --- |

where EIRP is effective isotropic radiated power (EIRP), k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLS is shadowing margin, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and GT is antenna-gain-to-noise-temperature.

Antenna-gain-to-noise-temperature GT can be derived by [2]

|  | GTdB=GR dBi-NfdB-10log10T0(K)+Ta(K)-T0(K)10-0.1Nf(dB) | (X.Y-4) |
| --- | --- | --- |

where GR is receive antenna gain,  Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature.

Receive antenna gain GR can be obtained by

|  | GRdBi=GR,edBi+10log10NR,a-LR,pdB | (X.Y-5) |
| --- | --- | --- |

where GR,e is receive antenna element gain, NR,a is the number of receive antenna elements, and LR,p is polar loss at the receiver side. If the signal is circular polarized signal, then LR,p can be assumed to be -3dB for single omni-directional receive antenna and [0dB] for cross-polarization receive antenna and circular polarization antenna (including parabolic antenna).

CIDR can be derived by carrier-to-interference ratio (CIR) and bandwidth as follows

|  | CIDR dBHz=CIRdB+B(dBHz) | (X.Y-6) |
| --- | --- | --- |

EIRP can be calculated by

|  | EIRP dBW=PT dBW-LC dB+GT dBi | (X.Y-7) |
| --- | --- | --- |

where PT is antenna transmission power, LC is cable loss, and GT is transmit antenna gain.

GT can be derived by

|  | GTdBi=GT,edBi+10log10NT,a-LT,pdB | (X.Y-8) |
| --- | --- | --- |

where GT,e is transmit antenna element gain, NT,a is the number of receive antenna elements, and LT,p is polar loss at the transmitter side. If the signal is circular polarized signal, then LT,p can be assumed to be -3dB for single omni-directional receive antenna and [0dB] for cross-polarization receive antenna and circular polarization antenna (including parabolic antenna)

X.Y.1.2 Throughput

The approximate maximal throughput can be calculated by [z1]

|  | Throughput (Mbps) = 10-3*j=1JTBSj 10-3⋅TBS | (X.Y-9) |
| --- | --- | --- |

where TBS is the total maximum number of transport block bits received within a 1ms TTI, as derived from TS 38.214 [z2].

X.Y.2 Simulation Assumptions

The simulations assumptions for link budget analysis are listed in Table X.Y.2-1, Table X.Y.2-2 and Table X.Y.2-3.

Table X.Y.2-1 Simulation assumptions for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| Bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Satellite altitude | 600km, 1200km, 35786km |
| SCS | 30kHz (S band), 120kHz (Ka band) |
| Elevation angle | 10, 30, 90 |
| Atmospheric loss | Equation (6.6-8) in TR 38.811 |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor (Note 1) | - 3 - FFS: 1 |
| Average C/I within a satellite beam | FFS |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable Note: Polarization re-use [Reference] should apply only if circular polarization for terminal antenna is considered. |
| Terminal type | - Ka band: VSAT, (M, N, P) = (4, 8, 2) - S band: (M, N, P) = (1,1,2) |
| Terminal speed | 0 km/h |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Fast fading channel model | - AWGN - FFS: NTN-CDL-D in TR 38.811 |
| Target BLER | - FFS: [10%], [1%], [0.1%], [0.001%] |
| Terminal RF parameters | Table X.3 |
| Satellite RF parameters | Set-1 in Table X.1 and Set-2 in Table X.2 |
| Doppler shift and variation | Compensated |
| Outcome | CIDR, MCS, Throughput (optional) |
| Note 1: Frequency reuse factor of 1 is not precluded from simulation studies. |  |

X.Y.3 Link Budget Analysis

End of changes