3GPP TSG-RAN WG1 Meeting #97	R1-1907808

Reno, USA, May 13th – 17th 2019

Agenda Item:	7.2.5.1

Source:	Thales

Title:	Summary#3 on simulation assumptions for NTN

Document for:	Discussion

# Introduction

This document aims to capture the potential agreements based on the companies’ contributions and the offline discussions. The proposals are focused on LLS parameters and assumptions:

# Potential agreements :

Proposal #0 : The updated table is proposed for agreement to capture the UE characteristics :

Table X.3 : UE characteristics for System Level Simulations

| Characteristics | VSAT | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of TR 38.811 with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (4,8,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65 deg) |
| Polarization | circular | Linear : +/-45°X-pol | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | 8dBi per element |
| Antenna temperature | 150 K | 290 K | 290 K |
| Noise figure | 1.2 dB | 7 dB | 9 dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [20 W (43 dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | 8dBi per element |
| Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. |  |  |  |

Proposal #1 : The following tables are proposed for agreement to capture the impairments introduced on the RF signal due to the satellite payload and movement.

Table X.9 : Impairments due to satellite payload and satellite movement

|  | S-band | Ka-band |
| --- | --- | --- |
| Satellite HPA model | Table Y.9 from R1-1907481 (ESA) | Table Y.8 from R1-1907481 (ESA) |
| Phase noise model (Note 2) | Table Y.6 from R1-1907481 (ESA) | Table Y.7 from R1-1907481 (ESA) |
| On-board oscillator long-term drift | FFS ppm | FFS ppm |
| Max Doppler shift (Note 1) | Scenario A : 0.15 ppm Scenario C2/D2 : - 1200 km : 20 ppm - 600 km : 24 ppm |  |
| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario A : n/a Scenario C2/D2 : - Satellite alt. = 1200 km : - beam diameter = 90 km (Set 1 - S-band) : 0.91 ppm - beam diameter = 40 km (Set 1 - Ka-band): 0.40 ppm - beam diameter = 190 km (Set 2 - S-band) : 1.91 ppm - beam diameter = 90 km (Set 2 - Ka-band) : 0.91ppm - Satellite alt. = 600 km : - beam diameter = 50 km (Set 1 - S-band) : 1.05 ppm - beam diameter = 20 km (Set 1 - Ka-band) : 0.42 ppm - beam diameter = 90 km (Set 2 - S-band) : 1.88 ppm - beam diameter = 50 km (Set 2 - Ka-band) : 1.05 ppm |  |
| Max Doppler rate | Scenario A : n/a Scenario C2/D2 : - Satellite alt. = 1200 km :0.09 ppm/s - Satellite alt. = 600 km : 0.27 ppm/s |  |
| Note 1 : Min. Elevation angle for both sat- user equipment is equal to 10 degree. Note 2 : For regenerative scenario, this can be considered as the phase noise model for the gnB. For transparent scenarios, it should be considered as an additional phase noise w.r.t the phase noise generated by the gnB and the UE. |  |  |

Table Y.6 : S-band satellite phase noise mask

| S-Band | 10 Hz | 100 Hz | 1 kHz | 10 kHz | 100 kHz | 1 MHz | 10 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| dBc/Hz (SSB) | -29 | -59 | -69 | -74 | -83 | -95 | -101 |

Table Y.7: Ka-band satellite phase noise mask

| Ka-Band | 100 Hz | 1 kHz | 10 kHz | 100 kHz | 1 MHz | 10 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| dBc/Hz (SSB) | -62 | -80 | -90 | -95 | -106 | -116 | -118 |

Table Y.8: Typical AM/AM and AM/PM characteristics for linearized TWTA.

| IBO [dB] | OBO [dB] | Phase [deg] |
| --- | --- | --- |
| -20 | -18 | 0 |
| -18 | -16 | 0 |
| -16 | -14 | -0.5 |
| -14 | -12 | -0.5 |
| -12 | -10 | -0.5 |
| -10 | -8 | -1 |
| -8 | -6 | -1 |
| -6 | -4 | -1 |
| -4 | -2 | -2.5 |
| -3 | -1 | -3 |
| -2 | -0.5 | -4 |
| -1 | -0.1 | -6 |
| 0 | 0 | -8 |
| 1 | -0.1 | -7 |
| 2 | -0.5 | -6 |
| 3 | -1 | -4 |
| 4 | -2 | -2 |
| 5 | -3 | -2 |

Table Y.9 : Typical AM/AM and AM/PM characteristics for SSPA

| IBO [dB] | OBO [dB] | Phase [deg] |
| --- | --- | --- |
| -20 | -18.7 | 0 |
| -18 | -16.7 | 0 |
| -16 | -14.7 | 0.5 |
| -14 | -12.7 | 1 |
| -12 | -10.7 | 2.5 |
| -10 | -8.6 | 3.5 |
| -8 | -6.5 | 5 |
| -6 | -4.5 | 6 |
| -4 | -2.6 | 7 |
| -3 | -1.9 | 7 |
| -2 | -1.2 | 7 |
| -1 | -0.6 | 6.5 |
| 0 | 0 | 6 |
| 1 | 0.5 | 5 |
| 2 | 0.9 | 4 |
| 3 | 1.1 | 3.5 |
| 4 | 1.3 | 3 |
| 5 | 1.3 | 3 |

| Companies | Comments / Proposals |
| --- | --- |

Proposal #10 : The following table is proposed for agreement on LLS parameters for DL synchronization performance evaluation :

Table X.10 : LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| SNR range | > -6 dB | > -18dB |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO (optional) : 10°, For LEO : 30° |  |
| Frequency Offset | - UE crystal accuracy: [10] ppm - Satellite : oscillator accuracy values provided in Table X.9 - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table X.9 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle Note 1 : The final frequency offset is the sum up of all the contributions Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table X.9] |  |
| Phase noise model | FFS |  |
| Metrics | CDF of timing and frequency residual offset at SNIR point corresponding to 90% likelihood. Note 4: FAR requirement = [1%] |  |

| Companies | Comments / Proposals |
| --- | --- |

Proposal #11 : The following table is proposed for agreement on LLS parameters for PRACH performance evaluation.

Table X.11 : LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Antenna Configuration at the TRP (satellite) | [1 or 2]Rx | [1 or 2]Rx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | FFS |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Initial timing Offset (Note 1) | FFS |  |
| Phase noise model | FFS |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate , FAR |  |
| Note1 : Ideal common delay compensation is assumed. |  |  |

| Companies | Comments / Proposals |
| --- | --- |

Proposal #12 : The following table is proposed for agreement on LLS parameters for data transmission performance evaluation.

Table X.12 : LLS parameters for data transmission performance evaluation

| Parameters | S-band |  | Ka-band |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz |  | DL 20GHz |
| Channel coding scheme | NR channel coding |  |  |
| Subcarrier spacing | 15kHz, 30kHz |  | 60kHz, 120kHz |
| Modulation order, Coding rate | To be reported by the companies |  |  |
| Channel estimation | Realistic estimation |  |  |
| Frequency offset | Residual Frequency error after DL synchronisation : FFS |  |  |
| Frequency drift | [Doppler rate values provided in Table X.9] |  |  |
| Frequency tracking | FFS |  |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |  |
| Channel model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |  |
| TRP antenna configuration | 1Tx |  | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | FFS |  |  |
| Metrics | BLER, Throughput |  |  |

| Companies | Comments / Proposals |
| --- | --- |

# Reference

[1] R1-1907087, On beam layout definition for NTN system level simulations, Thales
[2] R1-1907687, On beam layout definition for NTN system level simulations, Thales
[3] R1-1906463, Link budget and Doppler Aspects for evaluation of NR NTN, MediaTek Inc.
[4] R1-1906870, Discussion on the simulation assumption for NTN, ZTE
[5] R1-1906323, Simulation assumption for NTN evaluation, CATT
[6] R1-1906802, Considerations on performance evaluation for NTN, Intel Corporation
[7] R1-1906085, Considerations on Performance Evaluation in NTN, Nokia
[8] R1-1905992, Discussion on performance evaluation for NTN, Huawei
[9] R1-1906951, Remaining issues on NTN link-level and system-level simulation assumptions, Samsung
[10] R1-1907369, On Dowlink Performance in NTN, Fraunhofer IIS
[11] R1-1907481, Beam size computation and alternative satellite specifications, ESA
[12] R1-1907395, On NTN link level and system level evaluations, Ericsson
[13] R1-1906090, TP for Link Budget Analysis in TR 38.821, Nokia
[14] R1-1905997, Discussion on link budget for NTN, Huawei
[15] R1-1906876, Preliminary link budget for NTN, ZTE
[16] Radar System Engineering, Lecture 9, Antennas, Robert M.O’Donnel, IEEE New Hampshire