3GPP TSG RAN WG1 Meeting #97	R1-1907826

Reno, USA, 13th– 17th May, 2019

Title:	Draft LS on NR control for LTE sidelink

Response to:	-

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Huawei, [RAN1]

To:	RAN

Cc:	-

Contact Person:

Name:	                 Philippe Sartori

E-mail Address:	  philippe.sartori@huawei.com

# 1. Overall Description:

At RAN#83, RAN approved a WID on NR V2X (RP-190766) with the following objective:

2. Specify support for NR Uu to provide control for LTE sidelink

- […]

- Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2].

RAN1 to make a decision on which option is supported until RAN#84.

RAN1 reached the following agreement on NR Uu to provide control for LTE sidelink:

- RRC-based activation/deactivation is not supported

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

# 2. Actions:

RAN1 respectfully asks RAN to take the above into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #98		26 – 30 August 2019, Prague, CZ

TSG RAN WG1 Meeting #98bis		14 – 18 October 2019, TBD, CN