3GPP TSG RAN WG1 97			R1-190xxxx

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary on Tue offline session for MU-MIMO CSI

Document for:	Discussion and Decision

Introduction

The following agreement was made online on UCI design:

Agreement

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- On SCI for RI>1 (reported in UCI part 2), down-select among the two following alternatives:

  - Alt3.3: Per-layer SCI, where SCIi is a log2K0-bit indicator (i=0,1,…,(RI-1)), including further reducing the bitwidth if applicable (to log2minK0,2LMi)

  - Alt3.4: Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1))

- Further discuss offline on the possible down-selection

Agreement

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- RI (∈1,…,RIMAX) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1

  - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded

- For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…,RI-1, where i denotes the i-th layer) are reported in UCI part 2

  - FFS: If alt 3-4 is supported, size-2LMi-1 (i=0,1,…,RI-1, where i denotes the i-th layer) are reported in UCI part 2

- The following FD basis subset selection scheme is supported:

  - For N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

    - FFS (to be finalized in RAN1#97 Reno): The subset selection is indicated by an X1-bit combinatorial indicator (for each layer) in UCI part 2 where, depending on the outcome of SCI, X1 is either log2N3Mi (if Alt3.3 is selected) or log2N3-1Mi-1 (if Alt3.4 is selected)

  - For N3>19, IntS is window-based and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where α is higher-layer configured from two possible values

      - FFS (to be finalized in RAN1#97 Reno): the two values of α including whether this value is rank common or rank specific

      - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for L,p,β,α

    - FFS (to be finalized in RAN1#97 Reno): whether Minitial is reported in UCI part 2 (along with its possible values and bitwidth) or fixed

  - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

    - FFS (to be finalized in RAN1#97 Reno): depending on the outcome of SCI, X2 is either log2N3'Mi (if Alt3.3 is selected) or log2N3'-1Mi-1 (if Alt3.4 is selected)

Summary

## UCI design: two values of α

Offline agreement: In RAN1#98, finalize the values of α based on the following aspects

- Candidate values for α to be down selected/evaluated: at least {1.5, 2, 2.5}

  - The set of values is to be finalized via offline email discussion prior to RAN1#98

- Configuration of α:

  - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R

  - Whether α is rank-specific or rank-common

  - Note: This is to be discussed along with the supported parameter combinations for L,p,β,α

## UCI design: SCI and FD basis subset selection indicator

Since SCI design and some components of FD basis subset selection (combinatorial indicators and Minitial) are inter-related, they will be considered jointly. There are two (joint) alternatives.

Table 1 Two alternatives

|  | Alt A | Alt B |
| --- | --- | --- |
| SCI for RI>1 | Alt3.3: Per-layer SCI, where SCIi is a log2minK0,2LMi-bit indicator (i=0,1,…,(RI-1)) | Alt3.4: Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1)). The location of the strongest LC coefficient for layer i before index remapping is (li*,mi*) and SCIi=li* |
| Index remapping | None | (li*,mi*)→(li*,0); LC coefficient index: (li,mi)→(li,mi-mi*modMi); Codebook index (associated with LC coefficient index (li,mi)): kmi→kmi-kmi*modN3 |
| Combinatorial indicator for N3≤19 | log2N3Mi bits | log2N3-1Mi-1 bits |
| Combinatorial indicator for N3>19 | log2N3'Mi bits | log2N3'-1Mi-1 bits |
| Minitial | Fixed to 0 | Reported in UCI part 2, details on bitwidth and possible values are FFS |
| Companies’ views | Support (4): Fraunhofer/HHI, Qualcomm, LGE | Support (17): Huawei/HiSi, Ericsson, Samsung, Nokia/NSB, MotM/Lenovo, vivo, MediaTek, Intel, ZTE, OPPO, CATT, NEC, Spreadtrum, APT |

Qualcomm presented R1-1907767, arguing that (using a hypothetical LC coefficient profile) AltB could result in an uncaptured non-zero LC coefficient. Some companies questioned the validity of the argument given that the profile seems hypothetical (constructed) and SLS results demonstrating the claim are absent from the Tdoc.

Offline observation: Alt B represents the super-majority view.

Offline proposal: On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the above table.

- FFS: details on bitwidth and possible values for Minitial reporting in UCI part 2

## UCI design: RI=3-4 bitmap

Starting from the following possible agreement:

Possible Agreement

On the candidate UCI parameters listed in Table 2 of R1-1905629:

- The bitmap should have at least one entry of ‘1’ to for each polarization,

the offline discussion on this issue can be summarized as follows:

- Whether the proposal is necessary, e.g. with “proper” FD compression parameter configuration, what is the likelihood that all the LC coefficients for a polarization end up being “zero”?

  - Yes: Huawei/HiSi, CATT, Samsung

  - No: vivo, Intel, Qualcomm

- Should this be discussed in conjunction to the down scoping of FD compression parameter combinations? Or, how is this captured in the specification?

  - If captured in the specification, the specification will restrict the UE from reporting all “zero” in a bitmap for a polarization for each layer

Offline agreement: In RAN1#98, decide if the specification will restrict the UE from reporting all “zero” in the bitmap for a polarization for each layer