3GPP TSG RAN WG1 Meeting #97	R1-1907836

Reno, USA, May 13th – 17th, 2019

| Source: | Nokia, Nokia Shanghai Bell, Thales, MediaTek, Ericsson, ZTE |
| --- | --- |
| Title: | TP for Link Budget Analysis in TR 38.821 |
| Document for: | Discussion and Decision |
| Agenda Item: | 7.2.5.5 |

# 1 Introduction

A new Study Item on “Study on Solutions for NR to Support Non-Terrestrial Networks” was approved in RAN#80 meeting [1] and further updated in RAN#82 meeting [2] and RAN#83 meetings [3] with the considered scenarios of transparent GEO satellite and transparent/regenerative LEO satellite (moving beam on earth) for pedestrian UEs and on board vehicle UEs in NTN. This contribution provides TP for link budget analysis in TR 38.821.

# 2 Text Proposal for Link Budget Analysis in TR 38.821

Start of changes

Y.Z Link Budget Analysis

Y.Z.1 Link Budget Calculation

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

|  | CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB] | (Y.Z-1) |
| --- | --- | --- |

The formula for CNR calculation is

|  | CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLS [dB]-PLAD [dB]-B [dBHZ] | (Y.Z-2) |
| --- | --- | --- |

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLS is shadowing margin, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

Antenna-gain-to-noise-temperature GT can be derived by [2]

|  | GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB] | (Y.Z-3) |
| --- | --- | --- |

where GR is receive antenna gain,  Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature. Receive antenna gain GR can be obtained by

|  | GR dBi=GR,edBi+10log10NR,a-Lp[dB], &array antenna10log10η∙π2∙D[m]2λ[m]2, &parabolic antenna | (Y.Z-4) |
| --- | --- | --- |

where GR,e is receive antenna element gain, NR,a is the number of receive antenna elements, Lp is polarization loss, η is the antenna aperture efficiency (a dimensionless parameter with typical values for parabolic antennas from 0.55 to 0.70), D is the equivalent antenna diameter, and λ is the wavelength.

EIRP can be calculated by

|  | EIRP dBW=PT dBW-LC dB+GT [dBi] | (Y.Z-5) |
| --- | --- | --- |

where PT is antenna transmit power, LC is cable loss, and GT is transmit antenna gain and can be derived by

|  | GT dBi=GT,e dBi+10log10NT,a, &array antenna10log10η∙π2∙D[m]2λ[m]2, &parabolic antenna | (Y.Z-6) |
| --- | --- | --- |

where GT,e is transmit antenna element gain and NT,a is the number of transmit antenna elements.

Y.Z.2 Parameters

The parameter configuration for link budget analysis is listed in Table Y.Z.2-1.

Table Y.Z.2-1 Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| System bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Channel bandwidth | - UL in S band (handheld UE): 1 MHz - Others: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be challenge. |
| Satellite altitude | 600km, 1200km, 35786km |
| Elevation angle | 30 (LEO), 10 (GEO) |
| Atmospheric loss | Equation (6.6-8) in TR 38.811 |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| Average CIR within a satellite beam | Based on system-level calibration results |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | - Ka band: VSAT, (M, N, P) = (4, 8, 2) - S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Terminal RF parameters | Table X.3 |
| Satellite RF parameters | Set-1 in Table X.1 and Set-2 in Table X.2 |
| Polarization loss | - 3dB per linear polarization receive antenna - 0dB for circular polarization antenna receiver |
| Outcome | CNIR |

Y.Z.3 Link Budget Results

End of changes