# Summary of Offline Discussion for M-TRP based MIMO Enhancement

- M-DCI based NCJT

Possible Agreement

- For separate ACK/NACK feedback for PDSCHs received from different TRPs, the UE should be able to generate separate ACK/NACK codebooks identified by an index, if the index is configured and applied across all CCs

  - FFS: for the index per TRP basis, e.g. a higher layer signalling index, PRI in L1, CORESET group ID, slot or subslot index in L1

- Support joint HARQ-Ack feedback for PDSCHs received from different TRPs where multiple DCIs are used

- When the PUCCH resources are on the different slots, which are indicated by PDSCH-to-HARQ_feedback timing indicator fields of multiple DCIs for different TRPs, both type-1 HARQ-ACK codebook and type-2 HARQ-ACK codebook are supported.

  - FFS, additional specification impact from Rel-15

  - Note that it can include other M-DCI NCJT NW implementation cases in Rel-16

# [Offline Proposal]

- The index to be used to generate separated ACK/NACK codebook is a higher layer signalling index per CORESET

  - Index can take one of the two values. One index corresponds to one TRP.

  - Note that the index may not be configured for scenarios if there is no ambiguity of codebook generation at the UE, e.g. slot based PUCCH resource allocation per TRP

    - This does not preclude configuring the index for other purposes

  - Further clarify details on how to generate separated ACK/NACK codebook by email discussion including how to use such an index

- Further clarify details on how to generate joint ACK/NACK codebook by email discussion including how to use such an index

- S-DCI based NCJT

# [Offline Proposal]: Support following principles for DMRS port indication design for NCJT transmission based on single-PDCCH multi-TRP, at least for single front-load symbol and eMBB

- Antenna port field size is the same as Rel-15, at least for DCI format 1-1

- At least support following layer combinations from two TRPs indicated by antenna port field:

  - 1+1, 1+2, 2+1, 2+2 for single CW and SU, at least for DCI format 1-1

  - To be evaluated to determine whether introducing following design principles for DMRS entries in RAN1 98:

    - 1+3 and/or 3+1

    - MU cases, i.e. between NCJT UE+NCJT UE and NCJT UE+S-TRP UE

    - [Two CWs for the case of total layers of NCJT reception more than 4]

- M-TRP based URLLC

# [Offline Proposal 9]:

Downselect from following two options in RAN 97:

Option 1:

- Support both 2a and 2b

- Both scheme 2a and 2b are UE capabilities.

- For scheme 2b,

  - additional UE capability to inform the gNB whether the UE can support CW soft combing

  - Support up to two layer transmission and one CB per CW

Option 2:

- Both 2a and 2b are not supported in Rel-16

# [Offline Proposal 10]:

- For single-DCI based M-TRP URLLC schemes 3 & 4, support following design with respect to

  - The maximal number of transmission layers per transmission occasion, down-select one from the following options:

    - Option 1: up to single layer transmission

    - Option 2: up to two layers transmission

  - PDSCH repetition indication mechanism:

    - Number of repetitions, down-select one from following options:

      - Option 1: Dynamic indication

      - Option 2: High-layer configured as Rel-15

    - Resource allocation in time domain:

      - FFS for further details

      - FFS: whether a minimal gap between PDSCH mini-slot/slot groups is needed

    - Resource allocation at frequency domain:

      - Same frequency domain resource allocation across repetitions as Rel-15

    - For the number of TCI states across PDSCH repetitions, down-select one from following options:

      - Option 1: up to 2

        - Option 1-1: One TCI codepoint can indicate up to 2 TCI states as already agreed in Rel-16 for eMBB

        - Option 1-2: Joint field to indicate TCI states and RV sequences. Dedicated TCI or RV field is not needed.

          - For example, TCI states and RV sequences are jointly preconfigured and the combination of TCI states/RV sequences is jointly indicated in DCI.

      - Option 2: up to 4

        - Option 2-1: One TCI codepoint can indicate up to 4 TCI states

        - Option 2-2: Joint field to indicate TCI states and RV sequences. Dedicated TCI or RV field is not needed.

          - For example, TCI states and RV sequences are jointly preconfigured and the combination of TCI states/RV sequences is jointly indicated in DCI.

    - RV sequences for PDSCH repetitions

      - Option 1: support Rel-15 RV sequences at least

        - FFS whether RV sequences {0, 0, 0, 0} and {0, 3, 0, 3} are needed in Rel-16

      - Option 2: RV sequences are preconfigured by higher layer without restriction of specific orders in spec.

    - FFS how to map indicated TCI states and RV sequences to transmission  occasions

    - LDPC base graph and TBS shall be same across repetition.