3GPP TSG RAN WG1 Meeting #97             					  	R1-1907914

Reno, NV, USA, May 13th – 17th, 2019

Agenda item:		7.2.13.2

Source:				Nokia

Title:					Draft 38.300 CR on cross-carrier scheduling with different numerologies

Document for:		Discussion

# Introduction

This document proposes a draft stage 2 CR to TS38.300 on cross-carrier scheduling with different numerologies.

# Annex – Draft CR to 38.300 on Cross Carrier Scheduling with different numerologies

## 10.X	Cross Carrier Scheduling

Cross-carrier scheduling with the Carrier Indicator Field (CIF) allows the PDCCH of a serving cell to schedule resources on another serving cell but with the following restrictions:

-    Cross-carrier scheduling does not apply to PCell i.e. PCell is always scheduled via its PDCCH;

-    When an SCell is configured with a PDCCH, that cell’s PDSCH and PUSCH are always scheduled by the PDCCH on this SCell;

-    When an SCell is not configured with a PDCCH, that SCell’s PDSCH and PUSCH are always scheduled by a PDCCH on another serving cell.

When the scheduling PDCCH is using a different numerology than the scheduled PDSCH,

- 	The minimum allowed PDCCH-to-PDSCH timing is determined differently than if the numerologies are the same, ensuring that there is always time gap between the end of the PDCCH and start of the PDSCH.