3GPP TSG-RAN1 Meeting #97	R1-1907928

# Reno, USA, May 13- May 17, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | <CR#> | rev | <Rev#> | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on PUSCH scheduled by RAR UL grant and Msg3 PUSCH retransmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp, Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2019-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Mapping from virtual to physical resource blocks for CFRA PUSCH Subclause 6.3.1.7 of TS38.211 describes the mapping from virtual to physical resource blocks for msg3 corresponding to the msg3 initial transmission and retransmission in CBRA. Except for msg3, virtual resource block n is mapped to physical resource block n. According to the TS38.211, it seems that, for PUSCH scheduled by RAR UL grant in CFRA, virtual resource block n is also mapped to physical resoruce block n. However, TS38.213 specifies that the frequency allocation determination for PUSCH scheduled by RAR UL grant in both CBRA and CFRA are same. Therefore, current TS38.211 does not capture the correct mapping from virtual to physical resoruce blocks for PUSCH scheduled by RAR UL grant in CFRA. - Sequence generation of DMRS for CFRA PUSCH Subclause 6.4.1.1.2 of TS38.211 describes different sequence generation of DMRS for msg3 and CFRA PUSCH when transform precoding is enabled. Different from the common understanding that same UE behavior is applied for msg3 and CFRA PUSCH, the current TS38.211 has different DMRS sequence generation for PUSCH scheduled by RAR UL grant in CFRA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the same mapping from virtual to physical resource blocks is applied to the PUSCH transmission scheduled by RAR UL grant and msg3 retransmission. - Clarify that the same sequence generation of DMRS is applied to PUSCH scheduled by a RAR UL grant and Msg3 PUSCH retransmission when transform precoding is enabled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistency between TS38.211 and TS 38.213 regarding the mapping from virtual to physical resource blocks for PUSCH scheduled by RAR UL grant in CFRA. - Different sequence generation of DMRS for msg3 and CFRA PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7, 6.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis - Mapping from virtual to physical resource blocks for CFRA PUSCH - If UE implements the change and gNB does not, or gNB implements the change but UE does not, there will be a different understanding between UE and gNB on the mapping from virtual to physical resource blocks for PUSCH scheduled by RAR UL grant in CFRA. - Sequence generation of DMRS for CFRA PUSCH - If UE implements the change and gNB does not, or gNB implements the change but UE does not, there will be a different understanding between UE and gNB on sequence generation of DMRS for PUSCH scheduled by RAR UL grant in CFRA when transform precoding is enabled. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3.1.7	Mapping from virtual to physical resource blocks

Virtual resource blocks shall be mapped to physical resource blocks according to non-interleaved mapping.

For non-interleaved VRB-to-PRB mapping, virtual resource block n is mapped to physical resource block n except for PUSCH scheduled by RAR UL grant or Msg3 PUSCH retransmission in active uplink bandwidth part i starting at NBWP,istart, including all resource blocks of the initial uplink bandwidth part starting at NBWP,0start, and having the same subcarrier spacing and cyclic prefix as the initial uplink bandwidth part, in which case virtual resource block n is mapped to physical resource block n+NBWP,0start-NBWP,istart.

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where ru,vα,δ(m) is given by clause 5.2.2 with  and  for a PUSCH transmission.

The sequence group , where  is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and the PUSCH is neither a PUSCH scheduled by RAR UL grant nor a Msg3 PUSCH retransmission according to clause 8.3 in [5, TS 38.213].

-	nIDRS=NIDcell otherwise

where  and the sequence number v are given by:

-	if neither group, nor sequence hopping is enabled

-	if group hopping is enabled and sequence hopping is disabled

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame

-	if sequence hopping is enabled and group hopping is disabled

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame.

The hopping mode is controlled by higher-layer parameters:

-	forPUSCH transmission scheduled by RAR UL grant or Msg3 PUSCH retransmission, sequence hopping is disabled and group hopping is enabled or disabled by the higher-layer parameter groupHoppingEnabledTransformPrecoding;

-	for all other transmissions, sequence hopping and group hopping are enabled or disabled by the respective higher-layer parameters sequenceHopping and sequenceGroupHopping if these parameters are provided, otherwise, the same hopping mode as for Msg3 shall be used.

The UE is not expected to handle the case of combined sequence hopping and group hopping.

The quantity l above is the OFDM symbol number except for the case of double-symbol DMRS in which case l is the OFDM symbol number of the first symbol of the double-symbol DMRS.