3GPP TSG-RAN WG1 Meeting #97	R1- 1907944

# May 13th  – 17th,  2019

Reno, USA

Agenda item:	7.1.3

Source: 	Qualcomm Incorporated

Title: 	Summary#5 of CRs on UCI multiplexing on PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, a summary of CRs on UCI multiplexing on PUSCH is provided in Section 2.

# Summary of CRs

There are in total 6 CRs submitted to 7.1.3 which related to UCI multiplexing on PUSCH. 5 CRs are addressing scenarios with different numerologies between different UL channels. One CR proposes to fix a typo in 38.213. The 6 CRs are categorized in below sections.

## Timeline check for multiplexing PUCCH on PUSCH with different numerologies

Two CRs both from CATT are submitted regarding the timeline check for multiplexing UCI on PUSCH with different numerologies used for PUCCH/PUSCHs. These two CRs are related but still addressing different aspects on timeline check.

R1-1906297	Correction on timeline check for uplink physical channel overlapping	CATT

CATT updated the CR in R1-1907807.

R1-1906298	Correction on timeline definition for uplink physical channel overlapping	CATT

Proposal 1: down select to one option from the following options to clarify the calculation of  Tproc,1mux, Tproc,2mux, Tproc, releasemux for checking timeline conditions for UCI multiplexing on PUSCH.

Note: other options are not precluded.

Note: for  Tproc, CSImux and Z'proc,CSImux, no spec change is needed.

Option 1: The procedures to calculate Tproc,1mux, Tproc,2mux, Tproc, releasemux are captured as following.

Procedure to determine Tproc,1mux

- Set Counter = 1

- Sweep over the PDSCHs with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs/PUSCHs

  - Determine the UE PDSCH processing capability for that PDSCH

  - Select following 38.214 for that PDSCH

  - Sweep over the numerologies used for the PDCCH scheduling the PDSCH, the PDSCH, the corresponding HARQ-ACK transmission, and all PUSCHs in the group of overlapping PUCCHs/PUSCHs, for each numerology,

      - Select N1 based on the determined numerology and UE processing capability

      - Calculate Tproc,1mux,counter=N1+d1,1+1⋅2048+144⋅κ⋅2-μ⋅TC

      - counter = counter +1

- Set Tproc,1mux= max⁡{Tproc,1mux,1,…,Tproc,1mux,counter-1}

Procedure to determine Tproc,2mux

- Set Counter = 1

- Sweep over the PDSCHs with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs/PUSCHs,

  - Determine the UE PDSCH processing capability for that PDSCH

  - Select and to zeros

  - Sweep over the numerologies used for the PDCCH scheduling the PDSCH, and all the channels in the group of overlapping PUCCHs/PUSCHs, for each numerology,

      - Select N2 based on the determined numerology and UE processing capability

      - Calculate Tproc,2mux,counter=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d2,2

      - counter = counter +1

Note: the above sweeping is only for the case where there is no PUSCH in the overlapping group of channels.

- Sweep over each of the PUSCHs which is in the group of overlapping PUCCHs/PUSCHs

  - Determine the UE PUSCH processing capability for that PUSCH

  - Select and  following 38.214 for that PUSCH

  - Sweep over the numerologies used for the PDCCH scheduling the PUSCH, and all the PUSCHs in the group of overlapping PUCCHs/PUSCHs, for each numerology,

      - Select N2 based on the determined numerology and UE processing capability

      - Calculate Tproc,2mux,counter=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d2,2

      - counter = counter +1

- Set Tproc,2mux= max⁡{Tproc,2mux,1,…,Tproc,2mux,counter-1}

Procedure to determine Tproc, releasemux

- Set Counter = 1

- Sweep over the PDCCH providing the SPS PDSCH release with corresponding HARQ-ACK transmission on a PUCCH which is in the group of overlapping PUCCHs/PUSCHs

  - Determine the UE processing capability for that SPS PDSCH release

  - Sweep over the numerologies used for the PDCCH providing the SPS PDSCH release, the corresponding HARQ-ACK transmission, and all PUSCHs in the group of overlapping PUCCHs/PUSCHs, for each numerology,

      - Select N1 based on the determined numerology and UE processing capability

      - Calculate Tproc,releasemux, counter=N+1⋅2048+144⋅κ⋅2-μ⋅TC

      - counter = counter +1

- Set Tproc,releasemux= max⁡{Tproc,releasemux,1,…,Tproc,releasemux,counter-1}

Option 2: The procedures to calculate Tproc,1mux, Tproc,2mux, Tproc, releasemux are captured as following.

Procedure to determine Tproc,1mux

- Determines u which is corresponding to the smallest SCS among all PDCCHs/PDSCHs/PUCCHs/PUSCHs involved in UCI multiplexing (this step is already captured in current spec)

- If all PDSCHs/PUSCHs involved in UCI multiplexing are associated with UE processing capability 2.

  - Select N1 based on the determined u and UE processing capability 2

- Otherwise

  - Select N1 based on the determined u and UE processing capability 1

- is selected based on max {…} among all PDSCHs whose corresponding HARQ-ACK transmissions are involved in UCI multiplexing.

- Calculate Tproc,1mux=N1+d1,1+1⋅2048+144⋅κ⋅2-μ⋅TC

Procedure to determine Tproc,2mux

- Determines u which is corresponding to the smallest SCS among all PDCCH/PUCCH/PUSCH involved in UCI multiplexing (this step is already captured in current spec)

- If all PDSCHs/PUSCHs involved in UCI multiplexing are associated with UE processing capability 2.

  - Select N2 based on the determined u and UE processing capability 2

- Otherwise

  - Select N2 based on the determined u and UE processing capability 1

- is selected based on max {…} among all PUSCHs involved in UCI multiplexing.

- is selected based on max {} among all PUSCHs involved in UCI multiplexing.

- Calculate Tproc,2mux=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d2,2

Procedure to determine Tproc,releasemux

- Determines u which is corresponding to the smallest SCS among all PDCCH/ PUCCH/PUSCH involved in UCI multiplexing (this step is already captured in current spec)

- If all SPS releases/PUSCHs involved in UCI multiplexing are associated with UE processing capability 2.

  - Select N based on the determined u and UE processing capability 2

- Otherwise

  - Select N based on the determined u and UE processing capability 1

- Calculate Tproc,releasemux=N+1⋅2048+144⋅κ⋅2-μ⋅TC

Option 2a: The procedures to calculate Tproc,1mux, Tproc,2mux, Tproc, releasemux are captured as following.

Procedure to determine Tproc,1mux

- Determines u which is corresponding to the smallest SCS among all PDCCHs/PDSCHs/PUCCHs/PUSCHs involved in UCI multiplexing (this step is already captured in current spec)

- If all PDSCHs involved in UCI multiplexing are associated with UE processing capability 2.

  - Select N1 based on the determined u and UE processing capability 2

- Otherwise

  - Select N1 based on the determined u and UE processing capability 1

- is selected based on max {…} among all PDSCHs whose corresponding HARQ-ACK transmissions are involved in UCI multiplexing.

- Calculate Tproc,1mux=N1+d1,1+1⋅2048+144⋅κ⋅2-μ⋅TC

Procedure to determine Tproc,2mux

- Determines u which is corresponding to the smallest SCS among all PDCCH/PUCCH/PUSCH involved in UCI multiplexing (this step is already captured in current spec)

- If all PUSCHs involved in UCI multiplexing are associated with UE processing capability 2.

  - Select N2 based on the determined u and UE processing capability 2

- Otherwise

  - Select N2 based on the determined u and UE processing capability 1

- is selected based on max {…} among all PUSCHs involved in UCI multiplexing.

- is selected based on max {} among all PUSCHs involved in UCI multiplexing.

- Calculate Tproc,2mux=maxN2+d2,1+1⋅2048+144⋅κ⋅2-μ⋅TC, d2,2

Procedure to determine Tproc,releasemux

- Determines u which is corresponding to the smallest SCS among all PDCCH/ PUCCH/PUSCH involved in UCI multiplexing (this step is already captured in current spec)

- If all SPS releases whose corresponding HARQ-ACKs involved in UCI multiplexing are associated with UE processing capability 2.

  - Select N based on the determined u and UE processing capability 2

- Otherwise

  - Select N based on the determined u and UE processing capability 1

- Calculate Tproc,releasemux=N+1⋅2048+144⋅κ⋅2-μ⋅TC

## PUSCH selection for multiplexing PUCCH on PUSCH with different numerologies

There was a post RAN1 96bis email discussion on the topic of which PUSCH to be selected to multiplex UCI in case PUCCH overlap with multiple PUSCHs where the numerologies are different among PUCCH and PUSCHs.

Email discussion to decide potential solution to the issues identified in CR R1-1904538 – until 04/19, Yi (Qualcomm)

Unfortunately, during this email discussion no consensus was reached, and it was concluded that this issue will be reconsidered in RAN1#97.

Two CRs/contributions are submitted to this meeting to discuss this issue, as listed below. As these two CR/contribution are address the same issue, they should be discussed and treat together.

R1-1906302	Correction on PUSCH selection for overlapping of PUCCH and PUSCH	CATT

R1-1907441	Multiplexing of overlapping PUCCH and PUSCH with different numerologies		Nokia

Recommendation: The CR is not needed. No Spec update is needed.

Reason: Current spec already defined UE behavior in this scenario. If some companies think spec is not clear enough, a clarification can be captured in chairman’s notes.

Conclusion:

For the issue raised in the draft CR R1-1906302, the intended UE behavior per specification is commonly understood as follows:

- For UCI multiplexing, within a PUCCH group, on PUSCH, the following two steps are performed with step 1 first, then followed by step 2:

  - Step 1: UCI in overlapped PUCCH transmissions is multiplexed into one PUCCH resource (resource Z) on PCC. This step is done per PUCCH slot.

  - Step 2: UCI, that doesn’t include SR, in Z is multiplexed into one PUSCH, if Z overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below.

    - First priority: PUSCH with A-CSI as long as it overlaps with Z

    - Second priority: earliest PUSCH slot(s) based on the start of the slot(s)

    - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low)

      - Third priority: Dynamic grant PUSCHs > configured grant PUSCHs PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH

      - Fourth priority: PUSCHs on CC serving cell with smaller CC serving cell index > PUSCHs on CC serving cell with larger CC serving cell index

      - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission

Note: The clarification applies to both cases with the same (except the second priority part) and different numerologies among PUCCH and PUSCHs.

Proposals:

- The following is to be included in the 38.213 alignment CR (Section 9)

  - A UE does not expect that a PUCCH resource, which results from multiplexing of overlapped PUCCH transmissions if applicable, overlaps with more than one PUSCHs each with aperiodic CSI reports.

## PRACH overlap with PUCCH/PUSCH/SRS

One CR is submitted to address the ambiguity of slot definition in the scenario PRACH and PUCCH/PUSCH/SRS overlapping or back-to-back transition with different numerologies used between PRACH and PUCCH/PUSCH/SRS.

R1-1907502	Correction on simultaneous PRACH and other UL transmissions		Huawei, HiSilicon

Recommendation: Further discuss and conclude in RAN1 97 (this meeting).

Reason: The spirit of the CR seems reasonable. But how to deal with different numerologies among PUCCH/PUSCH/SRS needs further discussion.

Offline proposal: for PRACH overlapping with PUCCH/PUSCH/SRS in the same slot or the definition of gap of N symbols between PRACH and PUCCH/PUSCH/SRS, down selection from the following two options to resolve the issue with different numerologies among PUCCH/PUSCH/SRS,

Option 1: u is referenced to a slot with smallest numerology among PUSCH/PUCCH/SRS, with the following TP adopted for Subclause 8.1 in 38.213

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than  symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where for  or ,  for  or , and  is the smallest SCS of the PUSCH/PUCCH/SRS transmissions .

Supporting companies: QC, HW (first preference)

Option 2: u is referenced to a slot with numerology of PUSCH/PUCCH/SRS respectively, with the following TP adopted for Subclause 8.1 in 38.213

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than  symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where for  or ,  for  or , and  is the SCS configuration of the respective PUSCH/PUCCH/SRS transmission .

Supporting companies: ZTE, Nokia, FUJISU, Samsung, HW (second preference)

## CR to fix a typo

There is a CR submitted under 7.1.3 to fix a typo in Section 9.3 in 38.213.

R1-1906123	Draft TS 38.213 CR on UCI reporting in PUSCH		VIVO

Offline Proposal: Reject CR R1-1906123 and adopt CR1 included in feature lead summary R1-1907753.